IEEE P802.22
Wireless RANs

| Sensing and Geolocation Definition |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2013-02-04 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Ranga Reddy | Self | 2501 Farmcrest Drive, Apt 1133, Herndon, VA 20171 | 732-693-5812 | ranga.reddy@me.com |

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).   The material in this document is subject to change in form and content after further study.  The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair Apurva Mody <apurva.mody@ieee.org> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group.  If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Abstract

This contribution includes that text that was developed to address CID 8 in the LB2 database (DCN: 22-12/85r1). The following text defines SM-SSF-SAP (Sensing) and SM-GL-SAP (Geolocation) primtives.

Introduction

In during LB2 balloting it was determined that Cognitive Radio-related primitives would be consolidated into Clause 14. Comment 8 in the comment database (DCN: 22-12/85r1: https://mentor.ieee.org/802.22/dcn/12/22-12-0085-01-0mib-tga-lb2-comment-database.xls), requests that the cognitive radio related primitives in 10.7 be relocated to the appropriate sections within Clause 14. The primitives within section 10.7.1-10.7.3 and 10.7.6 have been relocated to the M-SAP (see DCN 22-13/16r0: https://mentor.ieee.org/802.22/dcn/13/22-13-0016-00-0mib-m-sap-definition.doc). The primitives in 10.7.4 and 10.7.5, relating to sensing and geolocation, will be re-located in sections w/in 14.2 that correspond to the SM-SSF-SAP and SM-GL-SAP. This contribution includes text to be included in the draft amendment that satisfies this comment.

Editorial Instruction

- Add a section 14.2.2, titled “Spectrum Manager-Spectrum Sensing Function SAP (SM-SSF-SAP)” to Seciton 14.2

- Add the the text below to section 14.2.2

[============================================Start of new text for 14.2.2========================================]

      - Spectrum Sensing Function Primitives

The IEEE 802.22 PHY layer shall provide local spectrum sensing services through its SSF accessed through the SM-SSF-SAP. Table 283 summarizes the spectrum sensing primitives supported through the SM-SSF-SAP interface. The primitives are discussed in the sub-clauses referenced in the table.

- — Spectrum Sensing Primitives supported by the SM-SSF-SAP

| Name | Request | Indication | Confirm |
| --- | --- | --- | --- |
| SM-SSF-SAP-CHANNEL-SENSING | 14.2.2.1.1 | 14.2.2.1.2 |  |
| SM-SSF-SAP-SENSING-RESULTS |  |  | 14.2.2.1.3 |

        - SM-SSF-SAP-CHANNEL-SENSING-REQUEST

          - Purpose

The SM-SSF-SAP-CHANNEL-SENSING-REQUEST primitive allows the SM to request the PHY SSF unit (in either the local SSA or remote SSA at a CPE) to perform spectrum sensing.  Table 284 specifies the parameters for the SM-SSF-SAP-CHANNEL-SENSING-REQUEST primitive.

          - SAP Type

SM-SSF-SAP

          - Operation Type

Information Request

          - Destination

SSA SSF

          - Data

- SM-SSF-SAP-CHANNEL-SENSING-REQUEST Parameters

| Name | Type | Length | Description |
| --- | --- | --- | --- |
| IETF Country Code | Character String | 16 bits | See Annex A |
| Channel Number | Integer | 8 bits | Then channel number which is to be sensed by the SSF. Values as Specified in Table 236. |
| Channel Bandwith | Integer | 4 bits | The bandwidth of the channel to be sensed by the SSF. Values as specified in Table 236. |
| Sensing Mode | Integer | 2 bits | The sensing mode specifies which SSF outputs are valid as specified in Table 239. |
| Signal Type Array | Array | 32 bits | An array indicating which signal types the SSF is to sense for as specified in Table 239. |
| Sensing Window Specification Array | Array | N X 32 bits | N is the number of signal types enumerated (that are equal to ‘1’) in the Signal Type Array. Sensing window specifications as given in Table 245 and Table 246. Each element in the Sensing Window Specification consists of: NumSensingPeriods, SensingPeriodDuration, SensingPeriodInterval |
| Maximum Probability of False Alarm Array | Array | N X 8 bits | N is the number of signal types enumerated (that are equal to ‘1’) in the Signal Type Array. This value is valid only for sensing modes 0 and 1. Each element specifies the maximum probability of false alarm for the corresponding signal type decision in the sensing present Array. Maximum Probability of False Alarm – 0x00 indicates ‘0’ and 0x01 indicates ‘0.001, and 0xFF = 0.255 (see Table 236) |
| Timestamp Length | Integer | 2 bytes | Length of Timestamp field (# of characters) |
| Timestamp | Character String | NMEA 0183 $ZDA string | Timestamp of the present request at time of transmission and as encoded in the $ZDA substring of the NMEA 0183 string |
| Padding |  | 2 bits | Set to ‘00’ |

          - When Generated

The SM-SSF-SAP-CHANNEL-SENSING-request primitive is generated by the SM and issued to the SSF to request the SSF to perform spectrum sensing.

          - Effect of Receipt

When the SSA receives the SM-SSF-SAP-CHANNEL-SENSING-REQUEST primitive, it requests the SSF to perform spectrum sensing. If the sensing request can be serviced the SSA replies by sending the SM-SSF-SAP-CHANNEL-SENSING-INDICATION to indicate to SM it can perform sensing, executes sensing, then sends sensing results back to SM in SM-SSF-SAP-SENSING-RESULTS-CONFIRMATION. If the sensing request cannot be serviced, the SSA will send back a failure code in the SM-SSF-SAP-CHANNEL-SENSING-INDICATION.

        - SM-SSF-SAP-CHANNEL-SENSING-INDICATION

          - Purpose

The SM-SSF-SAP-CHANNEL-SENSING-INDICATION primitive is used to inform the SM whether its request to the local PHY SSF or remote SSF at a CPE was successfully generated by the SM.  Table 285 specifies the parameters for the SM-SSF-SAP-CHANNEL-SENSING.confirm primitive.

          - SAP Type

SM-SSF-SAP

          - Operation Type

Event Indication

          - Destination

SM

          - Data

- SM-SSF-SAP-CHANNEL-SENSING-INDICATION Parameters

| Name | Type | Length | Description |
| --- | --- | --- | --- |
| IETF Country Code | ASCII | 16 bits | See Annex A. |
| Channel Number | Integer | 8 bits | The channel number which is to be sensed by the SSF. Range is specified in Table 236. |
| Sensing Mode | Integer | 2 bits | The sensing mode specifies which SSF outputs are valid as specified in Table 239. |
| Status | Enumeration | 2 bits | 00: INVALID_REQUEST 01: INVALID_SIGNAL_TYPES 10: Reserved 11: SUCCESS The value indicates whether the sensing request was successfully generated. |
| Invalid Signal Type Array | Array | 32 bits | An array indicating which signal types the SSF will not be able to sense as specified in Table 237. This attribute is valid only if the Status = INVALID_SIGNAL_TYPEs |
| Timestamp Length | Integer | 2 bytes | Length of Timestamp field (# of characters) |
| Timestamp | Character String | NMEA 0183 $ZDA string | Copied from the timestamp in the SM-SSF-SAP-CHANNEL-SENSING-REQUEST. |
| Padding |  | 4 bits | Set to ‘0000’ |

          - When Generated

The SM-SSF-SAP-CHANNEL-SENSING-INDICATION primitive is generated by the SSF and issued to its SM to indicate whether the received SM-SSF-SAP-CHANNEL-SENSING-REQUEST was valid and whether the SSF is able to perform sensing for the signal types as requested. If the SSF is able to perform the sensing in the requested sensing mode and with the requested probability of false alarm for all types of signals requested, the Status code shall be set to SUCCESS. If the SSF does not support the requested sensing mode, the status value should be INVALID_REQUEST. If one or more of the signal types in the request is not valid or the SSF does not have the capability to sensing a requested signal type, the status code should be set to INVALID_SIGNAL_TYPE and the corresponding invalid signal types shall be indicated in the Invalid Signal Type Array.

          - Effect of Receipt

When the SM receives the SM-SSF-SAP-CHANNEL-SENSING-INDICATION primitive, it will identify whether its request to the SSF was successfully received by the SSF.  The status parameter indicates the appropriate error code from 7.7.24 in case the request is invalid.

        - SM-SSF-SAP-SENSING-RESULTS-CONFIRMATION

          - Purpose

The SM-SSF-SAP-SENSING-RESULTS-CONFIRMATION primitive is used to inform the SM when the results of a previously issued request to the SSF were successfully generated by the SSF.  Table 286 specifies the parameters for the SM-SSF-SAP-SENSING-RESULTS-CONFIRMATION primitive.

          - SAP Type

SM-SSF-SAP

          - Operation Type

Information Confirmation

          - Destination

SM

          - Data

- SM—SSF-SAP-SENSING-RESULTS-CONFIRMATION Parameters

| Name | Type | Length | Description |
| --- | --- | --- | --- |
| IETF Country Code | ASCII | 16 bits | See Annccex A. |
| Channel Number | Integer | 8 bits | The channel number that is to be sensed by the SSF. Range as specified in Table 236. |
| Sensing Mode | Integer | 2 bits | The sensing mode specifies which SSF outputs are valid as specified in Table 237. |
| Signal Type Array | Array | 32 bits | An Array indicating which signal types the SSF is to sense as specified in Table 238. |
| Signal Present Array | Array | N X 2 | N is the number of signal types enumerated (that are equal to ‘1’) in the Signal Type Array. Each element in the Array is a signal present decision. Each decision can take on three possible values, as given in Table 240. |
| Confidence Array | Array | N X 8 | Confidence array is only valid for Sensing Mode 2. Each element in the confidence Array is a confidence metric for the sensing result for the corresponding signal type as defined in Table 240 0x00: No confidence 0x01to 0xFE: Reserved 0xFF: Full confidence |
| If(Sensing Mode == 3) { |  |  |  |
| RSSI Measurements | Integer | N X 8 bits | N is the number of signal types enumerated (that are equal to ‘1’) in the Signal Type Array. RSSI Measurement is only valid for Sensing Mode 3. Each RSSI measurement result is a signed integer number encoded with 8 bits (see Table 240). BS can ask the CPE for up to 255 measurements of the RSSI. In such case, each RSSI measurement will represent the mean of the multiple measurement results, ranging -104 dBm to +23.5 dBm in 0.5 dB steps |
| RSSI Standard Deviation | Integer | N X 8 bits | N is the number of signal types enumerated (that are equal to ‘1’) in the Signal Type Array. RSSI Measurement is only valid for Sensing Mode 3. BS can ask the CPE for up to 255 measurements of the RSSI. In such case, this parameter represents the result of the standard deviation calculation done on these multiple RSSI measurements results, ranging -104 dBm to +23.5 dBm in 0.5 dB steps |
| } |  |  |  |
| Timestamp Length | Integer | 2 bytes | Length of Timestamp field (# of characters) |
| Timestamp | Character String | NMEA 0183 $ZDA string | Copied from the timestamp in the SM-SSF-SAP-CHANNEL-SENSING-REQUEST. |
| Padding |  | 6 bit | Set to ‘000000’ |

          - When Generated

The SM-SSF-SAP-SENSING-RESULTS-CONFIRMATION primitive is generated by the SSF and issued to the SM to indicate the results of a previously issued request to the SSF have been generated.

          - Effect of Receipt

When the SM receives the SM-SSF-SAP-SENSING-RESULTS-CONFIRMATON it will obtain the sensing results to its request to the SSF.

[============================================End of new text for 14.2.2=========================================]

- Add a section 14.2.3, titled “Spectrum Manager-Geolocation SAP (SM-GL-SAP)” to Seciton 14.2

- Add the the text below to section 14.2.3

[============================================Start of new text for 14.2.3========================================]

      - Geolocation Primitives

The PHY layer provides local geolocation services through its satellite-based location acquisition unit to the SM/SSA through the SM-GL-SAP. Table 287 summarizes the geolocation primitives supported through the SM-GL-SAP interface. The primitives are discussed in the subclauses referenced in the table.

- — Geolocation Primitives supported by the SM-GL-SAP

| Name | Request | Indication | Confirm |
| --- | --- | --- | --- |
| SM-GL-SAP-GEOLOCATION | 14.2.3.1.1 | 14.2.3.1.2 |  |
| SM-GL-SAP-GEOLOCATION-RESULTS |  |  | 14.2.3.1.3 |

        - SM-GL-SAP-GEOLOCATION-REQUEST

          - Purpose

The SM-GL-SAP-GEOLOCATION-REQUEST primitive allows the SM to request the local PHY geolocation unit or the one at a CPE to perform geolocation.  Table 288 specifies the parameters for the SM-GL-SAP-GEOLOCATION-REQUEST primitive.

          - SAP Type

SM-GL-SAP

          - Operation Type

Information Request

          - Destination

SSA GL

          - Data

- SM-GL-SAP-GEOLOCATION-REQUEST Parameters

| Name | Type | Length | Description |
| --- | --- | --- | --- |
| NMEA Sentence Request | Character String | 6 octets | NMEA 0183, in ASCII, string type (e.g. GPGGA) |
| Timestamp Length | Integer | 2 bytes | Length of Timestamp field (# of characters) |
| Timestamp | Character String | NMEA 0183 $ZDA string | Timestamp of the present request at time of transmission and as encoded in the $ZDA substring of the NMEA 0183 string |

          - When Generated

The SM-GL-SAP-GEOLOCATION-REQUEST primitive is generated by the SM and issued to an SSA (either it’s own or remote at a CPE) GL to request the geolocation service to perform geolocation.

          - Effect of Receipt

When the Geolocation receives the SM-GL-SAP-GEOLOCATION-REQUEST primitive, it attempts execution of the geolocation service to perform geolocation. If the geolocation request can be satisified, it sends a SM-GL-SAP-GEOLOCATION-INDICATION with a Success status code, executes the geolocation and returns the result in SM-GL-SAP-GEOLOCATION-RESULTS-CONFIRMATION. If the geolocation request cannot be satisfied, it sends a SM-GL-SAP-GEOLOCATION-INDICATION with a failure status code.

        - SM-GL-SAP-GEOLOCATON-INDICATION

          - Purpose

The SM-GL-SAP-GEOLOCATION.confirm primitive is used to inform the SM whether its request to the PHY geolocation service was successfully generated by the Geolocation function of the SSA.  Table 289 specifies the parameters for the SM-GL-SAP-GEOLOCATION.confirm primitive.

          - SAP Type

SM-GL-SAP

          - Operation Type

Event Indication

          - Destination

SM

          - Data

- SM-GL-SAP-GEOLOCATION-INDICATION Parameters

| Name | Type | Length | Description |
| --- | --- | --- | --- |
| Status | Integer | 8 bits | The value indicates whether the requested query was successfully generated: 0x00 = Success 0x01 = Failure 0x02-0xFF = Reserved |
| Timestamp Length | Integer | 2 bytes | Length of Timestamp field (# of characters) |
| Timestamp | Character String | NMEA 0183 $ZDA string | Copied from the timestamp in the SM-GL-SAP-GEOLOCATION-REQUEST. |

          - When Generated

The SM-GL-SAP-GEOLOCATION-INDICATION primitive is generated by the SSA GL and issued to its SM to indicate whether the received SM-GL-SAP-GEOLOCATION-REQUEST was valid, in which case the SSA GL acquires the requested NMEA sentence from the geolocation service.

          - Effect of Receipt

When the SM receives the SM-GL-SAP-GEOLOCATION-INDICATION primitive, it will identify whether its request to the geolocation service was successfully received by the Geolocation.  The status parameter indicates the appropriate error code from 7.7.24 in case the geolocation service was not available.

        - SM-GL-SAP-RESULTS-CONFIRMATION

          - Purpose

The SM-GL-SAP-GEOLOCATION-RESULTS-CONFIRMATION primitive is used to inform the SM when a response to a previously issued request to the geolocation service was successfully received by the SSA GL.  Table 290 specifies the parameters for the SM-GL-SAP-GEOLOCATION-RESULTS-CONFIRMATION primitive.

          - SAP Type

SM-GL-SAP

          - Operation Type

Information Confirmation

          - Destination

SM

          - Data

- SM-GL-SAP-GEOLOCATION-RESULTS-CONFIRMATION Parameters

| Name | Type | Length | Description |
| --- | --- | --- | --- |
| Location Data Length | Integer | 8 bits | Length of location data string in octets (0-128 characters) |
| Location Data String | Character String | N octets | Where N is the Location Data Length in octets |
| Timestamp Length | Integer | 2 bytes | Length of Timestamp field (# of characters) |
| Timestamp | Character String | NMEA 0183 $ZDA string | Copied from the timestamp in the SM-GL-SAP-GEOLOCATION-REQUEST. |
| Timestamp Length | Integer | 2 bytes | Length of Timestamp field (# of characters) |
| Timestamp | Character String | NMEA 0183 $ZDA string | Copied from the timestamp in the SM-SSF-SAP-CHANNEL-SENSING-REQUEST. |

          - When Generated

The SM-GL-SAP-GEOLOCATION-RESULTS-CONFIRMATION primitive shall be generated by the SSA GL and issued to the SM to indicate the reception of a response to a query previously issued to the geolocation service.

          - Effect of Receipt

When the SM receives the SM-GL-SAP-GEOLOCATION-RESULTS-CONFIRMATION it shall identify whether the response to its request to the geolocation service was successfully received by the SSA GL, in which case, the SM will obtain NMEA string containing the latitude and longitude information. If the response is not successful the SM may decide to issue another request.

[============================================End of new text for 14.2.3=========================================]

References:

[1] “TGa Comment Database”, https://mentor.ieee.org/802.22/dcn/12/22-12-0085-01-0mib-tga-lb2-comment-database.xls