IEEE P802.22
Wireless RANs

| Review of the Tables 274 and 276 of the IEEE 802.22 Standard |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2013-02-17 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Gerald Chouinard | Self Employed | Ottawa, Canada | 819-684-2490 | gerald.chouinard@sympatico.ca |

Abstract

This document presents suggestions to fill the missing values in Tables 274 and 276 of Clause 12 of the IEEE 802.22 Standard.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Apurva Mody> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Review of the Tables 274 and 276 of the IEEE 802.22 Standard.

The values indicated in ‘track changes’ are suggestions to be considered for inclusion in the updated 802.22 Standard as part of the 802.22a Working Group Letter Ballot.

    - Coexistence

- — Coexistence parameters, timers, message IEs

| Entity/Scope | Name | Reference | Min value | Default value | Max value |
| --- | --- | --- | --- | --- | --- |
| BS | T32 | Wait for the Frame Contention Response message | 1 superframe | — | 32 superframes |
| BS | FCW | Frame Contention Window: number of superframes during which a BS (FC_DST) accumulates the frame contention requests before reacting to it and responding to the FC_SRC’s. | 0 superframe | — | 16 superframes |
| BS | SCWBackoff_Max | Integer denoting the maximum superframes for the SCW backoff window | — | — | 16 |
| BS | Frame_Contention_Min | Number of frames not available for contention at a BS in a coexistence situation | 0 | 2 | 8 |
| BS | FCN_range | Exponent in base 2 defining the extent of the range of the random number FCN | 4 |  | 16 |
| BS | SF_release | Number of super-frame after which the BS releases the frames wan by another BS through frame contention | — | 5 | — |
| CPE | TCBP (T34) | The minimum time between transmissions of a CBP packet carrying its MAC address for identification by nearby CPEs and BSs for coexistence purpose, as well as by spectrum monitoring systems to identify potential interference situations. Timing value may depend on the regulatory domain where the WRAN system operates (see 1747HError: Reference source not found) | 8 s | — | 15 min |
| BS | SCWBackoff_Timer (T35) | Backoff timer that controls exiting or continuation of Frame contention procedure | — | — | SCWBackoff_Max |

    - Cognitive radio capabilities (SM, SSA, incumbent protection, QP management )

- — Cognitive radio capability parameters, timers, message IEs

| Entity/Scope | Name | Reference | Min value | Default value | Max value |
| --- | --- | --- | --- | --- | --- |
| CPE | T29 | Wait for BLM-ACK timeout | 10 ms | — | 300 ms |
| BS | T31 | Wait for BLM-REP timeout | 1 MAC Frame | — | — |
| CPE | BLM-REP Retries | Number of retries allowed for sending BLM-REP | — | 3 | — |
| BS, CPE | Channel Availability Check Time | The time during which a channel SHALL be checked for the presence of licensed incumbent signals having a level above the Incumbent Detection Threshold prior to the commencement of WRAN operation in that channel, and in the case of TV, a related channel at an EIRP level that can affect the measured channel. | — | 30 s | — |
| BS, CPE | Non-Occupancy Period | The required period during which WRAN device transmissions SHALL NOT occur in a given channel because of the detected presence of an incumbent signal in that channel above the Incumbent Detection Threshold or, in the case of TV, above a given EIRP level. | 10 min | — | — |
| BS, CPE | Channel Detection Time | The maximum time taken by a WRAN device to detect a licensed incumbent signal above the Incumbent Detection Threshold within a given channel during normal WRAN operation. | — | ≤ 2 s to ≥ 90% Probability of Detection with a False Alarm rate of ≤ 10% | — |
| BS, CPE | Channel Setup Time | The window of time that may be taken by a WRAN CPE to transmit control information to a WRAN base station in order to establish operation with that base station at the prescribed power, or in the case of TV, at or below the allowable EIRP within a given channel. | — | 2 s | — |
| BS, CPE | Channel Opening Transmission Time (Aggregate transmission time) | The aggregate duration of control transmissions by WRAN devices during the Channel Setup Time, which starts at the end of the Channel Availability Check Time. | — | 100 ms | — |
| BS, CPE | Channel Move Time (In-service monitoring) | The time taken by a WRAN system to cease all interfering transmissions on the current channel upon detection of a licensed incumbent signal above the relevant Incumbent Detection Threshold or, in the case TV, to alternatively reduce its EIRP to that which is allowable within a given channel upon detection of a TV signal in the same or a related channel | — | 2 s | — |
| BS, CPE | Channel Closing Transmission Time (Aggregate transmission time) | The aggregate duration of control transmissions by the WRAN devices during the Channel Move/EIRP Reduction Time, which starts upon detection of a licensed incumbent signal above the relevant Incumbent Detection Threshold | — | 100 ms | — |
| BS, CPE | Channel Number | The channel number which is to be sensed by the SSF (Error: Reference source not found) | 0 | — | 255 |
| BS, CPE | Channel Bandwidth | The bandwidth of the channel to be sensed by the SSF (Error: Reference source not found) | — | 6/7/8 MHz (depending on regulatory domain) | — |
| BS, CPE | Maximum Probability of false alarm | In sensing modes 0 and 1 this value specifies the maximum probability of false alarm for each sensing mode decision in the signal present array | 0.0 | — | 0.255 |
| BS, CPE | NumSensingPeriods | Number of sensing periods field in a Sensing Window Specification Array entry | 0 | 1 | 127 |
| BS, CPE | SensingPeriodDuration | Duration of sensing period field (in units of OFDM symbols) in a Sensing Window Specification Array entry | 0 | 16 | 1023 |
| BS, CPE | SensingPeriodInterval | Periodicity of Sensing period field in a Sensing Window Specification Array entry | 0 | 200 | 2047 |
| BS, CPE | Sensing Window Specification Array | Array containing Sensing Window specification for each signal type enumerated in Signal Type Array | 1×sizeof(NumSensingPeriods+SensingPeriodDuration+SensingPeriodInterval) | — | 32×sizeof(NumSensingPeriods+SensingPeriodDuration+SensingPeriodInterval) |
| BS, CPE | Signal Type Array | Bitmap that indicates which signal types are to be sensed for in a given regulatory domain | — | Set on a regulatory domain-by-domain basis (see Error: Reference source not found, 1757HError: Reference source not found) | — |
| BS, CPE | Sensing Mode | The sensing mode a CPE supports. Negotiated during CPE initialization. Various modes are described in Table 238. | — | No Sensing | — |
| BS, CPE | Signal Present Decision | Indication of whether or not a signal of a specific type has been detected. All unused values are reserved | 0x00 (Absent) | 0x7F (No Decision could be made) | 0xFF (Signal Present) |
| BS, CPE | Confidence Metric | Confidence with which sensing can determine the signal type | 0x00 (No Confidence) | — | 0xFF (Full confidence) |
| BS, CPE | Mean RSSI | Mean of M RSSI measurements | –104 dBm | — | +23.5 dBm |
| BS, CPE | Standard Deviation of RSSI | Standard deviation of the M RSSI measurements | +0.0 dB | — | +25.5 dB |
| BS, CPE | RSSI_detection_threshold | Energy detection threshold indicating the presence of an incumbent, other WRAN system, or interference | –120 dBm | — | -10 dBm |
| BS, CPE | Microphone protection radius | Radius of the contour within which the WRAN system cannot operate due to potential interference with the microphone | 0.1 km | — | 100 km |
| BS, CPE | TCandidate_Channel_Refresh (T41) | Maximum time interval allowed before sensing is performed on the candidate channel to ensure that no incumbents are detected. | 1 s | 6 s | 10 s |
| BS, CPE | TBackup_Channel_Refresh (T42) | Maximum time interval allowed before sensing is performed on the backup channel to ensure that no incumbents are detected. | 1 s | 6 s | 10 s |
| BS, CPE | TCandidate_to_Backup_Transition (T43) | Minimum time duration without detection of any incumbent for a candidate channel to transition to the backup channel. | 1 s | 30 s | 100 s |
| BS, CPE | TCh_Move (T44) | Maximum time to ensure that the channel move information is successfully conveyed to all the associated CPEs and BS (self-coexistence mode). | 1 s | — | 10 s |
| BS, CPE | TNo_DB (T45) | Maximum WRAN operation time without access to the incumbent database service | 0.1 hr | — | 72 hrs |
| BS | TWait_Before_Channel_Move (T46) | Waiting time before which the BS moves to the first backup channel. This is used to make sure that all the CPEs are ready to move to the backup channel before BS switches operation to this backup channel. | 1 frame | — | 256x16 frames |
| CPE | TWait_Before_Channel_Move (T59) | Waiting time before which the CPE moves to its backup channels if it no longer hears from its BS. This is used to make sure that the CPE waits long enough after its UCS notification so that the BS has had time to move to a backup channel, if it decided to do so. | 1 frame | — | 256x16 frames |
| BS, CPE | TRefresh_Database_Info (T47) | The prescribed time by the WRAN operator to refresh the incumbent database service. | 0.1 hr | — | 72 hrs |
| BS, CPE | TClear_N (T48) | Lapser Timer keeps track of whether the Operating Channel N has been cleared using spectrum sensing | 0.1 s | — | 60 s |
| BS, CPE | TClear_N–1 (T49) | Lapser Timer keeps track of whether the Adjacent Channel N–1 has been cleared using spectrum sensing | 0.1 s | — | 60 s |
| BS, CPE | TClear_N+1 (T50) | Lapser Timer keeps track of whether the Adjacent Channel N+1 has been cleared using spectrum sensing | 0.1 s | — | 60 s |
| BS, CPE | TLoss_of_BS_Contact (T51) | Initiated when the CPE loses contact with the BS | 1 s | — | 600 s |
| BS, CPE | TRange1 (T52) | Used for terrestrial geolocation. Initiated when the downstream burst leaves the BS (i.e., at the start of the frame preamble). | 1 TU | — | 1000 TU |
| BS, CPE | TINsens (T53) | The parameter TINsens is used to verify that in-band sensing has been done within the required In-service monitoring period. The TINsens parameter is driven by the regulatory domain requirements. See 1758HError: Reference source not found. | 0.1 s | — | 60 s |
| BS, CPE | TOUTsens (T54) | The parameter TOUTsens is used to verify that out-of-band sensing has been done within the required “Acquiring a channel monitoring period” specified in 1759HError: Reference source not found (30 s in the US). This value would be used to either initialize a “lapse timer” for each channel in the backup/candidate list at each CPE so that it is compared to Tsens | 0.1 s | — | 60 s |
| BS, CPE | Tsensin (T55) | Tsensin parameter corresponds to the maximum length of time required to carry out the sensing process on an in-band channel (N, N–1, or N+1). Manufacturers need to specify the sensing time required to detect the specified signals with required accuracy (see 1761HError: Reference source not found). | 1 ms | — | 160 ms |
| CPE | Tsensout (T60) | Tsensout parameter corresponds to the maximum length of time required to carry out an out-of-band sensing process for a specified channel N (i.e., N, N–1, and N+1). Manufacturers need to specify the sensing time required to detect the specified signals out-of-band with required accuracy (see 1762HError: Reference source not found). | 20 ms |  | 160 ms |
| BS, CPE | ISO 3166 Country Code | 3 character, ASCII string denoting the regulatory domain of operation (e.g., “USA” is for United States of America) | — | 3 characters | — |
| BS, CPE | TACBP | Timing Advance of the CBP burst (see step 2 of 1763HError: Reference source not found). Note that the geolocation process will have to pre-adjust TACBP depending on the distance between the two CPE to be geolocated and the reference CPEs so that the delays measured by Vernier3 fall within the symbol cyclic prefix (e.g., 74.68 µs corresponding to 22.4 km). | –1024 TU | — | +1024 TU |