IEEE P802.22
Wireless RANs

| Management Message for Group Resource Allocation Update (GRA-UPD) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2013-07-16 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Sung-Hyun Hwang | ETRI | Korea | +82-42-860-1133 | shwang@etri.re.kr |
| Gwang-Zeen Ko | ETRI | Korea | +82-42-860-4862 | gogogo@etri.re.kr |
| Byung-Jang Jeong | ETRI | Korea | +82-42-860-6765 | bjjeong@etri.re.kr |
| Myung-Sun Song | ETRI | Korea | +82-42-860-5046 | mssong@etri.re.kr |
| Jae-Ick Choi | ETRI | Korea | +82-42-860-6160 | jichoi@etri.re.kr |

Abstract

This document proposes the management message to update the group resource allocation.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Carl R. Stevenson> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

- 1. Background

- The proposed group resource allocation (GRA) is proposed to meet two requirements in the Functional Requirements Document (22-12-0012r3). First, requirement #3 is regarding the device classes and complexity. Here, we have to support at least two classes of CPE with different complexity. The proposed group resource allocation is efficiently designed to support different classes of CPE. Second, requirement #5 is regarding the number of devices. Here, we have to support many number of devices, at least 2048 CPEs. In the network with several thounds of devices, we can significantly reduce the MAP overhead by using the proposed group resource allocation

- 2. Suggested text

Management Messages

As can be seen in 281HTable 19, the MAC defines a collection of management messages to support and implement its basic functions. All these messages are carried in the payload of a MAC PDU, and share the same message structure as depicted in 282HFigure 15. Management messages begin with a Type field that uniquely identifies the message in question, while its payload varies according to the message type. As for transmission, management messages can only be transmitted in Initial Ranging, Basic, Primary, Multicast Management, or Broadcast type of FIDs (see 283HTable 279, 284HTable 280, and 285HTable 281). No other types of FIDs shall carry management messages.

- Figure 15 — General management message structure

Each of the management messages shown in 286HTable 19 are described in the following subclauses.

- a— Management messages

| Type | Message | Description | Reference | Class of connection |
| --- | --- | --- | --- | --- |
| 0 | DCD | Downstream Channel Descriptor | 287HError: Reference source not found | Broadcast |
| 1 | DS-MAP | Downstream Access Definition | 288HError: Reference source not found | Broadcast |
| 2 | UCD | Upstream Channel Descriptor | 289HError: Reference source not found | Broadcast |
| 3 | US-MAP | Upstream Access Definition | 290HError: Reference source not found | Broadcast |
| 4 | RNG-REQ | Ranging Request | 291HError: Reference source not found | Initial Ranging or Basic |
| 5 | RNG-CMD | Ranging Command | 292HError: Reference source not found | Initial Ranging or Basic |
| 6 | REG-REQ | Registration Request | 293HError: Reference source not found | Primary Management |
| 7 | REG-RSP | Registration Response | 294HError: Reference source not found | Primary Management |
| 8 | DSA-REQ | Dynamic Service Addition Request | 295HError: Reference source not found | Primary Management |
| 9 | DSA-RSP | Dynamic Service Addition Response | 296HError: Reference source not found | Primary Management |
| 10 | DSA-ACK | Dynamic Service Addition Acknowledge | 297HError: Reference source not found | Primary Management |
| 11 | DSC-REQ | Dynamic Service Change Request | 298HError: Reference source not found | Primary Management |
| 12 | DSC-RSP | Dynamic Service Change Response | 299HError: Reference source not found | Primary Management |
| 13 | DSC-ACK | Dynamic Service Change Acknowledge | 300HError: Reference source not found | Primary Management |
| 14 | DSD-REQ | Dynamic Service Deletion Request | 301HError: Reference source not found | Primary Management |
| 15 | DSD-RSP | Dynamic Service Deletion Response | 302HError: Reference source not found | Primary Management |
| 16 | DSX-RVD | Dynamic Service Request acknowledgement before authentication | 303HError: Reference source not found | Primary Management |
| 17 | MCA-REQ | Multicast Assignment Request | 304HError: Reference source not found | Primary Management |
| 18 | MCA-RSP | Multicast Assignment Response | 305HError: Reference source not found | Primary Management |
| 19 | CBC-REQ | CPE Basic Capability Request | 306HError: Reference source not found | Basic |
| 20 | CBC-RSP | CPE Basic Capability Response | 307HError: Reference source not found | Basic |
| 21 | DREG-CMD | De/Re-register Command | 308HError: Reference source not found | Basic Primary Management |
| 22 | DREG-REQ | CPE De-registration Request | 309HError: Reference source not found | Primary Management |
| 23 | ARQ-Feedback | Standalone ARQ Feedback | 310HError: Reference source not found | Primary Management |
| 24 | ARQ-Discard | ARQ Discard | 311HError: Reference source not found | Primary Management |
| 25 | ARQ-Reset | ARQ Reset | 312HError: Reference source not found | Primary Management |
| 26 | CHS-REQ | Channel Switch Request | 313HError: Reference source not found | Primary Management or Broadcast |
| 27 | CHS-RSP | Channel Switch Response | 314HError: Reference source not found | Primary Management |
| 28 | CHQ-REQ | Channel Quiet Request | 315HError: Reference source not found | Primary Management, Multicast Management or Broadcast |
| 29 | CHQ-RSP | Channel Quiet Response | 316HError: Reference source not found | Primary Management, Multicast Management or Broadcast |
| 30 | IPC-UPD | Incumbent Prohibited Channels Update | 317HError: Reference source not found | Primary Management, Multicast Management or Broadcast |
| 31 | BLM-REQ | Bulk Measurement Request | 318HError: Reference source not found | Primary Management, Multicast Management or Broadcast |
| 32 | BLM-RSP | Bulk Measurement Response | 319HError: Reference source not found | Primary Management |
| 33 | BLM-REP | Bulk Measurement Report | 320HError: Reference source not found | Primary Management |
| 34 | BLM-ACK | Bulk Measurement Acknowledgement | 321HError: Reference source not found | Primary Management |
| 35 | TFTP-CPLT | Config File TFTP Complete | 322HError: Reference source not found | Primary Management |
| 36 | TFTP-RSP | Config File TFTP Complete Response | 323HError: Reference source not found | Primary Management |
| 37 | SCM-REQ | Security Control Management Request | 324HError: Reference source not found | Primary Management |
| 38 | SCM-RSP | Security Control Management Response | 325HError: Reference source not found | Primary Management |
| 39 | FRM_UPD | The first active frame allocation update in self-coexistence mode | 326HError: Reference source not found | Basic |
| 40 | CBP -RLY | CBP Relay | 327H7.7.1 | Primary Management, Multicast Management |
| 41 | GRA-CFG | Group Resource Allocation Configuration | 7.7.25 |  |
| 42 | GRA-UPD | Group Resource Allocation Update | 7.7.26 |  |

Subclauses 7.7.1 to 7.7.24 are same as IEEE 802.22-2011 standard.

Subclase 7.7.25 is the prposed management message for group resource allocation configuration (GRA-CFG)

Group Resource Allocation Update (GRA-UPD)

The format of Group Resource Allocation Update message is shown in Table 175. This message is used to update the group resource allocation configuration. The BS uses this message to create a new group and indicate the devices that is belonging to a group. The device can be added to or deleted from a group.

- — GRA-UPD message format

| Syntax | Size | Notes |
| --- | --- | --- |
| GRA-UPD_Message_Format() { |  |  |
| Management Message Type = 42 | 8 bits |  |
| Deletion Flag | 1 | Flag to indicate whether the device is added to or deleted from a group. 0: Added to a group 1: Deleted from a group |
| SID | 9 | Station ID that is added to or deleted from a group. |
| GID | 12 | Group ID to which the device is added to or deleted from a group. |
| If(Deletion Flag = 0) { |  |  |
| Device Bitmap Index | 7 | Indicates the new index of the device in a group’s device bitmap. |
| } |  |  |
| } |  |  |