IEEE P802.22
Wireless RANs

| Management Message for Group Backup Channel |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2013-07-16 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Sung-Hyun Hwang | ETRI | Korea | +82-42-860-1133 | shwang@etri.re.kr |
| Gwang-Zeen Ko | ETRI | Korea | +82-42-860-4862 | gogogo@etri.re.kr |
| Byung-Jang Jeong | ETRI | Korea | +82-42-860-6765 | bjjeong@etri.re.kr |
| Myung-Sun Song | ETRI | Korea | +82-42-860-5046 | mssong@etri.re.kr |
| Jae-Ick Choi | ETRI | Korea | +82-42-860-6160 | jichoi@etri.re.kr |

Abstract

This document proposes the management message for group backup channel.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Carl R. Stevenson> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

- 1. Background

- The proposed group resource allocation (GRA) is proposed to meet two requirements in the Functional Requirements Document (22-12-0012r3). First, requirement #3 is regarding the device classes and complexity. Here, we have to support at least two classes of CPE with different complexity. The proposed group resource allocation is efficiently designed to support different classes of CPE. Second, requirement #5 is regarding the number of devices. Here, we have to support many number of devices, at least 2048 CPEs. In the network with several thounds of devices, we can significantly reduce the MAP overhead by using the proposed group resource allocation

- 2. Suggested text

    - Downstream Channel Descriptor (DCD)

The format of a DCD message is shown in Table 20. This message shall be transmitted by the BS at a periodic interval (Error: Reference source not found) to define the characteristics of a downstream physical channel.

- — DCD message format

| Syntax | Size | Notes |
| --- | --- | --- |
| DCD_Message_Format() { |  |  |
| Management Message Type = 0 | 8 bits |  |
| Configuration Change Count | 8 bits | Incremented by one (modulo 256) by the BS whenever any of the values of this channel descriptor change. If the value of this count in a subsequent DCD remains the same, the CPE can quickly decide that the remaining fields have not changed and may be able to disregard the remainder of the message. This value is also referenced from the DS-MAP messages (see 330HError: Reference source not found). |
| DCD Channel Information Elements (IEs) | Variable in integer number of bytes | 331HTable 21 |
| Begin PHY Specific Section { |  |  |
| Number of downstream burst profiles: n | 6 bits | Number of burst profiles described in the current DCD message. Its maximum size corresponds to the maximum number of DIUC burst profiles contained in 332HError: Reference source not found. |
| Reserved | 2 bits | All bits shall be set to zero. |
| for (i = 1; i  n; i++) { |  | “n” is defined as the “Number of downstream burst profiles” to be described in the current DCD message. |
| Downstream_Burst_Profile | Variable | PHY specific (333HTable 23) |
| } |  |  |
| } |  |  |
| } |  |  |

      - DCD Channel Information Elements

These are the allowed information elements that can be included in the DCD message.

- — DCD channel information elements

| Name | Element ID (1 byte) | Length (bits) | Description |
| --- | --- | --- | --- |
| Downstream_Burst_Profile | 1 | Variable | Value reserved for the burst profile (see 334HTable 23) |
| EIRPBS | 2 | 8 | Signed in units of dBm in 0.5 dB steps with a range from –64 dBm (encoded 0x00) to +63.5 dBm (encoded 0xFF). Values outside this range shall be assigned the closest extreme. |
| TTG | 3 | 8 | 0x00-0xFF: range of TTG in 2.75 µs increments. Default set to 0x4D to allow for 210 μs for 30 km propagation. |
| RSSIR_BS_nom | 4 | 8 | Initial ranging nominal signal strength per subcarrier to be received at the BS by a 0 dBi antenna gain, i.e., corrected for the gain of the BS receive antenna in the direction of the CPE and for 0 coupling and cable loss (see 335HError: Reference source not found). Signed in units of dBm in 0.5 dB steps ranging from –104 dBm (encoded 0x00) to +23.5 dBm (encoded 0xFF). Values outside this range shall be assigned the closest extreme. |
| Channel Action | 5 | 3 | Action to be taken by all CPEs in a cell. 000: None 001: Switch 010-111: Reserved |
| Action Mode | 6 | 1 | This is valid only for channel switch (Action = 001). Indicates a restriction on transmission until the specified Channel Action is performed. The BS shall set the Action Mode field to either 0 or 1 on transmission. A value of 1 means that the CPE to which the frame containing this element is addressed shall transmit no further frames until the scheduled Channel Action is performed. An Action Mode set to 0 does not impose any requirement on the receiving CPE. |
| Action Superframe Number | 7 | 8 | The superframe number (modulo 256) at which Channel Action shall be performed. |
| Action Frame Number | 8 | 4 | Integer value greater than or equal to zero that indicates the starting frame number, within the Action Superframe Number, at which the Channel Action shall be performed by all CPEs. |
| Number of Backup channels | 9 | 4 | Number of backup channels in the backup and candidate channel list IE (See element 336HTable 22). |
| Backup and Candidate channel list. | 10 | Variable | See 337HTable 22 for specification |
| MAC version | 11 | 8 | IEEE 802.22 MAC version to which the message originator conforms. 0x01: IEEE P802.22 Standard 0x02 – 0xFF: Reserved |

- a— Backup and Candidate channel list

| Syntax | Size | Notes |
| --- | --- | --- |
| Backup_and_candidate_channel_list_IE_Format() { |  |  |
| Element ID = 10 | 8 bits |  |
| Length | 8 bits |  |
| Number of Channels in the list | 8 bits |  |
| For (i=0; i < Number of Channels in the list; i++) { |  | List of backup channels in order of priority to be used by CPEs in case of loss of communication with the BS due to incumbents. This list may also include candidate channels, in which case they will follow the backup channels in the list, and will also be included in order of priority. The number of backup channels in the list is indicated in DCD Element ID 9 (see 338HTable 21). The list shall be a disjoint set with the current operating channel. |
| Channel Number [i] | 8 bits |  |
| Group Flag | 1 bits | Flag to indicate whether the backup or candidate channel is used globally at the BS or locally at the Group 0: Used globally at the BS 1: Used locally at the group |
| If(Group Flag=1){ |  |  |
| GID [i] | 12 bits | Group ID at which the backup or candidate channel is used locally |
| } |  |  |
| } |  |  |
| } |  |  |

      - Downstream Burst Profile

- — Downstream burst profile format

| Syntax | Size | Notes |
| --- | --- | --- |
| Downstream_Burst_Profile_Format() { |  |  |
| Type = 1 | 8 bits |  |
| Length | 8 bits |  |
| DIUC | 6 bits | 339HError: Reference source not found |
| Reserved | 2 bits | All bits shall be set to zero |
| Information Elements (IEs) | Variable | 340HTable 24 |
| } |  |  |

- — Downstream burst profile Information elements

| Name | Element ID (1 byte) | Length (bytes) | Description |
| --- | --- | --- | --- |
| DIUC mandatory exit threshold | 151 | 1 | -64 dB (encoded 0x00) to +63.5 dB (encoded 0xFF) CINR at or below which this DIUC can no longer be used and where change to a more robust DIUC is required (in 0.5 dB units). |
| DIUC minimum entry threshold | 152 | 1 | –64 dB (encoded 0x00) to +63.5 dB (encoded 0xFF) The minimum CINR required to start using this DIUC when changing from a more robust DIUC is required (in 0.5 dB units) |