IEEE P802.22
Wireless RANs

| DS/US-MAP IE for Group Resource Allocation |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2013-07-16 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Sung-Hyun Hwang | ETRI | Korea | +82-42-860-1133 | shwang@etri.re.kr |
| Gwang-Zeen Ko | ETRI | Korea | +82-42-860-4862 | gogogo@etri.re.kr |
| Byung-Jang Jeong | ETRI | Korea | +82-42-860-6765 | bjjeong@etri.re.kr |
| Myung-Sun Song | ETRI | Korea | +82-42-860-5046 | mssong@etri.re.kr |
| Jae-Ick Choi | ETRI | Korea | +82-42-860-6160 | jichoi@etri.re.kr |

Abstract

This document proposes the DS/US-MAP IE for group resource allocation.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Carl R. Stevenson> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

- 1. Background

- The proposed group resource allocation (GRA) is proposed to meet two requirements in the Functional Requirements Document (22-12-0012r3). First, requirement #3 is regarding the device classes and complexity. Here, we have to support at least two classes of CPE with different complexity. The proposed group resource allocation is efficiently designed to support different classes of CPE. Second, requirement #5 is regarding the number of devices. Here, we have to support many number of devices, at least 2048 CPEs. In the network with several thounds of devices, we can significantly reduce the MAP overhead by using the proposed group resource allocation

- 2. Suggested text

Subclauses 7.7 to 7.7.1 are same as IEEE 802.22-2011 standard.

    - Downstream Map (DS-MAP)

The format of a DS-MAP message is shown in 341HTable 25. The DS-MAP message defines the access to the downstream information. The length of the DS-MAP shall be an integer number of bytes.

- a— DS-MAP message format

| Syntax | Size | Notes |
| --- | --- | --- |
| DS-MAP_Message_Format() { |  |  |
| Management Message Type = 1 | 8 bits |  |
| DCD Count | 8 bits | Matches the value of the configuration change count of the DCD, which describes the downstream burst profiles that apply to this map. |
| Begin PHY Specific Section { |  |  |
| Number of IRA IEs: n | 12 bits | Number of individual resource allocation (IRA) IEs in the downstream map |
| for (i = 1; i  n; i++) { |  |  |
| DS-MAP_IE() | Variable | PHY specific (7.7.2.2) |
| } |  |  |
| Number of GRA IEs: m | 12 bits | Number of group resource allocation (GRA) IEs in the downstream map |
| for (i = 1; i  m; i++) { |  |  |
| DS-MAP_GRA_IE() | Variable | PHY specific (7.7.2.2) |
| } |  |  |
| } |  |  |
| If(!byte_boundary) |  |  |
| Padding bits | 0-7 bits | Padding to octet alignment – All bits shall be set to 0. |
| } |  |  |

      - DS-MAP IE

The format of the DS-MAP IE is shown in Table 30.

- — DS-MAP information elements

| Syntax | Size | Description |
| --- | --- | --- |
| DS-MAP_IE() { |  |  |
| DIUC | 6 bits | 344H7.7.2.1.1 |
| If (DIUC == 62) |  |  |
| Extended DIUC Dependent IE | Variable | 345H7.7.2.1.2 |
| else { |  |  |
| SID | 9 bits | Station ID of CPE or multicast group. |
| } |  |  |
| Length | 12 bits | Number of OFDM slots linearly allocated to the DS burst specified by this IE. |
| Boosting | 3 bits | 111: +9 dB 110: +6 dB 101: +3 dB 100: 0 dB, normal (not boosted) 011: -3 dB 010: -6 dB 001: -9 dB 000: -12 dB |
| } |  |  |
| } |  |  |

        - DIUC Allocations

346HTable 27 illustrates the various DIUC values used in the MAC.

- — DIUC values

| DIUC | Usage |  |  |
| --- | --- | --- | --- |
| 0-12 | Reserved |  |  |
| 13 | Uncoded | NA | BPSK |
| 14 | Convolutional Code | FEC rate = 1/2 | QPSK |
| 15 | Convolutional Code | FEC rate = 2/3 | QPSK |
| 16 | Convolutional Code | FEC rate = 3/4 | QPSK |
| 17 | Convolutional Code | FEC rate = 5/6 | QPSK |
| 18 | Convolutional Code | FEC rate = 1/2 | 16-QAM |
| 19 | Convolutional Code | FEC rate = 2/3 | 16-QAM |
| 20 | Convolutional Code | FEC rate = 3/4 | 16-QAM |
| 21 | Convolutional Code | FEC rate = 5/6 | 16-QAM |
| 22 | Convolutional Code | FEC rate = 1/2 | 64-QAM |
| 23 | Convolutional Code | FEC rate = 2/3 | 64-QAM |
| 24 | Convolutional Code | FEC rate = 3/4 | 64-QAM |
| 25 | Convolutional Code | FEC rate = 5/6 | 64-QAM |
| 26 | CTC | FEC rate = 1/2 | QPSK |
| 27 | CTC | FEC rate = 2/3 | QPSK |
| 28 | CTC | FEC rate = 3/4 | QPSK |
| 29 | CTC | FEC rate = 5/6 | QPSK |
| 30 | CTC | FEC rate = 1/2 | 16-QAM |
| 31 | CTC | FEC rate = 2/3 | 16-QAM |
| 32 | CTC | FEC rate = 3/4 | 16-QAM |
| 33 | CTC | FEC rate = 5/6 | 16-QAM |
| 34 | CTC | FEC rate = 1/2 | 64-QAM |
| 35 | CTC | FEC rate = 2/3 | 64-QAM |
| 36 | CTC | FEC rate = 3/4 | 64-QAM |
| 37 | CTC | FEC rate = 5/6 | 64-QAM |
| 38 | LDPC | FEC rate = 1/2 | QPSK |
| 39 | LDPC | FEC rate = 2/3 | QPSK |
| 40 | LDPC | FEC rate = 3/4 | QPSK |
| 41 | LDPC | FEC rate = 5/6 | QPSK |
| 42 | LDPC | FEC rate = 1/2 | 16-QAM |
| 43 | LDPC | FEC rate = 2/3 | 16-QAM |
| 44 | LDPC | FEC rate = 3/4 | 16-QAM |
| 45 | LDPC | FEC rate = 5/6 | 16-QAM |
| 46 | LDPC | FEC rate = 1/2 | 64-QAM |
| 47 | LDPC | FEC rate = 2/3 | 64-QAM |
| 48 | LDPC | FEC rate = 3/4 | 64-QAM |
| 49 | LDPC | FEC rate = 5/6 | 64-QAM |
| 50 | SBTC | FEC rate = 1/2 | QPSK |
| 51 | SBTC | FEC rate = 2/3 | QPSK |
| 52 | SBTC | FEC rate = 3/4 | QPSK |
| 53 | SBTC | FEC rate = 5/6 | QPSK |
| 54 | SBTC | FEC rate = 1/2 | 16-QAM |
| 55 | SBTC | FEC rate = 2/3 | 16-QAM |
| 56 | SBTC | FEC rate = 3/4 | 16-QAM |
| 57 | SBTC | FEC rate = 5/6 | 16-QAM |
| 58 | SBTC | FEC rate = 1/2 | 64-QAM |
| 59 | SBTC | FEC rate = 2/3 | 64-QAM |
| 60 | SBTC | FEC rate = 3/4 | 64-QAM |
| 61 | SBTC | FEC rate = 5/6 | 64-QAM |
| 62 | Extended DIUC |  |  |
| 63 | End of Map |  |  |

        - DS-MAP Extended DIUC IE

A DS-MAP IE entry with a DIUC value of 62 indicates that the IE carries special information and conforms to the structure shown in 347HTable 28. A CPE shall ignore an extended IE entry with an extended DIUC value for which the CPE has no knowledge. In the case of a known extended DIUC value but with a length field longer than expected, the CPE shall process information up to the known length and ignore the remainder of the IE.

- — DS-MAP extended IE general format

| Syntax | Size | Notes |
| --- | --- | --- |
| DS_Extended_IE() { |  |  |
| Extended DIUC | 6 bits |  |
| Length | 8 bits | Length of this IE in bits. |
| Unspecified Data | Variable |  |
| } |  |  |

          - DS-MAP Dummy Extended IE

A CPE shall be able to decode the DS-MAP Dummy Extended IE. A BS shall not transmit this IE (unless under test). A CPE may skip decoding downlink bursts scheduled after the start time of this IE within the current frame.

- — DS-MAP Dummy Extended IE format

| Syntax | Size | Notes |
| --- | --- | --- |
| Dummy_IE() { |  |  |
| Extended DIUC | 6 bits | 0x00 |
| Length | 8 bits | Length of this IE in bits. |
| Unspecified Data | Variable |  |
| } |  |  |

      - DS-MAP GRA IE

The format of the DS-MAP GRA IE is shown in Table 30.

- — DS-MAP GRA information elements

| Syntax | Size | Description |
| --- | --- | --- |
| DS-MAP_GRA_IE() { |  |  |
| Resource Allocation Bitmap | Variable (1 bits * number of devices in the group) | Indicates whether the resources are allocated to the device in a group. The number of devices in the group is determined by the Device Bitmap Size in GRA Configuration Message. 0: not allocated in the frame 1: allocated in the frame |
| Resource Starting Index | 11 | Indicates the starting index of resource in the unit of OFDMA slot. In the DS subframe, the index starts right after the frame preamble from 0. In the US subframe, the index 0 starts from the ranging/BW request/UCS notification contention windows (not including SCW) if it exists. |
| Resource Size Bitmap | Variable (3 bits * number of devices in the group) | Indicates the resource allocation size for the device in the unit of OFDMA slot. 000: 1 001: 2 010: 4 011: 8 100: 16 101: 32 110: 64 111: 128 |
| Group DIUC Flag | 1 | Indicates whether the DIUC is fixed within group. 0: not fixed within group 1: fixed within group |
| Group Boosting Flag | 1 | Indicates whether the Boosting is fixed within group. 0: not fixed within group 1: fixed within group |
| If(Group DIUC Flag = 0) { |  |  |
| Group DIUC Bitmap | Variable (6 bits * number of devices in the group) | Specifies the DIUC of each device in a group |
| } |  |  |
| Else{ |  |  |
| DIUC | 6 | Same DIUC is used by all device in a group |
| } |  |  |
| If(Group Boosting Flag = 0) { |  |  |
| Group Boosting Bitmap | Variable (3 bits * number of devices in the group) | Specifies the Bossting of each device in a group |
| } |  |  |
| Else{ |  |  |
| Boosting | 3 | Same Boosting is used by all device in a group |
| } |  |  |
| } |  |  |

    - Upstream Channel Descriptor (UCD)

The format of a UCD message is shown in 348HTable 30. This message shall be transmitted by the BS at a periodic interval (349HError: Reference source not found) to define the characteristics of an upstream physical channel.

- — UCD message format

| Syntax | Size | Notes |
| --- | --- | --- |
| UCD_Message_Format() { |  |  |
| Management Message Type = 2 | 8 bits |  |
| Configuration Change Count | 8 bits | Incremented by one (modulo 256) by the BS whenever any of the values of this channel descriptor change. If the value of this count in a subsequent UCD remains the same, the CPE can quickly decide that the remaining fields have not changed and may be able to disregard the remainder of the message. This value is also referenced from the US-MAP messages (see 350HTable 34). |
| BW Request Backoff Start | 4 bits | Initial backoff window size in units of BW Request opportunity (see 351HTable 31) used by CPEs to contend to send BW requests to the BS, expressed as a power of 2. Values of n range 0–15. Refer in the note to section 6.16 on Contention Resolution. Include a sub-section that will describe the size and the content of the BW Request US burst and refer to it in the note. |
| BW Request Backoff End | 4 bits | Final backoff window size in units of BW Request opportunity (see Table 39) to contend to send BW requests to the BS, expressed as a power of 2. Values of n range 0–15. All declared opportunities for BW request in subsequent frames are concatenated in this potentially large number. |
| UCS Notification Backoff Start | 4 bits | Initial backoff window size in units of UCS Notification opportunity (see 352HTable 31) used by CPEs to contend to send UCS notifications to the BS. This is expressed as a power of 2. Values of n range 0–15. |
| UCS Notification Backoff End | 4 bits | Final backoff window size in units of UCS Notification opportunity (see 353HTable 31) used by CPEs to contend to send UCS notifications to the BS. This is expressed as a power of 2. Values of n range 0–15. All declared opportunities for UCS Notifications in subsequent frames are concatenated in this potentially large number. |
| Information Elements (IEs) for the overall channel | Variable | See 354H7.7.3.1. |
| Begin PHY Specific Section { |  |  |
| Number of upstream burst profiles: n | 6 bits | Number of upstream burst profiles described in the current UCD message. Its maximum size corresponds to the maximum number of UIUC burst profiles contained in 355HTable 36. |
| for (i = 1; i  n; i++) { |  | n = number of upstream burst profiles |
| Upstream_Burst_Profile | Variable | PHY specific (356HTable 32) |
| } |  |  |
| } |  |  |
| } |  |  |

      - UCD Channel IEs

Common channel encodings are provided in 357HTable 31.

- — UCD channel information elements

| Name | Element ID (1 byte) | Length (bytes) | Description |
| --- | --- | --- | --- |
| Upstream_Burst_Profile | 1 | Variable | Value reserved for the burst profile (see 358HTable 32) |
| Contention-based reservation timeout | 2 | 1 | Number of US-MAPs to receive before contention-based reservation is attempted again for the same connection |
| Bandwidth request opportunity size | 3 | 1 | Size (in OFDM slots) of PHY bursts, mapped horizontally in one sub-channel at a time as in the case of normal upstream data, that a CPE may use to format and transmit a bandwidth request message in a contention request opportunity. The value includes all PHY overhead as well as allowance for the BW Request MAC sub-header that the message will hold (see 359HError: Reference source not found). |
| UCS Notification request opportunity size | 4 | 1 | Size (in OFDM slots) of PHY bursts, mapped horizontally in one sub-channel at a time as in the case of normal upstream data, that a CPE may use to transmit a UCS notification. The value includes all PHY overhead for the GMH containing the UCS flag (see 360HError: Reference source not found). |
| Initial ranging codes | 150 | 1 | Number of initial ranging CDMA codes. Possible values are 0–255. |
| Periodic ranging codes | 151 | 1 | Number of periodic ranging CDMA codes. Possible values are 0–255. |
| Bandwidth request codes | 152 | 1 | Number of bandwidth request CDMA codes. Possible values are 0–255. |
| UCS notification codes | 153 | 1 | Number of UCS notification CDMA codes. Possible values are 0–255. |
| Start of CDMA codes group | 154 | 1 | Indicates the starting number, S, of the group of codes used for this upstream. All the ranging codes used on this upstream will be between S and ((S+N+M+L+I) mod 256). Where: N is the number of initial-ranging codes M is the number of periodic-ranging codes L is the number of bandwidth-request codes I is the number of UCS notification codes The range of values is 0 ≤ S ≤ 255. |

      - Upstream Burst Profile

The format of the upstream burst profile is shown in 361HTable 32, and the information elements contained in the upstream burst profiles are defined in 362HTable 33.

- — Upstream burst profile format

| Syntax |  | Size |  | Notes |
| --- | --- | --- | --- | --- |
| Upstream_Burst_Profile_Format() { |  |  |  |  |
| Type = 1 | 8 bits |  |  |  |
| Length | 8 bits |  |  |  |
| UIUC | 6 bits |  | 363HTable 36 |  |
| Reserved | 2 bits |  | All bits shall be set to zero. |  |
| Information Elements (IEs) | Variable |  | 364HTable 33 |  |
| } |  |  |  |  |

- — Upstream burst profile information elements

| Name | Element ID (1 byte) | Length (bytes) | Description |
| --- | --- | --- | --- |
| Ranging data ratio | 151 | 1 | Reduction factor, in units of 0.5 dB, between the EIRP per subcarrier used for this burst and the EIRP per subcarrier that should be used for CDMA Ranging. |
| Normalized CNR override | 152 | 7 | The first byte shall represent a signed integer which specifies, in dB, the first normalized CNR value in 365HError: Reference source not found (i.e., normalized CNR value corresponding to the CDMA code). Bytes 2-7: represent a list of numbers, where each number is encoded by one nibble, and is interpreted as a signed integer. The number encoded by each nibble represents the difference, in dB, in normalized CNR relative to the previous line in 366HError: Reference source not found. Thus the left most nibble of the second byte corresponds to the difference between the normalized CNR value for QPSK, rate: 1/2, and the normalized CNR value for the CDMA code. |

    - Upstream Map (US-MAP)

The format of a US-MAP message is shown in 367HTable 34. The US-MAP message defines the access to the upstream channel using US-MAP IEs and US-MAP GRA IEs.

- a— US-MAP message format

| Syntax | Size | Notes |
| --- | --- | --- |
| US-MAP_Message_Format() { |  |  |
| Management Message Type = 3 | 8 bits |  |
| UCD Count | 8 bits | Matches the value of the Configuration Change Count of the UCD, which describes the upstream burst profiles that apply to this map. |
| Allocation Start Time | 6 bits | Effective start time (in OFDM symbols from the start of the frame including all preambles) of the upstream allocation defined by the US-MAP. |
| Begin PHY Specific Section { |  |  |
| Number of IRA IEs: n | 12 bits | Number of individual resource allocation (IRA) IEs in the upstream map |
| for (i = 1; i  n; i++) { |  |  |
| US-MAP_IE() | Variable | PHY specific (7.7.4.1) Define upstream bandwidth allocations. Each US-MAP message shall contain at least one IE that marks the end of the last allocated burst. (UIUC=63 as defined in Table 36). |
| } |  |  |
| Number of GRA IEs: m | 12 bits | Number of group resource allocation (GRA) IEs in the upstream map |
| for (i = 1; i  m; i++) { |  |  |
| US-MAP_GRA_IE() | Variable | PHY specific (7.7.4.1) Define upstream bandwidth allocations. Each US-MAP message shall contain at least one IE that marks the end of the last allocated burst. (UIUC=63 as defined in Table 36). |
| } |  |  |
| } |  |  |
| If(!byte_boundary) |  |  |
| Padding bits | 0-7 bits | Padding to octet alignment –All bits shall be set to 0. |
| } |  |  |

      - US-MAP IE

The SID field carried by the US-MAP IE is associated with a unicast address. When specifically addressed to allocate a bandwidth grant, the FID shall be the Basic FID of the CPE. A UIUC shall be used to define the type of upstream access and the upstream burst profile associated with that access. An Upstream_Burst_Profile shall be included in the UCD for each UIUC to be used in the US-MAP. The beginning of the upstream subframe is clearly defined by the allocation start time which corresponds to the number of symbols from the first preamble symbol of the current frame (e.g., superframe preamble or frame preamble) plus the width of the TTG (see 370HError: Reference source not found).  The end of the upstream subframe is defined either by the SCH in the case of the scheduling of an intra-frame quiet period or by the US-MAP when a SCW is scheduled at the end of the frame by the presence of UIUC’s 0 or 1 in the US-MAP.

The US-MAP IE is shown in 371HTable 35, and is used to define the upstream bandwidth allocations. The first US-MAP IE shall start at the lowest numbered sub-channel on the first non-allocated symbol defined by the allocation start time field of the US-MAP message. These IEs shall represent the number of OFDM slots provided for the allocation. Each allocation IE shall start immediately following the previous allocation and shall advance in the time domain. If the end of the US subframe has been reached, the allocation shall continue on the next sub-channel at the first symbol (defined by the allocation start time field). The US subframe can also be defined in terms of columns as described in 372HError: Reference source not found.  A Burst Descriptor shall be specified in the UCD for each UIUC to be used in the US-MAP.

The SID field in this message can also refer to a group of CPEs, e.g., a multicast group. In this case, only UIUC = 0 or 1 shall be allowed to enable configuration of that group of CPEs to use an SCW (see 373HError: Reference source not found and 374HError: Reference source not found).

- — US-MAP information elements

| Syntax | Size | Description |
| --- | --- | --- |
| US-MAP_IE() { |  |  |
| SID | 9 bits | Station ID of the CPE. |
| UIUC | 6 bits | 375H7.7.4.1.1 (See 376HTable 36) |
| If ((UIUC  0) && (UIUC  1)) { |  |  |
| CBP Frame Number | 4 bits | Frame number where the active or passive CBP action is to take place. If the identified frame falls in the next superframe (e.g., current frame is 9 and the CBP Frame Number is 4), the CPE shall make sure that a SCW is still scheduled for this frame as indicated by the upcoming SCH. If not, the CBP action shall be cancelled. |
| If(UIUC==0) { |  | Active SCW mode (CPE to transmit a CBP burst as requested by the BS.) |
| Timing advance | 16 bits | Signed number in TU corresponding to the advance of the transmission of the CBP burst at the CPE. As the CPE starts to transmit the CBP burst as its fourth symbol before the end of the frame, zero advance corresponds to this signal being received by the BS at the beginning of its fourth symbol before the end of the frame when the CPE is co-located with the BS (see 377HError: Reference source not found).. |
| EIRP Density Level | 8 bits | EIRP per transmitted subcarrier (see 378HError: Reference source not found). Signed in units of 0.5 dB, ranging from -104 dBm (encoded 0x00) to +23.5 dBm (encoded 0xFF). |
| } |  |  |
| If(UIUC==1) { |  | Passive SCW mode (CPE to receive and demodulate the CBP burst and send content to the BS.) |
| Channel Number | 8 bits | Channel number in which the CPE shall listen to the medium for a coexistence beacon |
| Synchronization mode | 1 bit | = 0 The CPE will capture the CBP burst using its current synchronization (i.e., locked to its BS) for geolocation purposes. = 1 The CPE will re-synchronize on the received CBP burst using the preamble symbol and optionally pilot carriers to decode the payload for self-coexistence purposes. |
| } |  |  |
| } else if (UIUC 2) && (UIUC  3) ) { |  |  |
| Number of Sub-channels | 4 bits | Number of sub-channels reserved for the BW Request/UCS Notification opportunistic window. |
| } else if (UIUC 4) && (UIUC  6) ) { |  |  |
| Number of Sub-channels | 4 bits | Number of sub-channels reserved for the CDMA Ranging/BW Request/UCS Notification opportunistic window. Note that in case where UIUC=8 and any UIUC in the range 4 to 6 are allocated to a frame, the largest number of sub-channel specified shall prevail. Note also that when the CDMA ranging burst is to be used for terrestrially-based geolocation (see 379HError: Reference source not found), the number of sub-channels shall be at least 6. |
| Number of symbols | 5 bits | Number of symbols in the US ranging channel reserved for the opportunistic windows carrying either CDMA Periodic Ranging/BW Request/UCS Notification as specified by the respective UIUC. These shall be placed in the ranging channel following the initial ranging window if scheduled and consecutively (see 380HError: Reference source not found). |
| } else if (UIUC == 7) { |  |  |
| CDMA_ Allocation_IE () | 20 bits | See 381H7.7.4.1.2. |
| } else if (UIUC == 8) { |  | The first 5 symbols of the upstream subframe shall be reserved for the opportunistic initial ranging burst. |
| Number of Sub-channels | 4 bits | Number of sub-channels reserved for the initial ranging burst. Note that in case where UIUC=8 and any UIUC in the range 4 to 6 are allocated to a frame, the largest number of claimed sub-channels specified shall prevail. |
| } else if (UIUC == 9) { |  | US-MAP EIRP Control IE |
| US-MAP EIRP Control IE | Variable | See 382H7.7.4.1.3. |
| } else if (UIUC == 62) { |  |  |
| US_Extended_IE() | Variable | See 383H7.7.4.1.4. |
| } else { |  |  |
| Burst_Type | 1 bit | This value specifies the burst type for the burst specified by this US-MAP IE. 0: Bursts are mapped in the time axis over the full width of the upstream subframe before incrementing in the frequency axis; 1: Bursts are mapped in the time axis over segments of 7 symbols before incrementing in the frequency axis and then re-tracing to the lowest unused sub-channel in the next 7 symbol segment. The width of the last segment is to be between 7 and 13 symbols depending on the width of the upstream subframe |
| Duration | 12 bits | Number of OFDM slots linearly allocated to the US burst specified by this IE. (Up to 60 by 30 slots can be allocated to a US burst.) |
| MDP | 1 bit | Measurement Data Preferred Used by the BS to indicate to the CPE that this upstream allocation is to be preferably used by the CPE for the specific purpose of reporting back any measurement data. The measurement data to be reported is in connection to the specified Transaction ID. In case the CPE does not have anything to report, it can use this allocation for any other data. This is useful, for example, after a quiet period. 0: Measurement data not required (default) 1: Measurement data preferred |
| MRT | 1 bit | Measurement Report Type In case MDP == 1, this field indicates which type of report the BS wants the CPE to send back. 0: Detailed (see 384HError: Reference source not found through 385HError: Reference source not found) 1: Consolidated (see 386HError: Reference source not found) |
| CMRP | 1 bit | Channel Management Response Preferred Used by the BS to indicate to the CPE that this upstream allocation is to be used for confirming or not the receipt of the channel management command with the Transaction ID specified. 0: Channel management response not required (default) 1: Channel management response required |
| } |  |  |
| } |  |  |

        - UIUC Allocations

387HTable 36 specifies the UIUC incorporated into the MAC. In particular, the self-coexistence UIUCs (in both modes) have the same applicability to their DIUC counterpart (see 388H7.7.2.1.1).

- — UIUC values

| UIUC | Usage |  |  |
| --- | --- | --- | --- |
| 0 | Self-Coexistence (Active Mode) |  |  |
| 1 | Self-Coexistence (Passive Mode) |  |  |
| 2 | UCS Notification |  |  |
| 3 | BW Request |  |  |
| 4 | CDMA UCS Notification |  |  |
| 5 | CDMA BW Request |  |  |
| 6 | CDMA Periodic Ranging |  |  |
| 7 | CDMA Allocation IE (see 389HTable 37) |  |  |
| 8 | CDMA Initial Ranging |  |  |
| 9 | US-MAP EIRP Control IE |  |  |
| 10~12 | Reserved |  |  |
| 13 | Uncoded | NA | BPSK |
| 14 | Convolutional Code | FEC rate = 1/2 | QPSK |
| 15 | Convolutional Code | FEC rate = 2/3 | QPSK |
| 16 | Convolutional Code | FEC rate = 3/4 | QPSK |
| 17 | Convolutional Code | FEC rate = 5/6 | QPSK |
| 18 | Convolutional Code | FEC rate = 1/2 | 16-QAM |
| 19 | Convolutional Code | FEC rate = 2/3 | 16-QAM |
| 20 | Convolutional Code | FEC rate = 3/4 | 16-QAM |
| 21 | Convolutional Code | FEC rate = 5/6 | 16-QAM |
| 22 | Convolutional Code | FEC rate = 1/2 | 64-QAM |
| 23 | Convolutional Code | FEC rate = 2/3 | 64-QAM |
| 24 | Convolutional Code | FEC rate = 3/4 | 64-QAM |
| 25 | Convolutional Code | FEC rate = 5/6 | 64-QAM |
| 26 | CTC | FEC rate = 1/2 | QPSK |
| 27 | CTC | FEC rate = 2/3 | QPSK |
| 28 | CTC | FEC rate = 3/4 | QPSK |
| 29 | CTC | FEC rate = 5/6 | QPSK |
| 30 | CTC | FEC rate = 1/2 | 16-QAM |
| 31 | CTC | FEC rate = 2/3 | 16-QAM |
| 32 | CTC | FEC rate = 3/4 | 16-QAM |
| 33 | CTC | FEC rate = 5/6 | 16-QAM |
| 34 | CTC | FEC rate = 1/2 | 64-QAM |
| 35 | CTC | FEC rate = 2/3 | 64-QAM |
| 36 | CTC | FEC rate = 3/4 | 64-QAM |
| 37 | CTC | FEC rate = 5/6 | 64-QAM |
| 38 | LDPC | FEC rate = 1/2 | QPSK |
| 39 | LDPC | FEC rate = 2/3 | QPSK |
| 40 | LDPC | FEC rate = 3/4 | QPSK |
| 41 | LDPC | FEC rate = 5/6 | QPSK |
| 42 | LDPC | FEC rate = 1/2 | 16-QAM |
| 43 | LDPC | FEC rate = 2/3 | 16-QAM |
| 44 | LDPC | FEC rate = 3/4 | 16-QAM |
| 45 | LDPC | FEC rate = 5/6 | 16-QAM |
| 46 | LDPC | FEC rate = 1/2 | 64-QAM |
| 47 | LDPC | FEC rate = 2/3 | 64-QAM |
| 48 | LDPC | FEC rate = 3/4 | 64-QAM |
| 49 | LDPC | FEC rate = 5/6 | 64-QAM |
| 50 | SBTC | FEC rate = 1/2 | QPSK |
| 51 | SBTC | FEC rate = 2/3 | QPSK |
| 52 | SBTC | FEC rate = 3/4 | QPSK |
| 53 | SBTC | FEC rate = 5/6 | QPSK |
| 54 | SBTC | FEC rate = 1/2 | 16-QAM |
| 55 | SBTC | FEC rate = 2/3 | 16-QAM |
| 56 | SBTC | FEC rate = 3/4 | 16-QAM |
| 57 | SBTC | FEC rate = 5/6 | 16-QAM |
| 58 | SBTC | FEC rate = 1/2 | 64-QAM |
| 59 | SBTC | FEC rate = 2/3 | 64-QAM |
| 60 | SBTC | FEC rate = 3/4 | 64-QAM |
| 61 | SBTC | FEC rate = 5/6 | 64-QAM |
| 62 | Extended UIUC |  |  |
| 63 | End of Map |  |  |

        - CDMA Allocation IE

This IE is used by the BS with UIUC= 7 to assign a US bandwidth allocation to a CPE that signaled its wish to either associate through the Initial Ranging CDMA burst, to signal the presence of an incumbent by the CDMA UCS Notification or to request a BW allocation through the CDMA BW Request burst.

- — CDMA allocation IE format

| Syntax | Size | Notes |
| --- | --- | --- |
| CDMA_Allocation_IE() { |  |  |
| Code | 8 bits | Indicates the Code sent by the CPE. |
| Duration | 5 bits | Indicates the duration, in OFDMA slots, of the allocation. [Not necessarily on the same sub-channel.] |
| UIUC | 6 bits | UIUC to be used by the CPE for this allocation (see 390HTable 36). |
| Usage | 1 bit | If (Code = Incumbent) This field indicates whether the CPE shall transmit only the MAC header with the notification. 1: yes; 0: no |
| } |  |  |

        - US-MAP EIRP Control IE

When an EIRP change for the CPE is needed, this UIUC= 9 is used as shown in 391HTable 38. The EIRP control value is an 8-bit signed integer expressing the EIRP per subcarrier (in 0.5 dB units) that the CPE should apply instead of its current transmission EIRP (see 392HError: Reference source not found). The current FID used with this IE shall be the Basic FID of the CPE.  The EIRP accuracy shall be ± 1.5 dB when the level is at least 10 dB below the maximum regulatory power limit and ±0.5 dB elsewhere.

- — US-MAP EIRP control IE format

| Syntax | Size | Notes |
| --- | --- | --- |
| EIRP_Control_IE() { |  |  |
| UIUC | 6 bits | 0x09 |
| EIRP Control | 8 bits | Signed integer that indicates the EIRP per subcarrier that the CPE should apply to correct its current transmission EIRP. Signed in units of 0.5 dB, ranging from -104 dBm (encoded 0x00) to +23.5 dBm (encoded 0xFF). |
| } |  |  |

        - US-MAP Extended UIUC IE

A US-MAP IE entry with a UIUC value of 62 indicates that the IE carries special information and conforms to the structure shown in 393HTable 39. A BS/CPE shall ignore an extended IE entry with an extended UIUC value for which it has no knowledge. In the case of a known extended UIUC value but with a length field longer than expected, it shall process information up to the known length and ignore the remainder of the IE.

- — US-MAP extended IE general format

| Syntax | Size | Notes |
| --- | --- | --- |
| US_Extended_IE() { |  |  |
| Extended UIUC | 6 bits | Values specific to the Extended IE |
| Length | 8 bits | Length of this IE in bits. |
| Unspecified Data | Variable |  |
| } |  |  |

          - US-MAP Dummy Extended IE

A CPE shall be able to decode the US-MAP Dummy Extended IE. A BS shall not transmit this IE (unless under test). The Length field of 394HTable 40 specifies the size of the Unspecified Data Field.

- — US-MAP Dummy Extended IE format

| Syntax | Size | Notes |
| --- | --- | --- |
| Dummy_IE() { |  |  |
| Extended UIUC | 6 bits | 0x00 |
| Length | 8 bits | Length of this IE in bits. |
| Unspecified Data | Variable |  |
| } |  |  |

      - US-MAP GRA IE

The format of the US-MAP GRA IE is shown in Table 30.

- — US-MAP GRA information elements

| Syntax | Size | Description |
| --- | --- | --- |
| US-MAP_GRA_IE() { |  |  |
| Resource Allocation Bitmap | Variable (1 bits * number of devices in the group) | Indicates whether the resources are allocated to the device in a group. The number of devices in the group is determined by the Device Bitmap Size in GRA Configuration Message. 0: not allocated in the frame 1: allocated in the frame |
| Resource Starting Index | 11 | Indicates the starting index of resource in the unit of OFDMA slot. In the DS subframe, the index starts right after the frame preamble from 0. In the US subframe, the index 0 starts from the ranging/BW request/UCS notification contention windows (not including SCW) if it exists. |
| Resource Size Bitmap | Variable (3 bits * number of devices in the group) | Indicates the resource allocation size for the device in the unit of OFDMA slot. 000: 1 001: 2 010: 4 011: 8 100: 16 101: 32 110: 64 111: 128 |
| Group UIUC Flag | 1 | Indicates whether the UIUC is fixed within group. 0: not fixed within group 1: fixed within group |
| Group Burst_Type Flag | 1 | Indicates whether the Burst_Type is fixed within group. 0: not fixed within group 1: fixed within group |
| Group MDP Flag | 1 | Indicates whether the MDP is fixed within group. 0: not fixed within group 1: fixed within group |
| Group MRT Flag | 1 | Indicates whether the MRT is fixed within group. 0: not fixed within group 1: fixed within group |
| Group CMRP Flag | 1 | Indicates whether the CMRP is fixed within group. 0: not fixed within group 1: fixed within group |
| If(Group UIUC Flag = 0) { |  |  |
| Group UIUC Bitmap | Variable (6 bits * number of devices in the group) | Specifies the UIUC of each device in a group |
| } |  |  |
| Else{ |  |  |
| UIUC | 6 | Same UIUC is used by all device in a group |
| } |  |  |
| If(Group Burst_Type Flag = 0) { |  |  |
| Group Burst_Type Bitmap | Variable (1 bits * number of devices in the group) | Specifies the Burst_Type of each device in a group |
| } |  |  |
| Else{ |  |  |
| Burst_Type | 1 | Same Burst_Type is used by all device in a group |
| } |  |  |
| If(Group MDP Flag = 0) { |  |  |
| Group MDP Bitmap | Variable (1 bits * number of devices in the group) | Specifies the MDP of each device in a group |
| } |  |  |
| Else{ |  |  |
| MDP | 1 | Same MDP is used by all device in a group |
| } |  |  |
| If(Group MRT Flag = 0) { |  |  |
| Group MRT Bitmap | Variable (1 bits * number of devices in the group) | Specifies the MRT of each device in a group |
| } |  |  |
| Else{ |  |  |
| MRT | 1 | Same MRT is used by all device in a group |
| } |  |  |
| If(Group CMRP Flag = 0) { |  |  |
| Group CMRP Bitmap | Variable (1 bits * number of devices in the group) | Specifies the CMRP of each device in a group |
| } |  |  |
| Else{ |  |  |
| CMRP | 1 | Same CMRP is used by all device in a group |
| } |  |  |
| } |  |  |