IEEE P802.22
Wireless RANs

| Suggested MIMO Topics to the Std.802.22b |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2013-07-XX |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Gabriel Porto Vilardi | NICT | 3-4, Hikarino-oka, Yokosuka, 239-0847, Japan |  | gpvillardi@nict.go.jp |
| Zhang Xin | NICT | 20 Science Park Road, #01-09A/10 TeleTech Park, Singapore |  | amy.xinzhang@ieee.org |
| Pin-Hsun Lin | NICT | 3-4, Hikarino-oka, Yokosuka, 239-0847, Japan |  | pslin@nict.go.jp |
| Masayuki Oodo | NICT | 3-4, Hikarino-oka, Yokosuka, 239-0847, Japan |  | moodo@nict.go.jp |
| Chang-Woo Pyo | NICT | 3-4, Hikarino-oka, Yokosuka, 239-0847, Japan |  | cwpyo@nict.go.jp |
| Chunyi Song | NICT | 3-4, Hikarino-oka, Yokosuka, 239-0847, Japan |  | songe@ieee.org |
| Keiichi Mizutani | NICT | 3-4, Hikarino-oka, Yokosuka, 239-0847, Japan |  | songe@ieee.org |
| Hiroshi Harada | NICT | 3-4, Hikarino-oka, Yokosuka, 239-0847, Japan |  | harada@ieee.org |

Abstract

This document suggests some MIMO topics to be considered in 802.22b standard and intends to stir up MIMO related discussions within the group.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair Apurva N. Mody <apurva.mody@ieee.org> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Suggested MIMO Topics:

- Multiple-input, multiple-output (MIMO)

  - MIMO channel estimation and synchronization

    - MIMO pilot allocation

      - Pilot allocation for 2 antennas

      - Pilot allocation for 4 antennas

  - Spatial Diversity/Multiplexing/Beam-Forming/Relaying

    - Open Loop Schemes

      - Space Time Coding (STC)

        - Transmit diversity using 2 antennas (Alamouti O-STBC)

        - Spatial multiplexing using 2 antennas

        - Spatial multiplexing using 4 antennas

    - Closed Loop Schemes

      - Closed-Loop Transmit Diversity

        - Transmit diversity for 2 antennas

        - Transmit diversity for 4 antennas

      - Closed-Loop Spatial Multiplexing

        - Spatial multiplexing for 2 antennas

        - Spatial multiplexing for 4 antennas

      - Beam-Forming

        - Beam Forming for 2 antennas

        - Beam Forming for 4 antennas

      - Relaying

        - Relaying for 2 antennas

        - Relaying for 4 antennas

MIMO Techniques Purpose in IEEE 802.22b

Spatial Diversity: to improve the quality and reliability of a wireless link, e.g., O-STBC. Thus, it can be used to extend 802.22b coverage as compared to original 802.22.

- Closed-loop version further mitigates the channel thus enhancing reliability and coverage.

Spatial Multiplexing: to create independend and separated streams so as to increase throughput, e.g., BLAST. It can be used to achieve the required 802.22b throughput.

- Independent streams might have weak channel gains.

- Closed-loop version precodes the symbols to further enhance reliability thus enabling higher throughputs.

Spatial Selectivity (Beam-Forming): to create directional signal transmission and/or reception. It can be used to mitigate interference among users contributing to the required interference management capabilities of 802.22b.

Relaying: to create a multihop link whose length is increased as compared to a direct link, for the same transmit power. It can be used to extend the coverage as required by 802.22b.

- Mantaining shorter links is necessary to create favourable channel conditions, e.g., small delay spread, in order to achieve the higher throughputs required by 802.22b.

- Cooperative relaying, where nodes relay each other information, can further aggregate value to the interference management capabilities of 802.22b.

- Does the group agree with the above topics?

- Any suggestion for addition? Why?

- Any suggestion for removal? Why?