IEEE P802.22
Wireless RANs

| Proposed Text of MAC technical items related to 7.7.X Channel Allocation Manager management messages |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2013-09-18 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Keat Beng Toh | Hitachi Kokusai Electric Inc. | 32,Miyuki-cho,Kodaira-shi, Tokyo, 187-8511, Japan | +81-42-322-3111 | toh.keatbeng@h-kokusai.com |
| Keigo Hasegawa | Hitachi Kokusai Electric Inc. | 32,Miyuki-cho,Kodaira-shi, Tokyo, 187-8511, Japan | +81-42-322-3111 | hasegawa.keigo@h-kokusai.com |
| Masayuki Takekawa | Hitachi Kokusai Electric Inc. | 32,Miyuki-cho,Kodaira-shi, Tokyo, 187-8511, Japan | +81-42-322-3111 | takekawa.masayuki@h-kokusai.com |
| Kei Yanagisawa | Hitachi Kokusai Electric Inc. | 32,Miyuki-cho,Kodaira-shi, Tokyo, 187-8511, Japan | +81-42-322-3111 | yanagisawa.kei@h-kokusai.com |
| Seishi Sasaki | Hitachi Kokusai Electric Inc. | 32,Miyuki-cho,Kodaira-shi, Tokyo, 187-8511, Japan | +81-42-322-3111 | sasaki.seishi@h-kokusai.com |
| Masahiro Asano | Hitachi Kokusai Electric Inc. | 32,Miyuki-cho,Kodaira-shi, Tokyo, 187-8511, Japan | +81-42-322-3111 | asano.masahiro@h-kokusai.com |

Abstract

This document contains the proposed text of MAC technical items related to Channel Allocation Manager　management messages.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair Apurva N. Mody <apurva.mody@ieee.org> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Summary of this document

In this document, channel allocation manager management messages for IEEE 802.22b is proposed to support multi channel operation which provide higher data throughput than the IEEE Std. 802.22-2011.The subclause structure in the proposed text is shown as follows.

7.7.X Channel allocation manager (CAM-AIF/STP/STP-ACK/SWH/SWH-ACK)

7.7.X.1 Add new operating channel 		(CAM-AIF)

7.7.X.2 Stop operating channel			(CAM-STP)

7.7.X.3 Stop operating channel acknowledgement	(CAM-STP-ACK)

7.7.X.4 Switch operating channel			(CAM-SWH)

7.7.X.5 Switch operating channel acknowledgement	(CAM-SWH-ACK)

In this proposal, all the texts are described as new texts (without revision marks based on the 802.22 standard).

7.7 Management Messages

As can be seen in Table 19, the MAC defines a collection of management messages to support and implement its basic functions. All these messages are carried in the payload of a MAC PDU, and share the same message structure as depicted in Figure 15. Management messages begin with a Type field that uniquely identifies the message in question, while its payload varies according to the message type. As for transmission, management messages can only be transmitted in Initial Ranging, Basic, Primary, Multicast Management, or Broadcast type of FIDs (see Table 279, Table 280, and Table 281). No other types of FIDs shall carry management messages.

| Type (8-bit) | Payload (variable) |
| --- | --- |

Figure 15 — General management message structure

Each of the management messages shown in Table 19 are described in the following subclauses.

Table 19a — Management messages

| Type | Message | Description | Reference | Class of connection |
| --- | --- | --- | --- | --- |
| 0 | DCD | Downstream Channel Descriptor | 7.7 7.7.1 | Broadcast |
| 1 | DS-MAP | Downstream Access Definition | 7.7.2 | Broadcast |
| 2 | UCD | Upstream Channel Descriptor | 7.7.3 | Broadcast |
| 3 | US-MAP | Upstream Access Definition | 27.7.4 | Broadcast |
| 4 | RNG-REQ | Ranging Request | 7.7.5 | Initial Ranging or Basic |
| 5 | RNG-CMD | Ranging Command | 7.7.6 | Initial Ranging or Basic |
| 6 | REG-REQ | Registration Request | 7.7.7.1 | Primary Management |
| 7 | REG-RSP | Registration Response | 7.7.7.2 | Primary Management |
| 8 | DSA-REQ | Dynamic Service Addition Request | 7.7.8.1 | Primary Management |
| 9 | DSA-RSP | Dynamic Service Addition Response | 7.7.8.2 | Primary Management |
| 10 | DSA-ACK | Dynamic Service Addition Acknowledge | 7.7.8.3 | Primary Management |
| 11 | DSC-REQ | Dynamic Service Change Request | 7.7.8.4 | Primary Management |
| 12 | DSC-RSP | Dynamic Service Change Response | 7.7.8.5 | Primary Management |
| 13 | DSC-ACK | Dynamic Service Change Acknowledge | 7.7.8.6 | Primary Management |
| 14 | DSD-REQ | Dynamic Service Deletion Request | 7.7.8.7 | Primary Management |
| 15 | DSD-RSP | Dynamic Service Deletion Response | 7.7.8.8 | Primary Management |
| 16 | DSX-RVD | Dynamic Service Request acknowledgement before authentication | 7.7.8.10 | Primary Management |
| 17 | MCA-REQ | Multicast Assignment Request | 7.7.9 | Primary Management |
| 18 | MCA-RSP | Multicast Assignment Response | 7.7.10 | Primary Management |
| 19 | CBC-REQ | CPE Basic Capability Request | 7.7.11.1 | Basic |
| 20 | CBC-RSP | CPE Basic Capability Response | 7.7.11.2 | Basic |
| 21 | DREG-CMD | De/Re-register Command | 7.7.12 | Basic Primary Management |
| 22 | DREG-REQ | CPE De-registration Request | 7.7.13 | Primary Management |
| 23 | ARQ-Feedback | Standalone ARQ Feedback | 7.7.14 | Primary Management |
| 24 | ARQ-Discard | ARQ Discard | 7.7.15 | Primary Management |
| 25 | ARQ-Reset | ARQ Reset | 7.7.16 | Primary Management |
| 26 | CHS-REQ | Channel Switch Request | 7.7.17.1 | Primary Management or Broadcast |
| 27 | CHS-RSP | Channel Switch Response | 7.7.17.2 | Primary Management |
| 28 | CHQ-REQ | Channel Quiet Request | 7.7.17.3 | Primary Management, Multicast Management or Broadcast |
| 29 | CHQ-RSP | Channel Quiet Response | 7.7.17.4 | Primary Management, Multicast Management or Broadcast |
| 30 | IPC-UPD | Incumbent Prohibited Channels Update | 7.7.17.4 | Primary Management, Multicast Management or Broadcast |
| 31 | BLM-REQ | Bulk Measurement Request | 7.7.18.1 | Primary Management, Multicast Management or Broadcast |
| 32 | BLM-RSP | Bulk Measurement Response | 7.7.18.2 | Primary Management |
| 33 | BLM-REP | Bulk Measurement Report | 7.7.18.3 | Primary Management |
| 34 | BLM-ACK | Bulk Measurement Acknowledgement | 7.7.18.4 | Primary Management |
| 35 | TFTP-CPLT | Config File TFTP Complete | 7.7.19 | Primary Management |
| 36 | TFTP-RSP | Config File TFTP Complete Response | 7.7.20 | Primary Management |
| 37 | SCM-REQ | Security Control Management Request | 7.7.21 | Primary Management |
| 38 | SCM-RSP | Security Control Management Response | 7.7.21 | Primary Management |
| 39 | FRM_UPD | The first active frame allocation update in self-coexistence mode | 7.7.22 | Basic |
| 40 | CBP -RLY | CBP Relay | 7.7.23 | Primary Management, Multicast Management |
| 41 | CAM-AIF | Add new operating channel | 7.7.X.1 | Primary Management |
| 42 | CAM-STP | Stop operating channel | 7.7.X.2 | Primary Management |
| 43 | CAM-STP-ACK | Stop operating channel acknowledgement | 7.7.X.3 | Primary Management |
| 44 | CAM-SWH | Switch operating channel | 7.7.X.4 | Primary Management |
| 45 | CAM-SWH-ACK | Switch operating channel acknowledgement | 7.7.X.5 | Primary Management |

Subclauses 7.7.1 to 7.7.24 are remains the same as described in the IEEE 802.22-2011 standard.

Subclause 7.7.X presents the proposed management messages for the multi channel operation managed by the Channel Allocation Manager (CAM).

7.7.X Channel Allocation Manager (CAM-AIF/STP/STP-ACK/SWH/SWH-ACK)

This clause describes the channel allocation manager management messages for the basic multi channel operations such as add new operating channel operation (CAM-AIF), stop operating channel (CAM-STP/STP-ACK)and switch operating channel (CAM-SWH/SWH-ACK).

7.7.X.1 Add new operating channel (CAM-AIF)

The format of the add new operating channel message is shown in Table 1xx. This message is used to configure add new operating channel procedure during the multichannel operation. The aggregation information is needed by the CPE-CAM in order to indetify the aggregation information transmitted from the BS-CAM. This message includes the number of maximum aggregation channel allowed and the channel aggregation information for CPE.

Table 1xx — CAM-AIF message format

| Syntax | Size | Notes |
| --- | --- | --- |
| CAM-AIF_Message_Format() { |  |  |
| Management Message Type = 41 | 8 bits |  |
| Aggregation Information | 1 bit | 0: Aggregation on 1: Aggregation off |
| Maximum Aggregation Channels | 3 bits | The number of maximum aggregation channels allowed in CPE. |
| For (i=0;i < Maximum Aggregation Channels;i++){ |  | List of the channel informations that are available for channel aggregation in CPE. |
| Channel Number [i] | 8 bits |  |
| } |  |  |
| } |  |  |

7.7.X.2 Stop operating channel (CAM-STP)

The format of the stop operating channel message is shown in Table 1xx. This message is used to configure stop operating channel procedure during the multichannel operation. This message is sent by BS-CHU to the CPE-CHU in order to stop the operating channel in CPE-CHU. Transmission of this message may result from various conditions such as protection of incumbent services (BS incumbent sensing report, CPE incumbent sensing report), channel availability in database and BS channel scheduling.

Table 1xx — CAM-STP message format

| Syntax | Size | Notes |
| --- | --- | --- |
| CAM-STP_Message_Format() { |  |  |
| Management Message Type = 42 | 8 bits |  |
| Transaction ID | 16 bits |  |
| Confirmation Needed | 1 bit | 0: No confirmation needed 1: Confirmation needed |
| Stop Channel Number | 8 bits | Specified destination for channel stop operation request. |
| } |  |  |

7.7.X.3 Stop operating channel acknowledgement (CAM-STP-ACK)

The format of the stop operating channel acknowledgement message is shown in Table 1xx. This message shall be sent by CPE-CHU to the BS-CHU in response to a received CAM-STP. This message serves to confirm to the BS-CHU the reception of the CAM-STP message by the CPE-CHU.

Table 1xx — CAM-STP-ACK message format

| Syntax | Size | Notes |
| --- | --- | --- |
| CAM-STP-ACK_Message_Format() { |  |  |
| Management Message Type = 43 | 8 bits |  |
| Transaction ID | 16 bits |  |
| Confirmation Code | 8 bits | 7.7.24 |
| } |  |  |

7.7.X.4 Switch operating channel (CAM-SWH)

The format of the switch operating channel message is shown in Table 1xx. This message is used to configure switch operating channel procedure during the multichannel operation. This message is sent by BS-CHU to the CPE-CHU in order to switch the operating channel in CPE-CHU. Transmission of this message may result from various conditions such as protection of incumbent services (BS incumbent sensing report, CPE incumbent sensing report), channel availability in database and BS channel scheduling.

Table 1xx — CAM-SWH message format

| Syntax | Size | Notes |
| --- | --- | --- |
| CAM-SWH_Message_Format() { |  |  |
| Management Message Type = 44 | 8 bits |  |
| Transaction ID | 16 bits |  |
| Confirmation Needed | 1 bit | 0: No confirmation needed 1: Confirmation needed |
| Switch Mode | 1 bit | 0: no restriction on transmission until the scheduled channel switch 1: addressed CPE shall transmit no further frames until the schedules channel switch. |
| Switch Count | 8 bits | The number of frames until the BS sending the switching operating channel message switches to the new operating channel. |
| Switch Channel Number | 8 bits | Specified destination for channel switch request. |
| } |  |  |

7.7.X.5 Switch operating channel acknowledgement (CAM-SWH-ACK)

The format of the switch operating channel acknowledgement message is shown in Table 1xx. This message shall be sent by CPE-CHU to the BS-CHU in response to a received CAM-SWH. This message serves to confirm to the BS-CHU the reception of the CAM-SWH message by the CPE-CHU.

Table 1xx — CAM-SWH-ACK message format

| Syntax | Size | Notes |
| --- | --- | --- |
| CAM-SWH-ACK_Message_Format() { |  |  |
| Management Message Type = 45 | 8 bits |  |
| Transaction ID | 16 bits |  |
| Confirmation Code | 8 bits | 7.7.24 |
| } |  |  |