IEEE P802.22
Wireless RANs

| Proposed Text of MAC technical items related to 7.6.X MAC headers (Multi-channel operation mode) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2013-09-18 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Keat Beng Toh | Hitachi Kokusai Electric Inc. | 32,Miyuki-cho,Kodaira-shi, Tokyo, 187-8511, Japan | +81-42-322-3111 | toh.keatbeng@h-kokusai.com |
| Keigo Hasegawa | Hitachi Kokusai Electric Inc. | 32,Miyuki-cho,Kodaira-shi, Tokyo, 187-8511, Japan | +81-42-322-3111 | hasegawa.keigo@h-kokusai.com |
| Masayuki Takekawa | Hitachi Kokusai Electric Inc. | 32,Miyuki-cho,Kodaira-shi, Tokyo, 187-8511, Japan | +81-42-322-3111 | takekawa.masayuki@h-kokusai.com |
| Kei Yanagisawa | Hitachi Kokusai Electric Inc. | 32,Miyuki-cho,Kodaira-shi, Tokyo, 187-8511, Japan | +81-42-322-3111 | yanagisawa.kei@h-kokusai.com |
| Seishi Sasaki | Hitachi Kokusai Electric Inc. | 32,Miyuki-cho,Kodaira-shi, Tokyo, 187-8511, Japan | +81-42-322-3111 | sasaki.seishi@h-kokusai.com |
| Masahiro Asano | Hitachi Kokusai Electric Inc. | 32,Miyuki-cho,Kodaira-shi, Tokyo, 187-8511, Japan | +81-42-322-3111 | asano.masahiro@h-kokusai.com |

Abstract

This document contains the proposed text of MAC technical items related to MAC headers (Multi channel operation mode).

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair Apurva N. Mody <apurva.mody@ieee.org> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Summary of this document

In this document, MAC headers (Multi channel operation mode) for IEEE 802.22b is proposed to support multi-channel operation which provide higher data throughput than the IEEE Std. 802.22-2011.The subclause structure in the proposed text is shown as follows.

7.6 MAC PDU Formats

7.6.X MAC headers (Multi-channel operation mode)

7.6.X.1 Generic MAC header

7.6.X.2 MAC subheaders and special payloads

7.6.X.3 CBP MAC PDU format

7.6.X.4 Aggregation header

In this proposal, proposed texts in subclause 7.6 are described as revision texts and proposed texts in subclause 7.6.X are described as new texts (without revision marks based on the 802.22 standard).

  - MAC PDU Formats

The MAC PDU is illustrated in Figure14. Figure12 depicts how the MAC PDU fits in the overall frame structure when used for intra-cell communication. Each PDU begins with a fixed-length generic MAC header, which shall be followed by a Payload. The Payload shall consist of zero or more subheaders, zero or more IEs, and zero or more MAC SDUs and/or fragments thereof. The payload information may vary in length, so that a MAC PDU may represent a variable number of bytes. A CRC shall be applied to all the MAC PDUs transmitted on the broadcast, Initial Ranging, Basic and Multicast transport FIDs.  This is because the MAC PDUs transmitted on these FIDs do not have any other protection.  For all other FIDs, a MAC PDU may or may not carry a CRC since MAC PDUs belonging to all these FIDs contain an integrity check vector (see 8.4).  Such PDU will have to fit in a given integer number of OFDM slots once encoded and this will result in specific data bit lengths possible as indicated in Table 227. Bit padding is used to make each PDU fit these bit lengths as explained in 7.8.6.

- Figure 14 — MAC PDU format

The MAC PDU format for multi channel operation is illustrated in Figure 1X. Figure12 depicts how the MAC PDU fits in the overall frame structure when used for intra-cell communication. Each PDU begins with a fixed-length generic MAC header and an Aggregation header, which shall be followed by a Payload. The Payload shall consist of zero or more subheaders, zero or more IEs, and zero or more MAC SDUs and/or fragments thereof. The payload information may vary in length, so that a MAC PDU may represent a variable number of bytes. A CRC shall be applied to all the MAC PDUs transmitted on the broadcast, Initial Ranging, Basic and Multicast transport FIDs.  This is because the MAC PDUs transmitted on these FIDs do not have any other protection.  For all other FIDs, a MAC PDU may or may not carry a CRC since MAC PDUs belonging to all these FIDs contain an integrity check vector (see 8.4).  Such PDU will have to fit in a given integer number of OFDM slots once encoded and this will result in specific data bit lengths possible as indicated in Table 227. Bit padding is used to make each PDU fit these bit lengths as explained in 7.8.6.

| MAC Header | Aggregation Header | MAC Payload | CRC |
| --- | --- | --- | --- |

- Figure 1X — MAC PDU format (Multi channel operation mode)

- 7.6.X MAC Headers (Multi channel operation mode)

Three MAC headers are defined: the generic MAC header used for intra-cell communication (between BS and CPEs within a cell), the coexistence beacon MAC header used by the CBP protocol for inter-cell communication to foster appropriate self-coexistence amongst overlapping IEEE 802.22 cells, provide a way to identify the transmitting device for potential interference resolution, and intra-cell communication between the CPEs in the same cell to contribute to terrestrial geolocation, and the Aggregation header which is necessary for commencing the multi channel operation. The detail format of the Aggregation header  is described in 7.6.X.4.

In addition to these headers, there is also the possibility of including subheaders and special payloads associated to the generic MAC header. In the following subclauses the MAC headers, together with the subheaders and special payloads are described.

Subclauses inside the 7.6.X.1 to 7.6.X.3 are remains the same as described in subclauses 7.6.1.1 to 7.6.1.3 of the IEEE 802.22-2011 standard.

- 7.6.X.4 Aggregation header

The format of Aggregation header is shown in Table 1X. This Aggregation header  is used to manage the aggregation data sequence and aggregation type during the multi-channel operation. The Aggregation header with fixed-length size of 3 bytes shall be added to each PDU after the generic MAC header as shown in Figure 1X.

- Table 1X— Aggregation header format

| Syntax | Size | Notes |
| --- | --- | --- |
| Aggregation_Header_Format() { |  |  |
| Aggregation ID | 16 bits | Indicates the sequence management ID of the transmitted data during multi-channel operation. The value of Aggregation ID is from 0 to 8191. The Aggregation ID shall be incremented by one after each transmission and shall be reset to 0 after the maximum value (8191). |
| Aggregation Type | 8 bits | This field specifies the aggregation type of the transmission. 0x00: No aggregation. 0x01: Diversity mode. 0x02: Bulk transmission mode. 0x03-0xFF: Reserved. |
| } |  |  |