IEEE P802.22
Wireless RANs

| Proposed Text of Cognitive Radio Capability technical items related to 10.7.2 BS configuration and monitoring primitives |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2013-11-13 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Keat Beng Toh | Hitachi Kokusai Electric Inc. | 32,Miyuki-cho,Kodaira-shi, Tokyo, 187-8511, Japan | +81-42-322-3111 | toh.keatbeng@h-kokusai.com |
| Keigo Hasegawa | Hitachi Kokusai Electric Inc. | 32,Miyuki-cho,Kodaira-shi, Tokyo, 187-8511, Japan | +81-42-322-3111 | hasegawa.keigo@h-kokusai.com |
| Masayuki Takekawa | Hitachi Kokusai Electric Inc. | 32,Miyuki-cho,Kodaira-shi, Tokyo, 187-8511, Japan | +81-42-322-3111 | takekawa.masayuki@h-kokusai.com |
| Kei Yanagisawa | Hitachi Kokusai Electric Inc. | 32,Miyuki-cho,Kodaira-shi, Tokyo, 187-8511, Japan | +81-42-322-3111 | yanagisawa.kei@h-kokusai.com |
| Seishi Sasaki | Hitachi Kokusai Electric Inc. | 32,Miyuki-cho,Kodaira-shi, Tokyo, 187-8511, Japan | +81-42-322-3111 | sasaki.seishi@h-kokusai.com |
| Masahiro Asano | Hitachi Kokusai Electric Inc. | 32,Miyuki-cho,Kodaira-shi, Tokyo, 187-8511, Japan | +81-42-322-3111 | asano.masahiro@h-kokusai.com |

Abstract

This document contains the proposed text of Cognitive Radio Capability technical items related to BS configuration and monitoring primitives.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair Apurva N. Mody <apurva.mody@ieee.org> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Summary of this document

In this document, BS configuration and monitoring primitives for IEEE 802.22b is proposed to support multi-channel operation which provide higher data throughput than the IEEE Std. 802.22-2011.The subclause structure in the proposed text is shown as follows.

10.7.2 BS configuration and monitoring primitives

10.7.2.1 M-AVAIL-TV-CH-REPORT.request

10.7.2.2 M-AVAIL-TV-CH-REPORT.confirm

10.7.2.3 M-DISALLOWED-TV-CHS.indication

10.7.2.4 M-OPERATING-TV-CH.indication

10.7.2.5 M-OPERATING-TV-CHS.indication

In this proposal, all the texts are described as revision texts to the 802.22 standard.

10.7.2	BS Configuration and Monitoring Primitives

The BS SM occasionally sends the available channel list to its higher layers for additional channel classification.  The available channel list can be presented to its higher layers to have channels classified as disallowed.  The classification of an operating channel by the BS is also performed by its higher layers.  The M-SAP is an interface that provides a means of exchange information between the SM and the higher layers in the BS.  Table 252 summarizes the primitives supported by the SM to pass the available channel list and to receive disallowed channel classifications and the selected operating channel/channels through the M-SAP interface. The primitives are discussed in the subclauses referenced in the table.

- — Available Channel List Primitives supported by the M-SAP

| Name | Request | Indication | Confirm |
| --- | --- | --- | --- |
| M-AVAIL-TV-CH-REPORT | 10.7.2.1 |  | 10.7.2.2 |
| M-DISALLOWED-TV-CHS |  | 10.7.2.3 |  |
| M-OPERATING-TV-CH |  | 110.7.2.4 |  |
| M-OPERATING-TV-CHS |  | 110.7.2.5 |  |

      - M-AVAIL-TV-CH-REPORT.request

The M-AVAIL-TV-CH-REPORT.request primitive is sent by the BS SM to request the higher layers for a selection of an operating channel based on the available channel list information provided this primitive. Table 253 specifies the parameters for the M-AVAIL-TV-CH-REPORT.request primitive.

- — M-AVAIL-TV-CH-REPORT.request parameters

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| For (i=1; i≤ Number of Channels Avaliable; i++) { Channel_Number Maximum Allowed EIRP } | List of available channels and their Maximum Allowed EIRP |  | List of available channels and corresponding Maximum Allowed EIRP. |
| Mode |  |  | The expected response from the higher layers 0 = Test 1 = Request for disallowed channel classification 2 = Request for selection of operating channel 3 = Request for selection of operating channels in multi-channel operation mode |

        - When generated

The M-AVAIL-TV-CH-REPORT.request primitive is generated by the BS SM and issued to the higher layers either (depending on the mode) to request disallowed channel classification or selection of an operating channel or selection of operating channels in multi-channel operation mode during BS initialization as described in 7.14.1.

        - Effect on receipt

When the higher layers receive the M-AVAIL-TV-CH-REPORT.request primitive, they generate an M-AVAIL-TV-CH-REPORT.confirm primitive to notify the SM if the request was successfully received.

      - M-AVAIL-TV-CH-REPORT.confirm

The M-WRAN-SERVICE-REPORT.confirm primitive allows the higher layers to inform the SM if the receipt of the available channel list was successful. Table 254 specifies the parameters for the M-AVAIL-TV-CH-REPORT.confirm primitive.

- — M-WRAN-SERVICE-REPORT.confirm parameters

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Status | Enumeration | SUCCESS, INVALID_REQUEST | The value indicates whether the request to select a WRAN service was successfully generated. |

        - When generated

The M-AVAIL-TV-CH-REPORT.confirm primitive is generated by the higher layers and issued to its MIB when an M-AVAIL-TV-CH-REPORT.request primitive is received to indicate whether the available channel list was successfully generated.

        - Effect on receipt

When the SM of a BS receives the M-AVAIL-TV-CH-REPORT.confirm primitive and depending on the mode, it expects the higher layers either to return nothing, an M-DISALLOWED-TV-CHS.indication primitive with classified disallowed channels, or an M-OPERATING-TV-CH.indication with the selected channel or an M-OPERATING-TV-CHS.indication with the selected channels.

      - M-DISALLOWED-TV-CHS.indication

The M-DISALLOWED-TV-CHS.indication primitive is used by the higher layers to return the disallowed channels on the available channel list to the SM per its request. Table 255 specifies the parameters for the M-DISALLOWED-TV-CHS.indication primitive.

- — M-DISALLOWED-TV-CHS.indication parameters

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| For (i=1; i≤ Number of Channels Disallowed; i++) { Channel_Number } | List of disallowed channels |  | List of disallowed channels. |

        - When generated

The M-DISALLOWED-TV-CHS.indication primitive is generated by the higher layers and issued to the MIB to indicate the disallowed channels from the available channel list.

        - Effect on receipt

When the SM receives the M-DISALLOWED-TV-CHS.indication it will identify whether the response to its request for the higher layers to classify channels as disallowed from the available channel list was successfully received by the higher layers, in which case, the SM will obtain the classified disallowed channels and the BS will continue to the following steps of initialization and perform detection, described in 7.14.1.6. If the response is not successful the SM may decide to issue another request.

      - M-OPERATING-TV-CH.indication

The M-OPERATING-TV-CH.indication primitive is used by the higher layers to return the selected operating channel on the available channel list to the SM per its request. Table 256 specifies the parameters for the M-OPERATING-TV-CH.indication primitive.

- — M-OPERATING-TV-CH.indication parameters

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Channel_Number | The selected operating channel | 0-80 | The selected operating channel |

        - When generated

The M-OPERATING-TV-CH.indication primitive is generated by the higher layers and issued to the MIB to indicate the selected operating channel from the available channel list.

        - Effect on receipt

When the SM receives the M-OPERATING-TV-CH.indication it will identify whether the response to its request for the higher layers to select the operating channel from the available channel list was successfully received by the higher layers, in which case, the SM will obtain the selected operating channel and the BS will continue to commence operation on the selected channel. If the response is not successful the SM may decide to issue another request.

      - M-OPERATING-TV-CHS.indication

The M-OPERATING-TV-CHS.indication primitive is used by the higher layers to return the selected operating channels in multi-channel operation mode on the available channel list to the SM per its request. Table 25X specifies the parameters for the M-OPERATING-TV-CHS.indication primitive.

- Table 25X — M-OPERATING-TV-CHS.indication parameters

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| For (i=1; i≤ Number of ChannelsinMulti-channel Operation; i++) { Channel_Number } | The selected operating channels in multi-channel operation mode |  | The selected operating channels in multi-channel operation mode |

        - When generated

The M-OPERATING-TV-CHS.indication primitive is generated by the higher layers and issued to the MIB to indicate the selected operating channels in multi-channel operation mode from the available channel list.

        - Effect on receipt

When the SM receives the M-OPERATING-TV-CHS.indication it will identify whether the response to its request for the higher layers to select the operating channels in multi-channel operation mode from the available channel list was successfully received by the higher layers, in which case, the SM will obtain the selected operating channels and the BS will continue to commence multi-channel operation on the selected channels. If the response is not successful the SM may decide to issue another request.