IEEE P802.22
Wireless RANs

| Proposed Text of Cognitive Radio Capability technical items related to 10.7.3 CPE reports the resulting available WRAN service list |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2013-11-13 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Keat Beng Toh | Hitachi Kokusai Electric Inc. | 32,Miyuki-cho,Kodaira-shi, Tokyo, 187-8511, Japan | +81-42-322-3111 | toh.keatbeng@h-kokusai.com |
| Keigo Hasegawa | Hitachi Kokusai Electric Inc. | 32,Miyuki-cho,Kodaira-shi, Tokyo, 187-8511, Japan | +81-42-322-3111 | hasegawa.keigo@h-kokusai.com |
| Masayuki Takekawa | Hitachi Kokusai Electric Inc. | 32,Miyuki-cho,Kodaira-shi, Tokyo, 187-8511, Japan | +81-42-322-3111 | takekawa.masayuki@h-kokusai.com |
| Kei Yanagisawa | Hitachi Kokusai Electric Inc. | 32,Miyuki-cho,Kodaira-shi, Tokyo, 187-8511, Japan | +81-42-322-3111 | yanagisawa.kei@h-kokusai.com |
| Seishi Sasaki | Hitachi Kokusai Electric Inc. | 32,Miyuki-cho,Kodaira-shi, Tokyo, 187-8511, Japan | +81-42-322-3111 | sasaki.seishi@h-kokusai.com |
| Masahiro Asano | Hitachi Kokusai Electric Inc. | 32,Miyuki-cho,Kodaira-shi, Tokyo, 187-8511, Japan | +81-42-322-3111 | asano.masahiro@h-kokusai.com |

Abstract

This document contains the proposed text of Cognitive Radio Capability technical items related to CPE reports the resulting available WRAN service list.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair Apurva N. Mody <apurva.mody@ieee.org> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Summary of this document

In this document, CPE reports the resulting available WRAN service list for IEEE 802.22b is proposed to support multi-channel operation which provide higher data throughput than the IEEE Std. 802.22-2011.The subclause structure in the proposed text is shown as follows.

10.7.3 CPE reports the resulting available WRAN service list

10.7.3.1 M-WRAN-SERVICE-REPORT.request

10.7.3.2 M-WRAN-SERVICE-REPORT.confirm

10.7.3.3 M-WRAN-SERVICE-RESPONSE.indication

10.7.3.4 M-WRAN-SERVICES-RESPONSE.indication

In this proposal, all the texts are described as revision texts to the 802.22 standard.

10.7.3 	CPE Reports the Resulting Available WRAN Services List

The selection of WRAN service or WRAN services by the CPE is performed by its higher layers.  The M-SAP is an interface that provides a means of exchange information between the SA and the higher layers. Table 257 summarizes the primitives supported by the SM to pass the available WRAN services list and the selected WRAN service or WRAN services through the M-SAP interface. The primitives are discussed in the subclauses referenced in the table.

- — Available WRAN Services List Primitives supported by the M-SAP

| Name | Request | Indication | Confirm |
| --- | --- | --- | --- |
| M-WRAN-SERVICE-REPORT | 10.7.3.1 |  | 10.7.3.2 |
| M-WRAN-SERVICE-RESPONSE |  | 10.7.3.3 |  |
| M-WRAN-SERVICES-RESPONSE |  | 10.7.3.4 |  |

Subclauses 10.7.3.1 to 10.7.3.3 are remains the same as described in the IEEE 802.22-2011 standard.

- 10.7.3.4 	M-WRAN-SERVICES-RESPONSE.indication

The M-WRAN-SERVICES-RESPONSE.indication primitive is used by the higher layers to return selected WRAN channels from the available WRAN service list to the SA per its request. Table 26X specifies the parameters for the M-WRAN-SERVICES-RESPONSE.indication primitive.

- Table 26X — M-WRAN-SERVICES-RESPONSE.indication parameters

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| For (i=1; i≤ Number of ChannelsinMulti-channel Operation; i++) { Channel_Number } | The selected operating channels in multi-channel operation mode |  | The selected operating channels in multi-channel operation mode |

        - When generated

The M-WRAN-SERVICES-RESPONSE.indication primitive is generated by the higher layers and issued to the MIB to indicate the selected channels from the available WRAN service list.

        - Effect on receipt

When the SA receives the M-WRAN-SERVICES-RESPONSE.indication it will identify whether the response to its request for the higher layers to select channels from the available WRAN service list was successfully received by the higher layers, in which case, the SA will obtain the selected channels and CPE will continue to the following steps of initialization. If the response is not successful the SA may decide to issue another query.