IEEE P802.22
Wireless RANs

| Comment Resolution for Group Management (CID #223) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2014-01-16 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Sung-Hyun Hwang | ETRI | Korea | +82-42-860-1133 | shwang@etri.re.kr |
| Gwang-Zeen Ko | ETRI | Korea | +82-42-860-4862 | gogogo@etri.re.kr |
| Byung-Jang Jeong | ETRI | Korea | +82-42-860-6765 | bjjeong@etri.re.kr |
| Myung-Sun Song | ETRI | Korea | +82-42-860-5046 | mssong@etri.re.kr |
| Hyung-Do Choi | ETRI | Korea | +82-42-860-5814 | choihd@etri.re.kr |

Abstract

This is the comment resolution for CID #223. It is related to the Group Management of 802.22b systems.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Apurva N. Mody> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

- 1. Related Comment

- This is the comment resolution for CID #223 as described in the following table. To resolve the CID #223, I checked the related “Section 7.17 Multicast support” of IEEE Std. 802.22-2011.

- 2. Proposed Resolution

- Though the concept of multicast is similar with group resoure allocation (GRA), there are two critical differences. First, the multicast service has single (same) flow ID within group, but group resource allocation has different flow ID within group. Using group resource allocation, each station within group transports different traffic, and the overhead for resource allocation can be signifcantly reduced using bitmap format. Second, the multicast can only provide downstream service, but group resource allocation should provide both downstream and upstream service. In conclusion, group resource allocation has different purpose and function, compared to the multicast. Thus it would be better to define multicast and GRA separately.

- In addition, there was the discussion regarding the size of GID and the data type of group location. First, regarding the size of group ID, the group is composed of one H-CPE and many L-CPEs. Thus it is reasonable that the size of GID should be at least 3 bits less than the size of SID. It roughly means that a group has 8 CPEs (1 H-CPE and 7 L-CPEs). Second, regarding the data type of group location, to minimize the burden for carrying the ’48 bits of Group Location’, the group location could be approximated with the H-CPE location. We can extend to ‘area concept for group location’ after checking the signalling overhead.

- Based on the discussion above, I suggest the proposed remedy as follows:

- The following text is the email exchanges with commenter, Ranga Reddy, to clear the CID #223.

- 1) In the GRA-CFG msg, couldn’t the GID just be 9 bits, and be equal to the Station ID of the H-CPE of the group?

- [SH] The size of station ID is changed from 9 bits to 13 bits in Draft 1.0. Do you think that 12 bits of GID (4096 groups per BS) is too much?

- [RKR] We were discussing possibly shrinking the station ID to be inline with the base standard (the station ID as may be small), or keeping it larger, but we haven’t settled, or I haven’t seen the comment resolution on it yet. However, the size of the ID value used to identify the group is not what I have issue with. I would make the first SID stored in the SID Bitmap field in Table AP1 correspond with the SID of the H-CPE of the group. As far as I understand a group has a single H-CPE, which is uniquely identified by that CPE’s SID.

- [SH] I agree your opinion. In my thought, it is reasonable that the size of GID should be at least 3 bits less than the size of SID. It roughly means that a group has 8 CPEs (1 H-CPE and 7 L-CPEs). I will suggest modifying the related text of 802.22b draft in line with the discussion above.

- 2) What explicitly is the purpose (in the GRA-CFG) message, of identifying the location of the group? I assume you would send a GRA-CFG message to CPEs that are near each other and being assigned to the same group, and not to a CPE that is no located no where near the others being assigned to a group. One could also, if you consider the (1) issue above, locate the group by the (known to the MRBS) location of the H-CPE of the group.

- [SH] As described in Section 7.7.30.1, the location of group is represented by the location of H-CPE. The main purpose of group location is to manage the group backup channel efficiently, i.e., the group backup channel can be allocated considering the location and mobility of group, so that the BS and the CPEs can reduce the signaling overhead and can prevent QoS degradation by avoding the frequency channel switching (as described in Section 7.25).

- [RKR] Oh, I understand this better now. I would like to make a suggestion, in that I would consider two location points that bound the area (the lat/lon of the NW corner and SE corner). If you look that the database primitives in TGa draft the location for a channel request could be for a single point or a set of points defining a polygon. If you configure the H-CPE with the “area” it’s group covers it can optimize its query.

- [SH] I checked the related primitive, “Location Data Type”, in Table 269 of the 802.22a draft. It is good information. To minimize the burden for carrying the ’48 bits of Group Location’, I suggested that the group location could be approximated with the H-CPE location. In future, I will also extend to ‘area concept’ after checking the signalling overhead.

- 3) If you decide you still need to include a group location in the GRA-CFG message, you need to explicitly define how those 48 bits convey a location.

- [SH] The group location is the latitude and longitude of the H-CPE. Because the GRA-CFG message is only used when the BS create a new group and configure (setup) the group characteristics, I think it is not big burden to 802.22b network.

- [RKR] See my response to your feedback on questions 2.