IEEE P802.22
Wireless RANs

| Proposed Changes for Group Location Update |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2014-01-16 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Sung-Hyun Hwang | ETRI | Korea | +82-42-860-1133 | shwang@etri.re.kr |
| Gwang-Zeen Ko | ETRI | Korea | +82-42-860-4862 | gogogo@etri.re.kr |
| Byung-Jang Jeong | ETRI | Korea | +82-42-860-6765 | bjjeong@etri.re.kr |
| Myung-Sun Song | ETRI | Korea | +82-42-860-5046 | mssong@etri.re.kr |
| Hyung-Do Choi | ETRI | Korea | +82-42-860-5814 | choihd@etri.re.kr |

Abstract

This is the proposed changes to update the group location of group resource allocation. The backup and candidate channels are selected using the mobility and the location of H-CPE. If the location of H-CPE is changed, the location should be updated through GRA-UPD message.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Apurva N. Mody> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

- 1. Background

- The backup and candidate channels of group are selected using the mobility and the location of H-CPE. If the location of H-CPE within group is changed, the location should be updated through GRA-UPD message. However the current GRA-UPD message doesn’t include the group location information.

- 2. Proposed Changes

- 7.7.30.2 Group Resource Allocation Update (GRA-UPD)

- The format of Group Resource Allocation Update message is shown in Table AQ1. This message is used to update the group resource allocation configuration. The device can be added to or deleted from a group. This message is also used to update the location of group.

- Table AQ1 — GRA-UPD message format

| Syntax | Size | Notes |
| --- | --- | --- |
| GRA-UPD_Message_Format() { |  |  |
| Management Message Type = 56 | 8 bits |  |
| Device Flag { | 1 bit | Flag to indicate whether the device information is changed 0: Device information is not changed 1: Device information is changed |
| Deletion Flag | 1 bit | Flag to indicate whether the device is added to or deleted from a group. 0: Added to a group 1: Deleted from a group |
| SID | 9 bits | Station ID that is added to or deleted from a group. |
| GID | 12 bits | Group ID to which the device is added to or deleted from a group. |
| If(Deletion Flag = 0) { |  |  |
| Device Bitmap Index | 7 bits | Indicates the new index of the device in a group’s device bitmap. |
| } |  |  |
| } |  |  |
| Group Flag { | 1 bit | Flag to indicate whether the group information is changed 0: Group information is not changed 1: Group information is changed |
| Group Location | 48 bits | New latitude and longitude of a group |
| } |  |  |
| } |  |  |

- 7.25 Group Resource Allocation

- A large number of CPEs (at least 2048 CPEs) may be connected to the BS. Among them, some CPEs have similar traffic pattern, such as, payload size, traffic period, and data rate (PHY mode), etc. It is a burden of MAP overhead to allocate the resources to all CPEs individually. The Group Resource Allocation (GRA) is very efficient for a group of CPEs communicating using a same PHY mode and with a fixed payload size. The MAP overhead is significantly reduced by allocating the resources to the Group using bitmap format.

- The group is composed of one H-CPE and many L-CPEs. Within the group, H-CPE is a controller of a group consisting of many L-CPEs. The H-CPE capabilities include access to the database services, identification of the group, network entry with BS, etc. All the L-CPEs within the group are synchronized to the H-CPE.

- There are two types of group: fixed group and mobile (or portable) group. The type of group is determined according to the mobility of H-CPE. The H-CPE within fixed group is fixed on the building, house, tower, etc. The H-CPE within mobile (or portable) group is mobile (or portable) on the vehicles, etc.

- The BS configures the group resource allocation by using the Group Resource Allocation Configuration (GRA-CFG) message, as shown in Table AK1. The BS creates a new group, identifies the devices that are belonging to the group, and allocates the resources on a group basis. The GRA-CFG message includes the characteristics of group, such as Device Bitmap Size, Bitmap of Station ID, Group ID, Group Type, and Group Location. The group resource allocation configuration can be updated using the Group Resource Allocation Update (GRA-UPD) message, as shown in Table AL1. The BS uses GRA-UPD message to add a device to a group or delete a device from a group. The BS also uses GRA-UPD message to update the location of group. The DS/US-MAP message defines the access to the downstream/upstream resources, as shown in Table 25 and Table 34, respectively. The format of a DS/USMAP IEs is defined for the Individual Resource Allocation (IRA), as shown in Table 26 and Table 35, respectively. And the format of a DS/US-MAP GRA IEs is defined for the Group Resource Allocation (GRA), as shown in Table J1 and Table J1, respectively.

- …

- The other part of this section is not changed.