IEEE P802.22
Wireless RANs

| Data rates of IEEE 802.22b systems |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2014-03-18 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Sung-Hyun Hwang | ETRI | Korea | +82-42-860-1133 | shwang@etri.re.kr |
| Gwang-Zeen Ko | ETRI | Korea | +82-42-860-4862 | gogogo@etri.re.kr |
| Byung-Jang Jeong | ETRI | Korea | +82-42-860-6765 | bjjeong@etri.re.kr |
| Myung-Sun Song | ETRI | Korea | +82-42-860-5046 | mssong@etri.re.kr |
| Hyung-Do Choi | ETRI | Korea | +82-42-860-5814 | choihd@etri.re.kr |

Abstract

This document provides the data rate of 802.22b systems considering multiple channel and MIMO. The proposed tables also provide the data rates for all combination of cyclic prefix and channel bandwidth in the 802.22b systems.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Apurva N. Mody> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

- 1. Background

- The backgrounds of this contribution are summarized as follows:

- The 802.22b draft v1.0 only contains the data rate for single channel and SISO, not incluing the data rate for multiple channel and MIMO. Moreover, the table of data rate just provides partial information regarding 1/16 CP and 6MHz BW. The proposed tables provide the data rates for all combination of cyclic prefix and channel bandwidth.

- In the Table AU1, the spectral efficiency is not critical information to show the merit of 802.22b systems. Because the column of table is not enough, I suggest removing the spectral efficiencies in the Table AU1.

- The PHY mode for control signal transmission is already explained in the corresponding section. And because it is not paylod, it is not appropriate to include in the Table AU1.

- From the proposed table, the maximum data rate of 802.22b system is about 700Mbps with 4 channels and 4 spatial streams. It would be more advantageous to highlight the merits of 802.22b systems.

- 2. Proposed Tables

- Table AU1.1 – Data rate for 1 channel and 1 spatial stream

| - PHY mode | - Modulation | - Code - rate | Data rate (Mbits/s) |  |  |  | Data rate (Mbits/s) |  |  |  | Data rate (Mbits/s) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 6MHz BW |  |  |  | 7MHz BW |  |  |  | 8MHz BW |  |  |  |
|  |  |  | 1/4 CP | 1/8 CP | 1/16 CP | 1/32 CP | 1/4 CP | 1/8 CP | 1/16 CP | 1/32 CP | 1/4 CP | 1/8 CP | 1/16 CP | 1/32 CP |
| 0 | QPSK | 1/2 | 3.86 | 4.29 | 4.54 | 4.67 | 4.50 | 5.00 | 5.29 | 5.45 | 5.14 | 5.71 | 6.05 | 6.23 |
| 1 | QPSK | 2/3 | 5.14 | 5.71 | 6.05 | 6.23 | 6.00 | 6.67 | 7.06 | 7.27 | 6.85 | 7.61 | 8.06 | 8.31 |
| 2 | QPSK | 3/4 | 5.78 | 6.43 | 6.81 | 7.01 | 6.75 | 7.50 | 7.94 | 8.18 | 7.71 | 8.57 | 9.07 | 9.34 |
| 3 | QPSK | 5/6 | 6.43 | 7.14 | 7.56 | 7.79 | 7.50 | 8.33 | 8.82 | 9.09 | 8.57 | 9.52 | 10.08 | 10.38 |
| 4 | 16-QAM | 1/2 | 7.71 | 8.57 | 9.07 | 9.35 | 9.00 | 10.00 | 10.59 | 10.91 | 10.28 | 11.42 | 12.09 | 12.46 |
| 5 | 16-QAM | 2/3 | 10.28 | 11.43 | 12.10 | 12.47 | 12.00 | 13.33 | 14.12 | 14.55 | 13.70 | 15.23 | 16.12 | 16.61 |
| 6 | 16-QAM | 3/4 | 11.57 | 12.86 | 13.61 | 14.02 | 13.50 | 15.00 | 15.88 | 16.36 | 15.42 | 17.13 | 18.14 | 18.69 |
| 7 | 16-QAM | 5/6 | 12.86 | 14.28 | 15.12 | 15.58 | 15.00 | 16.67 | 17.65 | 18.18 | 17.13 | 19.03 | 20.15 | 20.76 |
| 8 | 64-QAM | 1/2 | 11.57 | 12.86 | 13.61 | 14.02 | 13.50 | 15.00 | 15.88 | 16.36 | 15.42 | 17.13 | 18.14 | 18.69 |
| 9 | 64-QAM | 2/3 | 15.43 | 17.14 | 18.15 | 18.70 | 18.00 | 20.00 | 21.18 | 21.82 | 20.56 | 22.84 | 24.18 | 24.92 |
| 10 | 64-QAM | 3/4 | 17.35 | 19.28 | 20.42 | 21.04 | 20.25 | 22.50 | 23.82 | 24.55 | 23.13 | 25.70 | 27.21 | 28.03 |
| 11 | 64-QAM | 5/6 | 19.28 | 21.43 | 22.69 | 23.37 | 22.50 | 25.00 | 26.47 | 27.27 | 25.70 | 28.55 | 30.23 | 31.15 |
| 12 | 256-QAM | 1/2 | 15.43 | 17.14 | 18.15 | 18.70 | 18.00 | 20.00 | 21.18 | 21.82 | 20.56 | 22.84 | 24.18 | 24.92 |
| 13 | 256-QAM | 2/3 | 20.57 | 22.85 | 24.20 | 24.93 | 24.00 | 26.67 | 28.24 | 29.09 | 27.41 | 30.45 | 32.24 | 33.22 |
| 14 | 256-QAM | 3/4 | 23.14 | 25.71 | 27.22 | 28.05 | 27.00 | 30.00 | 31.76 | 32.73 | 30.83 | 34.26 | 36.28 | 37.37 |
| 15 | 256-QAM | 5/6 | 25.71 | 28.57 | 30.25 | 31.16 | 30.00 | 33.33 | 35.29 | 36.36 | 34.26 | 38.07 | 40.31 | 41.53 |
| 16 | 256-QAM | 7/8 | 27.00 | 30.00 | 31.76 | 32.72 | 31.50 | 35.00 | 37.06 | 38.18 | 35.97 | 39.97 | 42.32 | 43.60 |
| 17 | 4D-48TCM | 10/11 | 19.58 | 21.76 | 23.04 | 23.73 | 22.85 | 25.39 | 26.88 | 27.69 | 26.09 | 28.99 | 30.70 | 31.63 |
| 18 | 4D-192TCM | 14/15 | 27.30 | 30.33 | 32.12 | 33.09 | 31.86 | 35.40 | 37.48 | 38.61 | 36.38 | 40.42 | 42.80 | 44.10 |

- Table AU1.2 – Data rate for 1 channel and 2 spatial streams

| - PHY mode | - Modulation | - Code - rate | Data rate (Mbits/s) |  |  |  | Data rate (Mbits/s) |  |  |  | Data rate (Mbits/s) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 6MHz BW |  |  |  | 7MHz BW |  |  |  | 8MHz BW |  |  |  |
|  |  |  | 1/4 CP | 1/8 CP | 1/16 CP | 1/32 CP | 1/4 CP | 1/8 CP | 1/16 CP | 1/32 CP | 1/4 CP | 1/8 CP | 1/16 CP | 1/32 CP |
| 0 | QPSK | 1/2 | 7.71 | 8.57 | 9.07 | 9.35 | 9.00 | 10.00 | 10.59 | 10.91 | 10.28 | 11.42 | 12.09 | 12.46 |
| 1 | QPSK | 2/3 | 10.28 | 11.43 | 12.10 | 12.47 | 12.00 | 13.33 | 14.12 | 14.55 | 13.70 | 15.23 | 16.12 | 16.61 |
| 2 | QPSK | 3/4 | 11.57 | 12.86 | 13.61 | 14.02 | 13.50 | 15.00 | 15.88 | 16.36 | 15.42 | 17.13 | 18.14 | 18.69 |
| 3 | QPSK | 5/6 | 12.86 | 14.28 | 15.12 | 15.58 | 15.00 | 16.67 | 17.65 | 18.18 | 17.13 | 19.03 | 20.15 | 20.76 |
| 4 | 16-QAM | 1/2 | 15.43 | 17.14 | 18.15 | 18.70 | 18.00 | 20.00 | 21.18 | 21.82 | 20.56 | 22.84 | 24.18 | 24.92 |
| 5 | 16-QAM | 2/3 | 20.57 | 22.85 | 24.20 | 24.93 | 24.00 | 26.67 | 28.24 | 29.09 | 27.41 | 30.45 | 32.24 | 33.22 |
| 6 | 16-QAM | 3/4 | 23.14 | 25.71 | 27.22 | 28.05 | 27.00 | 30.00 | 31.76 | 32.73 | 30.83 | 34.26 | 36.28 | 37.37 |
| 7 | 16-QAM | 5/6 | 25.71 | 28.57 | 30.25 | 31.16 | 30.00 | 33.33 | 35.29 | 36.36 | 34.26 | 38.07 | 40.31 | 41.53 |
| 8 | 64-QAM | 1/2 | 23.14 | 25.71 | 27.22 | 28.05 | 27.00 | 30.00 | 31.76 | 32.73 | 30.83 | 34.26 | 36.28 | 37.37 |
| 9 | 64-QAM | 2/3 | 30.85 | 34.28 | 36.30 | 37.40 | 36.00 | 40.00 | 42.35 | 43.64 | 41.11 | 45.68 | 48.37 | 49.83 |
| 10 | 64-QAM | 3/4 | 34.71 | 38.57 | 40.83 | 42.07 | 40.50 | 45.00 | 47.65 | 49.09 | 46.25 | 51.39 | 54.41 | 56.06 |
| 11 | 64-QAM | 5/6 | 38.57 | 42.85 | 45.37 | 46.75 | 45.00 | 50.00 | 52.94 | 54.55 | 51.39 | 57.10 | 60.46 | 62.29 |
| 12 | 256-QAM | 1/2 | 30.85 | 34.28 | 36.30 | 37.40 | 36.00 | 40.00 | 42.35 | 43.64 | 41.11 | 45.68 | 48.37 | 49.83 |
| 13 | 256-QAM | 2/3 | 41.14 | 45.71 | 48.40 | 49.86 | 48.00 | 53.33 | 56.47 | 58.18 | 54.82 | 60.91 | 64.49 | 66.44 |
| 14 | 256-QAM | 3/4 | 46.28 | 51.42 | 54.44 | 56.09 | 54.00 | 60.00 | 63.53 | 65.45 | 61.67 | 68.52 | 72.55 | 74.75 |
| 15 | 256-QAM | 5/6 | 51.42 | 57.13 | 60.49 | 62.33 | 60.00 | 66.67 | 70.59 | 72.73 | 68.52 | 76.13 | 80.61 | 83.05 |
| 16 | 256-QAM | 7/8 | 53.99 | 59.99 | 63.52 | 65.44 | 63.00 | 70.00 | 74.12 | 76.36 | 71.95 | 79.94 | 84.64 | 87.21 |
| 17 | 4D-48TCM | 10/11 | 39.16 | 43.51 | 46.07 | 47.47 | 45.70 | 50.77 | 53.76 | 55.39 | 52.18 | 57.98 | 61.39 | 63.25 |
| 18 | 4D-192TCM | 14/15 | 54.60 | 60.67 | 64.24 | 66.19 | 63.71 | 70.79 | 74.96 | 77.23 | 72.76 | 80.85 | 85.60 | 88.20 |

- Table AU1.3 – Data rate for 1 channel and 4 spatial streams

| - PHY mode | - Modulation | - Code - rate | Data rate (Mbits/s) |  |  |  | Data rate (Mbits/s) |  |  |  | Data rate (Mbits/s) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 6MHz BW |  |  |  | 7MHz BW |  |  |  | 8MHz BW |  |  |  |
|  |  |  | 1/4 CP | 1/8 CP | 1/16 CP | 1/32 CP | 1/4 CP | 1/8 CP | 1/16 CP | 1/32 CP | 1/4 CP | 1/8 CP | 1/16 CP | 1/32 CP |
| 0 | QPSK | 1/2 | 15.43 | 17.14 | 18.15 | 18.70 | 18.00 | 20.00 | 21.18 | 21.82 | 20.56 | 22.84 | 24.18 | 24.92 |
| 1 | QPSK | 2/3 | 20.57 | 22.85 | 24.20 | 24.93 | 24.00 | 26.67 | 28.24 | 29.09 | 27.41 | 30.45 | 32.24 | 33.22 |
| 2 | QPSK | 3/4 | 23.14 | 25.71 | 27.22 | 28.05 | 27.00 | 30.00 | 31.76 | 32.73 | 30.83 | 34.26 | 36.28 | 37.37 |
| 3 | QPSK | 5/6 | 25.71 | 28.57 | 30.25 | 31.16 | 30.00 | 33.33 | 35.29 | 36.36 | 34.26 | 38.07 | 40.31 | 41.53 |
| 4 | 16-QAM | 1/2 | 30.85 | 34.28 | 36.30 | 37.40 | 36.00 | 40.00 | 42.35 | 43.64 | 41.11 | 45.68 | 48.37 | 49.83 |
| 5 | 16-QAM | 2/3 | 41.14 | 45.71 | 48.40 | 49.86 | 48.00 | 53.33 | 56.47 | 58.18 | 54.82 | 60.91 | 64.49 | 66.44 |
| 6 | 16-QAM | 3/4 | 46.28 | 51.42 | 54.44 | 56.09 | 54.00 | 60.00 | 63.53 | 65.45 | 61.67 | 68.52 | 72.55 | 74.75 |
| 7 | 16-QAM | 5/6 | 51.42 | 57.13 | 60.49 | 62.33 | 60.00 | 66.67 | 70.59 | 72.73 | 68.52 | 76.13 | 80.61 | 83.05 |
| 8 | 64-QAM | 1/2 | 46.28 | 51.42 | 54.44 | 56.09 | 54.00 | 60.00 | 63.53 | 65.45 | 61.67 | 68.52 | 72.55 | 74.75 |
| 9 | 64-QAM | 2/3 | 61.70 | 68.56 | 72.59 | 74.79 | 72.00 | 80.00 | 84.71 | 87.27 | 82.22 | 91.36 | 96.73 | 99.67 |
| 10 | 64-QAM | 3/4 | 69.42 | 77.13 | 81.67 | 84.14 | 81.00 | 90.00 | 95.29 | 98.18 | 92.50 | 102.78 | 108.83 | 112.12 |
| 11 | 64-QAM | 5/6 | 77.13 | 85.70 | 90.74 | 93.49 | 90.00 | 100.00 | 105.88 | 109.09 | 102.78 | 114.20 | 120.92 | 124.58 |
| 12 | 256-QAM | 1/2 | 61.70 | 68.56 | 72.59 | 74.79 | 72.00 | 80.00 | 84.71 | 87.27 | 82.22 | 91.36 | 96.73 | 99.67 |
| 13 | 256-QAM | 2/3 | 82.27 | 91.41 | 96.79 | 99.72 | 96.00 | 106.67 | 112.94 | 116.36 | 109.63 | 121.81 | 128.98 | 132.89 |
| 14 | 256-QAM | 3/4 | 92.56 | 102.84 | 108.89 | 112.19 | 108.00 | 120.00 | 127.06 | 130.91 | 123.34 | 137.04 | 145.10 | 149.50 |
| 15 | 256-QAM | 5/6 | 102.84 | 114.27 | 120.99 | 124.65 | 120.00 | 133.33 | 141.18 | 145.45 | 137.04 | 152.27 | 161.22 | 166.11 |
| 16 | 256-QAM | 7/8 | 107.98 | 119.98 | 127.04 | 130.89 | 126.00 | 140.00 | 148.24 | 152.73 | 143.89 | 159.88 | 169.28 | 174.41 |
| 17 | 4D-48TCM | 10/11 | 78.32 | 87.02 | 92.14 | 94.94 | 91.39 | 101.54 | 107.52 | 110.78 | 104.37 | 115.96 | 122.79 | 126.51 |
| 18 | 4D-192TCM | 14/15 | 109.21 | 121.34 | 128.48 | 132.37 | 127.43 | 141.59 | 149.91 | 154.46 | 145.52 | 161.69 | 171.20 | 176.39 |

- Table AU1.4 – Data rate for 2 channels and 1 spatial stream

| - PHY mode | - Modulation | - Code - rate | Data rate (Mbits/s) |  |  |  | Data rate (Mbits/s) |  |  |  | Data rate (Mbits/s) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 6MHz BW |  |  |  | 7MHz BW |  |  |  | 8MHz BW |  |  |  |
|  |  |  | 1/4 CP | 1/8 CP | 1/16 CP | 1/32 CP | 1/4 CP | 1/8 CP | 1/16 CP | 1/32 CP | 1/4 CP | 1/8 CP | 1/16 CP | 1/32 CP |
| 0 | QPSK | 1/2 | 7.71 | 8.57 | 9.07 | 9.35 | 9.00 | 10.00 | 10.59 | 10.91 | 10.28 | 11.42 | 12.09 | 12.46 |
| 1 | QPSK | 2/3 | 10.28 | 11.43 | 12.10 | 12.47 | 12.00 | 13.33 | 14.12 | 14.55 | 13.70 | 15.23 | 16.12 | 16.61 |
| 2 | QPSK | 3/4 | 11.57 | 12.86 | 13.61 | 14.02 | 13.50 | 15.00 | 15.88 | 16.36 | 15.42 | 17.13 | 18.14 | 18.69 |
| 3 | QPSK | 5/6 | 12.86 | 14.28 | 15.12 | 15.58 | 15.00 | 16.67 | 17.65 | 18.18 | 17.13 | 19.03 | 20.15 | 20.76 |
| 4 | 16-QAM | 1/2 | 15.43 | 17.14 | 18.15 | 18.70 | 18.00 | 20.00 | 21.18 | 21.82 | 20.56 | 22.84 | 24.18 | 24.92 |
| 5 | 16-QAM | 2/3 | 20.57 | 22.85 | 24.20 | 24.93 | 24.00 | 26.67 | 28.24 | 29.09 | 27.41 | 30.45 | 32.24 | 33.22 |
| 6 | 16-QAM | 3/4 | 23.14 | 25.71 | 27.22 | 28.05 | 27.00 | 30.00 | 31.76 | 32.73 | 30.83 | 34.26 | 36.28 | 37.37 |
| 7 | 16-QAM | 5/6 | 25.71 | 28.57 | 30.25 | 31.16 | 30.00 | 33.33 | 35.29 | 36.36 | 34.26 | 38.07 | 40.31 | 41.53 |
| 8 | 64-QAM | 1/2 | 23.14 | 25.71 | 27.22 | 28.05 | 27.00 | 30.00 | 31.76 | 32.73 | 30.83 | 34.26 | 36.28 | 37.37 |
| 9 | 64-QAM | 2/3 | 30.85 | 34.28 | 36.30 | 37.40 | 36.00 | 40.00 | 42.35 | 43.64 | 41.11 | 45.68 | 48.37 | 49.83 |
| 10 | 64-QAM | 3/4 | 34.71 | 38.57 | 40.83 | 42.07 | 40.50 | 45.00 | 47.65 | 49.09 | 46.25 | 51.39 | 54.41 | 56.06 |
| 11 | 64-QAM | 5/6 | 38.57 | 42.85 | 45.37 | 46.75 | 45.00 | 50.00 | 52.94 | 54.55 | 51.39 | 57.10 | 60.46 | 62.29 |
| 12 | 256-QAM | 1/2 | 30.85 | 34.28 | 36.30 | 37.40 | 36.00 | 40.00 | 42.35 | 43.64 | 41.11 | 45.68 | 48.37 | 49.83 |
| 13 | 256-QAM | 2/3 | 41.14 | 45.71 | 48.40 | 49.86 | 48.00 | 53.33 | 56.47 | 58.18 | 54.82 | 60.91 | 64.49 | 66.44 |
| 14 | 256-QAM | 3/4 | 46.28 | 51.42 | 54.44 | 56.09 | 54.00 | 60.00 | 63.53 | 65.45 | 61.67 | 68.52 | 72.55 | 74.75 |
| 15 | 256-QAM | 5/6 | 51.42 | 57.13 | 60.49 | 62.33 | 60.00 | 66.67 | 70.59 | 72.73 | 68.52 | 76.13 | 80.61 | 83.05 |
| 16 | 256-QAM | 7/8 | 53.99 | 59.99 | 63.52 | 65.44 | 63.00 | 70.00 | 74.12 | 76.36 | 71.95 | 79.94 | 84.64 | 87.21 |
| 17 | 4D-48TCM | 10/11 | 39.16 | 43.51 | 46.07 | 47.47 | 45.70 | 50.77 | 53.76 | 55.39 | 52.18 | 57.98 | 61.39 | 63.25 |
| 18 | 4D-192TCM | 14/15 | 54.60 | 60.67 | 64.24 | 66.19 | 63.71 | 70.79 | 74.96 | 77.23 | 72.76 | 80.85 | 85.60 | 88.20 |

- Table AU1.5 – Data rate for 2 channels and 2 spatial streams

| - PHY mode | - Modulation | - Code - rate | Data rate (Mbits/s) |  |  |  | Data rate (Mbits/s) |  |  |  | Data rate (Mbits/s) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 6MHz BW |  |  |  | 7MHz BW |  |  |  | 8MHz BW |  |  |  |
|  |  |  | 1/4 CP | 1/8 CP | 1/16 CP | 1/32 CP | 1/4 CP | 1/8 CP | 1/16 CP | 1/32 CP | 1/4 CP | 1/8 CP | 1/16 CP | 1/32 CP |
| 0 | QPSK | 1/2 | 15.43 | 17.14 | 18.15 | 18.70 | 18.00 | 20.00 | 21.18 | 21.82 | 20.56 | 22.84 | 24.18 | 24.92 |
| 1 | QPSK | 2/3 | 20.57 | 22.85 | 24.20 | 24.93 | 24.00 | 26.67 | 28.24 | 29.09 | 27.41 | 30.45 | 32.24 | 33.22 |
| 2 | QPSK | 3/4 | 23.14 | 25.71 | 27.22 | 28.05 | 27.00 | 30.00 | 31.76 | 32.73 | 30.83 | 34.26 | 36.28 | 37.37 |
| 3 | QPSK | 5/6 | 25.71 | 28.57 | 30.25 | 31.16 | 30.00 | 33.33 | 35.29 | 36.36 | 34.26 | 38.07 | 40.31 | 41.53 |
| 4 | 16-QAM | 1/2 | 30.85 | 34.28 | 36.30 | 37.40 | 36.00 | 40.00 | 42.35 | 43.64 | 41.11 | 45.68 | 48.37 | 49.83 |
| 5 | 16-QAM | 2/3 | 41.14 | 45.71 | 48.40 | 49.86 | 48.00 | 53.33 | 56.47 | 58.18 | 54.82 | 60.91 | 64.49 | 66.44 |
| 6 | 16-QAM | 3/4 | 46.28 | 51.42 | 54.44 | 56.09 | 54.00 | 60.00 | 63.53 | 65.45 | 61.67 | 68.52 | 72.55 | 74.75 |
| 7 | 16-QAM | 5/6 | 51.42 | 57.13 | 60.49 | 62.33 | 60.00 | 66.67 | 70.59 | 72.73 | 68.52 | 76.13 | 80.61 | 83.05 |
| 8 | 64-QAM | 1/2 | 46.28 | 51.42 | 54.44 | 56.09 | 54.00 | 60.00 | 63.53 | 65.45 | 61.67 | 68.52 | 72.55 | 74.75 |
| 9 | 64-QAM | 2/3 | 61.70 | 68.56 | 72.59 | 74.79 | 72.00 | 80.00 | 84.71 | 87.27 | 82.22 | 91.36 | 96.73 | 99.67 |
| 10 | 64-QAM | 3/4 | 69.42 | 77.13 | 81.67 | 84.14 | 81.00 | 90.00 | 95.29 | 98.18 | 92.50 | 102.78 | 108.83 | 112.12 |
| 11 | 64-QAM | 5/6 | 77.13 | 85.70 | 90.74 | 93.49 | 90.00 | 100.00 | 105.88 | 109.09 | 102.78 | 114.20 | 120.92 | 124.58 |
| 12 | 256-QAM | 1/2 | 61.70 | 68.56 | 72.59 | 74.79 | 72.00 | 80.00 | 84.71 | 87.27 | 82.22 | 91.36 | 96.73 | 99.67 |
| 13 | 256-QAM | 2/3 | 82.27 | 91.41 | 96.79 | 99.72 | 96.00 | 106.67 | 112.94 | 116.36 | 109.63 | 121.81 | 128.98 | 132.89 |
| 14 | 256-QAM | 3/4 | 92.56 | 102.84 | 108.89 | 112.19 | 108.00 | 120.00 | 127.06 | 130.91 | 123.34 | 137.04 | 145.10 | 149.50 |
| 15 | 256-QAM | 5/6 | 102.84 | 114.27 | 120.99 | 124.65 | 120.00 | 133.33 | 141.18 | 145.45 | 137.04 | 152.27 | 161.22 | 166.11 |
| 16 | 256-QAM | 7/8 | 107.98 | 119.98 | 127.04 | 130.89 | 126.00 | 140.00 | 148.24 | 152.73 | 143.89 | 159.88 | 169.28 | 174.41 |
| 17 | 4D-48TCM | 10/11 | 78.32 | 87.02 | 92.14 | 94.94 | 91.39 | 101.54 | 107.52 | 110.78 | 104.37 | 115.96 | 122.79 | 126.51 |
| 18 | 4D-192TCM | 14/15 | 109.21 | 121.34 | 128.48 | 132.37 | 127.43 | 141.59 | 149.91 | 154.46 | 145.52 | 161.69 | 171.20 | 176.39 |

- Table AU1.6 – Data rate for 2 channels and 4 spatial streams

| - PHY mode | - Modulation | - Code - rate | Data rate (Mbits/s) |  |  |  | Data rate (Mbits/s) |  |  |  | Data rate (Mbits/s) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 6MHz BW |  |  |  | 7MHz BW |  |  |  | 8MHz BW |  |  |  |
|  |  |  | 1/4 CP | 1/8 CP | 1/16 CP | 1/32 CP | 1/4 CP | 1/8 CP | 1/16 CP | 1/32 CP | 1/4 CP | 1/8 CP | 1/16 CP | 1/32 CP |
| 0 | QPSK | 1/2 | 30.85 | 34.28 | 36.30 | 37.40 | 36.00 | 40.00 | 42.35 | 43.64 | 41.11 | 45.68 | 48.37 | 49.83 |
| 1 | QPSK | 2/3 | 41.14 | 45.71 | 48.40 | 49.86 | 48.00 | 53.33 | 56.47 | 58.18 | 54.82 | 60.91 | 64.49 | 66.44 |
| 2 | QPSK | 3/4 | 46.28 | 51.42 | 54.44 | 56.09 | 54.00 | 60.00 | 63.53 | 65.45 | 61.67 | 68.52 | 72.55 | 74.75 |
| 3 | QPSK | 5/6 | 51.42 | 57.13 | 60.49 | 62.33 | 60.00 | 66.67 | 70.59 | 72.73 | 68.52 | 76.13 | 80.61 | 83.05 |
| 4 | 16-QAM | 1/2 | 61.70 | 68.56 | 72.59 | 74.79 | 72.00 | 80.00 | 84.71 | 87.27 | 82.22 | 91.36 | 96.73 | 99.67 |
| 5 | 16-QAM | 2/3 | 82.27 | 91.41 | 96.79 | 99.72 | 96.00 | 106.67 | 112.94 | 116.36 | 109.63 | 121.81 | 128.98 | 132.89 |
| 6 | 16-QAM | 3/4 | 92.56 | 102.84 | 108.89 | 112.19 | 108.00 | 120.00 | 127.06 | 130.91 | 123.34 | 137.04 | 145.10 | 149.50 |
| 7 | 16-QAM | 5/6 | 102.84 | 114.27 | 120.99 | 124.65 | 120.00 | 133.33 | 141.18 | 145.45 | 137.04 | 152.27 | 161.22 | 166.11 |
| 8 | 64-QAM | 1/2 | 92.56 | 102.84 | 108.89 | 112.19 | 108.00 | 120.00 | 127.06 | 130.91 | 123.34 | 137.04 | 145.10 | 149.50 |
| 9 | 64-QAM | 2/3 | 123.41 | 137.12 | 145.19 | 149.59 | 144.00 | 160.00 | 169.41 | 174.55 | 164.45 | 182.72 | 193.47 | 199.33 |
| 10 | 64-QAM | 3/4 | 138.83 | 154.26 | 163.33 | 168.28 | 162.00 | 180.00 | 190.59 | 196.36 | 185.00 | 205.56 | 217.65 | 224.25 |
| 11 | 64-QAM | 5/6 | 154.26 | 171.40 | 181.48 | 186.98 | 180.00 | 200.00 | 211.76 | 218.18 | 205.56 | 228.40 | 241.84 | 249.16 |
| 12 | 256-QAM | 1/2 | 123.41 | 137.12 | 145.19 | 149.59 | 144.00 | 160.00 | 169.41 | 174.55 | 164.45 | 182.72 | 193.47 | 199.33 |
| 13 | 256-QAM | 2/3 | 164.54 | 182.83 | 193.58 | 199.45 | 192.00 | 213.33 | 225.88 | 232.73 | 219.26 | 243.63 | 257.96 | 265.77 |
| 14 | 256-QAM | 3/4 | 185.11 | 205.68 | 217.78 | 224.38 | 216.00 | 240.00 | 254.12 | 261.82 | 246.67 | 274.08 | 290.20 | 299.00 |
| 15 | 256-QAM | 5/6 | 205.68 | 228.53 | 241.98 | 249.31 | 240.00 | 266.67 | 282.35 | 290.91 | 274.08 | 304.53 | 322.45 | 332.22 |
| 16 | 256-QAM | 7/8 | 215.96 | 239.96 | 254.08 | 261.77 | 252.00 | 280.00 | 296.47 | 305.45 | 287.78 | 319.76 | 338.57 | 348.83 |
| 17 | 4D-48TCM | 10/11 | 156.64 | 174.05 | 184.29 | 189.87 | 182.78 | 203.09 | 215.04 | 221.55 | 208.74 | 231.93 | 245.57 | 253.01 |
| 18 | 4D-192TCM | 14/15 | 218.41 | 242.68 | 256.95 | 264.74 | 254.85 | 283.17 | 299.83 | 308.91 | 291.04 | 323.38 | 342.40 | 352.78 |

- Table AU1.7 – Data rate for 3 channels and 1 spatial stream

| - PHY mode | - Modulation | - Code - rate | Data rate (Mbits/s) |  |  |  | Data rate (Mbits/s) |  |  |  | Data rate (Mbits/s) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 6MHz BW |  |  |  | 7MHz BW |  |  |  | 8MHz BW |  |  |  |
|  |  |  | 1/4 CP | 1/8 CP | 1/16 CP | 1/32 CP | 1/4 CP | 1/8 CP | 1/16 CP | 1/32 CP | 1/4 CP | 1/8 CP | 1/16 CP | 1/32 CP |
| 0 | QPSK | 1/2 | 11.57 | 12.86 | 13.61 | 14.02 | 13.50 | 15.00 | 15.88 | 16.36 | 15.42 | 17.13 | 18.14 | 18.69 |
| 1 | QPSK | 2/3 | 15.43 | 17.14 | 18.15 | 18.70 | 18.00 | 20.00 | 21.18 | 21.82 | 20.56 | 22.84 | 24.18 | 24.92 |
| 2 | QPSK | 3/4 | 17.35 | 19.28 | 20.42 | 21.04 | 20.25 | 22.50 | 23.82 | 24.55 | 23.13 | 25.70 | 27.21 | 28.03 |
| 3 | QPSK | 5/6 | 19.28 | 21.43 | 22.69 | 23.37 | 22.50 | 25.00 | 26.47 | 27.27 | 25.70 | 28.55 | 30.23 | 31.15 |
| 4 | 16-QAM | 1/2 | 23.14 | 25.71 | 27.22 | 28.05 | 27.00 | 30.00 | 31.76 | 32.73 | 30.83 | 34.26 | 36.28 | 37.37 |
| 5 | 16-QAM | 2/3 | 30.85 | 34.28 | 36.30 | 37.40 | 36.00 | 40.00 | 42.35 | 43.64 | 41.11 | 45.68 | 48.37 | 49.83 |
| 6 | 16-QAM | 3/4 | 34.71 | 38.57 | 40.83 | 42.07 | 40.50 | 45.00 | 47.65 | 49.09 | 46.25 | 51.39 | 54.41 | 56.06 |
| 7 | 16-QAM | 5/6 | 38.57 | 42.85 | 45.37 | 46.75 | 45.00 | 50.00 | 52.94 | 54.55 | 51.39 | 57.10 | 60.46 | 62.29 |
| 8 | 64-QAM | 1/2 | 34.71 | 38.57 | 40.83 | 42.07 | 40.50 | 45.00 | 47.65 | 49.09 | 46.25 | 51.39 | 54.41 | 56.06 |
| 9 | 64-QAM | 2/3 | 46.28 | 51.42 | 54.44 | 56.09 | 54.00 | 60.00 | 63.53 | 65.45 | 61.67 | 68.52 | 72.55 | 74.75 |
| 10 | 64-QAM | 3/4 | 52.06 | 57.85 | 61.25 | 63.11 | 60.75 | 67.50 | 71.47 | 73.64 | 69.38 | 77.09 | 81.62 | 84.09 |
| 11 | 64-QAM | 5/6 | 57.85 | 64.28 | 68.06 | 70.12 | 67.50 | 75.00 | 79.41 | 81.82 | 77.09 | 85.65 | 90.69 | 93.44 |
| 12 | 256-QAM | 1/2 | 46.28 | 51.42 | 54.44 | 56.09 | 54.00 | 60.00 | 63.53 | 65.45 | 61.67 | 68.52 | 72.55 | 74.75 |
| 13 | 256-QAM | 2/3 | 61.70 | 68.56 | 72.59 | 74.79 | 72.00 | 80.00 | 84.71 | 87.27 | 82.22 | 91.36 | 96.73 | 99.67 |
| 14 | 256-QAM | 3/4 | 69.42 | 77.13 | 81.67 | 84.14 | 81.00 | 90.00 | 95.29 | 98.18 | 92.50 | 102.78 | 108.83 | 112.12 |
| 15 | 256-QAM | 5/6 | 77.13 | 85.70 | 90.74 | 93.49 | 90.00 | 100.00 | 105.88 | 109.09 | 102.78 | 114.20 | 120.92 | 124.58 |
| 16 | 256-QAM | 7/8 | 80.99 | 89.99 | 95.28 | 98.17 | 94.50 | 105.00 | 111.18 | 114.55 | 107.92 | 119.91 | 126.96 | 130.81 |
| 17 | 4D-48TCM | 10/11 | 58.74 | 65.27 | 69.11 | 71.20 | 68.54 | 76.16 | 80.64 | 83.08 | 78.28 | 86.97 | 92.09 | 94.88 |
| 18 | 4D-192TCM | 14/15 | 81.90 | 91.00 | 96.36 | 99.28 | 95.57 | 106.19 | 112.44 | 115.84 | 109.14 | 121.27 | 128.40 | 132.29 |

- Table AU1.8 – Data rate for 3 channels and 2 spatial streams

| - PHY mode | - Modulation | - Code - rate | Data rate (Mbits/s) |  |  |  | Data rate (Mbits/s) |  |  |  | Data rate (Mbits/s) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 6MHz BW |  |  |  | 7MHz BW |  |  |  | 8MHz BW |  |  |  |
|  |  |  | 1/4 CP | 1/8 CP | 1/16 CP | 1/32 CP | 1/4 CP | 1/8 CP | 1/16 CP | 1/32 CP | 1/4 CP | 1/8 CP | 1/16 CP | 1/32 CP |
| 0 | QPSK | 1/2 | 23.14 | 25.71 | 27.22 | 28.05 | 27.00 | 30.00 | 31.76 | 32.73 | 30.83 | 34.26 | 36.28 | 37.37 |
| 1 | QPSK | 2/3 | 30.85 | 34.28 | 36.30 | 37.40 | 36.00 | 40.00 | 42.35 | 43.64 | 41.11 | 45.68 | 48.37 | 49.83 |
| 2 | QPSK | 3/4 | 34.71 | 38.57 | 40.83 | 42.07 | 40.50 | 45.00 | 47.65 | 49.09 | 46.25 | 51.39 | 54.41 | 56.06 |
| 3 | QPSK | 5/6 | 38.57 | 42.85 | 45.37 | 46.75 | 45.00 | 50.00 | 52.94 | 54.55 | 51.39 | 57.10 | 60.46 | 62.29 |
| 4 | 16-QAM | 1/2 | 46.28 | 51.42 | 54.44 | 56.09 | 54.00 | 60.00 | 63.53 | 65.45 | 61.67 | 68.52 | 72.55 | 74.75 |
| 5 | 16-QAM | 2/3 | 61.70 | 68.56 | 72.59 | 74.79 | 72.00 | 80.00 | 84.71 | 87.27 | 82.22 | 91.36 | 96.73 | 99.67 |
| 6 | 16-QAM | 3/4 | 69.42 | 77.13 | 81.67 | 84.14 | 81.00 | 90.00 | 95.29 | 98.18 | 92.50 | 102.78 | 108.83 | 112.12 |
| 7 | 16-QAM | 5/6 | 77.13 | 85.70 | 90.74 | 93.49 | 90.00 | 100.00 | 105.88 | 109.09 | 102.78 | 114.20 | 120.92 | 124.58 |
| 8 | 64-QAM | 1/2 | 69.42 | 77.13 | 81.67 | 84.14 | 81.00 | 90.00 | 95.29 | 98.18 | 92.50 | 102.78 | 108.83 | 112.12 |
| 9 | 64-QAM | 2/3 | 92.56 | 102.84 | 108.89 | 112.19 | 108.00 | 120.00 | 127.06 | 130.91 | 123.34 | 137.04 | 145.10 | 149.50 |
| 10 | 64-QAM | 3/4 | 104.13 | 115.70 | 122.50 | 126.21 | 121.50 | 135.00 | 142.94 | 147.27 | 138.75 | 154.17 | 163.24 | 168.19 |
| 11 | 64-QAM | 5/6 | 115.70 | 128.55 | 136.11 | 140.24 | 135.00 | 150.00 | 158.82 | 163.64 | 154.17 | 171.30 | 181.38 | 186.87 |
| 12 | 256-QAM | 1/2 | 92.56 | 102.84 | 108.89 | 112.19 | 108.00 | 120.00 | 127.06 | 130.91 | 123.34 | 137.04 | 145.10 | 149.50 |
| 13 | 256-QAM | 2/3 | 123.41 | 137.12 | 145.19 | 149.59 | 144.00 | 160.00 | 169.41 | 174.55 | 164.45 | 182.72 | 193.47 | 199.33 |
| 14 | 256-QAM | 3/4 | 138.83 | 154.26 | 163.33 | 168.28 | 162.00 | 180.00 | 190.59 | 196.36 | 185.00 | 205.56 | 217.65 | 224.25 |
| 15 | 256-QAM | 5/6 | 154.26 | 171.40 | 181.48 | 186.98 | 180.00 | 200.00 | 211.76 | 218.18 | 205.56 | 228.40 | 241.84 | 249.16 |
| 16 | 256-QAM | 7/8 | 161.97 | 179.97 | 190.56 | 196.33 | 189.00 | 210.00 | 222.35 | 229.09 | 215.84 | 239.82 | 253.93 | 261.62 |
| 17 | 4D-48TCM | 10/11 | 117.48 | 130.54 | 138.21 | 142.40 | 137.09 | 152.32 | 161.28 | 166.16 | 156.55 | 173.95 | 184.18 | 189.76 |
| 18 | 4D-192TCM | 14/15 | 163.81 | 182.01 | 192.72 | 198.56 | 191.14 | 212.38 | 224.87 | 231.69 | 218.28 | 242.54 | 256.80 | 264.59 |

- Table AU1.9 – Data rate for 3 channels and 4 spatial streams

| - PHY mode | - Modulation | - Code - rate | Data rate (Mbits/s) |  |  |  | Data rate (Mbits/s) |  |  |  | Data rate (Mbits/s) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 6MHz BW |  |  |  | 7MHz BW |  |  |  | 8MHz BW |  |  |  |
|  |  |  | 1/4 CP | 1/8 CP | 1/16 CP | 1/32 CP | 1/4 CP | 1/8 CP | 1/16 CP | 1/32 CP | 1/4 CP | 1/8 CP | 1/16 CP | 1/32 CP |
| 0 | QPSK | 1/2 | 46.28 | 51.42 | 54.44 | 56.09 | 54.00 | 60.00 | 63.53 | 65.45 | 61.67 | 68.52 | 72.55 | 74.75 |
| 1 | QPSK | 2/3 | 61.70 | 68.56 | 72.59 | 74.79 | 72.00 | 80.00 | 84.71 | 87.27 | 82.22 | 91.36 | 96.73 | 99.67 |
| 2 | QPSK | 3/4 | 69.42 | 77.13 | 81.67 | 84.14 | 81.00 | 90.00 | 95.29 | 98.18 | 92.50 | 102.78 | 108.83 | 112.12 |
| 3 | QPSK | 5/6 | 77.13 | 85.70 | 90.74 | 93.49 | 90.00 | 100.00 | 105.88 | 109.09 | 102.78 | 114.20 | 120.92 | 124.58 |
| 4 | 16-QAM | 1/2 | 92.56 | 102.84 | 108.89 | 112.19 | 108.00 | 120.00 | 127.06 | 130.91 | 123.34 | 137.04 | 145.10 | 149.50 |
| 5 | 16-QAM | 2/3 | 123.41 | 137.12 | 145.19 | 149.59 | 144.00 | 160.00 | 169.41 | 174.55 | 164.45 | 182.72 | 193.47 | 199.33 |
| 6 | 16-QAM | 3/4 | 138.83 | 154.26 | 163.33 | 168.28 | 162.00 | 180.00 | 190.59 | 196.36 | 185.00 | 205.56 | 217.65 | 224.25 |
| 7 | 16-QAM | 5/6 | 154.26 | 171.40 | 181.48 | 186.98 | 180.00 | 200.00 | 211.76 | 218.18 | 205.56 | 228.40 | 241.84 | 249.16 |
| 8 | 64-QAM | 1/2 | 138.83 | 154.26 | 163.33 | 168.28 | 162.00 | 180.00 | 190.59 | 196.36 | 185.00 | 205.56 | 217.65 | 224.25 |
| 9 | 64-QAM | 2/3 | 185.11 | 205.68 | 217.78 | 224.38 | 216.00 | 240.00 | 254.12 | 261.82 | 246.67 | 274.08 | 290.20 | 299.00 |
| 10 | 64-QAM | 3/4 | 208.25 | 231.39 | 245.00 | 252.43 | 243.00 | 270.00 | 285.88 | 294.55 | 277.51 | 308.34 | 326.48 | 336.37 |
| 11 | 64-QAM | 5/6 | 231.39 | 257.10 | 272.22 | 280.47 | 270.00 | 300.00 | 317.65 | 327.27 | 308.34 | 342.60 | 362.75 | 373.75 |
| 12 | 256-QAM | 1/2 | 185.11 | 205.68 | 217.78 | 224.38 | 216.00 | 240.00 | 254.12 | 261.82 | 246.67 | 274.08 | 290.20 | 299.00 |
| 13 | 256-QAM | 2/3 | 246.82 | 274.24 | 290.37 | 299.17 | 288.00 | 320.00 | 338.82 | 349.09 | 328.90 | 365.44 | 386.94 | 398.66 |
| 14 | 256-QAM | 3/4 | 277.67 | 308.52 | 326.67 | 336.57 | 324.00 | 360.00 | 381.18 | 392.73 | 370.01 | 411.12 | 435.30 | 448.49 |
| 15 | 256-QAM | 5/6 | 308.52 | 342.80 | 362.96 | 373.96 | 360.00 | 400.00 | 423.53 | 436.36 | 411.12 | 456.80 | 483.67 | 498.33 |
| 16 | 256-QAM | 7/8 | 323.95 | 359.94 | 381.11 | 392.66 | 378.00 | 420.00 | 444.71 | 458.18 | 431.68 | 479.64 | 507.85 | 523.24 |
| 17 | 4D-48TCM | 10/11 | 234.96 | 261.07 | 276.43 | 284.81 | 274.17 | 304.63 | 322.55 | 332.33 | 313.10 | 347.89 | 368.36 | 379.52 |
| 18 | 4D-192TCM | 14/15 | 327.62 | 364.02 | 385.43 | 397.11 | 382.28 | 424.76 | 449.74 | 463.37 | 436.57 | 485.07 | 513.61 | 529.17 |

- Table AU1.10 – Data rate for 4 channels and 1 spatial stream

| - PHY mode | - Modulation | - Code - rate | Data rate (Mbits/s) |  |  |  | Data rate (Mbits/s) |  |  |  | Data rate (Mbits/s) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 6MHz BW |  |  |  | 7MHz BW |  |  |  | 8MHz BW |  |  |  |
|  |  |  | 1/4 CP | 1/8 CP | 1/16 CP | 1/32 CP | 1/4 CP | 1/8 CP | 1/16 CP | 1/32 CP | 1/4 CP | 1/8 CP | 1/16 CP | 1/32 CP |
| 0 | QPSK | 1/2 | 15.43 | 17.14 | 18.15 | 18.70 | 18.00 | 20.00 | 21.18 | 21.82 | 20.56 | 22.84 | 24.18 | 24.92 |
| 1 | QPSK | 2/3 | 20.57 | 22.85 | 24.20 | 24.93 | 24.00 | 26.67 | 28.24 | 29.09 | 27.41 | 30.45 | 32.24 | 33.22 |
| 2 | QPSK | 3/4 | 23.14 | 25.71 | 27.22 | 28.05 | 27.00 | 30.00 | 31.76 | 32.73 | 30.83 | 34.26 | 36.28 | 37.37 |
| 3 | QPSK | 5/6 | 25.71 | 28.57 | 30.25 | 31.16 | 30.00 | 33.33 | 35.29 | 36.36 | 34.26 | 38.07 | 40.31 | 41.53 |
| 4 | 16-QAM | 1/2 | 30.85 | 34.28 | 36.30 | 37.40 | 36.00 | 40.00 | 42.35 | 43.64 | 41.11 | 45.68 | 48.37 | 49.83 |
| 5 | 16-QAM | 2/3 | 41.14 | 45.71 | 48.40 | 49.86 | 48.00 | 53.33 | 56.47 | 58.18 | 54.82 | 60.91 | 64.49 | 66.44 |
| 6 | 16-QAM | 3/4 | 46.28 | 51.42 | 54.44 | 56.09 | 54.00 | 60.00 | 63.53 | 65.45 | 61.67 | 68.52 | 72.55 | 74.75 |
| 7 | 16-QAM | 5/6 | 51.42 | 57.13 | 60.49 | 62.33 | 60.00 | 66.67 | 70.59 | 72.73 | 68.52 | 76.13 | 80.61 | 83.05 |
| 8 | 64-QAM | 1/2 | 46.28 | 51.42 | 54.44 | 56.09 | 54.00 | 60.00 | 63.53 | 65.45 | 61.67 | 68.52 | 72.55 | 74.75 |
| 9 | 64-QAM | 2/3 | 61.70 | 68.56 | 72.59 | 74.79 | 72.00 | 80.00 | 84.71 | 87.27 | 82.22 | 91.36 | 96.73 | 99.67 |
| 10 | 64-QAM | 3/4 | 69.42 | 77.13 | 81.67 | 84.14 | 81.00 | 90.00 | 95.29 | 98.18 | 92.50 | 102.78 | 108.83 | 112.12 |
| 11 | 64-QAM | 5/6 | 77.13 | 85.70 | 90.74 | 93.49 | 90.00 | 100.00 | 105.88 | 109.09 | 102.78 | 114.20 | 120.92 | 124.58 |
| 12 | 256-QAM | 1/2 | 61.70 | 68.56 | 72.59 | 74.79 | 72.00 | 80.00 | 84.71 | 87.27 | 82.22 | 91.36 | 96.73 | 99.67 |
| 13 | 256-QAM | 2/3 | 82.27 | 91.41 | 96.79 | 99.72 | 96.00 | 106.67 | 112.94 | 116.36 | 109.63 | 121.81 | 128.98 | 132.89 |
| 14 | 256-QAM | 3/4 | 92.56 | 102.84 | 108.89 | 112.19 | 108.00 | 120.00 | 127.06 | 130.91 | 123.34 | 137.04 | 145.10 | 149.50 |
| 15 | 256-QAM | 5/6 | 102.84 | 114.27 | 120.99 | 124.65 | 120.00 | 133.33 | 141.18 | 145.45 | 137.04 | 152.27 | 161.22 | 166.11 |
| 16 | 256-QAM | 7/8 | 107.98 | 119.98 | 127.04 | 130.89 | 126.00 | 140.00 | 148.24 | 152.73 | 143.89 | 159.88 | 169.28 | 174.41 |
| 17 | 4D-48TCM | 10/11 | 78.32 | 87.02 | 92.14 | 94.94 | 91.39 | 101.54 | 107.52 | 110.78 | 104.37 | 115.96 | 122.79 | 126.51 |
| 18 | 4D-192TCM | 14/15 | 109.21 | 121.34 | 128.48 | 132.37 | 127.43 | 141.59 | 149.91 | 154.46 | 145.52 | 161.69 | 171.20 | 176.39 |

- Table AU1.11 – Data rate for 4 channels and 2 spatial streams

| - PHY mode | - Modulation | - Code - rate | Data rate (Mbits/s) |  |  |  | Data rate (Mbits/s) |  |  |  | Data rate (Mbits/s) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 6MHz BW |  |  |  | 7MHz BW |  |  |  | 8MHz BW |  |  |  |
|  |  |  | 1/4 CP | 1/8 CP | 1/16 CP | 1/32 CP | 1/4 CP | 1/8 CP | 1/16 CP | 1/32 CP | 1/4 CP | 1/8 CP | 1/16 CP | 1/32 CP |
| 0 | QPSK | 1/2 | 30.85 | 34.28 | 36.30 | 37.40 | 36.00 | 40.00 | 42.35 | 43.64 | 41.11 | 45.68 | 48.37 | 49.83 |
| 1 | QPSK | 2/3 | 41.14 | 45.71 | 48.40 | 49.86 | 48.00 | 53.33 | 56.47 | 58.18 | 54.82 | 60.91 | 64.49 | 66.44 |
| 2 | QPSK | 3/4 | 46.28 | 51.42 | 54.44 | 56.09 | 54.00 | 60.00 | 63.53 | 65.45 | 61.67 | 68.52 | 72.55 | 74.75 |
| 3 | QPSK | 5/6 | 51.42 | 57.13 | 60.49 | 62.33 | 60.00 | 66.67 | 70.59 | 72.73 | 68.52 | 76.13 | 80.61 | 83.05 |
| 4 | 16-QAM | 1/2 | 61.70 | 68.56 | 72.59 | 74.79 | 72.00 | 80.00 | 84.71 | 87.27 | 82.22 | 91.36 | 96.73 | 99.67 |
| 5 | 16-QAM | 2/3 | 82.27 | 91.41 | 96.79 | 99.72 | 96.00 | 106.67 | 112.94 | 116.36 | 109.63 | 121.81 | 128.98 | 132.89 |
| 6 | 16-QAM | 3/4 | 92.56 | 102.84 | 108.89 | 112.19 | 108.00 | 120.00 | 127.06 | 130.91 | 123.34 | 137.04 | 145.10 | 149.50 |
| 7 | 16-QAM | 5/6 | 102.84 | 114.27 | 120.99 | 124.65 | 120.00 | 133.33 | 141.18 | 145.45 | 137.04 | 152.27 | 161.22 | 166.11 |
| 8 | 64-QAM | 1/2 | 92.56 | 102.84 | 108.89 | 112.19 | 108.00 | 120.00 | 127.06 | 130.91 | 123.34 | 137.04 | 145.10 | 149.50 |
| 9 | 64-QAM | 2/3 | 123.41 | 137.12 | 145.19 | 149.59 | 144.00 | 160.00 | 169.41 | 174.55 | 164.45 | 182.72 | 193.47 | 199.33 |
| 10 | 64-QAM | 3/4 | 138.83 | 154.26 | 163.33 | 168.28 | 162.00 | 180.00 | 190.59 | 196.36 | 185.00 | 205.56 | 217.65 | 224.25 |
| 11 | 64-QAM | 5/6 | 154.26 | 171.40 | 181.48 | 186.98 | 180.00 | 200.00 | 211.76 | 218.18 | 205.56 | 228.40 | 241.84 | 249.16 |
| 12 | 256-QAM | 1/2 | 123.41 | 137.12 | 145.19 | 149.59 | 144.00 | 160.00 | 169.41 | 174.55 | 164.45 | 182.72 | 193.47 | 199.33 |
| 13 | 256-QAM | 2/3 | 164.54 | 182.83 | 193.58 | 199.45 | 192.00 | 213.33 | 225.88 | 232.73 | 219.26 | 243.63 | 257.96 | 265.77 |
| 14 | 256-QAM | 3/4 | 185.11 | 205.68 | 217.78 | 224.38 | 216.00 | 240.00 | 254.12 | 261.82 | 246.67 | 274.08 | 290.20 | 299.00 |
| 15 | 256-QAM | 5/6 | 205.68 | 228.53 | 241.98 | 249.31 | 240.00 | 266.67 | 282.35 | 290.91 | 274.08 | 304.53 | 322.45 | 332.22 |
| 16 | 256-QAM | 7/8 | 215.96 | 239.96 | 254.08 | 261.77 | 252.00 | 280.00 | 296.47 | 305.45 | 287.78 | 319.76 | 338.57 | 348.83 |
| 17 | 4D-48TCM | 10/11 | 156.64 | 174.05 | 184.29 | 189.87 | 182.78 | 203.09 | 215.04 | 221.55 | 208.74 | 231.93 | 245.57 | 253.01 |
| 18 | 4D-192TCM | 14/15 | 218.41 | 242.68 | 256.95 | 264.74 | 254.85 | 283.17 | 299.83 | 308.91 | 291.04 | 323.38 | 342.40 | 352.78 |

- Table AU1.12 – Data rate for 4 channels and 4 spatial streams

| - PHY mode | - Modulation | - Code - rate | Data rate (Mbits/s) |  |  |  | Data rate (Mbits/s) |  |  |  | Data rate (Mbits/s) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 6MHz BW |  |  |  | 7MHz BW |  |  |  | 8MHz BW |  |  |  |
|  |  |  | 1/4 CP | 1/8 CP | 1/16 CP | 1/32 CP | 1/4 CP | 1/8 CP | 1/16 CP | 1/32 CP | 1/4 CP | 1/8 CP | 1/16 CP | 1/32 CP |
| 0 | QPSK | 1/2 | 61.70 | 68.56 | 72.59 | 74.79 | 72.00 | 80.00 | 84.71 | 87.27 | 82.22 | 91.36 | 96.73 | 99.67 |
| 1 | QPSK | 2/3 | 82.27 | 91.41 | 96.79 | 99.72 | 96.00 | 106.67 | 112.94 | 116.36 | 109.63 | 121.81 | 128.98 | 132.89 |
| 2 | QPSK | 3/4 | 92.56 | 102.84 | 108.89 | 112.19 | 108.00 | 120.00 | 127.06 | 130.91 | 123.34 | 137.04 | 145.10 | 149.50 |
| 3 | QPSK | 5/6 | 102.84 | 114.27 | 120.99 | 124.65 | 120.00 | 133.33 | 141.18 | 145.45 | 137.04 | 152.27 | 161.22 | 166.11 |
| 4 | 16-QAM | 1/2 | 123.41 | 137.12 | 145.19 | 149.59 | 144.00 | 160.00 | 169.41 | 174.55 | 164.45 | 182.72 | 193.47 | 199.33 |
| 5 | 16-QAM | 2/3 | 164.54 | 182.83 | 193.58 | 199.45 | 192.00 | 213.33 | 225.88 | 232.73 | 219.26 | 243.63 | 257.96 | 265.77 |
| 6 | 16-QAM | 3/4 | 185.11 | 205.68 | 217.78 | 224.38 | 216.00 | 240.00 | 254.12 | 261.82 | 246.67 | 274.08 | 290.20 | 299.00 |
| 7 | 16-QAM | 5/6 | 205.68 | 228.53 | 241.98 | 249.31 | 240.00 | 266.67 | 282.35 | 290.91 | 274.08 | 304.53 | 322.45 | 332.22 |
| 8 | 64-QAM | 1/2 | 185.11 | 205.68 | 217.78 | 224.38 | 216.00 | 240.00 | 254.12 | 261.82 | 246.67 | 274.08 | 290.20 | 299.00 |
| 9 | 64-QAM | 2/3 | 246.82 | 274.24 | 290.37 | 299.17 | 288.00 | 320.00 | 338.82 | 349.09 | 328.90 | 365.44 | 386.94 | 398.66 |
| 10 | 64-QAM | 3/4 | 277.67 | 308.52 | 326.67 | 336.57 | 324.00 | 360.00 | 381.18 | 392.73 | 370.01 | 411.12 | 435.30 | 448.49 |
| 11 | 64-QAM | 5/6 | 308.52 | 342.80 | 362.96 | 373.96 | 360.00 | 400.00 | 423.53 | 436.36 | 411.12 | 456.80 | 483.67 | 498.33 |
| 12 | 256-QAM | 1/2 | 246.82 | 274.24 | 290.37 | 299.17 | 288.00 | 320.00 | 338.82 | 349.09 | 328.90 | 365.44 | 386.94 | 398.66 |
| 13 | 256-QAM | 2/3 | 329.09 | 365.65 | 387.16 | 398.89 | 384.00 | 426.67 | 451.76 | 465.45 | 438.53 | 487.25 | 515.92 | 531.55 |
| 14 | 256-QAM | 3/4 | 370.22 | 411.36 | 435.56 | 448.76 | 432.00 | 480.00 | 508.24 | 523.64 | 493.34 | 548.16 | 580.40 | 597.99 |
| 15 | 256-QAM | 5/6 | 411.36 | 457.07 | 483.95 | 498.62 | 480.00 | 533.33 | 564.71 | 581.82 | 548.16 | 609.07 | 644.89 | 664.44 |
| 16 | 256-QAM | 7/8 | 431.93 | 479.92 | 508.15 | 523.55 | 504.00 | 560.00 | 592.94 | 610.91 | 575.57 | 639.52 | 677.14 | 697.66 |
| 17 | 4D-48TCM | 10/11 | 313.29 | 348.10 | 368.57 | 379.74 | 365.56 | 406.18 | 430.07 | 443.10 | 417.47 | 463.86 | 491.14 | 506.03 |
| 18 | 4D-192TCM | 14/15 | 436.82 | 485.36 | 513.91 | 529.48 | 509.71 | 566.34 | 599.66 | 617.83 | 582.09 | 646.76 | 684.81 | 705.56 |