IEEE P802.22
Wireless RANs

| Group Update Message to Handle Multiple Devices (coment resolution for CID #88 and #89) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2014-08-28 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Sung-Hyun Hwang | ETRI | Korea | +82-42-860-1133 | shwang@etri.re.kr |
| Seung-Keun Park | ETRI | Korea | +82-42-860-5993 | seungkp@etri.re.kr |
| Gwang-Zeen Ko | ETRI | Korea | +82-42-860-4862 | gogogo@etri.re.kr |
| Myung-Sun Song | ETRI | Korea | +82-42-860-5046 | mssong@etri.re.kr |
| Hyung-Do Choi | ETRI | Korea | +82-42-860-5814 | choihd@etri.re.kr |

Abstract

This is the comment resolution for 2nd WG Letter Ballot CID #88 and #89. These are related to the Group Management of 802.22b systems.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Apurva N. Mody> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

- 1. Related Comment

- This is the comment resolution for 2nd WG Letter Ballot CID #88 and #89 as described in the following table.

- 2. Proposed Resolution

- Because the conditional syntax (if clause) and Flag parameters were combined together in the Table AS1, it caused some confusions. Therefore, the conditional syntax and Flag parameters are described separately, and structure are modified accordingly.

- Moreover, to allow adding or deleting multiple devices from the group at once, the message structure is modified to identify multiple devices that is added to the group or deleted from the group.

- 3. Proposed Text Changes

- 7.7.30.2 Group Resource Allocation Update (GRA-UPD)

- The format of Group Resource Allocation Update message is shown in Table AS1. This message is used to update the group resource allocation configuration. The device can be added to or deleted from a group. This message is also used to update the location of group.

- Table AS1 — GRA-UPD message format

| Syntax | Size | Notes |
| --- | --- | --- |
| GRA-UPD_Message_Format() { |  |  |
| Management Message Type = 56 | 8 bits |  |
| Device Flag | 1 bit | Flag to indicate whether the device information is changed 0: Device information is not changed 1: Device information is changed |
| Group Flag | 1 bit | Flag to indicate whether the group information is changed 0: Group information is not changed 1: Group information is changed |
| If(Device Flag = 1) { |  | Indicate that device information is changed |
| GID | 10 bits | Group ID to which the device(s) is added to or deleted from a group. |
| Number of Added Devices | 7 bits |  |
| Number of Deleted Devices | 7 bits |  |
| For (i=0; i< Number of Added Devices; i++) { |  |  |
| SID [i] | 13 bits | Station ID that is added to a group. |
| Device Bitmap Index | 7 bits | New index of the device in a group’s device bitmap. |
| } |  |  |
| For (i=0; i< Number of Deleted Devices; i++) { |  |  |
| SID [i] | 13 bits | Station ID that is deleted from a group. |
| } |  |  |
| } |  |  |
| If(Group Flag = 1) { |  | Indicate that group information is changed |
| Group Location | 48 bits | New latitude and longitude of a group |
| } |  |  |
| } |  |  |