IEEE P802.22
Wireless RANs

| MIMO pilot pattern for PHY Operation Mode 1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2014-11-05 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Sung-Hyun Hwang | ETRI | Korea | +82-42-860-1133 | shwang@etri.re.kr |
| Seung-Keun Park | ETRI | Korea | +82-42-860-5993 | seungkp@etri.re.kr |
| Gwang-Zeen Ko | ETRI | Korea | +82-42-860-4862 | gogogo@etri.re.kr |
| Myung-Sun Song | ETRI | Korea | +82-42-860-5046 | mssong@etri.re.kr |
| Hyung-Do Choi | ETRI | Korea | +82-42-860-5814 | choihd@etri.re.kr |

Abstract

This is the suggested text for MIMO pilot pattern in IEEE 802.22b PHY Operation Mode 1. It is the resolution for 2nd WG Letter Ballot CID #58.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Apurva N. Mody> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

- 1. Related Comment

- In the draft 2.0, there is no MIMO pilot pattern for PHY operation mode 1. This is the resolution for 2nd WG Letter Ballot CID # 58 as described in the following table.

- 2. Proposed Resolution

- In this contribution, we propose the pilot pattern for the PHY operation mode 1. For the consistency with legacy Std. 802.22-2011, we propose the MIMO pilot pattern which is repeated every 7 OFDM symbols and 7 subcarriers in the time and frequency domains, respectively.

- 3. Proposed Text Changes

- Change the section 7.4b.2 as follows:

9.15 Multiple-input, multiple-output (MIMO)

Multiple-input-multiple-output (MIMO) system has attracted a great deal of attention since the mid-1990’s. It consists of multiple antennas at both transmitter and receiver, and are a breakthrough in wireless communication system design. Not limited to time and frequency dimensions, MIMO exploits the spatial dimension created by multiple antennas to achieve improvements in capacity and reliability. Moreover, the improvements come with neither addition of bandwidth nor increment of power, therefore, making MIMO a highly spectral efficient and reliable technique.

The MIMO channel can be made more robust against fading channels than each of its single-input-single-output (SISO) components by exploiting spatial diversity. Furthermore, MIMO channel creates the possibility to increase the transmission rate compared to SISO channel through spatial multiplexing; however, these two features cannot be fully exploited at the same time. A fundamental trade-off between rate increment and robustness against fading exists and must be decided according to the users needs. The MIMO technique described hereafter is more appropriate for the link between base station and relay stations existing in the wireless network backbone of A-WRAN systems. On the other hand, implementing MIMO on the links between base station and CPEs or relay stations and CPEs is technically more challenging. This is due to the fact that the lower operating frequencies inherent to the TVWS bands require antennas with bigger physical sizes than the ones required by existing wireless communication systems operating at higher parts of the frequency spectrum.

9.15.1 MIMO pilot allocation

The pilot insertion pattern for two antennas and four antennas is shown in Figure XX1 and Figure XX2, respectively. The pilot pattern shall be repeated every 7 OFDM symbols and 7 subcarriers in the time and frequency domains, respectively. The pilot pattern is always the same, independent of the channel bandwidth. The pilot pattern shall also be the same for the downstream and upstream.

9.15.1.1 Pilot allocation for 2 antennas

The pilot insertion pattern for two antennas is shown in Figure XX1. The following physical pilot indices, Pk, after the last frame preamble in every downstream subframe, shall be used instead of the logical indices 0, 1, …, 6.

Pk=−840+7k+pilotsubcarrieroffset+DCflag , k=0, 1, 2, ..., 239

where:

k is the running subcarrier index from 0 to 239.

pilotsubcarrieroffset is the subcarrier offset to control the beginning of first pilot subcarrier within each OFDM symbol. The pilotsubcarrieroffset for antenna 0 is 0, 3, 5, and 1 for (OFDMAsymbolindex mod 7)= 0, 2, 4, and 6, respectively. The pilotsubcarrieroffset for antenna 1 is 2, 6, and 4 for (OFDMAsymbolindex mod 7)= 1, 3, and 5, respectively. The OFDMA symbol of index 0 should be the first OFDMA symbol in every downstream or upstream subframes.

DCflag is used to count the DC subcarrier in the calculation of pilot subcarrier index. When k is equal or larger than 120, the value is set to 1.

Figure XX1 – Pilot pattern for 2 TX antennas

9.15.1.2 Pilot allocation for 4 antennas

The pilot insertion pattern for four antennas is shown in Figure XX2. The following physical pilot indices, Pk, after the last frame preamble in every downstream subframe, shall be used instead of the logical indices 0, 1, …, 6.

Pk=−840+7k+pilotsubcarrieroffset+DCflag , k=0, 1, 2, ..., 239

where:

k is the running subcarrier index from 0 to 239.

pilotsubcarrieroffset is the subcarrier offset to control the beginning of first pilot subcarrier within each OFDM symbol. The pilotsubcarrieroffset for antenna 0 is 0 and 3 for (OFDMAsymbolindex mod 7)= 0 and 4, respectively. The pilotsubcarrieroffset for antenna 1 is 1 and 4 for (OFDMAsymbolindex mod 7)= 1 and 5, respectively. The pilotsubcarrieroffset for antenna 2 is 2 and 5 for (OFDMAsymbolindex mod 7)= 2 and 6, respectively. The pilotsubcarrieroffset for antenna 3 is 6 for (OFDMAsymbolindex mod 7)= 3. The OFDMA symbol of index 0 should be the first OFDMA symbol in every downstream or upstream subframes.

DCflag is used to count the DC subcarrier in the calculation of pilot subcarrier index. When k is equal or larger than 120, the value is set to 1.

Figure XX2 – Pilot pattern for 4 TX antennas