IEEE Standard for Information Technology—

Telecommunications and information exchange between systems

Wireless Regional Area Networks (WRAN)—

Specific requirements

Part 22.3: Spectrum Characterization and Occupancy Sensing

Sponsor

LAN/MAN Standards Committee
of the
IEEE Computer Society

Approved Date XX

IEEE-SA Standards Board

Abstract: This standard specifies the architecture, abstraction layers, interfaces and metadata requirements for Spectrum Characterization and Occupancy Sensing (SCOS) system, a defines performance parameters, units and measures. This SCOS system comprises one or more semi-autonomous Spectrum Sensing Devices which scan electromagnetic spectrum, digitize it and perform processing, transmitting the resultant data with appropriate metadata to a central storage and processing system, according to rules, policies or instructions imposed on the Spectrum Sensing Devices by a management system.

Keywords: radio spectrum sensing, spectrum monitoring, signal characterization, cognitive radio, IEEE 802.22.3, WRAN standards



IEEE Standards documents are developed within the IEEE Societies and the Standards Coordinating Committees of the IEEE Standards Association (IEEE-SA) Standards Board. The IEEE develops its standards through a consensus development process, approved by the American National Standards Institute, which brings together volunteers representing varied viewpoints and interests to achieve the final product. Volunteers are not necessarily members of the Institute and serve without compensation. While the IEEE administers the process and establishes rules to promote fairness in the consensus development process, the IEEE does not independently evaluate, test, or verify the accuracy of any of the information or the soundness of any judgments contained in its standards.

Use of an IEEE Standard is wholly voluntary. The IEEE disclaims liability for any personal injury, property or other damage, of any nature whatsoever, whether special, indirect, consequential, or compensatory, directly or indirectly resulting from the publication, use of, or reliance upon this, or any other IEEE Standard document.

The IEEE does not warrant or represent the accuracy or content of the material contained herein, and expressly disclaims any express or implied warranty, including any implied warranty of merchantability or fitness for a specific purpose, or that the use of the material contained herein is free from patent infringement. IEEE Standards documents are supplied “AS IS.”

The existence of an IEEE Standard does not imply that there are no other ways to produce, test, measure, purchase, market, or provide other goods and services related to the scope of the IEEE Standard. Furthermore, the viewpoint expressed at the time a standard is approved and issued is subject to change brought about through developments in the state of the art and comments received from users of the standard. Every IEEE Standard is subjected to review at least every five years for revision or reaffirmation, or every ten years for stabilization. When a document is more than five years old and has not been reaffirmed, or more than ten years old and has not been stabilized, it is reasonable to conclude that its contents, although still of some value, do not wholly reflect the present state of the art. Users are cautioned to check to determine that they have the latest edition of any IEEE Standard.

In publishing and making this document available, the IEEE is not suggesting or rendering professional or other services for, or on behalf of, any person or entity. Nor is the IEEE undertaking to perform any duty owed by any other person or entity to another. Any person utilizing this, and any other IEEE Standards document, should rely upon his or her independent judgment in the exercise of reasonable care in any given circumstances or, as appropriate, seek the advice of a competent professional in determining the appropriateness of a given IEEE standard.

Interpretations: Occasionally questions may arise regarding the meaning of portions of standards as they relate to specific applications. When the need for interpretations is brought to the attention of IEEE, the Institute will initiate action to prepare appropriate responses. Since IEEE Standards represent a consensus of concerned interests, it is important to ensure that any interpretation has also received the concurrence of a balance of interests. For this reason, IEEE and the members of its societies and Standards Coordinating Committees are not able to provide an instant response to interpretation requests except in those cases where the matter has previously received formal consideration. A statement, written or oral, that is not processed in accordance with the IEEE-SA Standards Board Operations Manual shall not be considered the official position of IEEE or any of its committees and shall not be considered to be, nor be relied upon as, a formal interpretation of the IEEE. At lectures, symposia, seminars, or educational courses, an individual presenting information on IEEE standards shall make it clear that his or her views should be considered the personal views of that individual rather than the formal position, explanation, or interpretation of the IEEE.

Comments for revision of IEEE Standards are welcome from any interested party, regardless of membership affiliation with IEEE. Suggestions for changes in documents should be in the form of a proposed change of text, together with appropriate supporting comments. Recommendations to change the status of a stabilized standard should include a rationale as to why a revision or withdrawal is required. Comments and recommendations on standards, and requests for interpretations should be addressed to:

Secretary, IEEE-SA Standards Board

445 Hoes Lane

Piscataway, NJ 08854

USA

Authorization to photocopy portions of any individual standard for internal or personal use is granted by The Institute of Electrical and Electronics Engineers, Inc., provided that the appropriate fee is paid to Copyright Clearance Center. To arrange for payment of licensing fee, please contact Copyright Clearance Center, Customer Service, 222 Rosewood Drive, Danvers, MA 01923 USA; +1 978 750 8400. Permission to photocopy portions of any individual standard for educational classroom use can also be obtained through the Copyright Clearance Center.

Introduction

This introduction is not part of IEEE Std 802.22-2011, IEEE Standard for Information Technology—Telecommunications and information exchange between systems—Wireless Regional Area Networks (WRAN)—Specific requirements—Part 22: Cognitive Wireless RAN Medium Access Control (MAC) and Physical Layer (PHY) Specifications: Policies and Procedures for Operation in the TV Bands.

This standard specifies the architecture, abstraction layers, interfaces and metadata requirements for Spectrum Characterization and Occupancy Sensing (SCOS) system, a defines performance parameters, units and measures. This SCOS system comprises one or more semi-autonomous Spectrum Sensing Devices which scan electromagnetic spectrum, digitize it and perform processing, transmitting the resultant data with appropriate metadata to a central storage and processing system, according to rules, policies or instructions imposed on the Spectrum Sensing Devices by a management system.

Notice to users

Laws and regulations

Users of these documents should consult all applicable laws and regulations. Compliance with the provisions of this standard does not imply compliance to any applicable regulatory requirements. Implementers of the standard are responsible for observing or referring to the applicable regulatory requirements. IEEE does not, by the publication of its standards, intend to urge action that is not in compliance with applicable laws, and these documents may not be construed as doing so.

Copyrights

This document is copyrighted by the IEEE. It is made available for a wide variety of both public and private uses. These include both use, by reference, in laws and regulations, and use in private self-regulation, standardization, and the promotion of engineering practices and methods. By making this document available for use and adoption by public authorities and private users, the IEEE does not waive any rights in copyright to this document.

Updating of IEEE documents

Users of IEEE standards should be aware that these documents may be superseded at any time by the issuance of new editions or may be amended from time to time through the issuance of amendments, corrigenda, or errata. An official IEEE document at any point in time consists of the current edition of the document together with any amendments, corrigenda, or errata then in effect. In order to determine whether a given document is the current edition and whether it has been amended through the issuance of amendments, corrigenda, or errata, visit the IEEE Standards Association web site at http://ieeexplore.ieee.org/xpl/standards.jsp, or contact the IEEE at the address listed previously.

For more information about the IEEE Standards Association or the IEEE standards development process, visit the IEEE-SA web site at http://standards.ieee.org.

Errata

Errata, if any, for this and all other standards can be accessed at the following URL: 
http://standards.ieee.org/reading/ieee/updates/errata/index.html. Users are encouraged to check this URL for errata periodically.

Interpretations

Current interpretations can be accessed at the following URL: http://standards.ieee.org/reading/ieee/interp/
index.html.

Patents

Attention is called to the possibility that implementation of this standard may require use of subject matter covered by patent rights. By publication of this standard, no position is taken with respect to the existence or validity of any patent rights in connection therewith. A patent holder or patent applicant has filed a statement of assurance that it will grant licenses under these rights without compensation or under reasonable rates, with reasonable terms and conditions that are demonstrably free of any unfair discrimination to applicants desiring to obtain such licenses. Other Essential Patent Claims may exist for which a statement of assurance has not been received. The IEEE is not responsible for identifying Essential Patent Claims for which a license may be required, for conducting inquiries into the legal validity or scope of Patents Claims, or determining whether any licensing terms or conditions provided in connection with submission of a Letter of Assurance, if any, or in any licensing agreements are reasonable or non-discriminatory. Users of this standard are expressly advised that determination of the validity of any patent rights, and the risk of infringement of such rights, is entirely their own responsibility. Further information may be obtained from the IEEE Standards Association.

Participants

At the time this standard was submitted to the IEEE-SA for approval, the following voting members had participated in the IEEE P802.22.3 Task Group:

TBC

Major contributions to this standard were made by the following individuals:

TBC

The following members of the balloting committee voted on this Error! Unknown document property name.Error! Unknown document property name.. Balloters may have voted for approval, disapproval, or abstention.

When the IEEE-SA Standards Board approved this on TBC, it had the following membership:

Richard H. Hulett, Chair

John Kulick, Vice Chair

Robert M. Grow, Past Chair

Judith Gorman, Secretary

*Member Emeritus

Also included are the following nonvoting IEEE-SA Standards Board liaisons:

Satish Aggarwal, NRC Representative

Richard DeBlasio, DOE Representative

Michael Janezic, NIST Representative

Patricia Gerdon

IEEE Standards Program Manager, Document Development

Catherine Berger

IEEE Standards Project Editor

Contents

1.	Overview	10

1.1	Scope	10

1.2	Purpose	11

1.3	Reference Applications	11

2.	Normative References	12

3.	Abbreviations and acronyms	13

4.	Functional Requirements	13

4.1	Introduction	13

4.2	Managed objects	14

4.3	Regulatory requirements	14

4.4	Device classes and complexity	14

4.5	Number of devices	15

4.6	Network Topology	15

4.7	Real-time applications	15

4.8	Security	15

4.9	Security of sensed data	16

4.10	Channelization	16

4.11	Reporting to the Spectrum Sensing Management System	16

4.12	Sensor Location	16

5.	System architecture	17

5.1	Reference architecture	17

5.2	Management Reference Architecture	18

6.	Architecture System Requirements	18

7.	System Definitions and Interfaces	20

7.1	System Units and Parameters	20

7.2	Metadata Formats	21

8.	Security Systems	21

Annex A (normative) IEEE 802.22 regulatory domains and regulatory classes requirements	22

A.1 Regulatory domains, regulatory classes, and professional installation	22

A.2 Radio performance requirements	23

Annex B (informative) Sensing	24

B.1 References	24

Annex C (informative) Bibliography	25

IEEE Standard for Information Technology—

Telecommunications and information exchange between systems

Wireless Regional Area Networks (WRAN)—

Specific requirements

Part 22.3: Standard for Spectrum Characterization and Occupancy Sensing

IMPORTANT NOTICE: This standard is not intended to ensure safety, security, health, or environmental protection. Implementers of the standard are responsible for determining appropriate safety, security, environmental, and health practices or regulatory requirements.

This IEEE document is made available for use subject to important notices and legal disclaimers. 
These notices and disclaimers appear in all publications containing this document and may 
be found under the heading “Important Notice” or “Important Notices and Disclaimers 
Concerning IEEE Documents.” They can also be obtained on request from IEEE or viewed at http://standards.ieee.org/IPR/disclaimers.html.

- Overview

  - Scope

This Standard defines a Spectrum Characterization and Occupancy Sensing (SCOS) System. It specifies measurement parameters and device behaviors. It includes protocols for reporting measurement information that enable coalescing the results from multiple such devices. The standard leverages interfaces and primitives that are derived from IEEE Std. 802.22-2011, and uses any on-line transport mechanism available to achieve the control and management of the system. Interfaces and primitives are provided for conveying value added sensing information to various spectrum sharing database services. This standard specifies a device operating in the bands below 1 GHz and a second device operating from 2.7 GHz to 3.7 GHz.

Rather the the traditional IEEE approach, which has always been around a normative standard that defines what a product (device) would be, the task group is taking a “platform” approach.

Instead of designing a standard for a spectrum sensor device, we want to develop a standard for a platform that allows “spectrum sensing as a service”. It is a platform with five abstracted layers, each of which communicates by a set of standardised interfaces, to allow whoever needs the spectrum data from the sensing system’s back end to request it in the way they want, and do with it what they find useful.

This means we don’t have to pre-suppose the use case, manipulation technique, etc.

The requestor uses the platform to query a particular device's capabilities, asks for the scan to be done and data sent back to the platform, and then takes and processes the data as their application requires. Instead of mandating a hardware standard, we pack sensing data with metadata describing the device's parameters (e.g. antenna gain, amplifier/SDR noise floor, software libraries used, temperature, geolocation).

It also allows the back end system to apply policies in terms of what the sensing device may do – for e.g. not allow hi-res raw scans in sensitive bands (e.g. military radar). These policies could be imposed by a national regulator, etc in a very granular way.

  - Purpose

The purpose is to specify operating characteristics of the components of the Spectrum Characterization and Occupancy Sensing System. The intent of this standard is to create an open platform where elements of the architecture are characterised in terms of the capabilities they offer to other elements, defined through abstractions and interfaces with standardised metadata sets attached to scan data, allowing users of the data to manipulate it in independent systems according to their requirements.

  - Reference Applications

    - White Space device radio operator

Either the network operator or device operator using spectrum sensing to identify primary or other secondary users of particular channels. Spectrum sensing either built into the radio devices or standalone sensing units.

    - National regulators

National radio regulators would use a system comprising spectrum sensing devices to feed into a national spectrum utilization database for assignment management and planning purposes, and generating historical records for compliance monitoring and enforcement. Devices deployed in various scenarios:

- Fixed devices at key locations and high sites

- Mobile devices on vehicles that travel widely and can create a sample set of spectrum utilization through snapshots at time or location intervals

- Devices either at fixed locations or periodically moved to create location-based spectrum utilization datasets

- Nationally deployed in a swarm of a given device density to create real-time national spectrum utilization maps.

    - Scientific community:

Scientists using sensitive radio frequency systems (e.g. radio-telescopes) struggle with RF interference. SCOS devices can let them identify RFI and the location of their sources.

    - Law enforcement and public order

Law enforcement and other authorities are increasingly dealing with problems stemming from radio-controlled or radio-connected systems.

Illegal drone use:  These include people flying radio-controlled unmanned aerial vehicles (drones) in prohibited places. SCOS systems can be used to detect characteristic transmissions of drone operation in areas such as in the airfield flight traffic area.

Detecting jamming devices: A problem area for security staff and law enforcement is the use of radio jammers to interfere with remote control devices like vehicle keyless entry systems or radio links for alarm systems. SCOS devices can be used to identify and locate jamming systems.

Detecting unauthorized mobile phone use: Controlled and high security areas such as prisons will frequently prohibit the use of cellular phones in certain areas, but may not jam operating frequencies because of other regulations. Identifying and locating transmissions allows direct action to be taken on equipment users.

- Normative References

- Abbreviations and acronyms

RF – Radio Frequency

RFI – Radio Frequency Interference

SCOS – Spectrum Characterization and Occupancy Sensing

SSD – Spectrum Sensing Device

Spectrum Sensing Management System – SSMS

- Functional Requirements

  - Introduction

Various national regulators and government authorities are developing regulatory and policy frameworks to allow cooperative spectrum sharing approaches in order to optimize spectrum utilization. There is emphasis on greater spectrum efficiencies, spectrum sharing and spectrum utilization, which requires not only database-driven configuration of the radios, but systems that can provide spectrum occupancy at a particular location and at a particular time.

The IEEE 802.22.3 standard described in this document will help fulfil this need by creating a Spectrum Characterization and Occupancy Sensing (SCOS) system. This will improve knowledge of spectrum utilization and support shared spectrum applications, hence benefitting the regulators and users alike.

The Spectrum Occupancy Sensing (SCOS) System has many applications which include:

1. On-demand spectrum survey and report

2. Collaborative spectrum measurement and calibration

3. Labeling of systems using the spectrum

4. Spectrum planning

5. Spectrum mapping

6. Coverage analysis for wireless deployment

7. Terrain and topology - shadowing and fading analysis

8. Quantification of the available spectrum through spectrum observatories [2, 13],

9. Complement the database access for spectrum sharing by adding in-situ awareness and faster decision making.

10. Space-Time-Frequency spectrum hole identification and prediction where non-time-sensitive tasks can be performed at certain times and at certain locations, when the spectrum use is sparse or non-existent

11. Identification and geolocation of interference sources.

The Spectrum Characterization Occupancy Sensing (SCOS) systems may be deployed to characterize many bands such as VHF/UHF, L, S, C and X bands.

  - Managed objects

Anything here?

  - Regulatory requirements

This standard should provide mechanisms to meet the regulatory requirements of national operators that have defined parameters or requirements for spectrum sensing in various applications. For example, in the FCC rules for the VHF/UHF TV bands, the FCC requires a spectrum sensing detection accuracy as specified by the Table 1: FCC Sensing sensitivity requirements.

Table 1: FCC Sensing sensitivity requirements

| Regulatory domain | Type of signal | Sensing detection threshold (in dBm) | Data fusion rule for distributed sensinga | Monitoring requirements |
| --- | --- | --- | --- | --- |
| USA | ATSC | –114 (averaged over 6 MHz) | “OR” rule | Detection threshold referenced to an omni-directional receive antenna with a gain of 0 dBi |
| USA | NTSC | –114 (averaged over 100 kHz) | “OR” rule | Detection threshold referenced to an omni-directional receive antenna with a gain of 0 dBi |
| USA | Wireless microphone | –107 (averaged over 200 kHz) | “OR” rule | Detection threshold referenced to an omni-directional receive antenna with a gain of 0 dBi |

aThe value “1” indicates detection.

Other requirements for the 2.7 GHz to 3.7 GHz band shall be defined based on the evolving regulations. For example, the spectrum sensing devices in the 2.7 GHz to 3.7 GHz can sense for Radar Signals and provide that information to the Spectrum Access System (SAS) that is being defined in these bands.

  - Device classes and complexity

Following device categories may be considered:

- Energy Efficient Devices: This standard should provide mechanisms of energy efficient operations, eg. solar powered or battery operated spectrum sensors for monitoring applications.

- Small form factor devices: Devices that can fit the spectrum sensing function within a small form factor (e. g. a USB dongle, cell phone etc.)

- Advanced Spectrum Sensing Devices:  Advanced Spectrum Sensing Devices with capable Radio Frequency Front Ends (RFFE) and dedicated resources for spectrum sensing may be considered.

  - Number of devices

This standard shall support at least one Spectrum Sensing Device to cover a location or area, communicating with a back-end Spectrum Sensing Management System (SSMS), but will extend to describing an architecture and interfaces for multiple SSDs potentially communicating with multiple SSMS instances.

  - Network Topology

The network topology from the SSD point of view will be point-to-multi-point, where SSDs can transmit sensing and control data to one or more data ingest points using unicast or multicast methods.

  - Real-time applications

The sensing devices will be performing spectrum sensing functions in real time, based on the demand from the Spectrum Sensing System Manager as well as semi-autonomously based on pre-configured schedules. However, the spectrum sensing reporting may be carried out on a Best Effort (BE) basis, since the SCOS System uses any available transport mechanism (e. g. 802.11, 802.22, Ethernet, Cable, Cellular etc.).

The SCOS system will benefit if sensing reports from various sensors are provided on a reasonable time-scales (e. g. minutes) so that the information is not stale. However, this is not a mandatory requirement. Also, the messaging format may be defined such that it does not produce excessive overhead penalty on the transport layer being used.

  - Security

  - Security of Spectrum Databases and Cognitive Radio Systems

This standard may provide enhancement to existing or alternate cognitive security mechanisms which are enabled as a result of spectrum sensing. This may result in additional information to the database which includes enhanced situational awareness, as well as authentication of the protected devices.

This includes ensuring that data derived from SCOS devices and SSMS are not used as an attack vector against White Space Databases, regulator spectrum management databases, etc.

    - Inter-layer security

      - Intra-device Layers (physical interfaces)

This standard defines security mechanisms to ensure integrity of sensing chain from antenna to data store.

- Antenna to amplifier/filters: physical security of device in terms of cable/connectors (tampering such as substituting antenna, physical such as connector corrosion)

- Amplifier to SDR: cable connectors or PCB connections

- SDR to processing unit: cable connectors or PCB connections

- Enclosure for active elements: Protection against moisture, dust ingress. Screening against RFI from external sources. Screening to protect antenna elements against RFI from active elements.

      - Network Layer

Since this standard uses any available transport mechanism for data transmission, it will not recommend its own security mechanisms, but will use the existing security mechanisms of the transport mechanism being used (e.g. network 802.11 using Transport Layer Security,

    - Application Layer

Data transmissions should be secured on the application layer using mechanisms to guarantee the integrity and confidentiality of sensing and control data transmissions  …

… possibly Secure Sockets Layer?

  - Security of sensed data

This Standard shall not support mechanisms to go into the internals of the signals. For example, any kind of demodulation of the signals that may interfere with the privacy of the users shall not be not be supported. However, the SCOS system shall support sophisticated spectrum sensing methods such as cyclostationary processing that can detect signals and characterize their modulation.

  - Channelization

This standard may specify a Spectrum Manager entity that can command various sensors to go and sense in certain bands, or it may even specify the spectrum sensors to ignore certain bands from sensing.

This standard may provide one primary channelization map for all the spectrum sensors (e. g. 5 MHz).

  - Reporting to the Spectrum Sensing Management System

This standard defines interfaces to and from the Spectrum Sensing Device to the SSMS service to provide value added information back to the database on spectrum sensing device health, capabilities, activities, etc.

  - Sensor Location

The SCOS device should convey the location of the sensors to the aggregation entity such as the SSMS. This feature allows the SM or the aggregation entity to localize the proximity of the signal source location allowing more efficient spectrum management.

- System architecture

  - Reference architecture

    - Interface – Detecting

Physical interface: Interconnect from filter/amp to SDR

API: Controller to antenna motor

Metadata:	Antenna gain

Antenna type

Insertion loss (into amp)

Amplifier gain

Filter parameters

Noise metric

    - Interface – Sensing

Physical interface: N/A

API:	Scan request and parameters

Sensing-to-Packaging transfer

Metadata:	Scan parameters

SSD hardware ID/key

(Management System metadata)

(Detecting Metadata)

    - Interface – Packaging

Physical interface:	Best effort IP transport

SSL encryption

Transmission Control

API:	Packaging-to-MQ

SCOS-as-a-Service Request

Metadata:	Packaging information (time, certificates, etc)

Network information (media, connection)

(Sensing Metadata)

(Detecting Metadata)

    - Interface – Transmission

Physical interface:	Best effort IP transport

SSL encryption

Transmission Control

API:	MQ-to-Ingest

Ingest-to-Data Store

Metadata:	Package audit information

(Packaging Metadata)

(Sensing Metadata)

(Detecting Metadata)

    - Interfaces – Management

Physical interface:	Best effort IP transport

SSL encryption

Transmission Control

API:	Management-to-MQ

SCOS-as-a-Service Request

Metadata:	Management system certs

  - Management Reference Architecture

- Architecture System Requirements

- Radio energy is detected up by an antenna on Spectrum Sensing Device and transferred through an interconnect to a signal pre-conditioner containing mixers/filters/amplifier segments, according to pre-determined hardware parameters

- Signal is transferred to the SSD’s SDR to produce baseband signal, quantised by ADC and passed in digital form to be processed by an accepted sensing technique

- This sensing data is packaged along with key metadata within the SSD and stored locally for transmission. Metadata includes:
– scan time, scan duration, scan location, device identifiers…
– hardware parameters of antenna and radio front end

- The package is (a) transmitted to a remote system that (b) ingests and validates data, and (c) stores for further processing

- The back-end management system exchanges control information with SSDs

- e.g. device management, operation validation, integrity of information chain verification, maintenance tasks

    - Detecting

Radio energy is detected up by an antenna on Spectrum Sensing Device and transferred through an interconnect to a signal pre-conditioner containing mixers/filters/amplifier segments, according to pre-determined hardware parameters

    - Sensing

Signal is transferred to the SSD’s SDR to produce baseband signal, quantised by ADC and passed in digital form to be processed by an accepted sensing technique

    - Packaging

This sensing activity is packaged along with key metadata

    - Transmission

The package is transmitted to a remote system that ingests and validates data, and stores for further processing

    - Ingest

Package must be reliably, securely and scaleably received and transmitted to data store

    - Storage

Data moved into structured database

    - Management and Maintenance

The back-end management system exchanges control information with SSDs

Manage them

- Device health reports – power, temperature, location, GPS health, OS/environment health, network health, storage health, scheduled and by query

- Manage by device, group, Class of device

Validate their operation

- Run test scans against known data points (e.g. from WSDB)

Verify integrity of information chain

- Software tools to validate process chain

Perform maintenance

- Push updates to devices (software, OS, firmware, certifications)

- Perform remote reboots, resets

- Shell into device to do diagnostics

    - User Device Control

Ability for authorized user to instgruct Sensing Devices to perfrom particular scans

    - User Data Store Interrogation

Ability for authorized user to pull information from data store via API.

    - Policy Management and Enforcement

Ability to cascade policies down from master server to SSD management servers and there to SSDs to determine:

- Possible sensing techniques

- Scan parameters

- Data upload qualities from SSD to data store

- System Definitions and Interfaces

Definitions of units, measurements for each defined parameter in the standard

  - System Units and Parameters

    - Key minimum sensing parameters

    - Standard system units

  - Metadata Formats

Definition of metadata to be captured at each layer

- Security Systems

      - Threat overview

      - Authorisation, authentication, identity

      - Security design between each architecture layer

        - Physical security

        - Data transmission security

      - Security and redundancy model for data stores

Review of 802.22 sections

Areas of 802.22 that are potentially directly relevant/usable in 802.22.3

  - 5.1.3.2 Spectrum Sensing Automation

  - 5.1.3.3 Security sublayer 2

  - 7 Management messages, esp

    - 7.1.1.1.1 NMEA location

    - 7.1.1.1.2 IP version

    - 7.1.1.1.3 Spectrum sensing capability

      - Antenna model

      - Antenna gain

      - Sensor ID

      - Operational Capability

      - Registration timer (use for WSDB to SSD, not CPE/BS

    - 7.1.6 Measurement management

    - 7.2 MAC PDU

    - 7.3 Initialisation, authentication, registration

  - 8 Security sublayer

  - 9 some may be relevant in antenna, receiver spec

  - 10.1 and 2 Spectrum Manager falls out of scope – back end function

  - 10.3 SSA

  - 10.4 Spectrum Sensing Funtion

  - 10.5 Geolocation

  - 10.6 Database service

  - 10.7 Primitives largely falls outside scope (Cognitive Radio control) until 10.7.4

# (normative)
IEEE 802.22 regulatory domains and regulatory classes requirements

This annex describes the various technical parameters and specifications required by the various regulatory domains for operation of the IEEE Std 802.22 in the TV bands.

## Regulatory domains, regulatory classes, and professional installation

Table A.2 specifies the regulatory domains and licensing regime where the IEEE 802.22 systems are planned to be authorized to operate in the TV bands.

Table A.2—Regulatory domains

| Geographic area | Regulatory domain ISO 3166 (3 Bytes) | Licensing regime | Approval authority |
| --- | --- | --- | --- |
| United States | USA | Unlicensed | FCC |
| Canada | CAN | Licensed | IC |
| United Kingdom | GBR | — | OFCOM |
| — | — | — | — |

1921HTable A.3 specifies the authorized regulatory classes under their respective regulatory domains.

Table A.3—Regulatory classes

| Regulatory domain | Regulatory class and profile |  |
| --- | --- | --- |
|  | Fixed | Personal portable |
| USA | Stationary fixed | Mode I & IIa |
| CAN | Stationary fixed | N/A |
| — | — | — |

aThe behavioral limits sets for Modes I and II are defined in the FCC Report and Order. However, IEEE Std 802.22 will only operate in portable nomadic Mode II.

Table A.4 specifies the requirement for professional installation of the WRAN BS and CPEs.

Table A.4—Professional installation requirement

| Regulatory domain | Type of terminal |  | Definition of professional installer |
| --- | --- | --- | --- |
|  | BS | CPE |  |
| USA | Professionally installed | Professionally installed | A professional installer is a competent individual or team of individuals with experience in installing radio communications equipment and who normally provides service on a fee basis—such an individual or team can generally be expected to be capable of ascertaining the geographic coordinates of a site and entering them into the device for communication to a database. |
| CAN | Professionally installed | N/A | Same as for USA. |
| — | — | — | — |

## Radio performance requirements

### Sensitivity and Noise

# (informative)
Sensing

This annex contains descriptions of a number of sensing techniques. A sensing technique is an implementation of the spectrum sensing function.

…

## References

# (informative)
Bibliography

At the time of publication, the editions indicated were valid. All standards and specifications are subject to revision, and parties to agreements based on this standard are encouraged to investigate the possibility of applying the most recent editions of the references listed below.

The Institute of Electrical and Electronics Engineers, Inc.

3 Park Avenue, New York, NY 10016-5997, USA

Copyright © 2011 by the Institute of Electrical and Electronics Engineers, Inc.

All rights reserved. Published 1 July  2011. Printed in the United States of America.

IEEE and 802 are registered trademarks in the U.S. Patent & Trademark Office, owned by the Institute of Electrical and Electronics 
Engineers, Incorporated.

PDF:	ISBN 978-0-7381-6723-7	STD97146

Print:	ISBN 978-0-7381-6724-4	STDPD97146

IEEE prohibits discrimination, harassment and bullying. For more information, visit http://www.ieee.org/web/aboutus/whatis/policies/p9-26.html.

No part of this publication may be reproduced in any form, in an electronic retrieval system or otherwise, without the prior written permission of the publisher.