IEEE P802.22 Wireless RANs

| Errata – 802.22 base std, CBC-REQ/RSP Message Format errata |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2017-02-16 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Ivan Reede | AmeriSys | Montreal, Quebec, Canada | 514-620-8522 | i_reede@amerisys.com |
| Gerald Chouinard | AmeriSys | Gatineau, Quebec, Canada | 514-620-8522 | gerald@amerisys.com |
| Luc Stan | AmeriSys | Montreal, Quebec, Canada | 514-620-8522 | l_stan@amerisys.com |

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair Apurva N. Mody <apurva.mody@ieee.org> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Abstract

In our work to create test vectors and testing an implementation for compliance to the base 802.22 standard, it was discovered that there is something missing in the specification of CBC-REQ/RSP information elements related to the capabilities of the CPE modulator and demodulator covered in Tables 109 and 110. It is proposed to include the proposed changes to the IEEE Std. 802.22TM-2011.

Proposed modification to the IEEE Std 802.22TM- 2011

Needed clarification and correction to subclause 7.7.11.3.2.2

The CBC-REQ/RSP management messages allow the CPE and BS to coordinate their capabilities for PHY-specific parameters. In particular, Tables 109 and 110 allow the exchange of information regarding the CPE modulation and demodulation capabilities. This is done using the modulation/FEC index used for defining the DIUC (Table 27) and UIUC (Table 36) to which a 1 or a 0 has to be associated to indicate whether the modulator and demodulator options are supported (1) by a CPE for the downstream reception, or not supported (0).

Tables 109 and 110 only specify one parameter of 8 bits to fill this purpose. Since the DIUC and UIUC index Tables contain 64 possibilities, the specification of the support of all these options can be fully defined by 64 bits, a Table of 8 x 8 bits or, 8 bytes. Rather than qualifying the support for specific DIUC or UIUC one at the time where the 6-bit index for each DIUC or UIUC would need to be followed by the 1-bit flag indicating the support for the given DIUC or UIUC, it is proposed to send the entire 64-bit flag Table using 8 bytes.

The following modification is proposed to the text appearing in subclause 7.7.11.3.2.2.1:

“This field indicates the different demodulator options supported by a CPE for the downstream reception. A bit value of 0 indicates “not supported” while 1 indicates “supported.” The field contains 64 bits, one for each DIUC option, arranged in 8 bytes where the MSB of the first byte corresponds to DIUC option “0” and the second bit from the left corresponds to DIUC option “1”, and whereas the support flag for DIUC option “63” is given by the LSB of the eighth byte.”

Modify Table 109 as follows:

Table 109— CPE Demodulator information element

| Element ID | Length (bytes) | Value | Scope |
| --- | --- | --- | --- |
| 3 | 8 | Bit flags indicating support for all 64 DIUC options, see Table 27. | CBC-REQ, CBC-RSP |

The following modification is proposed to the text appearing in subclause 7.7.11.3.2.2.1:

“This field indicates the different modulator options supported by a CPE for the upstream transmission. A bit value of 0 indicates “not supported” while 1 indicates “supported.” The field contains 64 bits, one for each UIUC option, arranged in 8 bytes where the MSB of the first byte corresponds to UIUC option “0” and the second bit from the left corresponds to UIUC option “1”, and whereas the support flag for UIUC option “63” is given by the LSB of the eighth byte.”

Modify Table 110 as follows:

Table 110— CPE Modulator information element

| Element ID | Length (bytes) | Value | Scope |
| --- | --- | --- | --- |
| 4 | 8 | Bit flags indicating support for all 64 UIUC options, see Table 36. | CBC-REQ, CBC-RSP |

______________________