IEEE P802.22 Wireless RANs

| Errata – 802.22 base std, REG-REQ/RSP Message Format errata |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2017-02-22 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Ivan Reede | AmeriSys | Montreal, Quebec, Canada | 514-620-8522 | i_reede@amerisys.com |
| Gerald Chouinard | AmeriSys | Gatineau, Quebec, Canada | 514-620-8522 | gerald@amerisys.com |
| Luc Stan | AmeriSys | Gatineau, Quebec, Canada | 514-620-8522 | l_stan@amerisys.com |

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair Apurva N. Mody <apurva.mody@ieee.org> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Abstract

In our work to create test vectors and testing an implementation for compliance to the base 802.22 standard, it was discovered that there is something missing in the specification of the information elements of the REG-REQ/RSP management messages related to the Registration command issued by the base station in response to a Registration request received from the CPE. It is proposed to include the proposed changes to the IEEE Std. 802.22TM-2011.

Proposed modification to the IEEE Std 802.22TM- 2011

Needed correction to subclause 7.7.7.3

There are errors in a number of Tables describing the Information Elements related to the REG-REQ/RSP management messages between Tables 47 and 63. The Element IDs are erroneous in certain cases. It is proposed to make the following modifications to resolve these problems:

In subclause 7.7.7.3.2, the "Element ID" should be changed from 3 to 2 to provide a continuous incrementation of the Element IDs.

In subclause 7.7.7.3.3, the Element ID should be changed from 4 to 3 to provide a continuous incrementation of the Element IDs.

In subclause 7.7.7.3.4.1, the Element ID should be changed from 5 to 4 to provide a continuous incrementation of the Element IDs.

In subclause 7.7.7.3.4.2, the Element ID should be changed from 6 to 5 to provide a continuous incrementation of the Element IDs.

In subclause 7.7.7.3.4.3, the Element ID should be changed from 2 to 6 to provide a continuous incrementation of the Element IDs.

All following Element ID for the REG-REQ/RSP message should stay as is up to 17 that the last one in the list.

Some of these changes have already been implemented in the Tables below as part of the re-formatting of these Table.

The REG-REQ/RSP management messages exchanged between the BS and a CPE to get access to the network will contain a number of Information Elements (IEs) selected from the list given in Tables 47 to 63. Unlike in many management messages where all the listed IEs are contained in the message, not all of these IEs will be sent in a same REG-REQ/RSP message.

Since specific IEs will be selected from the list for different purposes, the Element ID for each information element will need to be transmitted before each of these information elements so that the CPE is informed on the one that is transmitted next to avoid any ambiguity, it is proposed to modify the following Tables as shown below to indicate the transmission of the specific Element ID using 8 bits before the content of this Information Element. Note that Tables 46, 47, 56 and 58 are already in the proper format.

Table 48— Convergence Sublayer Specification information element

| Syntax | Size | Description | Scope |
| --- | --- | --- | --- |
| Convergence Sublayer Specification () { |  |  | REG-REQ, REG-RSP |
| Element ID | 1 byte | 2 |  |
| Specification | 1 byte | 0x00: Ethernet CS 0x01: IP CS 0x02–0xFF: Reserved |  |
| } |  |  |  |

Table 49— IP Version information element

| Syntax | Size | Description | Scope |
| --- | --- | --- | --- |
| IP Version () { |  |  | REG-REQ, REG-RSP |
| Element ID | 1 byte | 3 |  |
| Specification | 1 byte | 0x00: IPv4 0x01: IPv6 |  |
| } |  |  |  |

Table 50— IP ROHC Support information element

| Syntax | Size | Description | Scope |
| --- | --- | --- | --- |
| IP ROHC Support () { |  |  | REG-REQ, REG-RSP |
| Element ID | 1 byte | 4 |  |
| Specification | 1 byte | 0x00: No IP ROHC support 0x01: IP ROHC supported 0x02–0xFF: Reserved |  |
| } |  |  |  |

Table 51— ARQ Support information element

| Syntax | Size | Description | Scope |
| --- | --- | --- | --- |
| ARQ Support () { |  |  | REG-REQ, REG-RSP |
| Element ID | 1 byte | 5 |  |
| Specification | 1 byte | 0x00: No ARQ support 0x01: ARQ supported on Secondary Management flow only 0x02: ARQ supported on Transport flows only 0x03: ARQ supported on Secondary Management and Transport flows 0x04–0xFF: Reserved |  |
| } |  |  |  |

Table 52— ARQ information element

| Syntax | Size | Description | Scope |
| --- | --- | --- | --- |
| ARQ () { |  |  | REG-REQ, REG-RSP |
| Element ID | Variable | 6 |  |
| Specification | 1 byte | Compound |  |
| } |  |  |  |

Table 53— DSx Flow Control information element

| Syntax | Size | Description | Scope |
| --- | --- | --- | --- |
| DSx Flow Control () { |  |  | REG-REQ, REG-RSP |
| Element ID | 1 byte | 7 |  |
| Specification | 1 byte | 0: no limit (default) 1–255: indicate maximum concurrent transactions |  |
| } |  |  |  |

Table 54— MCA Flow Control information element

| Syntax | Size | Description | Scope |
| --- | --- | --- | --- |
| MCA Flow Control () { |  |  | REG-REQ, REG-RSP |
| Element ID | 1 byte | 8 |  |
| Specification | 1 byte | 0: no limit (default) 1–255: indicate maximum concurrent transactions |  |
| } |  |  |  |

Table 55— Maximum number of Multicast Groups supported information element

| Syntax | Size | Description | Scope |
| --- | --- | --- | --- |
| Maximum number of Multicast Groups supported () { |  |  | REG-REQ, REG-RSP |
| Element ID | 1 byte | 9 |  |
| Specification | 1 byte | 0–255 0 (default) |  |
| } |  |  |  |

In Table 56, change the name of the IE from “MS_IE()” to “Management Support()”.

Table 57— Manufacturer-specific Antenna Model information element

| Syntax | Size | Description | Scope |
| --- | --- | --- | --- |
| Manufacturer-specific Antenna Model () { |  |  | REG-REQ, REG-RSP |
| Element ID | 1 byte | 11 |  |
| Specification | 2 bytes | Data stream identifying the antenna model |  |
| } |  |  |  |

In Table 58, change the column name “Notes” to “Description” to align with the other Tables and change the name of the IE from “AG IE()” to “CPE Antenna Gain()”.

Table 59— CPE Residual Delay information element

| Syntax | Size | Description | Scope |
| --- | --- | --- | --- |
| CPE Residual Delay () { |  |  | REG-REQ, REG-RSP |
| Element ID | 1 byte | 13 |  |
| Specification | 3 bytes | CPE Residual Delay Signed integer representing the CPE residual delay in nano-seconds. Positive value indicates a delay. Negative value indicates an advance. Accuracy shall be within ± 30 ns for 10 m distance accuracy. |  |
| } |  |  |  |

Table 60— Method for allocating IP addresses on secondary management connections

| Syntax | Size | Description | Scope |
| --- | --- | --- | --- |
| IP addresses on secondary management connections () { |  |  | REG-REQ, REG-RSP |
| Element ID | 1 byte | 14 |  |
| Specification | 1 byte | Bit 0: DHCPv4 Bit 1: Mobile IPv4 Bit 2: DHCPv6 Bit 3: IPv6 Stateless Address Autoconfiguration Bit 4–7: Reserved, shall be set to 0. |  |
| } |  |  |  |

Table 61— Permanent Station ID information element

| Syntax | Size | Description | Scope |
| --- | --- | --- | --- |
| Permanent Station ID () { |  |  | REG-REQ |
| Element ID | 1 byte | 15 |  |
| Specification | 2 bytes | Permanent SID (Bit 0000 000b bbbb bbbb) |  |
| } |  |  |  |

Table 62— CPE Operational Capability information element

| Syntax | Size | Description | Scope |
| --- | --- | --- | --- |
| CPE Operational Capability () { |  |  | REG-REQ, REG-RSP |
| Element ID | 1 byte | 16 |  |
| Specification | 1 byte | 0x00- Fixed 0x01: Portable 0x02–0xFF: Reserved |  |
| } |  |  |  |

Table 63— CPE Registration Timer information element

| Syntax | Size | Description | Scope |
| --- | --- | --- | --- |
| CPE Registration Timer () { |  |  | REG-REQ, REG-RSP |
| Element ID | 1 byte | 17 |  |
| Specification | 2 bytes | = 0x0000, Reserved = 0x0001–0xFFFF, in units of 160 ms |  |
| } |  |  |  |

______________________