IEEE P802.22 Wireless RANs

| Clarification and modification related to Station ID and Flow ID in the 802.22 base standard |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2017-03-03 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Ivan Reede | AmeriSys | Montreal, Quebec, Canada | 514-620-8522 | i_reede@amerisys.com |
| Gerald Chouinard | AmeriSys | Gatineau, Quebec, Canada | 514-620-8522 | gerald@amerisys.com |
| Luc Stan | AmeriSys | Gatineau, Quebec, Canada | 514-620-8522 | l_stan@amerisys.com |

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair Apurva N. Mody <apurva.mody@ieee.org> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Abstract

In our work to create test vectors and testing an implementation for compliance to the base 802.22 standard, it was discovered that some clarifications are required for the use of the Station ID (SID) and Flow ID (FID) of the IEEE Std. 802.22TM-2011. Some modifications are proposed to the subclause 12.2 of the standard to resolve this issue.

Proposed modification to the IEEE Std 802.22TM- 2011

Needed correction to subclause 12.2

Modification of Table 277

In Table 277, the ranges of values are defined for the assignment of the 9-bit field to different types of Station IDs. However, no null SID has been reserved for system testing purposes as is usual in many other digital transmissions systems. It is proposed to include this null SID to which no CPE would listen to with the value of 0x000 while the “Cell SID” to which all active CPEs would listen to would be represented by all 1’s in the 9-bit field (0x1FF). Table 277 should be modified to appear as follows:

Table 277— Station ID allocation

| SID | Value | Description |
| --- | --- | --- |
| Null | 0x000 | Cell-wide SID reserved for system testing to which no CPE should respond |
| Unicast | 0x001 – m | SID for individual CPEs |
| Multicast | m+1 – 0x1FE | SID for multicast groups |
| Cell | 0x1FF | Cell-wide SID reserved for broadcast and initial ranging to which all CPEs should respond |

Modification of Table 278

In order to allow the 802.22 standard to broadcast data streams to all its CPEs, giving the possibility to, for example, broadcast sound and video programs, the four data flow IDs used for unicast and multicast should also be included in Table 278 for Cell SIDs. In Table 278, should be modified to appear as follows:

Table 278— Flow ID allocation for Cell SID

| FID | Value | Description |
| --- | --- | --- |
| Broadcast | 000 | Traffic on broadcast connection mapped to the Broadcast Flow assigned to Cell SID |
| Initial Ranging | 001 | Traffic on initial ranging connection mapped to the Initial Ranging Flow assigned to Cell SID |
| BE | 010 | Data sent on Cell transport connection using the BE scheduling class mapped to BE Flow assigned to Cell SID |
| nrtPS | 011 | Data sent on Cell transport connection using the nrtPS scheduling class mapped to nrtPS Flow assigned to Cell SID |
| rtPS | 100 | Data sent on Cell transport connection using the rtPS scheduling class mapped to the rtPS Flow assigned to Cell SID |
| UGS | 101 | Data sent on Cell transport connection using the UGS scheduling class mapped to the UGS Flow assigned to Cell SID |
| Reserved | 110-111 | Reserved for future use |

Modification of Table 279

The minor augmentation to Table 279 is needed to align the text with the other Table of the subclause. In Table 278, should be modified to appear as follows:

Table 279— Flow ID allocation for Unicast SID

| FID | Value | Description |
| --- | --- | --- |
| Basic | 000 | Traffic on basic connection mapped to the Basic Flow assigned to a CPE SID |
| Primary Management | 001 | Traffic on primary management connection mapped to Primary Management Flow assigned to a CPE SID |
| BE | 010 | Data sent on unicast transport connection using the BE scheduling class mapped to BE Flow assigned to a CPE SID |
| nrtPS | 011 | Data sent on unicast transport connection using the nrtPS scheduling class mapped to nrtPS Flow assigned to a CPE SID |
| rtPS | 100 | Data sent on unicast transport connection using the rtPS scheduling class mapped to the rtPS Flow assigned to a CPE SID |
| UGS | 101 | Data sent on unicast transport connection using the UGS scheduling class mapped to the UGS Flow assigned to a CPE SID |
| Reserved | 110 | Flow ID reserved for future use |
| Secondary Management | 111 | Traffic on secondary management connection mapped to Secondary Management Flow assigned to a CPE SID |

Modification of Table 280

There is an error in naming Table 280. Its title should be modified to read as follows:

Table 280— Flow ID allocation for Multicast SID.

______________________