IEEE P802.22 Wireless RANs

| Rationalization of the carrier encoding levels in the 802.22 base standard |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2017-05-04 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Ivan Reede | AmeriSys | Montreal, Quebec, Canada | 514-620-8522 | i_reede@amerisys.com |
| Gerald Chouinard | AmeriSys | Gatineau, Quebec, Canada | 514-620-8522 | gerald@amerisys.com |
| Luc Stan | AmeriSys | Montreal, Quebec, Canada | 514-620-8522 | l_stan@amerisys.com |

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair Apurva N. Mody <apurva.mody@ieee.org> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Abstract

In our work to create test vectors and testing an implementation for compliance to the base 802.22 standard, it was discovered that, unless specific ratios between modulation levels based on simple integer numbers are included in the Standard, the equipment manufacturers will need to use a large number of encoding bits for the amplitude levels currently specified for the transmitted carriers to sufficiently approach the values of the irrational numbers resulting from the scaling factors specified in Table 226 and the boosting levels specified in Table 26. It is proposed to include a new sub-clause containing a Table of normalization factors corresponding to simpler integer number ratios for the various modulations and boosting levels to allow for a reduction of the number of bits required while avoiding the presence of systemic noise levels that would be induced by the use of different levels approaching the theoretical values as currently specified in sub-clauses 7.7.2.1 and 9.8.1 of the IEEE Std. 802.22TM-2011.

Proposed modification to the IEEE Std 802.22TM- 2011

Inclusion of a new sub-clause 9.8.3

At the end of sub-clause 9.8.1, the modulation dependent normalization factor KMOD is introduced to scale the amplitude of the frequency carriers for equivalent power for the QPSK, 16-QAM and 64-QAM modulations used for data transmission. Unfortunately, these scaling factors are introducing irrational numbers that are difficult to represent by simple binary coded values. In order to avoid the excessive number of bits that would be required in practice to approximate these irrational numbers and the likelihood of systemic errors that would result due to different approximations used by different manufacturers in their implementation, there is a need to specify simpler normalization factors corresponding to simpler integer number ratios for the various modulations and boosting levels.

Also, this KMOD is also specified in sub-clause 9.8.2 as being equal to 1 for the BPSK modulation of the pilot carriers.

Furthermore, in order to keep a consistent power transmission across all types of symbols, including the training symbols, the amplitude of the carriers in the LTS and the STS symbols need to be specified. Since one carrier over 4 will be used in the STS symbol, each carrier can be 4 times as powerful as the un-boosted data carrier.  Its amplitude therefore needs to be boosted by a factor of 6 dB, i.e., 2 in the simpler rational voltage ratio proposed for Table 26 in another contribution. Similarly, since one carrier over 2 will be used in the LTS symbol, each carrier can be 2 times as powerful as the un-boosted data carrier. In this case, its amplitude needs to be boosted by a factor of 3 dB, i.e., 3/2 in the simpler rational voltage ratio.

It is felt that a new Table is needed to summarize all these normalization factors for the frequency carrier amplitude. It is therefore proposed to add a new sub-clause 9.8.3 following the current sub-clause 9.8.2 to explain this rationalization of the carrier amplitude normalization factors for all different modulation constellations and boosting levels used in the IEEE Std. 802.22TM-2011 and to insert a Table that proposes simple values for these normalization factors based on rational numbers.

The following new sub-clause 9.8.3 should be inserted following the current sub-clause 9.8.2 of the Standard:

- 9.8.3	Normalized carrier amplitude levels

The carrier amplitude scaling factors specified for various modulations (9.8.1) and boosting levels (7.7.2.1) result in many cases in irrational numbers that are difficult to represent by simple binary coded values. In order to avoid the excessive number of bits that would be required in practice to approximate these irrational numbers and the likelihood of systemic errors that would result due to different approximations used by different manufacturers in their implementation, specific normalized values are given in Table 228 to simplify the coding of these carrier amplitudes and avoid any such systemic errors.

Table 228 — Normalized carrier amplitude values for all modulation and boosting levels used in the Standard

| BOOSTING |  | BPSK STS | BPSK LTS | BPSK Pilots | QPSK Data | 16-QAM Data |  | 64-QAM Data |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 1 1 | 3 | --- | --- | --- | 162 | 72 | 216 | 36 | 108 | 180 | 252 |
| 1 1 0 | 2 | --- | --- | --- | 108 | 48 | 144 | 24 | 72 | 120 | 168 |
| 1 0 1 | 3/2 | --- | --- | --- | 81 | 36 | 108 | 18 | 54 | 90 | 126 |
| 1 0 0 | 1 | 156 | 117 | 78 | 54 | 24 | 72 | 12 | 36 | 60 | 84 |
| 0 1 1 | 2/3 | --- | --- | --- | 36 | 16 | 48 | 8 | 24 | 40 | 56 |
| 0 1 0 | 1/2 | --- | --- | --- | 27 | 12 | 36 | 6 | 18 | 30 | 42 |
| 0 0 1 | 1/3 | --- | --- | --- | 18 | 8 | 24 | 4 | 12 | 20 | 28 |
| 0 0 0 | 0 | --- | --- | --- | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

Renumber the following Tables accordingly.

________