IEEE P802.22 Wireless RANs

| Modification of the basic sampling frequency |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2017-08-10 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Ivan Reede | AmeriSys | Montreal, Quebec, Canada | 514-620-8522 | i_reede@amerisys.com |
| Gerald Chouinard | AmeriSys | Gatineau, Quebec, Canada | 514-620-8522 | gerald@amerisys.com |
| Luc Stan | AmeriSys | Montreal, Quebec, Canada | 514-620-8522 | l_stan@amerisys.com |

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair Apurva N. Mody <apurva.mody@ieee.org> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Abstract

In our work to create test vectors and testing an implementation for compliance to the base 802.22 standard, it was discovered that the basic sampling frequencies are not easily amenable to PLL lock for the base stations to synchronize with the reference 10 MHz as stated in sub-clause 7.23.

It is proposed to align the basis sampling frequencies specified in the IEEE Std. 802.22TM-2011 to this effect.

Proposed modification to the IEEE Std 802.22TM- 2011

Modification of the basic sampling frequency

Modification of the base sampling frequency used in the IEEE Std. 802.22TM-2011, sub-clause 9.1.2.1 (6.856 MHz for 6 MHz channel bandwidth as indicated in Table 199) for 6.8 MHz is proposed to align with a proper processing clock for the treatment of the OFDM symbols and allow for the base station to use simple PLL lock with the reference 10 MHz clock available, for example, from the GPS. This will, among other things, have the effect of slightly reducing the signal corner pass frequency to 2.789 MHz, leaving a band of 211 kHz for the transition band to meet the filtering rejection requirement at the edges of the channel to allow for somewhat more easily meeting the tight FCC adjacent channel rejection mask at 3 MHz. However, this lower 6.8 MHz frequency sampling would normally result in this sampling frequency being closer to the Nyquist frequency, thus a very demanding gap of 1.222 MHz (6.8-2*2.789) for the filtering to achieve the necessary rejection against aliasing, but the use of over-sampling will make it easier. Similar rational is applied to the other 7 MHz and 8 MHz bandwidth systems. It is proposed to modify Tables 199, 200 and 203 accordingly.

The following modifications should be implemented in the following sub-clauses of the Standard:

a) In sub-clause 9.1.2.1, modify the Table 199 as follows:

|  | 6 MHz based channels | 7 MHz based channels | 8 MHz based channels |
| --- | --- | --- | --- |
| Basic sampling frequency, FS (MHz) | 6.8 | 7.9375 | 9.0625 |
| Inter-carrier spacing, f (Hz) = FS/2048 | 3.320… | 3.875… | 4.425… |
| FFT/IFFT period, TFFT (s)=1/f | 301.176… | 258.015… | 225.986… |
| Time Unit (ns) TU=TFFT/2048 | 147.058… | 125.984… | 110.344… |

a) In sub-clause 9.1.2.2 modify the Table 200 as follows:

|  |  | CP = TFFT/32 | CP = TFFT/16 | CP = TFFT/8 | CP = TFFT/4 |
| --- | --- | --- | --- | --- | --- |
| TSYM = TFFT + TCP (s) | 6 MHz | 310.588 | 320 | 338.823… | 376.470… |
|  | 7 MHz | 266.078… | 274.141… | 290.267… | 322.519… |
|  | 8 MHz | 233.048… | 240.110… | 254.234… | 282.482… |

b) In sub-clause 9.4, modify Table 203 as follows:

| Cyclic Prefix | Number of symbols per frame 1 | Transmit-receive turnaround gap 2 (TTG) | Receive-transmit turnaround gap 3 (RTG) |
| --- | --- | --- | --- |
| BW | 6 MHz 7 MHz 8 MHz | 6 MHz 7 MHz 8 MHz | 6 MHz 7 MHz 8 MHz |
| 1/4 | 24 29 33 | 1428 TU 1666 TU 1903 TU | 2572TU 909 TU 1682 TU |
| 1/8 | 27 33 37 | 1428 TU 1666 TU 1903 TU | 1548 TU 1165 TU 658 TU |
| 1/16 | 29 34 39 | 1428 TU 1666 TU 1903 TU | 524 TU 781 TU 914 TU |
| 1/32 | 30 35 40 | 1428 TU 1666 TU 1903 TU | 204 TU 781 TU 1234 TU |

Reference:

22-06-0264-15-0000_OFDMA_Parameters.xls

________