IEEE P802.22 Wireless RANs

| More flexible RF channel definition |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2017-08-11 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Ivan Reede | AmeriSys | Montreal, Quebec, Canada | 514-620-8522 | i_reede@amerisys.com |
| Gerald Chouinard | AmeriSys | Gatineau, Quebec, Canada | 514-620-8522 | gerald@amerisys.com |
| Luc Stan | AmeriSys | Montreal, Quebec, Canada | 514-620-8522 | l_stan@amerisys.com |

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair Apurva N. Mody <apurva.mody@ieee.org> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Abstract

In our work to create test vectors and testing an implementation for compliance to the base 802.22 standard, it was discovered that the current regular RF channel raster definition could be extended to allow for more precise positioning of the WRAN channel in the RF spectrum. It is proposed to insert some text in clause 9 to include a possible variation to the current regular RF channel raster in the IEEE Std. 802.22TM-2011 to allow more flexibility use of the spectrum.

Proposed modification to the IEEE Std 802.22TM- 2011

Inclusion of a more flexible definition of the RF channels in clause 9

It was discovered that the current regular RF channel raster definition could be extended to allow for more flexible positioning of the WRAN channel in the RF spectrum if the occupied channel of a WRAN system is defined by the two channel edge frequencies. This way, the WRAN channel could be anywhere within an RF spectrum. Example use cases are where a system could scale the sampling frequency (see contribution ZZZ), or overlap channels, allowing, for example, a separation of a fraction of a regular channel from a DTV or from another WRAN channel. This could be put to use to avoid adjacent channel interference. It could also be advantageous  in maximizing the use of  the RF spectrum, for example allowing the operation of a network  where not 3, but only 2 subsequent channels are vacant all the while keeping a 3 MHz guard band to other transmitters. (This could be explained by the following example: if channels x and x+3 are in use by other systems, operating the WRAN on channel x+1 may cause interference to a receiver tuned to channel x, operating the WRAN on channel x+2 may cause interference to a receiver tuned to channel x+3, while operating the WRAN on channel x+1.5 may not cause interference to either receivers on channel x or on channel x+3).

Modifications to the Standard may be required to replace or extend the RF channel numbering currently used for the regular, regulatory dependent raster by allowing for a finer granularity. The Standard already uses 8 bits for specifying RF channels which gives a maximum of 256 steps, giving some leeway for such extension.

It is proposed, as a first step to insert some text in clause 9 to include such variation to the current regular RF channel raster in the IEEE Std. 802.22TM-2011 to allow for more flexible use of the spectrum. This proposal also intends to harmonize the 802.22 std with the approved WiFAR PHY layer changes.

Insert the following paragraph as the third paragraph of clause 9:

More flexible variation of the regular frequency raster (the FCC currently only uses channels 2 to 51 for broadcasting) is possible if the occupied channel of a WRAN system is defined by the two channel edge frequencies. This way, the WRAN can operate anywhere within a regular channel in the case of a system using a scaled sampling frequency (see 9.1.4), or overlapping two regular channels, allowing, for example, a separation of a fraction of a regular channel from a DTV or from another WRAN channel to precisely control the adjacent channel interference while maximizing the use of the RF spectrum. Channel designations, mostly excess channel designations (outside those used by local regulatory bodies) may be redefined by the Base Station to these flexible channel extensions.

________