IEEE P802.22
Wireless RANs

| MIB & Management Plane Procedure Updates |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2018-06-04 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Ranga Reddy | N/A |  | 732-693-5812 | Ranga.reddy@me.com |

MIB & Management Plane Procedure Updates

Ranga Reddy (N/A)

This document (and subsequent revisions) will serve as an outline for the additional MIBs and management plance procedures (MPPs).  This document is organized as follows:

- (I) updates to section 14.1.x/14.2.x

- (II) additions to the C-SAP in 15.2.4

For each new section added to 14.1.x, MIB definitions in ASN.1 encoding format will be added to the corresponding section in 14.2.x

Revision 0: Initial outline of new items to be added in each section

# Structure of Modifications to Sections 14.1.x/14.2.x

### 14.1.8 wranIfABsMib (A-BS MIB Group)

### 14.1.8.1 wranIfABsFm

Fault Management

### 14.1.8.1.1 wranIfABsTrapControl

(bitmap to enable/disable notifications)

### 14.1.8.1.2 wranIfABsTrapDefinition

(definition of notifications)

### 14.1.8.1.3 wranIfABsNotificationObjectsTable

(definition of information captured with each notification)

### 14.1.8.1.4 wranIfABsFmMibGroups

(description of what MIB groups are part of this MIB definition)

### 14.1.8.1.5 wranIfABsFmMibCompliance

(description of which MIB objects are mandatory/optional with regard to compliance of using wranIfABsFm of information captured with each notification)

### 14.1.8.2 wranIfABsCm

Configuration Management

### 14.1.8.2.1 wranIfABsCpeBasicCapabilityReqTable

Contents of rx’d CBC-REQs (A-CPE related IEs)

### 14.1.8.2.2 wranIfABsCpeBasicCapabilityRspTable

Contents of Tx’d CBC-RSPs (A-CPE related IEs)

### 14.1.8.2.3 wranIfABsBasicCapabilitiesDefTable

Contents of defaults A-BS to use for CBC-RSPs (A-CPE related IEs)

### 14.1.8.2.4 wranIfABsCpeBasicCapabilitiyCmn

defines objects common to 14.1.8.2.1, 14.1.8.2.2, 14.1.8.2.3

### 14.1.8.2.5 wranIfABsGraConfigTable

Configuration of all current group resource configurations

### 14.1.8.2.6 wranIfABsMultiChnConfigTable

Configuration of all CHU/CAM configurations

### 14.1.8.2.7 wranIfABsRelayConfigTable

Configuration of all DS/US zone configuration configurations

### 14.1.8.2.8 wranIfABsPhy

MIB group related to PHY operation

### 14.1.8.2.8.1 wranIfABsOfdmaPhyUsChannelTable

Table that defines attributes of US channels

### 14.1.8.2.8.2 wranIfABsOfdmaPhyDsChannelTable

Table that defines attributes of DS channels

### 14.1.8.2.8.3 wranIfABsOfdmaUsRegionTable

Table that defines configuration of US subframe

### 14.1.8.2.9.4 wranIfABsOfdmaDsRegionTable

Table that defines configuration of DS subframe

### 14.1.8.2.9 wranIfABsCmMibGroups

(description of what MIB groups are part of this MIB definition)

### 14.1.8.2.10 wranIfABsCmMibCompliance

(description of which MIB objects are mandatory/optional with regard to compliance of using wranIfABsCm of information captured with each notification)

### 14.1.8.3 wranIfABsPm

Performance Management

### 14.1.8.3.1 wranIfABsPmConfigurationTable

Configuration of which performance metrics tables are enabled

### 14.1.8.3.2 wranIfABsArqMetricsTable

Two-Link ARQ metrics

### 14.1.8.1.9 wranIfABsPmMibGroups

(description of what MIB groups are part of this MIB definition)

### 14.1.8.1.10 wranIfABsPmMibCompliance

(description of which MIB objects are mandatory/optional with regard to compliance of using wranIfABsPm of information captured with each notification)

### 14.1.9 wranIfACpeMib (A-CPE MIB Group)

### 14.1.9.1 wranIfACpeConfigurationTable

Table containing current values of configuration related to A-CPE operation

### 14.1.9.2 wranIfACpeTrapControl

(bitmap to enable/disable notifications)

### 14.1.9.3 wranIfACpeTrapDefinition

(definition of notifications)

### 14.1.9.4 wranIfACpeNotificationObjectsTable

(definition of information captured with each notification)

### 14.1.9.1.5 wranIfACpeMibGroups

(description of what MIB groups are part of this MIB definition)

### 14.1.9.1.6 wranIfACpeMibCompliance

(description of which MIB objects are mandatory/optional with regard to compliance of using wranIfACpe of information captured with each notification)

# Structure of Modifications to Sections 15.2.4.x

### 15.2.4.7 Group Resource Allocation Management Primitives

Primitves related to using GRA allocation

### 15.2.4.7.1 C-GROUP-RESOURCE-ALLOC-CONFIG

Setting up GRA configuration at CPE

### 15.2.4.7.2 C-GROUP-RESOURCE-ALLOC-UPDATE

Updating  GRA configuration at CPE

### 15.2.4.8 Relay Mode Primitives

Primitves related to using Relay Mode an A-CPE can be configured for

### 15.2.4.8.1 C-RELAY-MODE-TYPE-CONFIG

Setting up the relaying mode used configuration at an A-CPE

### 15.2.4.8.2 C-RELAY-MODE-TYPE-CONFIRM

Confirmation of relaying mode configuration at an A-CPE

### 15.2.4.8.3 C-RELAY-ZONE-CONFIRM

Configuring zones at a relay A-CPE

### 15.2.4.8.4 C-RELAY-TEST-REQUEST

Request transmission of test burst

### 15.2.4.8.5 C-RELAY-TEST-RESPONSE

Response to test burst request

### 15.2.4.8.6 C-RELAY-TEST-REPORT

Transmssion of test burst analysis report

### 15.2.4.8.7 C-RELAY-TEST-CONFIRM

Confirmation of the reception of test request message

### 15.2.4.8.8 C-RELAY-SCHEDULE-UPDATE

Confirmation of relaying zone configuration at an A-CPE

### 15.2.4.9 Multi-channel Primitives

Primitves related to using multi-channel (i.e. CHU/CAM) configration

### 15.2.4.9.1 C-MULTI-CHANNEL-CAM-ADD

Setting up a CAM to use at A-CPE

### 15.2.4.9.2 C-MULTI-CHANNEL-CAM-STOP-REQUST

Request an A-CPE to stop using a CAM

### 15.2.4.9.3 C-MULTI-CHANNEL-CAM-STOP-CONFIRM

A-CPE confirmation of BS request to stop using a CAM

### 15.2.4.9.4 C-MULTI-CHANNEL-CAM-SWITCH-REQUEST

Request an A-CPE to switch the active CAM

### 15.2.4.9.5 C-MULTI-CHANNEL-CAM-SWITCH-CONFIRM

A-CPE confirmation of BS request to switch the active CAM being used

- add new section 15.2.4.7 “Group Resource Allocation Primitives” after section 15.2.4.6

---start of text modification---

      - Group Resource Allocation Management Primitives

Group Resource Allocation management primitives pertain to monitoring and managing information regarding setting up group resource allocation (GRA) configurations for groups of CPEs. GRA configurations help reduce (MAP) overhead from groups of CPEs operating in the same PHY Mode and a fixed payload size.

There are three multicast management primitives: C-GROUP-RESOURCE-ALLOC-CONFIG, and C-GROUP-RESOURCE-ALLOC-UPDATE.

        - C-GROUP-RESOURCE-ALLOC-CONFIG

          - Purpose

Add CPE(s) to a GRA configuration.

          - SAP type

C-SAP

          - Operation type

Information Confirmation

          - Destination

CPE

          - Data

Table xxx—C-GROUP-RESOURCE-ALLOC-CONFIG parameters

| Name | Type | Length | Description |
| --- | --- | --- | --- |
| Transaction ID | Integer | 16 bits | Identifier of GRA configuration transaction, selected by the BS. |
| Device Bitmap Size | Integer | 4 bits | Bitmap indicating the configuration of the maximum # of CPEs that can be added to a GRA group |
| SID Bitmap | Integer | N*13 bits | Bitmap/List of SIDs of CPEs being added to group, where N equal to configuration option specified by “Device Bitmap Size” |
| GID | Integer | 10 bits | ID of GRA group. |
| GRA Group Type | Integer | 1 bits | 0: Fixed Group 1: Portable or Mobile Group |
| Group Location | Integer | 48 bits | Latitude/Longitude of new group |
| Padding |  | Variable | Depending on total size of message, all set to 0. |

          - When generated

- BS sends this primitive when multiple CPEs are assigned to the same GRA configuration. One copy of this primitive is sent to each CPE it wants to add to the GRA.

Information in this primitive is carried to the CPE or CPEs in question via the GRA-CFG message defined in 7.7.30.1.

          - Effect of receipt

When CPE receives this primitive (via the GRA-CFG MAC management message), it takes the contents and configures GRA group configuration and begins monitoring DS-/US-MAPs for DRZ GRA Map IEs (see 7.7.2.1.2.5, 7.7.4.1.4.5) for allocations in the DS/US. The Backup/Candidate Channel list can be updated on a group basis, as defined by the procedure in 7.25.

        - C- GROUP-RESOURCE-ALLOC-UPDATE

          - Purpose

Remove a CPE from a GRA group.

          - SAP type

C-SAP

          - Operation type

Information Confirmation

          - Destination

CPE

          - Data

Table xxx—C-GROUP-RESOURCE-ALLOC-UPDATE parameters

| Name | Type | Length | Description |
| --- | --- | --- | --- |
| Transaction ID | Integer | 16 bits | Identifier of Multicast group setup transaction, selected by the BS. |
| Device Flag | Integer | 1 bit | 1bit flag to indicate whether or not device info in GRA group is being changed |
| Group Flag | Integer | 1 bit | 1bit flag to indicate whether or not group info (i.e. location) for GRA group is being changed |
| If(Device Flag ==1) { |  |  |  |
| GID | Integer | 10 bits | ID of GRA group |
| Number of Added Devices | Integer | 7 bits | # of devices being added to group |
| Number of Deleted Devices | Integer | 7 bits | # of devices being deleted from group |
| For(i=0;i<Number of Added Devices;i++) { |  |  |  |
| SID | Integer | 13 bits | SID of CPE added to group |
| Device Bitmap Index | Integer | 7 bits | New index of device in GRA group’s device bitmap |
| } |  |  |  |
| For(i=0;i<Number of Deleted Devices;i++) { |  |  |  |
| SID | Integer | 13 bits | SID of CPE added to group |
| } |  |  |  |
| } |  |  |  |
| If(Group Flag==1) { |  |  |  |
| New Group Location | Integer | 48 bit | New Group Location |
| } |  |  |  |
| Padding |  | Variable | Depedent on setting of “Device Flag” and “Group Flag”, as well as total # of Deleted/Added devices, all bits set to 0. |

          - When generated

- BS sends this primitive when it wants to remove a CPE from a GRA group configuration.

- BS sends this primitive when it wants to add a CPE to an existing GRA group configuration.

- BS sends this primitive when it wants to update location of existing GRA group.

Information in this primitive is carried to the CPE or CPEs in question via the GRA-UPD message defined in 7.7.30.2.

          - Effect of receipt

When CPE receives this primitive (via the GRA-CFG MAC management message), and has been instructed that it is being removed from the group, it stops monitoring DS-/US-MAPs for DRZ GRA MAP IEs (see 7.7.2.1.2.5, 7.7.4.1.4.5) and processing its’ allocations. CPEs that received the primitive and are being newly added to group, will take the contents and configures GRA group configuration and begins monitoring DS-/US-MAPs for DRZ GRA Map IEs for allocations in the DS/US. The Backup/Candidate Channel list can be updated on a group basis, as defined by the procedure in 7.25.

---end of text modification---

- add new section 15.2.4.8 “Relay Mode Primitives” after section 15.2.4.7

---start of text modification---

      - Relay Mode Primitives

Relay Mode primitives pertain to monitoring and managing information regarding configuring an A-CPE to operate in relay mode, i.e. as Centralized or Distriubted Scheduling CPE. There are 8 relay mode primitives: C-RELAY-MODE-TYPE-CONFIG, C-RELAY-MODE-TYPE-CONFIRM, C-RELAY-ZONE-CONFIG, C-RELAY-ZONE-CONFIRM, C-RELAY-TEST-REQUEST, C-RELAY-TEST-RESPONSE, C-RELAY-TEST-CONFIRM, and C-RELAY-SCHEDULE-UPDATE.

        - C-RELAY-MODE-TYPE-CONFIG

          - Purpose

Request configuration of Relaying Mode (e.g. either none, Centralized Scheduling, Distributed).

          - SAP type

C-SAP

          - Operation type

Information Request

          - Destination

BS

          - Data

Table xxx—C-RELAY-MODE-TYPE-CONFIG parameters

| Name | Type | Length | Description |
| --- | --- | --- | --- |
| Relay Mode | Integer | 8 bits | Bitmap. Bit==0 mode not supported, Bit==1 mode is supported Bit0: No Relaying Bit1: enable/disable Centralized Scheduling Bit2: enable/disable Distributed Scheduling Bit3-7: Reserved, set to 0 |

          - When generated

- A-CPE sends this primitive when entering/re-entering the network and request relay mode operation.

Information in this primitive is carried to the A-BS via the “Relay CPE Mode” CBC-/REG-REQ IE message defined in 7.7.11.3.4.

          - Effect of receipt

When A-BS receives this primitive (via the CBC-REQ MAC management message), it takes the contents and configures Relay Mode type and triggers the C-RELAY-MODE-TYPE-CONFIRM primitive (i.e. configures relay mode via “Relay CPE Mode” IE in CBC-/REG-RSP.

        - C-RELAY-MODE-TYPE-CONFIRM

          - Purpose

Confirm relay mode operation at an A-CPE.

          - SAP type

C-SAP

          - Operation type

Information Confirmation

          - Destination

CPE

          - Data

Table xxx—C-RELAY-MODE-TYPE-CONIRM parameters

| Name | Type | Length | Description |
| --- | --- | --- | --- |
| Relay Mode | Integer | 8 bits | Bitmap. Bit==0 mode not supported, Bit==1 mode is supported Bit0: No Relaying Bit1: enable/disable Centralized Scheduling Bit2: enable/disable Distributed Scheduling Bit3-7: Reserved, set to 0 |

          - When generated

- BS sends this primitive when it receives “Relay CPE Mode” IE in CBC-REQ.

- BS sends this primitive when it receives “Relay CPE Mode” IE in REG-REQ

- BS when triggered by NCMS to re-configured relay mode at an A-CPE

Information in this primitive is carried to the CPE via the “Relay CPE Mode” IE in the CBC-/REG-RSP defined in 7.7.11.3.4.

          - Effect of receipt

When CPE receives this primitive (via the CBC-/REG-RSP MAC management message), and it will cease any current relaying operation and discard any current zone configuration, prior to resuming relay operation in the new mode that is configured.

        - C-RELAY-ZONE-CONFIRM

          - Purpose

Configure Relay/Access zones at an A-CPE configured for Relaying.

          - SAP type

C-SAP

          - Operation type

Information Confirmation

          - Destination

CPE

          - Data

Table xxx—C-RELAY-ZONE parameters

| Name | Type | Length | Description |
| --- | --- | --- | --- |
| Number of Zones | Integer | 8 bits | # of zones including all access and relay zones. Must be >0. Zone 1 is always the access zone |
| For(i=1;i<=Number of Zones;i++) { |  |  |  |
| Zone Index | Integer | 8 bits | Index from 0..Number of zones-1 |
| Zone Mode | Integer | 2 bits | 00: access zone 01: centralized relay zone 10: distributed relay zone 11: reserved |
| Start Position of Segment | Integer | 4 bits | 0x0: 0 0x1: 1 0x2: 2 0x3: If zone mode is set to ‘Access’ or ‘Centralized Relay’ |
| Segment Bandwidth | Integer | 2 bits | 00: one segment from start position 01: two segments from start position 10: three segments from start position 11: If zone mode is set to ‘Access’ or ‘Centralized Relay’ |
| PHY Mode | Integer | 1 bit | 0: PHY-OM1 1: PHY-OM2 |
| } |  |  |  |
| For (j=0;j<Number of Zones;j++) { |  |  |  |
| OFDMA Symbol Offset | Integer | 7 bits | The zone starts at this offset, counted after the preamble of the frame. |
| Zone Duration | Integer | 5 bits | The zone ends after zone duration starting from symbol offset. The unit of duration is in OFDMA symbol |
| If (zone mode==2) { |  |  |  |
| SID | Integer | 13 bits | SID of Relay A-CPE configured for distributed relaying |
| } |  |  |  |
| } |  |  |  |

          - When generated

- BS/NCMS sends this primitive via DS-MAP to configure DS relay/access zones.

- BS/NCMS sends this primitive via US-MAP to configure US relay/access zones

Information in this primitive is carried to the CPE via the “DS Multi-zone Configuration IE” (see 7.7.2.1.2.2) to configure DS relay/access zones, and via “US Multi-zone Configuration IE” (see 7.7.4.1.4.2) to configure US relay access zones.

          - Effect of receipt

When a relay CPE receives this primitive (via the DS-/US-MAP MAC management message), it will immediately configure it’s acess/relay zones accordingly and begin US/DS relay/access zone operation.

        - C-RELAY-TEST-REQUEST

          - Purpose

Request a test to confirm relay DS burst profile when connecting through centralized scheduling A-CPE.

          - SAP type

C-SAP

          - Operation type

Information Request

          - Destination

BS

          - Data

Table xxx—C-RELAY-TEST-REQUEST parameters

| Name | Type | Length | Description |
| --- | --- | --- | --- |
| Transaction ID | Integer | 16 bits | Identifier of relay DS burst profile transaction |
| Station ID | Integer | 13 bits | SID of centralized scheduling A-CPE to test relay burst profile |

          - When generated

- After S-CPE completes CRZ initial ranging procedure, as per procedure in 7.15.1.3.

Information in this primitive is carried to the A-BS via the DTT-REQ message defined in 7.7.27.1.1.

          - Effect of receipt

When A-BS receives this primitive and triggers C-RELAY-TEST-RESPONSE to inform S-CPE where/when allocation to test burst profile will be provided.

        - C-RELAY-TEST-RESPONSE

          - Purpose

Inform S-CPE that requested relay burst profile test when/where the allocation to use for test will be provided.

          - SAP type

C-SAP

          - Operation type

Information Confirmation

          - Destination

CPE

          - Data

Table xxx—C-RELAY-TEST-RESPONSE parameters

| Name | Type | Length | Description |
| --- | --- | --- | --- |
| Transaction ID | Integer | 16 bits | Identifier of relay DS burst profile transaction |
| Action Frame # Offset | Integer | 4 bits | Integer value >0 that indicates the starting frame # for relay burst profile test |
| Status | Integers | 8 bits | See 7.7.24 |

          - When generated

- After A-BS receives DTT-REQ from S-CPE, as per procedure in 7.15.1.3.

Information in this primitive is carried to the A-BS via the DTT-RSP message defined in 7.7.27.2. After transmission of this primitive, transmits test frames to S-CPE through centralized scheduling A-CPE (see 7.15.1.3).

          - Effect of receipt

When S-CPE receives this primitive, it performs the test and then triggers the C-RELAY-TEST-REPORT primitives.

        - C-RELAY-TEST-REPORT

          - Purpose

Inform A-BS of measured relay DS burst profile measurement by S-CPE.

          - SAP type

C-SAP

          - Operation type

Information Confirmation

          - Destination

BS

          - Data

Table xxx—C-RELAY-TEST-REPORT parameters

| Name | Type | Length | Description |
| --- | --- | --- | --- |
| Transaction ID | Integer | 16 bits | Identifier of relay DS burst profile transaction |
| Downstream burst profile | Integer | 6 bits | Burst profile that can be received by CPE |

          - When generated

- After S-CPE receives DTT-RSP from A-BS, as per procedure in 7.15.1.3. It conducts measurement to determine the relevant burst profile using the allocation in the DTT-RSP.

Information in this primitive is carried to the A-BS via the DTT-RPT message defined in 7.7.27.3. After transmission of this primitive, transmits test frames to S-CPE through centralized scheduling A-CPE (see 7.15.1.3).

          - Effect of receipt

A-BS transits from using DS to using relay DS to communicate with S-CPE, confirms setting of relay DS burst profile by triggering C-RELAY-TEST-CONFIRM primitive.

        - C-RELAY-TEST-CONFIRM

          - Purpose

Inform S-CPE that requested relay burst profile test was received and A-BS has transitioned to relay DS using new burst profile.

          - SAP type

C-SAP

          - Operation type

Information Confirmation

          - Destination

CPE

          - Data

Table xxx—C-RELAY-TEST-CONFIRM parameters

| Name | Type | Length | Description |
| --- | --- | --- | --- |
| Transaction ID | Integer | 16 bits | Identifier of relay DS burst profile transaction |
| Confirmation Code | Integer | 8 bits | See 7.7.24 |
| Action Frame # offset | Integer | 4 bits | Integer value >0 indicating frame # when transitioning to relay DS burst profile will occure |

          - When generated

- After A-BS receives DTT-RPT from S-CPE, as per procedure in 7.15.1.3. It selects frame # to start transition to using relay DS burst profile.

Information in this primitive is carried to the A-BS via the DTT-CFM message defined in 7.7.27.4. After transmission of this primitive, transits to using new relay DS burst profile in the indicated frame offset.

          - Effect of receipt

S-CPE waits until indicated frame to begin monitoring for preamble, FCH, and DS-MAP from A-BS.

        - C-RELAY-SCHEDULE-UPDATE

          - Purpose

Coordinate US allocations at S-CPEs.

          - SAP type

C-SAP

          - Operation type

Information Confirmation

          - Destination

CPE

          - Data

Table xxx—C-RELAY-SCHEDULE-UPDATE parameters

| Name | Type | Length | Description |
| --- | --- | --- | --- |
| Transaction ID | Integer | 16 bits | Identifier of US allocation coordination transaction |
| N_FIDs | Integer | 8 bits | # of FIDs US allocations configured for |
| For(i=0;i<N_FID;i++) { |  |  |  |
| FID | Integer | 8 bits | FID of the CPE |
| Allocation Frame Offset | Integer | 8 bits | Start of allocation, In terms of number frames |
| Bandwidth | Intege | 8 bits | In # of bytes |
| } |  |  |  |

          - When generated

- When A-BS/distributed scheduling A-CPE needs to service UGS grants, it may used this primitive to inidicate how much allocation will be provided in the future

- When A-BS/distributed scheduling A-CPE needs to service rtPS grants, indicate to CPE when it will scheduling polling opportunities in the future.

Information in this primitive is carried to the S-CPE via the Relay-SCHE message defined in 7.7.28.

          - Effect of receipt

S-CPE use US allocation provided in primitive to transmit on US allocations .

---end of text modification---

- Add new section 15.2.4.9 “Multi-channel Primitives” after section 15.2.4.8

---start of text modification---

      - Multi-channel Primitives

Multi-channel primitives pertain to monitoring and managing information regarding maintaining multi-channel operation. There are five multi-channel primitives: C-MULTI-CHANNEL-CAM-ADD, C-MULTI-CHANNEL-CAM-STOP-REQUST, C-MULTI-CHANNEL-CAM-STOP-CONFIRM, C-MULTI-CHANNEL-SWITCH-REQUEST, and C-MULTI-CHANNEL-SWITCH-CONFIRM.

        - C-MULTI-CHANNEL-CAM-ADD

          - Purpose

Setup a new channel allocation manager at a CPE when enabling multi-channel operation.

          - SAP type

C-SAP

          - Operation type

Information Confirmation

          - Destination

CPE

          - Data

Table xxx—C-MULTI-CHANNEL-CAM-ADD parameters

| Name | Type | Length | Description |
| --- | --- | --- | --- |
| Transaction ID | Integer | 16 bits | Identifier of CAM add operation transaction |
| Aggregation Inforation | Integer | 1 bit | 0: Aggregation on 1: Aggregation off |
| Maximum Aggregation Channels | Integer | 6 bits | The # of maximum aggregation channels |
| For (i=0;i<Maximum Aggregation Channels; i++) { |  |  |  |
| Channel Number | Integer | 8 bits | Channel # added to CAM |
| } |  |  |  |
| Padding | Integer | 1 bit | Set to 0 |

          - When generated

- A-BS CAM sends this primitive when setting up a CAM at a CPE.

Information in this primitive is carried to the CPE via the CAM-ADD message defined in 7.7.29.2.

          - Effect of receipt

When A-CPE (CHU) receives this message it forwards it to the CPE-CAM (see procedure in 7.24.2.2).

        - C-MULTI-CHANNEL-CAM-STOP-REQUEST

          - Purpose

Used by CPE-CHU to configure the stop operating channel procedure at a CPE-CHU during multichannel operation.

          - SAP type

C-SAP

          - Operation type

Information Request

          - Destination

CPE

          - Data

Table xxx—C-MULTI-CHANNEL-CAM-STOP-REQUEST parameters

| Name | Type | Length | Description |
| --- | --- | --- | --- |
| Transaction ID | Integer | 16 bits | Identifier of CAM stop operation transaction |
| Confirmation Needed | Integer | 1 bit | 0: No confirmation needed 1: Confirmation needed |
| Stop channel number | Integer | 8 bits | Channel that stop operation is requested for |

          - When generated

- BS-CHU sends this primitive when it has determined it needs to stop operating on a channel to protect incumbent.

- BS-CHU sends this primitive when it has received an update to channel availability from database.

- BS-CHU sends this primitive when it has determined a new channel to operate on.

Information in this primitive is carried to the CPE via the CAM-STP message defined in 7.7.29.3.

          - Effect of receipt

The CPE-CHU forwards the stop operation request to its’ CPE-CAM, and sets the stop operation timer (see procedure in 7.24.2.2).

        - C-MULTI-CHANNEL-CAM-STOP-CONFIRM

          - Purpose

Let the BS-CHU that the stop operation has been executed.

          - SAP type

C-SAP

          - Operation type

Information Confirmation

          - Destination

BS

          - Data

Table xxx—C-MULTI-CHANNEL-CAM-STOP-CONFIRM

| Name | Type | Length | Description |
| --- | --- | --- | --- |
| Transaction ID | Integer | 16 bits | Transaction ID copied from C-MULTI-CHANNEL-CAM-STOP-REQUST primitive |
| Confirmation Code | Integer | 8 bits | See 7.7.24 |

          - When generated

- CPE-CHU sends this when it has received indication from CPE-CAM that stop operation has been approved, and a confirmation was requested in C-MULTI-CHANNEL-CAM-STOP-REQUEST

Information in this primitive is carried to the BS via the CAM-STP-ACK message (see 7.7.29.4).

          - Effect of receipt

BS-CHU checks stop operation timer and stops it, informs BS-CAM that stop operation has been executed (see procedure in 7.24.2.2).

        - C-MULTI-CHANNEL-CAM-SWITCH-REQUEST

          - Purpose

Sent by BS-CHU to CPE-CHU to indicate a switch in operating channel.

          - SAP type

C-SAP

          - Operation type

Information Request

          - Destination

CPE

          - Data

Table xxx—C-MULTI-CHANNEL-SWITCH-REQUEST parameters

| Name | Type | Length | Description |
| --- | --- | --- | --- |
| Transaction ID | Integer | 16 bits | Identifier of channel switch request transaction |
| Confirmation Needed | Integer | 1 bit | 0: No confirmation needed 1: Confirmation Needed |
| Switch Mode | Integer | 1 bit | 0: No restrictions of transmissions until scheduled channel switch 1: addressed CPE shall end transmissions until scheduled channel switch |
| Switch Count | Integer | 8 bits | The # of frames (from reception of this primitive), that the BS will switch to new channel |
| Switch Channel Number | Integer | 8 bits | Target channel that is being switched to. |

          - When generated

- BS-CHU sends this primitive when it has determined it needs to switch operating channel to protect incumbent.

- BS-CHU sends this primitive when it has received an update to channel availability from database.

- BS-CHU sends this primitive when it has determined a new channel to operate on.

Information in this primitive is carried to the CPE via the CAM-SWH message defined in 7.7.29.5.

          - Effect of receipt

The CPE-CHU forwards switch operation request to its’ CPE-CAM, and sets the switch operation timer (se procedure in 7.24.2.2).

        - C-MULTI-CHANNEL-SWITCH-CONFIRM

          - Purpose

Inform BS-CHU that channel switch operation has been executed by CPE-CHU.

          - SAP type

C-SAP

          - Operation type

Information Confirmation

          - Destination

CPE

          - Data

Table xxx—C-MULTI-CHANNEL-SWITCH-CONFIRM parameters

| Name | Type | Length | Description |
| --- | --- | --- | --- |
| Transaction ID | Integer | 16 bits | Transaciton ID copied from C-MULTI-CHANNEL-SWITCH-REQUEST |
| Confirmation Code | Integer | 8 bits | See 7.7.24 |

          - When generated

- CPE-CHU schedules channel switch. After CPE-CHU receives notification from CPE-CAM that channel switch operation will be executed, and the confirmation requested bit was set in the C-MULTI-CHANNEL-SWITCH-REQUEST.

Information in this primitive is carried to the BS-CHU via the CAM-SWH-ACK message defined in 7.7.29.6.

          - Effect of receipt

When BS-CHU receives this primitive and schedules the channel switch operation.

---end of text modification---

4) add new section 14.1.8 “wranIfABsMib”

---start of text modification---

14.1.8  wranIfABsMib

This MIB represents MIB functionality related to A-BS operation only. It is based on the FCAPS reference model, but is limited to the following items: Fault, Configuration, and Performance. The following list defines MIBs that compose  wranIfABsMib:

- wranIfABsFm: Fault management

- wranIfABsCm: Configuration management

- wranIfABsPm: Performance management/measurement

---end of text modification---

5) add new section 14.1.8.1 “wranIfABsFm” in section 14.1.8

---start of text modification---

14.1.8.1 wranIfABsFm

Exceptions and fault events can be reported by using the traps defined in this MIB. wranIfBsFm is made up of the following MIBs: wranIfABsTrapControl, wranIfABsTrapDefinition, wranIfABsNotificationObjectsTable, wranIfABsFmMibGroups, and wranIfABsFmMibCompliance.

14.1.8.1.1 wranIfABsTrapControl

This MIB is a bitmap that is used to enable/disable the following A-BS traps: wranIfABsGroupResourceAllocationNotification, wranIfABsMultiChannelConfigNotification, wranIfABsRelayCpeConfigNotification, and wranIfABsTwoLinkArqNotification. Enabling of a trap, indicates that the trap will be recorded in wranIfABsNotificationObjectsTable.

14.1.8.1.2 wranIfABsTrapDefinition

This object defines A-BS related traps for related to configuration and performance management.

14.1.8.1.2.1 wranIfABsGroupResourceAllocationTrap

This trap contains information related to the updating of group configurations used for group resource allocation.

14.1.8.1.2.2 wranIfABsMultiChannelConfigTrap

This trap contains information related to the updating of CHU/CAM configurations used for multi-channel operation.

14.1.8.1.2.3 wranIfABsRelayCpeConfigTrap

This trap contains information related to the changing/updating configuration of relay CPEs operating in the network.

14.1.8.1.2.4 wranIfABsTwoLinkArqTrap

This trap contains information related to the upating of Two-Link ARQ metrics.

14.1.8.1.3 wranIfABsNotificationObjectsTable

This table contains objects that represent notifications reported in A-BS fault traps. The table is made up of one or more entries.

14.1.8.1.3.1 wranIfABsNotificationObjectsEntry

This MIB is a compound object that represents an entry in the MIB object wranIfABsNotificationObjectsTable.

14.1.8.1.3.1.1 wranIfABsNotificationObjectsIndex

Index of entry in the table.

14.1.8.1.3.1.2 wranIfABsNotificationMacAddr

The MAC Address of the CPE reporting the notification.

14.1.8.1.3.1.3 wranIfABsNotificationMacAddr

The MAC Address of the CPE reporting the notification.

14.1.8.1.3.1.4 wranIfABsGroupResourceAllocationConfigType

Indication as to whether or not CPE has had a GRA configuration added, modififed, or deleted.

14.1.8.1.3.1.5 wranIfABsGroupResourceAllocationConfigIndex

Index into wranIfABsGroupResourceAllocConfigTable that indicates contains the current GRA configuration/status for the CPE.

14.1.8.1.3.1.6 wranIfABsMultiChannelConfigType

Indication as to whether or not CPE has had a CAM/CHU configuration added, modified, or deleted.

14.1.8.1.3.1.7 wranIfABsMultiChannelConfigIndex

Index into wranIfABsRelayConfigTable that indicates contains the current CAM/CHU configuration/status for the CPE.

14.1.8.1.3.1.8 wranIfABsRelayCpeConfigIndex

Index into wranIfABsCapabilitiesConfigTable that indicates contains the current relay configuration/status for the CPE.

14.1.8.1.3.1.9 wranIfABsTwoLinkArqIndex

Index into wranIfABsArqMetricsTable that indicates contains the current Two-Link ARQ statistics for the CPE.

14.1.8.1.4 wranIfABsFmMibGroups

This object helps define which MIB groups are available in this module (wranIfABsFm) and which MIB objects are part of each group.

14.1.8.1.5 wranIfABsFmMibCompliance

Defines which objects are optional and which ones are mandatory for wranIfABsFm compliance.

---end of text modification---

6) add new section 14.1.8.2 “wranIfABsCm” in section 14.1.8.1

---start of text modification---

14.1.8.2 wranIfABsCm

This MIB contains various objects related to Configruation Management at the A-BS. Within this MIB, the following tables are handled:

- wranIfABsCpeBasicCapabilityReqTable

- wranIfABsCpeBasicCapabilityRspTable

- wranIfABsCpeBasicCapabilityDefTable

- wranIfABsCpeBasicCapabilityCmn

- wranIfABsGraConfigTable

- wranIfABsMultiChnConfigTable

- wranIfABsRelayConfigTable

- wranIfABsRelayConfigTable

- wranIfABsPhy

- wranIfABsCmMibGroups

- wranIfABsCmMibComplieance

14.1.8.2.1 wranIfABsCpeBasicCapabilityReqTable

This table contains objects that define capabilities negotiated by A-CPEs during network entry. The table is made up of one or more entries, defined by wranIfABsCpeBasicCapabilityReqEntry.

14.1.8.2.1.1 wranIfABsCpeBasicCapabilityReqEntry

This is object is a compound object that contains information on capabilities A-CPEs have declared to A-BS during network entry. This list of objects that make up wranIfABsCpeBasicCapabilityReqEntry can be found in wranIfABsCpeBasicCapabilityCmn (see 14.1.8.2.4).

14.1.8.2.2 wranIfABsCpeBasicCapabilityRspTable

This table contains objects that define capabilities the A-BS has configured for A-CPEs during network entry. The table is made up of one or more entries, defined by wranIfABsCpeBasicCapabilityRspEntry.

14.1.8.2.2.1 wranIfABsCpeBasicCapabilityReqEntry

This is object is a compound object that contains information on capabilities the A-BS have configured for A-CPEs during network entry. This list of objects that make up wranIfABsCpeBasicCapabilityReqEntry can be found in wranIfABsCpeBasicCapabilityCmn (see 14.1.8.2.4).

14.1.8.2.3 wranIfABsCpeBasicCapabilityDefTable

This table contains objects that define default values of CBC-REQ/RSP IEs for A-CPEs. This table can be used to judge /verify a CPE’s CBC-REQ and used to construct the CBC-RSP message. This table has one entry defined by wranIfABsCpeBasicCapabilityDefEntry.

14.1.8.2.3.1 wranIfABsCpeBasicCapabilityDefEntry

This is object is a compound object that defines a entry in wranIfABsCpeBasicCapabilityDefTable. The objects that make up this compound object are defined in wranIfABsCpeBasicCapabilityCmn (see 14.1.8.2.4).

14.1.8.2.4 wranIfABsCpeBasicCapabilityCmn

This MIB object is a group containing objects that are common to wranIfABsCpeBasicCapabilityReqEntry, wranIfABsCpeBasicCapabilityRspEntry, and wranIfABsBasicCapabilitiesDefEntry objects.

14.1.8.2.4.1 wranIfABsBasicCapabilityIndex

Index of entry into wranIfABsCpeBasicCapabilityReqTable, wranIfABsCpeBasicCapabilityRspTable, and wranIfABsCpeBasicCapabilityDefTable.

14.1.8.2.4.2 wranIfABsBasicCapabilityCrzModulator

Defines modulator setting for Centralized Scheduling A-CPE

14.1.8.2.4.3 wranIfABsBasicCapabilityCrzDemodulator

Defines demodulator setting for Centralized Scheduling A-CPE

14.1.8.2.4.4 wranIfABsBasicCapabilityRelayCpeMode

Defines demodulator setting for Centralized Scheduling A-CPE

14.1.8.2.4.5 wranIfABsBasicCapabilityMultichannelSupport

Defines what multi-channel capability the A-CPE supports

14.1.8.2.5 wranIfABsGraConfigTable

This MIB object defines a table containing information on all groups created for supporting group resource allocation (GRA) operation.

14.1.8.2.5.1 wranIfABsGraConfigEntry

Compound object that defines an entry into wranIfABsGraConfigTable.

14.1.8.2.5.1.1 wranIfABsGraConfigIndex

Index of entry into wranIfABsGraConfigTable.

14.1.8.2.5.1.2 wranIfABsGraCpeMacAddress

MAC Address of CPE

14.1.8.2.5.1.3 wranIfABsGraCpeSid

Current Station ID of CPE

14.1.8.2.5.1.4 wranIfABsGraGroupId

Id of GRA group that CPE belongs to.

14.1.8.2.5.1.5 wranIfABsGraGroupType

GRA group type, fixed or portable.

14.1.8.2.5.1.6 wranIfABsGraGroupLat

Latitude portion of group location

14.1.8.2.5.1.7 wranIfABsGraGroupLon

Longitude portion of group location

14.1.8.2.5.1.8 wranIfABsGraMaxGroupSize

Current maximum size of GRA group.

14.1.8.2.6 wranIfABsMultiChnConfigTable

This MIB object defines a table containing information for all CAM/CHUs configured for CPEs.

14.1.8.2.6.1 wranIfABsMultiChnConfigEntry

Compound object that defines an entry into wranIfABsMultiChnConfigTable.

14.1.8.2.6.1.1 wranIfABsMultiChnConfigIndex

Index of entry into wranIfABsMultiChnConfigTable.

14.1.8.2.6.1.2 wranIfABsMultiChnMacAddress

MAC Address of CPE

14.1.8.2.6.1.4 wranIfABsMultiChnMaxAggChns

Maximum number of aggregation channels allowed at CPE

14.1.8.2.6.1.5 wranIfABsMultiChnCurNumAggChns

Current number of aggregation channels used by CPE

14.1.8.2.6.1.6 wranIfABsMultiChnAggChnList

Octet string containing list of channels assigned to CPE for use as aggregation channels

14.1.8.2.7 wranIfABsRelayConfigTable

This MIB object defines a table containing DS/US zone configuration for Relay CPEs.

14.1.8.2.7.1 wranIfABsRelayConfigEntry

Compound object that defines an entry into wranIfABsRealyConfigTable. Each CPE has multiple instances of this object in the table, one for each zone the Relay CPE is configured for.

14.1.8.2.7.1.1 wranIfABsRelayConfigIndex

Index of entry into wranIfABsRelayConfigTable.

14.1.8.2.7.1.2 wranIfABsRelayConfigMacAddress

MAC Address of relay CPE.

14.1.8.2.7.1.3 wranIfABsRelayZoneIndex

Index of relay zone.

14.1.8.2.7.1.4 wranIfABsRelayZoneMode

Mode of zone, i.e. access, centralized relay, distributed relay

14.1.8.2.7.1.5 wranIfABsRelayZoneSegStartPos

Start position of segment, if zone mode is distributed relay.

14.1.8.2.7.1.6 wranIfABsRelayZoneSegBandwidth

Bandwidth of segment, if zone mode is distributed relay.

14.1.8.2.7.1.7 wranIfABsRelayZonePhyMode

PHY mode, i.e. OM1 or OM2, of relay zone.

14.1.8.2.7.1.8 wranIfABsRelayZoneOfdmaSymOffset

Zone starts at this OFDMA symbol offset, counted after the preamble of the frame.

14.1.8.2.7.1.9 wranIfABsRelayCpeSid

SID of distributed scheduling relay CPE.

14.1.8.2.7.1.10 wranIfABsRelayZoneDir

Indication if this is an US or DS relay zone.

14.1.8.2.8 wranIfABsPhy

MIB object contains managed objects related to PHY configuration.

14.1.8.2.8.1 wranIfABsOfdmaPhyUsChannelTable

Table that defines attributes of US channels. Made of multiple entries, defined by wranIfABsOfdmaPhyUsChannelEntry.

14.1.8.2.8.1.1 wranIfABsOfdmaPhyUsChannelEntry

Compound object that defines entry in wranIfABsOfdmaPhyUsChannelTable.

14.1.8.2.8.1.1.1 wranIfABsOfdmaPhyUsChannelIndex

Index of entry in table.

14.1.8.2.8.1.1.2 wranIfABsOfdmaPhyUsSid

SID of CPE whose US is defined in this entry.

14.1.8.2.8.1.1.3 wranIfABsOfdmaPhyUsMacAddress

MAC Address of CPE whose US is defined in this entry.

14.1.8.2.8.1.1.4 wranIfABsOfdmaPhyUsOM

PHY operational mode of CPE US.

14.1.8.2.8.1.1.5 wranIfABsOfdmaPhyUsSegment

Distributed relay zone US segment that CPE is operating in. Defaults to ‘0’ indicating CPE operating through CRZ or directly attached to A-BS.

14.1.8.2.8.1.1.6 wranIfABsOfdmaPhyUsChannelNum

Channel number of US.

14.1.8.2.8.2 wranIfABsOfdmaPhyDsChannelTable

Table that defines attributes of DS channels. Made of multiple entries, defined by wranIfABsOfdmaPhyDsChannelEntry.

14.1.8.2.8.2.1 wranIfABsOfdmaPhyDsChannelEntry

Compound object that defines entry in wranIfABsOfdmaPhyDsChannelTable.

14.1.8.2.8.2.1.1 wranIfABsOfdmaPhyDsChannelIndex

Index of entry in table.

14.1.8.2.8.2.1.2 wranIfABsOfdmaPhyDsSid

SID of CPE whose DS is defined in this entry.

14.1.8.2.8.2.1.3 wranIfABsOfdmaPhyDsMacAddress

MAC Address of CPE whose DS is defined in this entry.

14.1.8.2.8.2.1.4 wranIfABsOfdmaPhyDsOM

PHY operational mode of CPE US.

14.1.8.2.8.2.1.5 wranIfABsOfdmaPhyDsSegmentConfig

Distributed relay zone US segment that CPE is operating in. Defaults to ‘0’ indicating CPE operating through CRZ or directly attached to A-BS.

14.1.8.2.8.2.1.6 wranIfABsOfdmaPhyDsChannelNum

Channel # of DS.

14.1.8.2.8.2.1.7 wranIfABsOfdmaPhyDsCellType

Classes of BS that CPE can be used to attempt network entry.

14.1.8.2.8.2.1.8 wranIfABsOfdmaPhyDsMacVersion

Version of MAC run in the DS/US.

14.1.8.2.8.3 wranIfABsOfdmaPhyUsRegionTable

Table that defines configuration of US subframe. Made of multiple entries, defined by wranIfABsOfdmaPhyUsRegionEntry.

14.1.8.2.8.3.1 wranIfABsOfdmaPhyUsRegionEntry

Compound object that defines entry in wranIfABsOfdmaPhyUsRegionTable.

14.1.8.2.8.3.1.1 wranIfABsOfdmaPhyUsRegionIndex

Index of entry in table.

14.1.8.2.8.3.1.2 wranIfABsOfdmaPhyUsRegionSegmentNum

Segment of US region, defaults to ‘0’ if this segment is  the US of a CRZ relay.

14.1.8.2.8.3.1.3 wranIfABsOfdmaPhyUsRegionDuration

Numberf of OFDMA slots allocated to region.

14.1.8.2.8.4 wranIfABsOfdmaPhyDsRegionTable

Table that defines configuration of DS subframe. Made of multiple entries, defined by wranIfABsOfdmaPhyDsRegionEntry.

14.1.8.2.8.4.1 wranIfABsOfdmaPhyDsRegionEntry

Compound object that defines entry in wranIfABsOfdmaPhyDsRegionTable.

14.1.8.2.8.4.1.1 wranIfABsOfdmaPhyDsRegionIndex

Index of entry in table.

14.1.8.2.8.4.1.2 wranIfABsOfdmaPhyDsRegionSegmentNum

Segment of US region, defaults to ‘0’ if this segment is  the DS of a CRZ relay.

14.1.8.2.8.4.1.3 wranIfABsOfdmaPhyDsRegionSegmentNum

Numberf of OFDMA slots allocated to region.

14.1.8.2.9 wranIfABsCmMibGroups

This object helps define which MIB groups are available in this module (wranIfABsCm) and which MIB objects are part of each group.

14.1.8.2.10 wranIfABsCmMibCompliance

Defines which objects are optional and which ones are mandatory for wranIfABsCm compliance.

---end of text modification---

7) add new section 14.1.8.3 “wranIfABsPm” in section 14.1.8.2

---start of text modification---

14.1.8.3 wranIfABsPm

This MIB contains various objects related to Perfmance Management at the A-BS. Within this MIB, the following tables are handled:

- wranIfABsPmConfigurationTable

- wranIfABsArqMetricsTable

- wranIfABsPmMibGroups

- wranIfABsPmMibComplieance

14.1.8.3.1 wranIfABsPmConfigurationTable

The configuration of statistics capture and measurement at the A-BS is captured in this table. There is one entry int this table for each BS sector. The entries are defined by wranIfBsPmConfigurationEntry.

14.1.8.3.1.1 wranIfABsPmConfigurationEntry

This object defines a compound object that represents an entry in wranIfABsPmConfigurationTable.

14.1.8.3.1.1.1 wranIfABsGranularityInterval

The interval in which statistics captured in wranIfABsTrapControl related to performance measurement reported to the NCMS.

14.1.8.3.1.1.2 wranIfABsTwoLinkArqConfig

This object indicates whether or not Two Link ARQ measurements (wranIfABsArqMetricsTable).

14.1.8.3.2 wranIfABsArqMetricsTable

This MIB provides a table to track performance of Two Link ARQ being used with S-CPEs attached to A-BS through distributed scheduling A-CPE. The entries are defined by wranIfABsArqMetricsEntry.

14.1.8.3.2.1 wranIfABsArqMetricsEntry

This object defines a compound object that represents an entry in wranIfABsArqMetricsTable.

14.1.8.3.2.1.1 wranIfABsRelayLinkDsNumArqBlocks

The total # of ARQ blocks, include retransmissions, that the A-BS has sent on the relay link (to distributed scheduling A-CPE) during the reporting period.

14.1.8.3.2.1.2 wranIfABsAccessLinkDsNumArqBlocks

The total # of ARQ blocks, include retransmissions, that A-CPEs has sent on the access link during the reporting period.

14.1.8.3.2.1.3 wranIfABsRelayLinkDsNumArqBlocksDropped

Total # of ARQ blocks that were dropped, that the A-BS has sent during the reporting period.

14.1.8.3.2.1.4 wranIfABsAccessLinkDsNumArqBlocksDropped

The total # of ARQ blocks that were dropped, that A-CPEs has sent on the access link during the reporting period.

14.1.8.3.2.1.5 wranIfABsRelayLinkDsArqBlockErrorRate

Error Rate on DS Relay Link.

14.1.8.3.2.1.6 wranIfABsAccessLinkDsArqBlockErrorRate

Error Rate on DS Access Link.

14.1.8.3.2.1.7 wranIfABsRelayLinkDsArqBlockRetransmission

The # of ARQ blocks that were retransmitted by the A-BS during the reporting period.

14.1.8.3.2.1.8 wranIfABsAccessLinkDsArqBlockRetransmission

The # of ARQ blocks that were retransmitted by the A-CPE during the reporting period.

14.1.8.3.2.1.9 wranIfABsRelayLinkUsNumArqBlocks

The total # of ARQ blocks, include retransmissions, that the A-BS has received on the relay link (from distributed scheduling A-CPE) during the reporting period.

14.1.8.3.2.1.10 wranIfABsAccessLinkUsNumArqBlocks

The total # of ARQ blocks, include retransmissions, that A-CPEs has received on the access link during the reporting period.

14.1.8.3.2.1.11 wranIfABsRelayLinkUsNumArqBlocksDropped

Total # of ARQ blocks that were dropped, that the A-BS has received during the reporting period.

14.1.8.3.2.1.12 wranIfABsAccessLinkUsNumArqBlocksDropped

The total # of ARQ blocks that were dropped, that A-CPEs has received on the access link during the reporting period.

14.1.8.3.2.1.13 wranIfABsRelayLinkUsArqBlockErrorRate

Error Rate on US Access Link.

14.1.8.3.2.1.14 wranIfABsAccessLinkUsArqBlockErrorRate

Error Rate on US Relay Link.

14.1.8.3.2.1.15 wranIfABsRelayLinkUsArqBlockRetransmission

The # of ARQ blocks that were retransmitted by the A-CPE during the reporting period on the relay link.

14.1.8.3.2.1.16 wranIfABsAccessLinkUsArqBlockRetransmission

The # of ARQ blocks that were retransmitted by the S-CPE during the reporting period on the access link.

14.1.8.3.2.1.17 wranIfABsRelayLinkDsEfficiency

Efficiency of transmission on DS relay link.

14.1.8.3.2.1.18 wranIfABsAccessLinkDsEfficiency

Efficiency of transmission on DS access link.

14.1.8.3.2.1.19 wranIfABsRelayLinkUsEfficiency

Efficiency of transmission on US relay link.

14.1.8.3.2.1.20 wranIfABsAccessLinkUsEfficiency

Efficiency of transmission on US access link.

14.1.8.3.3 wranIfABsPmMibGroups

This object helps define which MIB groups are available in this module (wranIfABsPm) and which MIB objects are part of each group.

14.1.8.3.4 wranIfABsPmMibCompliance

Defines which objects are optional and which ones are mandatory for wranIfBsPm compliance.

---end of text modification---

8) add new section 14.1.9 “wranIfACpeMib”

---start of text modification---

14.1.9  wranIfACpeMib

This MIB represents MIB functionality related to managing A-CPEs. It is broken up into the following sublclauses:

- wranIfACpeConfigurationTable: Definition of system parameters, timers, and constants related to CPE operation

- wranIfACpeTrapControl: Enabling/disabling of traps concerning events related to CPE operation

- wranIfACpeTrapDefinition: Definition of traps and objects that can be reported by traps

- wranIfACpeNotificationObjectsTable: Table of information related to configured traps

- wranIfACpeMibGroups: Defines which MIB groups are available in this module and which MIB objects are a part of each group

- wranIfACpeMibCompliance: Defines which MIB objects are optional, and which ones are mandatory for CPE conformance

---end of text modification---

9) add new section 14.1.9.1 “wranIfACpeConfigurationTable” after 14.1.9

---start of text modification---

14.1.9.1 wranIfABsCpeSystemParametersTable

This table contains objects that define system constants related to A-BS/A-CPE operation.

14.1.9.1.1 wranIfABsCpeSystemParametersEntry

This is a compound object that defines an entry in wranIfABsCpeSystemParametersTable.

14.1.9.1.1.1 wranIfABsCpeSystemParametersIndex

This is a compound object that defines an entry in wranIfABsCpeSystemParametersTable.

14.1.9.1.1.2 wranIfABsT61

Timeout for waiting for container ACK message.

14.1. 9.1.1.3 wranIfABsT62

Timeout for waiting for local cell update acknowledgement.

14.1. 9.1.1.4 wranIfABsT63

Timeout for DTT-RSP message.

14.1. 9.1.1.5 wranIfABsT64

Timeout for DTT-RPT message.

14.1. 9.1.1.6 wranIfABsT65

Timeout for DTT-CFM message.

14.1. 9.1.1.7 wranIfABsT66

Timeout for CAM-STP message.

14.1.9.1.1.8 wranIfABsT67

Timeout for CAM-SWH message.

---end of text modification---

10) add new section 14.1.9.2 “wranIfACpeMibGroups” after 14.1.9.1

---start of text modification---

14.1.9.2 wranIfACpeMibGroups

This object helps define which MIB groups are available in this module (wranIfACpe) and which MIB objects are part of each group.

---end of text modification---

11) add new section 14.1.9.3 “wranIfACpeMibCompliance” after 14.1.9.2

---start of text modification---

14.1.9.3 wranIfACpeMibCompliance

This defines which MIB objects are optional and mandatory for CPE conformance

---end of text modification---