IEEE P802.22
Wireless RANs

| MIB & Management Plane Procedure Updates |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2018-07-31 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Ranga Reddy | N/A |  | 732-693-5812 | Ranga.reddy@me.com |

This contribution seeks to amend LB2 draft to MIB and management plane procedures for newer capabilities introduced in IEEE Std. 802.22b-2015.

R0: Original Version of this document

R1: Adding Text for primitives

R2: Adding text for MIB descriptions

R3: updating section numbering to confirm to numbering in new template

R4: Adding text for MIB definitions

R5: Additional corrections to existing/new MIB, and management plane procedures base on errors found to additons made to produce Draft ver3. Includes making change to SID size to align base standard with amendment text being added.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Carl R. Stevenson> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

MIB & Management Plane Procedure Updates

Ranga Reddy (N/A)

This document (and subsequent revisions) will serve as an outline for the additional MIBs and management plance procedures (MPPs).  This document is organized as follows:

- (I) updates to section 14.1.x/14.2.x

- (II) additions to the C-SAP in 15.2.4

For each new section added to 14.1.x, MIB definitions in ASN.1 encoding format will be added to the corresponding section in 14.2.x

Revision 0: Initial outline of new items to be added in each section

# Structure of Modifications to Sections 14.1.x/14.2.x

### 14.1.8 wranIfABsMib (A-BS MIB Group)

### 14.1.8.1 wranIfABsFm

Fault Management

### 14.1.8.1.1 wranIfABsTrapControl

(bitmap to enable/disable notifications)

### 14.1.8.1.2 wranIfABsTrapDefinition

(definition of notifications)

### 14.1.8.1.3 wranIfABsNotificationObjectsTable

(definition of information captured with each notification)

### 14.1.8.1.4 wranIfABsFmMibGroups

(description of what MIB groups are part of this MIB definition)

### 14.1.8.1.5 wranIfABsFmMibCompliance

(description of which MIB objects are mandatory/optional with regard to compliance of using wranIfABsFm of information captured with each notification)

### 14.1.8.2 wranIfABsCm

Configuration Management

### 14.1.8.2.1 wranIfABsCpeBasicCapabilityReqTable

Contents of rx’d CBC-REQs (A-CPE related IEs)

### 14.1.8.2.2 wranIfABsCpeBasicCapabilityRspTable

Contents of Tx’d CBC-RSPs (A-CPE related IEs)

### 14.1.8.2.3 wranIfABsBasicCapabilitiesDefTable

Contents of defaults A-BS to use for CBC-RSPs (A-CPE related IEs)

### 14.1.8.2.4 wranIfABsCpeBasicCapabilitiyCmn

defines objects common to 14.1.8.2.1, 14.1.8.2.2, 14.1.8.2.3

### 14.1.8.2.5 wranIfABsGraConfigTable

Configuration of all current group resource configurations

### 14.1.8.2.6 wranIfABsMultiChnConfigTable

Configuration of all CHU/CAM configurations

### 14.1.8.2.7 wranIfABsRelayConfigTable

Configuration of all DS/US zone configuration configurations

### 14.1.8.2.8 wranIfABsPhy

MIB group related to PHY operation

### 14.1.8.2.8.1 wranIfABsOfdmaPhyUsChannelTable

Table that defines attributes of US channels

### 14.1.8.2.8.2 wranIfABsOfdmaPhyDsChannelTable

Table that defines attributes of DS channels

### 14.1.8.2.8.3 wranIfABsOfdmaUsRegionTable

Table that defines configuration of US subframe

### 14.1.8.2.9.4 wranIfABsOfdmaDsRegionTable

Table that defines configuration of DS subframe

### 14.1.8.2.9 wranIfABsCmMibGroups

(description of what MIB groups are part of this MIB definition)

### 14.1.8.2.10 wranIfABsCmMibCompliance

(description of which MIB objects are mandatory/optional with regard to compliance of using wranIfABsCm of information captured with each notification)

### 14.1.8.3 wranIfABsPm

Performance Management

### 14.1.8.3.1 wranIfABsPmConfigurationTable

Configuration of which performance metrics tables are enabled

### 14.1.8.3.2 wranIfABsArqMetricsTable

Two-Link ARQ metrics

### 14.1.8.1.9 wranIfABsPmMibGroups

(description of what MIB groups are part of this MIB definition)

### 14.1.8.1.10 wranIfABsPmMibCompliance

(description of which MIB objects are mandatory/optional with regard to compliance of using wranIfABsPm of information captured with each notification)

### 14.1.9 wranIfACpeMib (A-CPE MIB Group)

### 14.1.9.1 wranIfACpeConfigurationTable

Table containing current values of configuration related to A-CPE operation

### 14.1.9.2 wranIfACpeTrapControl

(bitmap to enable/disable notifications)

### 14.1.9.3 wranIfACpeTrapDefinition

(definition of notifications)

### 14.1.9.4 wranIfACpeNotificationObjectsTable

(definition of information captured with each notification)

### 14.1.9.1.5 wranIfACpeMibGroups

(description of what MIB groups are part of this MIB definition)

### 14.1.9.1.6 wranIfACpeMibCompliance

(description of which MIB objects are mandatory/optional with regard to compliance of using wranIfACpe of information captured with each notification)

# Structure of Modifications to Sections 15.2.4.x

### 15.2.4.7 Group Resource Allocation Management Primitives

Primitves related to using GRA allocation

### 15.2.4.7.1 C-GROUP-RESOURCE-ALLOC-CONFIG

Setting up GRA configuration at CPE

### 15.2.4.7.2 C-GROUP-RESOURCE-ALLOC-UPDATE

Updating  GRA configuration at CPE

### 15.2.4.8 Relay Mode Primitives

Primitves related to using Relay Mode an A-CPE can be configured for

### 15.2.4.8.1 C-RELAY-MODE-TYPE-CONFIG

Setting up the relaying mode used configuration at an A-CPE

### 15.2.4.8.2 C-RELAY-MODE-TYPE-CONFIRM

Confirmation of relaying mode configuration at an A-CPE

### 15.2.4.8.3 C-RELAY-ZONE-CONFIRM

Configuring zones at a relay A-CPE

### 15.2.4.8.4 C-RELAY-TEST-REQUEST

Request transmission of test burst

### 15.2.4.8.5 C-RELAY-TEST-RESPONSE

Response to test burst request

### 15.2.4.8.6 C-RELAY-TEST-REPORT

Transmssion of test burst analysis report

### 15.2.4.8.7 C-RELAY-TEST-CONFIRM

Confirmation of the reception of test request message

### 15.2.4.8.8 C-RELAY-SCHEDULE-UPDATE

Confirmation of relaying zone configuration at an A-CPE

### 15.2.4.9 Multi-channel Primitives

Primitves related to using multi-channel (i.e. CHU/CAM) configration

### 15.2.4.9.1 C-MULTI-CHANNEL-CAM-ADD

Setting up a CAM to use at A-CPE

### 15.2.4.9.2 C-MULTI-CHANNEL-CAM-STOP-REQUST

Request an A-CPE to stop using a CAM

### 15.2.4.9.3 C-MULTI-CHANNEL-CAM-STOP-CONFIRM

A-CPE confirmation of BS request to stop using a CAM

### 15.2.4.9.4 C-MULTI-CHANNEL-CAM-SWITCH-REQUEST

Request an A-CPE to switch the active CAM

### 15.2.4.9.5 C-MULTI-CHANNEL-CAM-SWITCH-CONFIRM

A-CPE confirmation of BS request to switch the active CAM being used

# Organization of text Modifications

The following pages of this contribution contain text modification instructions to be applied to the draft. These modification instructions are organized as follows:

- Instructions 1-3 define the application of updates for sections 15.2.4.7, 15.2.4.8, 15.2.4.9 in the new draft

- Instructions 4-11 define the applications of updates for sections 14.1.8 and 14.1.9 in the new draft

- Instructions 12-15 define the application of updates for sections 14.2.8 and 14.2.9 in the new draft

- Instruction 16-26 contains changes to existing text in the draft (from 802.22a-2014) 14.1.1-14.1.7, 14.2.1-14.2.7, and 15.2.1-15.2.3, 15.2.4.1-15.2.4.6 against draft

- Instruction 27-45 contains a list corrections to new MIBs/management plane procedures that have to be applied to new text in sections 14.1.8, 14.1.9,  and 15.2.4.7-15.2.4.9

- add new section 15.2.4.7 “Group Resource Allocation Primitives” after section 15.2.4.6

---start of text modification---

15.2.4.7	Group Resource Allocation Management Primitives

Group Resource Allocation management primitives pertain to monitoring and managing information regarding setting up group resource allocation (GRA) configurations for groups of CPEs. GRA configurations help reduce (MAP) overhead from groups of CPEs operating in the same PHY Mode and a fixed payload size.

There are three multicast management primitives: C-GROUP-RESOURCE-ALLOC-CONFIG, and C-GROUP-RESOURCE-ALLOC-UPDATE.

        - C-GROUP-RESOURCE-ALLOC-CONFIG

          - Purpose

Add CPE(s) to a GRA configuration.

          - SAP type

C-SAP

          - Operation type

Information Confirmation

          - Destination

CPE

          - Data

Table xxx—C-GROUP-RESOURCE-ALLOC-CONFIG parameters

| Name | Type | Length | Description |
| --- | --- | --- | --- |
| Transaction ID | Integer | 16 bits | Identifier of GRA configuration transaction, selected by the BS. |
| Device Bitmap Size | Integer | 4 bits | Bitmap indicating the configuration of the maximum # of CPEs that can be added to a GRA group |
| SID Bitmap | Integer | N*9 bits | Bitmap/List of SIDs of CPEs being added to group, where N equal to configuration option specified by “Device Bitmap Size” |
| GID | Integer | 10 bits | ID of GRA group. |
| GRA Group Type | Integer | 1 bits | 0: Fixed Group 1: Portable or Mobile Group |
| Group Location | Integer | 48 bits | Latitude/Longitude of new group |
| Padding |  | Variable | Depending on total size of message, all set to 0. |

          - When generated

- BS sends this primitive when multiple CPEs are assigned to the same GRA configuration. One copy of this primitive is sent to each CPE it wants to add to the GRA.

Information in this primitive is carried to the CPE or CPEs in question via the GRA-CFG message defined in 7.7.30.1.

          - Effect of receipt

When CPE receives this primitive (via the GRA-CFG MAC management message), it takes the contents and configures GRA group configuration and begins monitoring DS-/US-MAPs for DRZ GRA Map IEs (see 7.7.2.1.2.5, 7.7.4.1.4.5) for allocations in the DS/US. The Backup/Candidate Channel list can be updated on a group basis, as defined by the procedure in 7.25.

        - C- GROUP-RESOURCE-ALLOC-UPDATE

          - Purpose

Remove a CPE from a GRA group.

          - SAP type

C-SAP

          - Operation type

Information Confirmation

          - Destination

CPE

          - Data

Table xxx—C-GROUP-RESOURCE-ALLOC-UPDATE parameters

| Name | Type | Length | Description |
| --- | --- | --- | --- |
| Transaction ID | Integer | 16 bits | Identifier of Multicast group setup transaction, selected by the BS. |
| Device Flag | Integer | 1 bit | 1bit flag to indicate whether or not device info in GRA group is being changed |
| Group Flag | Integer | 1 bit | 1bit flag to indicate whether or not group info (i.e. location) for GRA group is being changed |
| If(Device Flag ==1) { |  |  |  |
| GID | Integer | 10 bits | ID of GRA group |
| Number of Added Devices | Integer | 7 bits | # of devices being added to group |
| Number of Deleted Devices | Integer | 7 bits | # of devices being deleted from group |
| For(i=0;i<Number of Added Devices;i++) { |  |  |  |
| SID | Integer | 9 bits | SID of CPE added to group |
| Device Bitmap Index | Integer | 7 bits | New index of device in GRA group’s device bitmap |
| } |  |  |  |
| For(i=0;i<Number of Deleted Devices;i++) { |  |  |  |
| SID | Integer | 9 bits | SID of CPE added to group |
| } |  |  |  |
| } |  |  |  |
| If(Group Flag==1) { |  |  |  |
| New Group Location | Integer | 48 bit | New Group Location |
| } |  |  |  |
| Padding |  | Variable | Depedent on setting of “Device Flag” and “Group Flag”, as well as total # of Deleted/Added devices, all bits set to 0. |

          - When generated

- BS sends this primitive when it wants to remove a CPE from a GRA group configuration.

- BS sends this primitive when it wants to add a CPE to an existing GRA group configuration.

- BS sends this primitive when it wants to update location of existing GRA group.

Information in this primitive is carried to the CPE or CPEs in question via the GRA-UPD message defined in 7.7.30.2.

          - Effect of receipt

When CPE receives this primitive (via the GRA-CFG MAC management message), and has been instructed that it is being removed from the group, it stops monitoring DS-/US-MAPs for DRZ GRA MAP IEs (see 7.7.2.1.2.5, 7.7.4.1.4.5) and processing its’ allocations. CPEs that received the primitive and are being newly added to group, will take the contents and configures GRA group configuration and begins monitoring DS-/US-MAPs for DRZ GRA Map IEs for allocations in the DS/US. The Backup/Candidate Channel list can be updated on a group basis, as defined by the procedure in 7.25.

---end of text modification---

- add new section 15.2.4.8 “Relay Mode Primitives” after section 15.2.4.7

---start of text modification---

15.2.4.8	Relay Mode Primitives

Relay Mode primitives pertain to monitoring and managing information regarding configuring an A-CPE to operate in relay mode, i.e. as Centralized or Distriubted Scheduling CPE. There are 8 relay mode primitives: C-RELAY-MODE-TYPE-CONFIG, C-RELAY-MODE-TYPE-CONFIRM, C-RELAY-ZONE-CONFIG, C-RELAY-ZONE-CONFIRM, C-RELAY-TEST-REQUEST, C-RELAY-TEST-RESPONSE, C-RELAY-TEST-CONFIRM, and C-RELAY-SCHEDULE-UPDATE.

        - C-RELAY-MODE-TYPE-CONFIG

          - Purpose

Request configuration of Relaying Mode (e.g. either none, Centralized Scheduling, Distributed).

          - SAP type

C-SAP

          - Operation type

Information Request

          - Destination

BS

          - Data

Table xxx—C-RELAY-MODE-TYPE-CONFIG parameters

| Name | Type | Length | Description |
| --- | --- | --- | --- |
| Relay Mode | Integer | 8 bits | Bitmap. Bit==0 mode not supported, Bit==1 mode is supported Bit0: No Relaying Bit1: enable/disable Centralized Scheduling Bit2: enable/disable Distributed Scheduling Bit3-7: Reserved, set to 0 |

          - When generated

- A-CPE sends this primitive when entering/re-entering the network and request relay mode operation.

Information in this primitive is carried to the A-BS via the “Relay CPE Mode” CBC-/REG-REQ IE message defined in 7.7.11.3.4.

          - Effect of receipt

When A-BS receives this primitive (via the CBC-REQ MAC management message), it takes the contents and configures Relay Mode type and triggers the C-RELAY-MODE-TYPE-CONFIRM primitive (i.e. configures relay mode via “Relay CPE Mode” IE in CBC-/REG-RSP.

        - C-RELAY-MODE-TYPE-CONFIRM

          - Purpose

Confirm relay mode operation at an A-CPE.

          - SAP type

C-SAP

          - Operation type

Information Confirmation

          - Destination

CPE

          - Data

Table xxx—C-RELAY-MODE-TYPE-CONIRM parameters

| Name | Type | Length | Description |
| --- | --- | --- | --- |
| Relay Mode | Integer | 8 bits | Bitmap. Bit==0 mode not supported, Bit==1 mode is supported Bit0: No Relaying Bit1: enable/disable Centralized Scheduling Bit2: enable/disable Distributed Scheduling Bit3-7: Reserved, set to 0 |

          - When generated

- BS sends this primitive when it receives “Relay CPE Mode” IE in CBC-REQ.

- BS sends this primitive when it receives “Relay CPE Mode” IE in REG-REQ

- BS when triggered by NCMS to re-configured relay mode at an A-CPE

Information in this primitive is carried to the CPE via the “Relay CPE Mode” IE in the CBC-/REG-RSP defined in 7.7.11.3.4.

          - Effect of receipt

When CPE receives this primitive (via the CBC-/REG-RSP MAC management message), and it will cease any current relaying operation and discard any current zone configuration, prior to resuming relay operation in the new mode that is configured.

        - C-RELAY-ZONE-CONFIRM

          - Purpose

Configure Relay/Access zones at an A-CPE configured for Relaying.

          - SAP type

C-SAP

          - Operation type

Information Confirmation

          - Destination

CPE

          - Data

Table xxx—C-RELAY-ZONE parameters

| Name | Type | Length | Description |
| --- | --- | --- | --- |
| Number of Zones | Integer | 8 bits | # of zones including all access and relay zones. Must be >0. Zone 1 is always the access zone |
| For(i=1;i<=Number of Zones;i++) { |  |  |  |
| Zone Index | Integer | 8 bits | Index from 0..Number of zones-1 |
| Zone Mode | Integer | 2 bits | 00: access zone 01: centralized relay zone 10: distributed relay zone 11: reserved |
| Start Position of Segment | Integer | 4 bits | 0x0: 0 0x1: 1 0x2: 2 0x3: If zone mode is set to ‘Access’ or ‘Centralized Relay’ |
| Segment Bandwidth | Integer | 2 bits | 00: one segment from start position 01: two segments from start position 10: three segments from start position 11: If zone mode is set to ‘Access’ or ‘Centralized Relay’ |
| PHY Mode | Integer | 1 bit | 0: PHY-OM1 1: PHY-OM2 |
| } |  |  |  |
| For (j=0;j<Number of Zones;j++) { |  |  |  |
| OFDMA Symbol Offset | Integer | 7 bits | The zone starts at this offset, counted after the preamble of the frame. |
| Zone Duration | Integer | 5 bits | The zone ends after zone duration starting from symbol offset. The unit of duration is in OFDMA symbol |
| If (zone mode==2) { |  |  |  |
| SID | Integer | 9 bits | SID of Relay A-CPE configured for distributed relaying |
| } |  |  |  |
| } |  |  |  |

          - When generated

- BS/NCMS sends this primitive via DS-MAP to configure DS relay/access zones.

- BS/NCMS sends this primitive via US-MAP to configure US relay/access zones

Information in this primitive is carried to the CPE via the “DS Multi-zone Configuration IE” (see 7.7.2.1.2.2) to configure DS relay/access zones, and via “US Multi-zone Configuration IE” (see 7.7.4.1.4.2) to configure US relay access zones.

          - Effect of receipt

When a relay CPE receives this primitive (via the DS-/US-MAP MAC management message), it will immediately configure it’s acess/relay zones accordingly and begin US/DS relay/access zone operation.

        - C-RELAY-TEST-REQUEST

          - Purpose

Request a test to confirm relay DS burst profile when connecting through centralized scheduling A-CPE.

          - SAP type

C-SAP

          - Operation type

Information Request

          - Destination

BS

          - Data

Table xxx—C-RELAY-TEST-REQUEST parameters

| Name | Type | Length | Description |
| --- | --- | --- | --- |
| Transaction ID | Integer | 16 bits | Identifier of relay DS burst profile transaction |
| Station ID | Integer | 9 bits | SID of centralized scheduling A-CPE to test relay burst profile |

          - When generated

- After S-CPE completes CRZ initial ranging procedure, as per procedure in 7.15.1.3.

Information in this primitive is carried to the A-BS via the DTT-REQ message defined in 7.7.27.1.1.

          - Effect of receipt

When A-BS receives this primitive and triggers C-RELAY-TEST-RESPONSE to inform S-CPE where/when allocation to test burst profile will be provided.

        - C-RELAY-TEST-RESPONSE

          - Purpose

Inform S-CPE that requested relay burst profile test when/where the allocation to use for test will be provided.

          - SAP type

C-SAP

          - Operation type

Information Confirmation

          - Destination

CPE

          - Data

Table xxx—C-RELAY-TEST-RESPONSE parameters

| Name | Type | Length | Description |
| --- | --- | --- | --- |
| Transaction ID | Integer | 16 bits | Identifier of relay DS burst profile transaction |
| Action Frame # Offset | Integer | 4 bits | Integer value >0 that indicates the starting frame # for relay burst profile test |
| Status | Integers | 8 bits | See 7.7.24 |

          - When generated

- After A-BS receives DTT-REQ from S-CPE, as per procedure in 7.15.1.3.

Information in this primitive is carried to the A-BS via the DTT-RSP message defined in 7.7.27.2. After transmission of this primitive, transmits test frames to S-CPE through centralized scheduling A-CPE (see 7.15.1.3).

          - Effect of receipt

When S-CPE receives this primitive, it performs the test and then triggers the C-RELAY-TEST-REPORT primitives.

        - C-RELAY-TEST-REPORT

          - Purpose

Inform A-BS of measured relay DS burst profile measurement by S-CPE.

          - SAP type

C-SAP

          - Operation type

Information Confirmation

          - Destination

BS

          - Data

Table xxx—C-RELAY-TEST-REPORT parameters

| Name | Type | Length | Description |
| --- | --- | --- | --- |
| Transaction ID | Integer | 16 bits | Identifier of relay DS burst profile transaction |
| Downstream burst profile | Integer | 6 bits | Burst profile that can be received by CPE |

          - When generated

- After S-CPE receives DTT-RSP from A-BS, as per procedure in 7.15.1.3. It conducts measurement to determine the relevant burst profile using the allocation in the DTT-RSP.

Information in this primitive is carried to the A-BS via the DTT-RPT message defined in 7.7.27.3. After transmission of this primitive, transmits test frames to S-CPE through centralized scheduling A-CPE (see 7.15.1.3).

          - Effect of receipt

A-BS transits from using DS to using relay DS to communicate with S-CPE, confirms setting of relay DS burst profile by triggering C-RELAY-TEST-CONFIRM primitive.

        - C-RELAY-TEST-CONFIRM

          - Purpose

Inform S-CPE that requested relay burst profile test was received and A-BS has transitioned to relay DS using new burst profile.

          - SAP type

C-SAP

          - Operation type

Information Confirmation

          - Destination

CPE

          - Data

Table xxx—C-RELAY-TEST-CONFIRM parameters

| Name | Type | Length | Description |
| --- | --- | --- | --- |
| Transaction ID | Integer | 16 bits | Identifier of relay DS burst profile transaction |
| Confirmation Code | Integer | 8 bits | See 7.7.24 |
| Action Frame # offset | Integer | 4 bits | Integer value >0 indicating frame # when transitioning to relay DS burst profile will occure |

          - When generated

- After A-BS receives DTT-RPT from S-CPE, as per procedure in 7.15.1.3. It selects frame # to start transition to using relay DS burst profile.

Information in this primitive is carried to the A-BS via the DTT-CFM message defined in 7.7.27.4. After transmission of this primitive, transits to using new relay DS burst profile in the indicated frame offset.

          - Effect of receipt

S-CPE waits until indicated frame to begin monitoring for preamble, FCH, and DS-MAP from A-BS.

        - C-RELAY-SCHEDULE-UPDATE

          - Purpose

Coordinate US allocations at S-CPEs.

          - SAP type

C-SAP

          - Operation type

Information Confirmation

          - Destination

CPE

          - Data

Table xxx—C-RELAY-SCHEDULE-UPDATE parameters

| Name | Type | Length | Description |
| --- | --- | --- | --- |
| Transaction ID | Integer | 16 bits | Identifier of US allocation coordination transaction |
| N_FIDs | Integer | 8 bits | # of FIDs US allocations configured for |
| For(i=0;i<N_FID;i++) { |  |  |  |
| FID | Integer | 8 bits | FID of the CPE |
| Allocation Frame Offset | Integer | 8 bits | Start of allocation, In terms of number frames |
| Bandwidth | Intege | 8 bits | In # of bytes |
| } |  |  |  |

          - When generated

- When A-BS/distributed scheduling A-CPE needs to service UGS grants, it may used this primitive to inidicate how much allocation will be provided in the future

- When A-BS/distributed scheduling A-CPE needs to service rtPS grants, indicate to CPE when it will scheduling polling opportunities in the future.

Information in this primitive is carried to the S-CPE via the Relay-SCHE message defined in 7.7.28.

          - Effect of receipt

S-CPE use US allocation provided in primitive to transmit on US allocations .

---end of text modification---

- Add new section 15.2.4.9 “Multi-channel Primitives” after section 15.2.4.8

---start of text modification---

15.2.4.9	Multi-channel Primitives

Multi-channel primitives pertain to monitoring and managing information regarding maintaining multi-channel operation. There are five multi-channel primitives: C-MULTI-CHANNEL-CAM-ADD, C-MULTI-CHANNEL-CAM-STOP-REQUST, C-MULTI-CHANNEL-CAM-STOP-CONFIRM, C-MULTI-CHANNEL-SWITCH-REQUEST, and C-MULTI-CHANNEL-SWITCH-CONFIRM.

        - C-MULTI-CHANNEL-CAM-ADD

          - Purpose

Setup a new channel allocation manager at a CPE when enabling multi-channel operation.

          - SAP type

C-SAP

          - Operation type

Information Confirmation

          - Destination

CPE

          - Data

Table xxx—C-MULTI-CHANNEL-CAM-ADD parameters

| Name | Type | Length | Description |
| --- | --- | --- | --- |
| Transaction ID | Integer | 16 bits | Identifier of CAM add operation transaction |
| Aggregation Inforation | Integer | 1 bit | 0: Aggregation on 1: Aggregation off |
| Maximum Aggregation Channels | Integer | 6 bits | The # of maximum aggregation channels |
| For (i=0;i<Maximum Aggregation Channels; i++) { |  |  |  |
| Channel Number | Integer | 8 bits | Channel # added to CAM |
| } |  |  |  |
| Padding | Integer | 1 bit | Set to 0 |

          - When generated

- A-BS CAM sends this primitive when setting up a CAM at a CPE.

Information in this primitive is carried to the CPE via the CAM-ADD message defined in 7.7.29.2.

          - Effect of receipt

When A-CPE (CHU) receives this message it forwards it to the CPE-CAM (see procedure in 7.24.2.2).

        - C-MULTI-CHANNEL-CAM-STOP-REQUEST

          - Purpose

Used by CPE-CHU to configure the stop operating channel procedure at a CPE-CHU during multichannel operation.

          - SAP type

C-SAP

          - Operation type

Information Request

          - Destination

CPE

          - Data

Table xxx—C-MULTI-CHANNEL-CAM-STOP-REQUEST parameters

| Name | Type | Length | Description |
| --- | --- | --- | --- |
| Transaction ID | Integer | 16 bits | Identifier of CAM stop operation transaction |
| Confirmation Needed | Integer | 1 bit | 0: No confirmation needed 1: Confirmation needed |
| Stop channel number | Integer | 8 bits | Channel that stop operation is requested for |

          - When generated

- BS-CHU sends this primitive when it has determined it needs to stop operating on a channel to protect incumbent.

- BS-CHU sends this primitive when it has received an update to channel availability from database.

- BS-CHU sends this primitive when it has determined a new channel to operate on.

Information in this primitive is carried to the CPE via the CAM-STP message defined in 7.7.29.3.

          - Effect of receipt

The CPE-CHU forwards the stop operation request to its’ CPE-CAM, and sets the stop operation timer (see procedure in 7.24.2.2).

        - C-MULTI-CHANNEL-CAM-STOP-CONFIRM

          - Purpose

Let the BS-CHU that the stop operation has been executed.

          - SAP type

C-SAP

          - Operation type

Information Confirmation

          - Destination

BS

          - Data

Table xxx—C-MULTI-CHANNEL-CAM-STOP-CONFIRM

| Name | Type | Length | Description |
| --- | --- | --- | --- |
| Transaction ID | Integer | 16 bits | Transaction ID copied from C-MULTI-CHANNEL-CAM-STOP-REQUST primitive |
| Confirmation Code | Integer | 8 bits | See 7.7.24 |

          - When generated

- CPE-CHU sends this when it has received indication from CPE-CAM that stop operation has been approved, and a confirmation was requested in C-MULTI-CHANNEL-CAM-STOP-REQUEST

Information in this primitive is carried to the BS via the CAM-STP-ACK message (see 7.7.29.4).

          - Effect of receipt

BS-CHU checks stop operation timer and stops it, informs BS-CAM that stop operation has been executed (see procedure in 7.24.2.2).

        - C-MULTI-CHANNEL-CAM-SWITCH-REQUEST

          - Purpose

Sent by BS-CHU to CPE-CHU to indicate a switch in operating channel.

          - SAP type

C-SAP

          - Operation type

Information Request

          - Destination

CPE

          - Data

Table xxx—C-MULTI-CHANNEL-SWITCH-REQUEST parameters

| Name | Type | Length | Description |
| --- | --- | --- | --- |
| Transaction ID | Integer | 16 bits | Identifier of channel switch request transaction |
| Confirmation Needed | Integer | 1 bit | 0: No confirmation needed 1: Confirmation Needed |
| Switch Mode | Integer | 1 bit | 0: No restrictions of transmissions until scheduled channel switch 1: addressed CPE shall end transmissions until scheduled channel switch |
| Switch Count | Integer | 8 bits | The # of frames (from reception of this primitive), that the BS will switch to new channel |
| Switch Channel Number | Integer | 8 bits | Target channel that is being switched to. |

          - When generated

- BS-CHU sends this primitive when it has determined it needs to switch operating channel to protect incumbent.

- BS-CHU sends this primitive when it has received an update to channel availability from database.

- BS-CHU sends this primitive when it has determined a new channel to operate on.

Information in this primitive is carried to the CPE via the CAM-SWH message defined in 7.7.29.5.

          - Effect of receipt

The CPE-CHU forwards switch operation request to its’ CPE-CAM, and sets the switch operation timer (se procedure in 7.24.2.2).

        - C-MULTI-CHANNEL-SWITCH-CONFIRM

          - Purpose

Inform BS-CHU that channel switch operation has been executed by CPE-CHU.

          - SAP type

C-SAP

          - Operation type

Information Confirmation

          - Destination

CPE

          - Data

Table xxx—C-MULTI-CHANNEL-SWITCH-CONFIRM parameters

| Name | Type | Length | Description |
| --- | --- | --- | --- |
| Transaction ID | Integer | 16 bits | Transaciton ID copied from C-MULTI-CHANNEL-SWITCH-REQUEST |
| Confirmation Code | Integer | 8 bits | See 7.7.24 |

          - When generated

- CPE-CHU schedules channel switch. After CPE-CHU receives notification from CPE-CAM that channel switch operation will be executed, and the confirmation requested bit was set in the C-MULTI-CHANNEL-SWITCH-REQUEST.

Information in this primitive is carried to the BS-CHU via the CAM-SWH-ACK message defined in 7.7.29.6.

          - Effect of receipt

When BS-CHU receives this primitive and schedules the channel switch operation.

---end of text modification---

4) add new section 14.1.8 “wranIfABsMib”

---start of text modification---

14.1.8  wranIfABsMib

This MIB represents MIB functionality related to A-BS operation only. It is based on the FCAPS reference model, but is limited to the following items: Fault, Configuration, and Performance. The following list defines MIBs that compose  wranIfABsMib:

- wranIfABsFm: Fault management

- wranIfABsCm: Configuration management

- wranIfABsPm: Performance management/measurement

---end of text modification---

5) add new section 14.1.8.1 “wranIfABsFm” in section 14.1.8

---start of text modification---

14.1.8.1 wranIfABsFm

Exceptions and fault events can be reported by using the traps defined in this MIB. wranIfBsFm is made up of the following MIBs: wranIfABsTrapControl, wranIfABsTrapDefinition, wranIfABsNotificationObjectsTable, wranIfABsFmMibGroups, and wranIfABsFmMibCompliance.

14.1.8.1.1 wranIfABsTrapControl

This MIB is a bitmap that is used to enable/disable the following A-BS traps: wranIfABsGroupResourceAllocationNotification, wranIfABsMultiChannelConfigNotification, wranIfABsRelayCpeConfigNotification, and wranIfABsTwoLinkArqNotification. Enabling of a trap, indicates that the trap will be recorded in wranIfABsNotificationObjectsTable.

14.1.8.1.2 wranIfABsTrapDefinition

This object defines A-BS related traps for related to configuration and performance management.

14.1.8.1.2.1 wranIfABsGroupResourceAllocationTrap

This trap contains information related to the updating of group configurations used for group resource allocation.

14.1.8.1.2.2 wranIfABsMultiChannelConfigTrap

This trap contains information related to the updating of CHU/CAM configurations used for multi-channel operation.

14.1.8.1.2.3 wranIfABsRelayCpeConfigTrap

This trap contains information related to the changing/updating configuration of relay CPEs operating in the network.

14.1.8.1.2.4 wranIfABsTwoLinkArqTrap

This trap contains information related to the upating of Two-Link ARQ metrics.

14.1.8.1.3 wranIfABsNotificationObjectsTable

This table contains objects that represent notifications reported in A-BS fault traps. The table is made up of one or more entries.

14.1.8.1.3.1 wranIfABsNotificationObjectsEntry

This MIB is a compound object that represents an entry in the MIB object wranIfABsNotificationObjectsTable.

14.1.8.1.3.1.1 wranIfABsNotificationObjectsIndex

Index of entry in the table.

14.1.8.1.3.1.2 wranIfABsNotificationMacAddr

The MAC Address of the CPE reporting the notification.

14.1.8.1.3.1.3 wranIfABsGroupResourceAllocationConfigType

Indication as to whether or not CPE has had a GRA configuration added, modififed, or deleted.

14.1.8.1.3.1.4 wranIfABsGroupResourceAllocationConfigIndex

Index into wranIfABsGroupResourceAllocConfigTable that indicates contains the current GRA configuration/status for the CPE.

14.1.8.1.3.1.5 wranIfABsMultiChannelConfigType

Indication as to whether or not CPE has had a CAM/CHU configuration added, modified, or deleted.

14.1.8.1.3.1.6 wranIfABsMultiChannelConfigIndex

Index into wranIfABsMultiChnConfigTable that indicates contains the current CAM/CHU configuration/status for the CPE.

14.1.8.1.3.1.7 wranIfABsRelayCpeConfigIndex

Index into wranIfABsRelayConfigTable that indicates contains the current relay configuration/status for the CPE.

14.1.8.1.3.1.8 wranIfABsTwoLinkArqIndex

Index into wranIfABsArqMetricsTable that indicates contains the current Two-Link ARQ statistics for the CPE.

14.1.8.1.4 wranIfABsFmMibGroups

This object helps define which MIB groups are available in this module (wranIfABsFm) and which MIB objects are part of each group.

14.1.8.1.5 wranIfABsFmMibCompliance

Defines which objects are optional and which ones are mandatory for wranIfABsFm compliance.

---end of text modification---

6) add new section 14.1.8.2 “wranIfABsCm” in section 14.1.8.1

---start of text modification---

14.1.8.2 wranIfABsCm

This MIB contains various objects related to Configruation Management at the A-BS. Within this MIB, the following tables are handled:

- wranIfABsCpeBasicCapabilityReqTable

- wranIfABsCpeBasicCapabilityRspTable

- wranIfABsCpeBasicCapabilityDefTable

- wranIfABsCpeBasicCapabilityCmn

- wranIfABsGraConfigTable

- wranIfABsMultiChnConfigTable

- wranIfABsRelayConfigTable

- wranIfABsPhy

- wranIfABsCmMibGroups

- wranIfABsCmMibCompliance

14.1.8.2.1 wranIfABsCpeBasicCapabilityReqTable

This table contains objects that define capabilities negotiated by A-CPEs during network entry. The table is made up of one or more entries, defined by wranIfABsCpeBasicCapabilityReqEntry.

14.1.8.2.1.1 wranIfABsCpeBasicCapabilityReqEntry

This is object is a compound object that contains information on capabilities A-CPEs have declared to A-BS during network entry. This list of objects that make up wranIfABsCpeBasicCapabilityReqEntry can be found in wranIfABsCpeBasicCapabilityCmn (see 14.1.8.2.4).

14.1.8.2.2 wranIfABsCpeBasicCapabilityRspTable

This table contains objects that define capabilities the A-BS has configured for A-CPEs during network entry. The table is made up of one or more entries, defined by wranIfABsCpeBasicCapabilityRspEntry.

14.1.8.2.2.1 wranIfABsCpeBasicCapabilityReqEntry

This is object is a compound object that contains information on capabilities the A-BS have configured for A-CPEs during network entry. This list of objects that make up wranIfABsCpeBasicCapabilityReqEntry can be found in wranIfABsCpeBasicCapabilityCmn (see 14.1.8.2.4).

14.1.8.2.3 wranIfABsCpeBasicCapabilityDefTable

This table contains objects that define default values of CBC-REQ/RSP IEs for A-CPEs. This table can be used to judge /verify a CPE’s CBC-REQ and used to construct the CBC-RSP message. This table has one entry defined by wranIfABsCpeBasicCapabilityDefEntry.

14.1.8.2.3.1 wranIfABsCpeBasicCapabilityDefEntry

This is object is a compound object that defines a entry in wranIfABsCpeBasicCapabilityDefTable. The objects that make up this compound object are defined in wranIfABsCpeBasicCapabilityCmn (see 14.1.8.2.4).

14.1.8.2.4 wranIfABsCpeBasicCapabilityCmn

This MIB object is a group containing objects that are common to wranIfABsCpeBasicCapabilityReqEntry, wranIfABsCpeBasicCapabilityRspEntry, and wranIfABsBasicCapabilitiesDefEntry objects.

14.1.8.2.4.1 wranIfABsBasicCapabilityIndex

Index of entry into wranIfABsCpeBasicCapabilityReqTable, wranIfABsCpeBasicCapabilityRspTable, and wranIfABsCpeBasicCapabilityDefTable.

14.1.8.2.4.2 wranIfABsBasicCapabilityCrzModulator

Defines modulator setting for Centralized Scheduling A-CPE

14.1.8.2.4.3 wranIfABsBasicCapabilityCrzDemodulator

Defines demodulator setting for Centralized Scheduling A-CPE

14.1.8.2.4.4 wranIfABsBasicCapabilityRelayCpeMode

Defines Relay CPE operational mode

14.1.8.2.4.5 wranIfABsBasicCapabilityMultichannelSupport

Defines what multi-channel capability the A-CPE supports

14.1.8.2.4.6 wranIfABsBasicCapabilityMacAddress

MAC Address of A-CPE

14.1.8.2.5 wranIfABsGraConfigTable

This MIB object defines a table containing information on all groups created for supporting group resource allocation (GRA) operation.

14.1.8.2.5.1 wranIfABsGraConfigEntry

Compound object that defines an entry into wranIfABsGraConfigTable.

14.1.8.2.5.1.1 wranIfABsGraConfigIndex

Index of entry into wranIfABsGraConfigTable.

14.1.8.2.5.1.2 wranIfABsGraCpeMacAddress

MAC Address of CPE

14.1.8.2.5.1.3 wranIfABsGraCpeSid

Current Station ID of CPE

14.1.8.2.5.1.4 wranIfABsGraGroupId

Id of GRA group that CPE belongs to.

14.1.8.2.5.1.5 wranIfABsGraGroupType

GRA group type, fixed or portable.

14.1.8.2.5.1.6 wranIfABsGraGroupLat

Latitude portion of group location

14.1.8.2.5.1.7 wranIfABsGraGroupLon

Longitude portion of group location

14.1.8.2.5.1.8 wranIfABsGraMaxGroupSize

Current maximum size of GRA group.

14.1.8.2.6 wranIfABsMultiChnConfigTable

This MIB object defines a table containing information for all CAM/CHUs configured for CPEs.

14.1.8.2.6.1 wranIfABsMultiChnConfigEntry

Compound object that defines an entry into wranIfABsMultiChnConfigTable.

14.1.8.2.6.1.1 wranIfABsMultiChnConfigIndex

Index of entry into wranIfABsMultiChnConfigTable.

14.1.8.2.6.1.2 wranIfABsMultiChnMacAddress

MAC Address of CPE

14.1.8.2.6.1.4 wranIfABsMultiChnMaxAggChns

Maximum number of aggregation channels allowed at CPE

14.1.8.2.6.1.5 wranIfABsMultiChnCurNumAggChns

Current number of aggregation channels used by CPE

14.1.8.2.6.1.6 wranIfABsMultiChnAggChnList

Octet string containing list of channels assigned to CPE for use as aggregation channels

14.1.8.2.7 wranIfABsRelayConfigTable

This MIB object defines a table containing DS/US zone configuration for Relay CPEs.

14.1.8.2.7.1 wranIfABsRelayConfigEntry

Compound object that defines an entry into wranIfABsRealyConfigTable. Each CPE has multiple instances of this object in the table, one for each zone the Relay CPE is configured for.

14.1.8.2.7.1.1 wranIfABsRelayConfigIndex

Index of entry into wranIfABsRelayConfigTable.

14.1.8.2.7.1.2 wranIfABsRelayConfigMacAddress

MAC Address of relay CPE.

14.1.8.2.7.1.3 wranIfABsRelayZoneIndex

Index of relay zone.

14.1.8.2.7.1.4 wranIfABsRelayZoneMode

Mode of zone, i.e. access, centralized relay, distributed relay

14.1.8.2.7.1.5 wranIfABsRelayZoneSegStartPos

Start position of segment, if zone mode is distributed relay.

14.1.8.2.7.1.6 wranIfABsRelayZoneSegBandwidth

Bandwidth of segment, if zone mode is distributed relay.

14.1.8.2.7.1.7 wranIfABsRelayZonePhyMode

PHY mode, i.e. OM1 or OM2, of relay zone.

14.1.8.2.7.1.8 wranIfABsRelayZoneOfdmaSymOffset

Zone starts at this OFDMA symbol offset, counted after the preamble of the frame.

14.1.8.2.7.1.9 wranIfABsRelayCpeSid

SID of distributed scheduling relay CPE.

14.1.8.2.7.1.10 wranIfABsRelayZoneDir

Indication if this is an US or DS relay zone.

14.1.8.2.8 wranIfABsPhy

MIB object contains managed objects related to PHY configuration.

14.1.8.2.8.1 wranIfABsOfdmaPhyUsChannelTable

Table that defines attributes of US channels. Made of multiple entries, defined by wranIfABsOfdmaPhyUsChannelEntry.

14.1.8.2.8.1.1 wranIfABsOfdmaPhyUsChannelEntry

Compound object that defines entry in wranIfABsOfdmaPhyUsChannelTable.

14.1.8.2.8.1.1.1 wranIfABsOfdmaPhyUsChannelIndex

Index of entry in table.

14.1.8.2.8.1.1.2 wranIfABsOfdmaPhyUsSid

SID of CPE whose US is defined in this entry.

14.1.8.2.8.1.1.3 wranIfABsOfdmaPhyUsMacAddress

MAC Address of CPE whose US is defined in this entry.

14.1.8.2.8.1.1.4 wranIfABsOfdmaPhyUsOM

PHY operational mode of CPE US.

14.1.8.2.8.1.1.5 wranIfABsOfdmaPhyUsSegment

Distributed relay zone US segment that CPE is operating in. Defaults to ‘0’ indicating CPE operating through CRZ or directly attached to A-BS.

14.1.8.2.8.1.1.6 wranIfABsOfdmaPhyUsCenterFrequency

Upstream center frequency in kHz.

14.1.8.2.8.2 wranIfABsOfdmaPhyDsChannelTable

Table that defines attributes of DS channels. Made of multiple entries, defined by wranIfABsOfdmaPhyDsChannelEntry.

14.1.8.2.8.2.1 wranIfABsOfdmaPhyDsChannelEntry

Compound object that defines entry in wranIfABsOfdmaPhyDsChannelTable.

14.1.8.2.8.2.1.1 wranIfABsOfdmaPhyDsChannelIndex

Index of entry in table.

14.1.8.2.8.2.1.2 wranIfABsOfdmaPhyDsSid

SID of CPE whose DS is defined in this entry.

14.1.8.2.8.2.1.3 wranIfABsOfdmaPhyDsMacAddress

MAC Address of CPE whose DS is defined in this entry.

14.1.8.2.8.2.1.4 wranIfABsOfdmaPhyDsOM

PHY operational mode of CPE US.

14.1.8.2.8.2.1.5 wranIfABsOfdmaPhyDsSegmentConfig

Distributed relay zone US segment that CPE is operating in. Defaults to ‘0’ indicating CPE operating through CRZ or directly attached to A-BS.

14.1.8.2.8.2.1.6 wranIfABsOfdmaPhyDsCenterFrequency

Centery frequency of DS in kHz.

14.1.8.2.8.2.1.7 wranIfABsOfdmaPhyDsCellType

Classes of BS that CPE can be used to attempt network entry.

14.1.8.2.8.2.1.8 wranIfABsOfdmaPhyDsMacVersion

Version of MAC run in the DS/US.

14.1.8.2.8.3 wranIfABsOfdmaPhyUsRegionTable

Table that defines configuration of US subframe. Made of multiple entries, defined by wranIfABsOfdmaPhyUsRegionEntry.

14.1.8.2.8.3.1 wranIfABsOfdmaPhyUsRegionEntry

Compound object that defines entry in wranIfABsOfdmaPhyUsRegionTable.

14.1.8.2.8.3.1.1 wranIfABsOfdmaPhyUsRegionIndex

Index of entry in table.

14.1.8.2.8.3.1.2 wranIfABsOfdmaPhyUsRegionSegmentNum

Segment of US region, defaults to ‘0’ if this segment is  the US of a CRZ relay.

14.1.8.2.8.3.1.3 wranIfABsOfdmaPhyUsRegionDuration

Number of OFDMA slots allocated to region.

14.1.8.2.8.4 wranIfABsOfdmaPhyDsRegionTable

Table that defines configuration of DS subframe. Made of multiple entries, defined by wranIfABsOfdmaPhyDsRegionEntry.

14.1.8.2.8.4.1 wranIfABsOfdmaPhyDsRegionEntry

Compound object that defines entry in wranIfABsOfdmaPhyDsRegionTable.

14.1.8.2.8.4.1.1 wranIfABsOfdmaPhyDsRegionIndex

Index of entry in table.

14.1.8.2.8.4.1.2 wranIfABsOfdmaPhyDsRegionSegmentNum

Segment of US region, defaults to ‘0’ if this segment is  the DS of a CRZ relay.

14.1.8.2.8.4.1.3 wranIfABsOfdmaPhyDsRegionSegmentNum

Numberf of OFDMA slots allocated to region.

14.1.8.2.9 wranIfABsCmMibGroups

This object helps define which MIB groups are available in this module (wranIfABsCm) and which MIB objects are part of each group.

14.1.8.2.10 wranIfABsCmMibCompliance

Defines which objects are optional and which ones are mandatory for wranIfABsCm compliance.

---end of text modification---

7) add new section 14.1.8.3 “wranIfABsPm” in section 14.1.8.2

---start of text modification---

14.1.8.3 wranIfABsPm

This MIB contains various objects related to Perfmance Management at the A-BS. Within this MIB, the following tables are handled:

- wranIfABsPmConfigurationTable

- wranIfABsArqMetricsTable

- wranIfABsPmMibGroups

- wranIfABsPmMibComplieance

14.1.8.3.1 wranIfABsPmConfigurationTable

The configuration of statistics capture and measurement at the A-BS is captured in this table. There is one entry int this table for each BS sector. The entries are defined by wranIfBsPmConfigurationEntry.

14.1.8.3.1.1 wranIfABsPmConfigurationEntry

This object defines a compound object that represents an entry in wranIfABsPmConfigurationTable.

14.1.8.3.1.1.1 wranIfABsPmConfigurationIndex

Index of entry into table, defaults to 1.

14.1.8.3.1.1.2 wranIfABsGranularityInterval

The interval in which statistics captured in wranIfABsTrapControl related to performance measurement reported to the NCMS.

14.1.8.3.1.1.3 wranIfABsTwoLinkArqConfig

This object indicates whether or not Two Link ARQ measurements (wranIfABsArqMetricsTable).

14.1.8.3.2 wranIfABsArqMetricsTable

This MIB provides a table to track performance of Two Link ARQ being used with S-CPEs attached to A-BS through distributed scheduling A-CPE. The entries are defined by wranIfABsArqMetricsEntry.

14.1.8.3.2.1 wranIfABsArqMetricsEntry

This object defines a compound object that represents an entry in wranIfABsArqMetricsTable.

14.1.8.3.2.1.1 wranIfABsArqMetricsIndex

The index of each entry into wranIfABsArqMetricsTable.

14.1.8.3.2.1.2 wranIfABsRelayLinkDsNumArqBlocks

The total # of ARQ blocks, include retransmissions, that the A-BS has sent on the relay link (to distributed scheduling A-CPE) during the reporting period.

14.1.8.3.2.1.3 wranIfABsAccessLinkDsNumArqBlocks

The total # of ARQ blocks, include retransmissions, that A-CPEs has sent on the access link during the reporting period.

14.1.8.3.2.1.4 wranIfABsRelayLinkDsNumArqBlocksDropped

Total # of ARQ blocks that were dropped, that the A-BS has sent during the reporting period.

14.1.8.3.2.1.5 wranIfABsAccessLinkDsNumArqBlocksDropped

The total # of ARQ blocks that were dropped, that A-CPEs has sent on the access link during the reporting period.

14.1.8.3.2.1.6 wranIfABsRelayLinkDsArqBlockErrorRate

Error Rate on DS Relay Link.

14.1.8.3.2.1.7 wranIfABsAccessLinkDsArqBlockErrorRate

Error Rate on DS Access Link.

14.1.8.3.2.1.8 wranIfABsRelayLinkDsArqBlockReTx

The # of ARQ blocks that were retransmitted by the A-BS during the reporting period.

14.1.8.3.2.1.9 wranIfABsAccessLinkDsArqBlockReTx

The # of ARQ blocks that were retransmitted by the A-CPE during the reporting period.

14.1.8.3.2.1.10 wranIfABsRelayLinkUsNumArqBlocks

The total # of ARQ blocks, include retransmissions, that the A-BS has received on the relay link (from distributed scheduling A-CPE) during the reporting period.

14.1.8.3.2.1.11 wranIfABsAccessLinkUsNumArqBlocks

The total # of ARQ blocks, include retransmissions, that A-CPEs has received on the access link during the reporting period.

14.1.8.3.2.1.12 wranIfABsRelayLinkUsNumArqBlocksDropped

Total # of ARQ blocks that were dropped, that the A-BS has received during the reporting period.

14.1.8.3.2.1.13 wranIfABsAccessLinkUsNumArqBlocksDropped

The total # of ARQ blocks that were dropped, that A-CPEs has received on the access link during the reporting period.

14.1.8.3.2.1.14 wranIfABsRelayLinkUsArqBlockErrorRate

Error Rate on US Access Link.

14.1.8.3.2.1.15 wranIfABsAccessLinkUsArqBlockErrorRate

Error Rate on US Relay Link.

14.1.8.3.2.1.16 wranIfABsRelayLinkUsArqBlockRetransmission

The # of ARQ blocks that were retransmitted by the A-CPE during the reporting period on the relay link.

14.1.8.3.2.1.17 wranIfABsAccessLinkUsArqBlockRetransmission

The # of ARQ blocks that were retransmitted by the S-CPE during the reporting period on the access link.

14.1.8.3.2.1.18 wranIfABsRelayLinkDsEfficiency

Efficiency of transmission on DS relay link.

14.1.8.3.2.1.19 wranIfABsAccessLinkDsEfficiency

Efficiency of transmission on DS access link.

14.1.8.3.2.1.20 wranIfABsRelayLinkUsEfficiency

Efficiency of transmission on US relay link.

14.1.8.3.2.1.21 wranIfABsAccessLinkUsEfficiency

Efficiency of transmission on US access link.

14.1.8.3.3 wranIfABsPmMibGroups

This object helps define which MIB groups are available in this module (wranIfABsPm) and which MIB objects are part of each group.

14.1.8.3.4 wranIfABsPmMibCompliance

Defines which objects are optional and which ones are mandatory for wranIfBsPm compliance.

---end of text modification---

8) add new section 14.1.9 “wranIfACpeMib”

---start of text modification---

14.1.9  wranIfACpeMib

This MIB represents MIB functionality related to managing A-CPEs. It is broken up into the following sublclauses:

- wranIfACpeSystemParametersTable: Definition of system parameters, timers, and constants related to CPE operation

- wranIfACpeMibGroups: Defines which MIB groups are available in this module and which MIB objects are a part of each group

- wranIfACpeMibCompliance: Defines which MIB objects are optional, and which ones are mandatory for CPE conformance

---end of text modification---

9) add new section 14.1.9.1 “wranIfACpeConfigurationTable” after 14.1.9

---start of text modification---

14.1.9.1 wranIfACpeSystemParametersTable

This table contains objects that define system constants related to A-BS/A-CPE operation.

14.1.9.1.1 wranIfACpeSystemParametersEntry

This is a compound object that defines an entry in wranIfABsCpeSystemParametersTable.

14.1.9.1.1.1 wranIfACpeSystemParametersIndex

This is a compound object that defines an entry in wranIfABsCpeSystemParametersTable.

14.1.9.1.1.2 wranIfABsT61

Timeout for waiting for container ACK message.

14.1. 9.1.1.3 wranIfABsT62

Timeout for waiting for local cell update acknowledgement.

14.1. 9.1.1.4 wranIfABsT63

Timeout for DTT-RSP message.

14.1. 9.1.1.5 wranIfABsT64

Timeout for DTT-RPT message.

14.1. 9.1.1.6 wranIfABsT65

Timeout for DTT-CFM message.

14.1. 9.1.1.7 wranIfABsT66

Timeout for CAM-STP message.

14.1.9.1.1.8 wranIfABsT67

Timeout for CAM-SWH message.

---end of text modification---

10) add new section 14.1.9.2 “wranIfACpeMibGroups” after 14.1.9.1

---start of text modification---

14.1.9.2 wranIfACpeMibGroups

This object helps define which MIB groups are available in this module (wranIfACpe) and which MIB objects are part of each group.

---end of text modification---

11) add new section 14.1.9.3 “wranIfACpeMibCompliance” after 14.1.9.2

---start of text modification---

14.1.9.3 wranIfACpeMibCompliance

This defines which MIB objects are optional and mandatory for CPE conformance

---end of text modification---

12) add new section 14.2.8 “wranIfABsMib” after 14.2.7, and add text for “wranIfABsFm”

---start of text modification---

14.2.8 wranIfABsMib

IEEE802dot22-WRAN-IF-ABS-MIB		::=		BEGIN

IMPORTS

MODULE-IDENTITY,

OBJECT-TYPE,

NOTIFICATION-TYPE,

Unsigned32, Integer32

FROM SNMPv2-SMI

SnmpAdminString

FROM SNMP-FRAMEWORK-MIB

TEXTAL-CONVENTION,

RowStatus, TruthValue,

TimeStamp, DateAndTime

FROM SNMPv2-TC

InetAddressType, InetAddress

FROM INET-ADDRESS-MIB

OBJECT-GROUP,

MODULE-COMPLIANCE,

NOTIFICATION-GROUP

FROM SNMPv2-CONF;

wranIfABsMib	MODULE-IDENTITY

LAST-UPDATED		“201808300000Z” – August 30, 2018

ORGANIZATION		“IEEE 802.22”

CONTACT-INFO

“WG E-mail:	STDS-802-22@LISTSERVE.IEEE.ORG

WG Chair:	Apurva N. Mody

E-mail:	apurva.mody@ieee.org

Maintenance Editor:	Ranga Reddy

E-mail:			ranga.reddy@ieee.org”

DESCRIPTIOIN

“This material was added to support newer functionality

Introduced with the IEEE Std 802.22b-2015 Copyright (c)

2015. This MIB Module defines managed objects for Advanced

Base Station based on IEEE Std 802.22b-2015 and is under

iso(1).std(0).iso8022(8802).wran(22).wranIfAbsMib(8)”

REVISION		“201808300000Z”

DESRIPTION

“The first version of IEEE802dot22-WRAN-IF-ABS-MIB that

Provides for management of the Advanced Base Station.”

::= {iso std(0) iso8802(8802) wran(22) 8}

wranIfABsFm			OBJECT IDENTIFIER ::= { wranIfABsMib 1 }

wranIfABsCm			OBJECT IDENTIFIER ::= { wranIfABsMib 2 }

wranIfABsPm			OBJECT IDENTIFIER ::= { wranIfABsMib 3 }

-- wranIfABsFm: Exceptions and fault events related to ABS operation can be

-- reported using the traps defined in this MIB.

wranIfABsTrapControl			OBJECT IDENTIFIER ::= { wranIfABsFm 1 }

wranIfABsTrapDefinition			OBJECT IDENTIFIER ::= { wranIfABsFm 2 }

wranIfABsNotificationObjectsTable	OBJECT IDENTIFIER ::= { wranIfABsFm 3 }

wranIfABsFmMibGroups			OBJECT IDENTIFIER ::= { wranIfABsFm 4 }

wranIfABsFmMibCompliance		OBJECT IDENTIFIER ::= { wranIfABsFm 5 }

wranIfABsTrapControl	OBJECT-TYPE

SYNTAX	BITS { gra(0),

multiChannel(1),

relayCpeConfig(2),

twoLinkArq(3) {

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

“This object is a bitmap that allows enabling/disabling

of traps related to setup/configuration of Group

Resource Allocation configuration (Bit0), Multi-Channel

configuration (Bit1), Relay CPE configuration (Bit2), and

Two-Link ARQ metrics notification (Bit3).”

::= { wranIfABsFm 1 }

wranIfABsTrapDefinition				OBJECT IDENTIFIER

::= { wranIfABsFm 2 }

wranIfABsGroupResourceAllocationTrap	OBJECT IDENTIFIER

::= { wranIfABsTrapDefinition 1 }

wranIfABsMultiChannelConfigTrap		OBJECT IDENTIFIER

::= { wranIfABsTrapDefinition 2 }

wranIfABsRelayCpeConfigTrap			OBJECT IDENTIFIER

::= { wranIfABsTrapDefinition 3 }

wranIfABsTwoLinkArqTrap				OBJECT IDENTIFIER

::= { wranIfABsTrapDefinition 4 }

wranIfABsGroupResourceAllocationTrap	NOTIFICATION-TYPE

OBJECTS	{ wranIfABsGroupResourceAllocationConfigType,

wranIfABsGroupResourceAllocationConfigIndex }

STATUS	current

DESCRIPTION

“This trap contains information related to updating the

configuration of groups for Group Resource Allocation

operation stored in wranIfABsGraConfigTable.”

::= { wranIfAbsTrapDefinition 1 }

wranIfABsMultiChannelConfigTrap	NOTIFICATION-TYPE

OBJECTS	{ wranIfABsMultiChannelConfigType,

wranIfABsMultiChannelConfigIndex }

STATUS	current

DESCRIPTION

“This trap contains information related to updating the

configuration of CAMs/CHUs for Multi-channel operation

stored in wranIfABsMultiChnTable.”

::= { wranIfAbsTrapDefinition 2 }

wranIfABsRelayCpeConfigTrap	NOTIFICATION-TYPE

OBJECTS	{ wranIfABsRelayCpeConfigIndex }

STATUS	current

DESCRIPTION

“This trap contains information related to updating the

configuration of Relay CPE operation stored in

wranIfABsRelayConfigTable.”

::= { wranIfAbsTrapDefinition 3 }

wranIfABsTwoLinkArqTrap	NOTIFICATION-TYPE

OBJECTS	{ wranIfABsTwoLinkArqIndex }

STATUS	current

DESCRIPTION

“This trap contains information related to updating of

Two-Link ARQ metrics stored in wranIfABsArqMetricsTable.”

::= { wranIfAbsTrapDefinition 4 }

wranIfABsNotificationObjectsTable	OBJECT-TYPE

SYNTAX	SEQUENCE OF wranIfABsNotificationObjectsEntry

STATUS	current

DESCRIPTION

“This table contains objects that represent notiifcations

Reported in A-BS fault traps. The table is made up of one or

more entries.”

::= { wranIfABsFm 3 }

wranIfABsNotificationObjectsEntry	OBJECT-TYPE

SYNTAX	wranIfABsNotificationObjectsEntry

MAX-ACCESS	not-accessible

STATUS	current

DESCRIPTION

“This MIB is a compound object that represents an entry in the

MIB object wranIfABsNotificationObjectsTable.”

INDEX	{ wranIfABsNotificationObjectsIndex }

::= { wranIfABsNotificationObjectsTable 1 }

wranIfABsNotificationObjectsEntry		::= SEQUENCE {

wranIfABsNotificationObjectsIndex			INTEGER,

wranIfABsNotificationMacAddr				MacAddress,

wranIfABsGroupResourceAllocationConfigType	INTEGER,

wranIfABsGroupResourceAllocationConfigIndex	Integer32,

wranIfABsMultiChannelConfigType			INTEGER,

wranIfABsMultiChannelConfigIndex			Integer32,

wranIfABsRelayCpeConfigIndex				Integer32,

wranIfABsTwoLinkArqIndex				Integer32 }

wranIfABsNotificationObjectsIndex	OBJECT-TYPE

SYNTAX	INTEGER (1..1)

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

“Index of entry in table.”

::= { wranIfABsNotificationObjectsEntry 1 }

wranIfABsNotificationMacAddr	OBJECT-TYPE

SYNTAX	MacAddress

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

“MAC address reporting the notification.”

::= { wranIfABsNotificationObjectsEntry 2 }

wranIfABsGroupResourceAllocationConfigType	OBJECT-TYPE

SYNTAX	INTEGER { added(0), modified(1), deleted(2) }

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

“Indication as to whether or not CPE has had a GRA

Configuration added, modified, or deleted.”

::= { wranIfABsNotificationObjectsEntry 3 }

wranIfABsGroupResourceAllocationConfigIndex	OBJECT-TYPE

SYNTAX	Integer32 { 1..2048 }

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

“Index into wranIfABsGroupResourceAllocationConfigTable

that indicates contains the current GRA configuration/

status for the CPE.”

::= { wranIfABsNotificationObjectsEntry 4 }

wranIfABsMultiChannelConfigType	OBJECT-TYPE

SYNTAX	INTEGER { added(0), modified(1), deleted(2) }

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

“Indication as to whether or not CPE has had a

CAM/CHU added, modified, or deleted.”

::= { wranIfABsNotificationObjectsEntry 5 }

wranIfABsMultiChannelConfigIndex	OBJECT-TYPE

SYNTAX	Integer32 { 1..2048 }

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

“Index into wranIfABsMultiChnConfigTable that contains

the current CAM/CHU configuration/status for the CPE.”

::= { wranIfABsNotificationObjectsEntry 6 }

wranIfABsRelayCpeConfigIndex	OBJECT-TYPE

SYNTAX	Integer32 { 1..2048 }

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

“Index into wranIfABsRelayConfigTable that contains the current

relay configuration/status for the CPE.”

::= { wranIfABsNotificationObjectsEntry 7 }

wranIfABsTwoLinkArqIndex	OBJECT-TYPE

SYNTAX	Integer32 { 1..2048 }

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

“Index into wranIfABsArqMetricsTable that indicates contains the

current Two-Link ARQ statistics for the CPE.”

::= { wranIfABsNotificationObjectsEntry 8 }

-- wranIfABsFmMibGroups: This object helps define which MIB groups are available

-- in this module (wranIfABsFm) and which MIB objects are part of each group.

wranIfABsFmMibGroups			OBJECT IDENTIFIER

::= { wranIfABsFm 4 }

wranIfABsFmTrapControlGroup		OBJECT IDENTIFIER

::= { wranIfABsFmMibGroups 1 }

wranIfABsFmNotificationGroup		OBJECT IDENTIFIER

::= { wranIfABsFmMibGroups 2 }

wranIfABsFmTrapControlGroup		OBJECT-GROUP

OBJECTS	{ wranfABsTrapControl }

STATUS	current

DESCRIPTION

“This group contains objects related to enabling/disabling traps

at A-BS.”

::= { wranIfABsFmMibGroups 1 }

wranIfABsFmNotificationGroup		OBJECT-GROUP

OBJECTS	{ wranIfABsGroupResourceAllocationTrap,

wranIfABsMultiChannelConfigTrap,

wranIfABsRelayCpeConfigTrap,

wranIfABsTwoLinkArqTrap,

wranIfABsGroupResourceAllocationTrap,

wranIfABsMultiChannelConfigTrap,

wranIfABsRelayCpeConfigTrap,

wranIfABsTwoLinkArqTrap,

wranIfABsNotificationObjectsIndex,

wranIfABsNotificationMacAddr,

wranIfABsGroupResourceAllocationConfigType,

wranIfABsGroupResourceAllocationConfigIndex,

wranIfABsMultiChannelConfigType,

wranIfABsMultiChannelConfigIndex,

wranIfABsRelayCpeConfigIndex,

wranIfABsTwoLinkArqIndex }

STATUS	current

DESCRIPTION

“This group contains objects related to traps used for management

of the A-BS.”

::= { wranIfABsFmMibGroups 2 }

wranIfABsFmMibCompliance	MODULE-COMPLIANCE

STATUS		current

DESCRIPTION

“MIB objects that are optional and mandatory for

wranIfABsFm compliance.”

MODULE		wranIfABsFm

MANDATORY-GROUPS	{ wranIfABsFmTrapControlGroup }

-- OPTIONAL-GROUPS	{ wranIfABsFmNotificationGroup }

::= { wranIfABsFm 5 }

---end of text modification---

13) add text “wranIfABsCm” after “wranIfABsFm”

---start of text modification---

-- wranIfABsCm: This MIB contains various objects relaed to Configuratio Management

-- at the A-BS.

wranIfABsCpeBasicCapabilityReqTable	OBJECT IDENTIFIER ::= { wranIfABsCm 1 }

wranIfABsCpeBasicCapabilityRspTable	OBJECT IDENTIFIER ::= { wranIfABsCm 2 }

wranIfABsCpeBasicCapabilityDefTable	OBJECT IDENTIFIER ::= { wranIfABsCm 3 }

wranIfABsCpeBasicCapabilityCmn	OBJECT IDENTIFIER ::= { wranIfABsCm 4 }

wranIfABsGraConfigTable			OBJECT IDENTIFIER ::= { wranIfABsCm 5 }

wranIfABsMutliChnConfigTable		OBJECT IDENTIFIER ::= { wranIfABsCm 6 }

wranIfABsRealyConfigTable		OBJECT IDENTIFIER ::= { wranIfABsCm 7 }

wranIfABsPhy				OBJECT IDENTIFIER ::= { wranIfABsCm 8 }

wranIfABsCmMibGroups			OBJECT IDENTIFIER ::= { wranIfABsCm 9 }

wranIfABsCmMibCompliance		OBJECT IDENTIFIER ::= { wranIfABsCm 10 }

wranIfABsCpeBasicCapabilityReqTable		OBJECT-TYPE

SYNTAX	SEQUENCE of wranIfABsCpeBasicCapabilityReqEntry

MAX-ACCESS	not-accessible

STATUS	current

DESCRIPTION

“This table contains objects that define capabilities negotiated

by A-CPEs during network entry. The table is made up of one or

more entries, defined by wranIfABsCpeBasicCapabilityReqEntry.”

::= { wranIfABsCm 1 }

wranIfABsCpeBasicCapabilityReqEntry		OBJECT-TYPE

SYNTAX	wranIfABsCpeBasicCapabilityReqEntry

MAX-ACCESS	not-accessible

STATUS	current

DESCRIPTION

“This object is a compound object that contains information on

Capabilities A-CPEs have declared to A-BS during network entry.

This list of objects that make up

wranIfABsCpeBasicCapabilityReqEntry can be found in

wranIfABsCpeBasicCapabilityCmn (see 14.1.8.2.4).”

INDEX { wranIfABsCpeBasicCapabilityReqIndex }

::= { wranIfABsCpeBasicCapabilityReqTable 1 }

wranIfABsCpeBasicCapabilityReqEntry		::= SEQUENCE {

wranIfABsCpeBasicCapabilityReqIndex

wranIfABsBasicCapabilityIndex,

wranIfABsCpeBasicCapabilityReqCrzModulator

wranIfABsBasicCapabilityCrzModulator,

wranIfABsCpeBasicCapabilityReqCrzDemodulator

wranIfABsBasicCapabilityCrzDemodulator,

wranIfABsCpeBasicCapabilityReqRelayCpeMode

wranIfABsBasicCapabilityRelayCpeMode,

wranIfABsCpeBasicCapabilityReqMutlichannelSupport

wranIfABsBasicCapabilityMultichannelSupport,

wranIfABsCpeBasicCapabilityReqMacAddress

wranIfABsBasicCapabilityMacAddress }

wranIfABsBasicCapabilityReqIndex	OBJECT-TYPE

SYNTAX	wranIfABsBasicCapabilityIndex

MAX-ACCESS	not-accessible

STATUS	current

DESCRIPTION

“Defined by wranIfABsBasicCapablityIndex.”

::= { wranIfABsCpeBasicCapabilityReqEntry 1 }

wranIfABsCpeBasicCapabilityReqCrzModulator	OBJECT-TYPE

SYNTAX	wranIfABsBasicCapabilityCrzModulator

MAX-ACCESS	not-accessible

STATUS	current

DESCRIPTION

“Defined by wranIfABsBasicCapabilityCrzModulator.”

::= { wranIfABsCpeBasicCapabilityReqEntry 2 }

wranIfABsCpeBasicCapabilityReqCrzDemodulator	OBJECT-TYPE

SYNTAX	wranIfABsBasicCapabilityCrzDemodulator

MAX-ACCESS	not-accessible

STATUS	current

DESCRIPTION

“Defined by wranIfABsBasicCapabilityCrzDemodulator.”

::= { wranIfABsCpeBasicCapabilityReqEntry 3 }

wranIfABsCpeBasicCapabilityReqRelayCpeMode	OBJECT-TYPE

SYNTAX	wranIfABsBasicCapabilityRelayCpeMode

MAX-ACCESS	not-accessible

STATUS	current

DESCRIPTION

“Defined by wranIfABsBasicCapabilityRelayCpeMode.”

::= { wranIfABsCpeBasicCapabilityReqEntry 4 }

wranIfABsCpeBasicCapabilityReqMutlichannelSupport	OBJECT-TYPE

SYNTAX	wranIfABsBasicCapabilityMultichannelSupport

MAX-ACCESS	not-accessible

STATUS	current

DESCRIPTION

“Defined by wranIfABsBasicCapabilityMultichannelSupport.”

::= { wranIfABsCpeBasicCapabilityReqEntry 5 }

wranIfABsCpeBasicCapabilityReqMacAddress	OBJECT-TYPE

SYNTAX	wranIfABsBasicCapabilityMacAddress

MAX-ACCESS	not-accessible

STATUS	current

DESCRIPTION

“Defined by wranIfABsBasicCapabilityMacAddress.”

::= { wranIfABsCpeBasicCapabilityReqEntry 6 }

wranIfABsCpeBasicCapabilityRspTable		OBJECT-TYPE

SYNTAX	SEQUENCE of wranIfABsCpeBasicCapabilityRspEntry

MAX-ACCESS	not-accessible

STATUS	current

DESCRIPTION

“This table contains objects the A-BS has configured for

A-CPEs during network entry. The table is made up of one or

Entries, defined by wranIfABsCpeBasicCapabilityRspEntry.”

::= { wranIfABsCm 2 }

wranIfABsCpeBasicCapabilityRspEntry		OBJECT-TYPE

SYNTAX	wranIfABsCpeBasicCapabilityRspEntry

MAX-ACCESS	not-accessible

STATUS	current

DESCRIPTION

“This object is a compound object that contains information on

Capabilities the A-BS have configured for A-CPEs during network

entry.”

INDEX { wranIfABsCpeBasicCapabilityRspIndex }

::= { wranIfABsCpeBasicCapabilityRspTable 1 }

wranIfABsCpeBasicCapabilityRspEntry		::= SEQUENCE {

wranIfABsCpeBasicCapabilityRspIndex

wranIfABsBasicCapabilityIndex,

wranIfABsCpeBasicCapabilityRspCrzModulator

wranIfABsBasicCapabilityCrzModulator,

wranIfABsCpeBasicCapabilityRspCrzDemodulator

wranIfABsBasicCapabilityCrzDemodulator,

wranIfABsCpeBasicCapabilityRspRelayCpeMode

wranIfABsBasicCapabilityRelayCpeMode,

wranIfABsCpeBasicCapabilityRspMutlichannelSupport

wranIfABsBasicCapabilityMultichannelSupport,

wranIfABsCpeBasicCapabilityRspMacAddress

wranIfABsBasicCapabilityMacAddress }

wranIfABsBasicCapabilityRspIndex	OBJECT-TYPE

SYNTAX	wranIfABsBasicCapabilityIndex

MAX-ACCESS	not-accessible

STATUS	current

DESCRIPTION

“Defined by wranIfABsBasicCapablityIndex.”

::= { wranIfABsCpeBasicCapabilityRspEntry 1 }

wranIfABsCpeBasicCapabilityRspCrzModulator	OBJECT-TYPE

SYNTAX	wranIfABsBasicCapabilityCrzModulator

MAX-ACCESS	not-accessible

STATUS	current

DESCRIPTION

“Defined by wranIfABsBasicCapabilityCrzModulator.”

::= { wranIfABsCpeBasicCapabilityRspEntry 2 }

wranIfABsCpeBasicCapabilityRspCrzDemodulator	OBJECT-TYPE

SYNTAX	wranIfABsBasicCapabilityCrzDemodulator

MAX-ACCESS	not-accessible

STATUS	current

DESCRIPTION

“Defined by wranIfABsBasicCapabilityCrzDemodulator.”

::= { wranIfABsCpeBasicCapabilityRspEntry 3 }

wranIfABsCpeBasicCapabilityRspRelayCpeMode	OBJECT-TYPE

SYNTAX	wranIfABsBasicCapabilityRelayCpeMode

MAX-ACCESS	not-accessible

STATUS	current

DESCRIPTION

“Defined by wranIfABsBasicCapabilityRelayCpeMode.”

::= { wranIfABsCpeBasicCapabilityRspEntry 4 }

wranIfABsCpeBasicCapabilityRspMutlichannelSupport	OBJECT-TYPE

SYNTAX	wranIfABsBasicCapabilityMultichannelSupport

MAX-ACCESS	not-accessible

STATUS	current

DESCRIPTION

“Defined by wranIfABsBasicCapabilityMultichannelSupport.”

::= { wranIfABsCpeBasicCapabilityRspEntry 5 }

wranIfABsCpeBasicCapabilityRspMacAddress	OBJECT-TYPE

SYNTAX	wranIfABsBasicCapabilityMacAddress

MAX-ACCESS	not-accessible

STATUS	current

DESCRIPTION

“Defined by wranIfABsBasicCapabilityMacAddress.”

::= { wranIfABsCpeBasicCapabilityRspEntry 6 }

wranIfABsCpeBasicCapabilityDefTable		OBJECT-TYPE

SYNTAX	SEQUENCE of wranIfABsCpeBasicCapabilityDefEntry

MAX-ACCESS	not-accessible

STATUS	current

DESCRIPTION

“This table contains objects that default values of CBC-REQ/RSP

IEs for A-CPEs. This table can be used to judge/verify a CPE’s

CBC-REQ and used to construct the CBC-RSP message. This table

has one entry defined by wranIfABsCpeBasicCapabilityDefEntry.”

::= { wranIfABsCm 4 }

wranIfABsCpeBasicCapabilityDefEntry		OBJECT-TYPE

SYNTAX	wranIfABsCpeBasicCapabilityDefEntry

MAX-ACCESS	not-accessible

STATUS	current

DESCRIPTION

“This object is a compound object that defines an entry in

wranIfABsCpeBasicCapabilityDefTable.”

INDEX { wranIfABsCpeBasicCapabilityDefIndex }

::= { wranIfABsCpeBasicCapabilityDefTable 1 }

wranIfABsCpeBasicCapabilityDefEntry		::= SEQUENCE {

wranIfABsCpeBasicCapabilityDefIndex

wranIfABsBasicCapabilityIndex,

wranIfABsCpeBasicCapabilityDefCrzModulator

wranIfABsBasicCapabilityCrzModulator,

wranIfABsCpeBasicCapabilityDefCrzDemodulator

wranIfABsBasicCapabilityCrzDemodulator,

wranIfABsCpeBasicCapabilityDefRelayCpeMode

wranIfABsBasicCapabilityRelayCpeMode,

wranIfABsCpeBasicCapabilityDefMutlichannelSupport

wranIfABsBasicCapabilityMultichannelSupport,

wranIfABsCpeBasicCapabilityDefMacAddress

wranIfABsBasicCapabilityMacAddress }

wranIfABsBasicCapabilityDefIndex	OBJECT-TYPE

SYNTAX	wranIfABsBasicCapabilityIndex

MAX-ACCESS	not-accessible

STATUS	current

DESCRIPTION

“Defined by wranIfABsBasicCapablityIndex.”

::= { wranIfABsCpeBasicCapabilityDefEntry 1 }

wranIfABsCpeBasicCapabilityDefCrzModulator	OBJECT-TYPE

SYNTAX	wranIfABsBasicCapabilityCrzModulator

MAX-ACCESS	not-accessible

STATUS	current

DESCRIPTION

“Defined by wranIfABsBasicCapabilityCrzModulator.”

::= { wranIfABsCpeBasicCapabilityDefEntry 2 }

wranIfABsCpeBasicCapabilityDefCrzDemodulator	OBJECT-TYPE

SYNTAX	wranIfABsBasicCapabilityCrzDemodulator

MAX-ACCESS	not-accessible

STATUS	current

DESCRIPTION

“Defined by wranIfABsBasicCapabilityCrzDemodulator.”

::= { wranIfABsCpeBasicCapabilityDefEntry 3 }

wranIfABsCpeBasicCapabilityDefRelayCpeMode	OBJECT-TYPE

SYNTAX	wranIfABsBasicCapabilityRelayCpeMode

MAX-ACCESS	not-accessible

STATUS	current

DESCRIPTION

“Defined by wranIfABsBasicCapabilityRelayCpeMode.”

::= { wranIfABsCpeBasicCapabilityDefEntry 4 }

wranIfABsCpeBasicCapabilityDefMutlichannelSupport	OBJECT-TYPE

SYNTAX	wranIfABsBasicCapabilityMultichannelSupport

MAX-ACCESS	not-accessible

STATUS	current

DESCRIPTION

“Defined by wranIfABsBasicCapabilityMultichannelSupport.”

::= { wranIfABsCpeBasicCapabilityDefEntry 5 }

wranIfABsCpeBasicCapabilityDefMacAddress	OBJECT-TYPE

SYNTAX	wranIfABsBasicCapabilityMacAddress

MAX-ACCESS	not-accessible

STATUS	current

DESCRIPTION

“Defined by wranIfABsBasicCapabilityMacAddress.”

::= { wranIfABsCpeBasicCapabilityDefEntry 6 }

-- wranIfABsCpeBasicCapabilityCmn: This MIB object is a group containing objects

-- that are common to wranIfABsCpeBasicCapabilityReqEntry,

-- wranIfABsCpeBasicCapabilityRspEntry, wranIfABsCpeBasicCapabilityDefEntry.

wranIfABsCpeBasicCapabilityCmn	OBJECT IDENTIFIER

::= { wranIfABsCm 4 }

wranIfABsBasicCapabilityIndex		OBJECT IDENTIFIER

::= { wranIfABsCpeBasicCapabilityCmn 1 }

wranIfABsBasicCapabilityCrzModulator		OBJECT IDENTIFIER

::= { wranIfABsCpeBasicCapabilityCmn 2 }

wranIfABsBasicCapabilityCrzDemodulator		OBJECT IDENTIFIER

::= { wranIfABsCpeBasicCapabilityCmn 3 }

wranIfABsBasicCapabilityRelayCpeMode		OBJECT IDENTIFIER

::= { wranIfABsCpeBasicCapabilityCmn 4 }

wranIfABsBasicCapabilityMultichannelSupport	OBJECT IDENTIFIER

::= { wranIfABsCpeBasicCapabilityCmn 5 }

wranIfABsBasicCapabilityMacAddress			OBJECT IDENTIFIER

::= { wranIfABsCpeBasicCapabilityCmn 6 }

wranIfABsBasicCapabilityIndex		::= TEXTUAL-CONVENTION

STATUS	current

DESCRIPTION

“Index of entry into table.”

SYNTAX	Integer32 (1..511)

wranIfABsBasicCapabilityCrzModulator	::= TEXTUAL-CONVENTION

STATUS	current

DESCRIPTION

“A bit map that encodes UIUCs that the CPE supports. This

reflects the options defined in section 7.7.4.1.1.”

SYNTAX	BITS { uiuc0(0), uiuc1(1), uiuc2(2), uiuc3(3), uiuc4(4),

uiuc5(5), uiuc6(6), uiuc7(7), uiuc8(8), uiuc9(9),

uiuc10(10), uiuc11(11), uiuc12(12), uiuc13(13),

uiuc14(14), uiuc15(15), uiuc16(16), uiuc17(17),

uiuc18(18), uiuc19(19), uiuc20(20), uiuc21(21),

uiuc22(22), uiuc23(23), uiuc24(24), uiuc25(25),

uiuc26(26), uiuc27(27), uiuc28(28), uiuc29(29),

uiuc30(30), uiuc31(31), uiuc32(32), uiuc33(33),

uiuc34(34), uiuc35(35), uiuc36(36), uiuc37(37),

uiuc38(38), uiuc39(39), uiuc40(40), uiuc41(41),

uiuc42(42), uiuc43(43), uiuc44(44), uiuc45(45),

uiuc46(46), uiuc47(47), uiuc48(48), uiuc49(49),

uiuc50(50), uiuc51(51), uiuc52(52), uiuc53(53),

uiuc54(54), uiuc55(55), uiuc56(56), uiuc57(57),

uiuc58(58), uiuc59(59), uiuc60(60), uiuc61(61),

uiuc62(62), uiuc63(63) }

wranIfABsBasicCapabilityCrzDemodulator	::= TEXTUAL-CONVENTION

STATUS	current

DESCRIPTION

“A bit map that encodes DIUCs that the CPE supports. This

reflects the options defined in section 7.7.2.1.1.”

SYNTAX	BITS { diuc0(0), diuc1(1), diuc2(2), diuc3(3), diuc4(4),

diuc5(5), diuc6(6), diuc7(7), diuc8(8), diuc9(9),

diuc10(10), diuc11(11), diuc12(12), diuc13(13),

diuc14(14), diuc15(15), diuc16(16), diuc17(17),

diuc18(18), diuc19(19), diuc20(20), diuc21(21),

diuc22(22), diuc23(23), diuc24(24), diuc25(25),

diuc26(26), diuc27(27), diuc28(28), diuc29(29),

diuc30(30), diuc31(31), diuc32(32), diuc33(33),

diuc34(34), diuc35(35), diuc36(36), diuc37(37),

diuc38(38), diuc39(39), diuc40(40), diuc41(41),

diuc42(42), diuc43(43), diuc44(44), diuc45(45),

diuc46(46), diuc47(47), diuc48(48), diuc49(49),

diuc50(50), diuc51(51), diuc52(52), diuc53(53),

diuc54(54), diuc55(55), diuc56(56), diuc57(57),

diuc58(58), diuc59(59), diuc60(60), diuc61(61),

diuc62(62), diuc63(63) }

wranIfABsBasicCapabilityRelayCpeMode		::= TEXTUAL-CONVENTION

STATUS	current

DESCRIPTION

“Relay CPE operational mode.”

SYNTAX	INTEGER ( noRelay(0), crzRelay(1), drzRelay(2) )

wranIfABsBasicCapabilityMultiChannelSupport	::= TEXTUAL-CONVENTION

STATUS	current

DESCRIPTION

“Multi-channel operational mode.”

SYNTAX	INTEGER ( noMultiChannelDisabled(0),

multiChannelEnabled(1) )

wranIfABsBasicCapabilityMacAddress			::= TEXTUAL=CONVENTION

STATUS	current

DESCRIPTION

“The MAC Address of CPE attempting basic capability

configuration.”

SYNTAX	MacAddress

wranIfABsGraConfigTable		OBJECT-TYPE

SYNTAX	SEQUENCE of wranIfABsGraConfigEntry

MAX-ACCESS	not-accessible

STATUS	current

DESCRIPTION

“This MIB object defines a table containing information on all

groups created for supporting group resource allocation (GRA)

operation.”

::= { wranIfABsCm 5 }

wranIfABsGraConfigEntry		OBJECT-TYPE

SYNTAX	wranIfABsGraConfigEntry

MAX-ACCESS	not-accessible

STATUS	current

DESCRIPTION

“Compound object that defines an entry into

wranIfABsGraConfigTable.”

INDEX { wranIfABsGraConfigIndex }

::= { wranIfABsGraConfigTable 1 }

wranIfABsGraConfigEntry					::= SEQUENCE {

wranIfABsGraConfigIndex					Integer32,

wranIfABsGraCpeMacAddr					MacAddress,

wranIfABsGraGroupSid					Integer32,

wranIfABsGraGroupId					Integer32,

wranIfABsGraGroupType					INTEGER,

wranIfABsGraGroupLat					REAL,

wranIfABsGraGroupLon					REAL,

wranIfABsGraMaxGroupSize				Integer32 }

wranIfABsGraConfigIndex	OBJECT-TYPE

SYNTAX	Integer32 (1..16384)

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

“Index of entry in table.”

::= { wranIfABsGraConfigEntry 1 }

wranIfABsGraCpeMacAddr	OBJECT-TYPE

SYNTAX	MacAddress

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

“MAC address of CPE whose GRA configuration is stored.”

::= { wranIfABsGraConfigEntry 2 }

wranIfABsGraGroupSid	OBJECT-TYPE

SYNTAX	Integer32{ 0..511 }

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

“Current Station ID of CPE.”

::= { wranIfABsGraConfigEntry 3 }

wranIfABsGraGroupId	OBJECT-TYPE

SYNTAX	INTEGER { fixedGroup(0), portableMobileGroup(1) }

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

“GRA Group type.”

::= { wranIfABsGraConfigEntry 4 }

wranIfABsGraGroupType	OBJECT-TYPE

SYNTAX	Integer32 { 1..1024 }

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

“GRA Group type.”

::= { wranIfABsGraConfigEntry 5 }

wranIfABsGraGroupLat	OBJECT-TYPE

SYNTAX	REAL { mantissa, base, exponent }

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

“Latitude of Group location defined as mantissa*base^exponent.

Base shall be set to 10. Latitude=31.195848, mantissa=31195848,

base=10, exponent=-6.”

::= { wranIfABsGraConfigEntry 6 }

wranIfABsGraGroupLon	OBJECT-TYPE

SYNTAX	REAL { mantissa, base, exponent }

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

“Longitude of Group location defined as mantissa*base^exponent.

Base shall be set to 10. Longitude=-73.195848, mantissa=-

73195848, base=10, exponent=-6.”

::= { wranIfABsGraConfigEntry 7 }

wranIfABsGraMaxGroupSize	OBJECT-TYPE

SYNTAX	Integer32 { 1..128 }

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

“Current maximum size of GRA group CPE assigned to.”

::= { wranIfABsGraConfigEntry 8 }

wranIfABsMultiChnConfigTable		OBJECT-TYPE

SYNTAX	SEQUENCE of wranIfABsMultiChnConfigEntry

MAX-ACCESS	not-accessible

STATUS	current

DESCRIPTION

“This MIB object defines a table containing information for all

CAM/CHUs configured for CPEs.”

::= { wranIfABsCm 6 }

wranIfABsMultiChnConfigEntry		OBJECT-TYPE

SYNTAX	wranIfABsMultiChnConfigEntry

MAX-ACCESS	not-accessible

STATUS	current

DESCRIPTION

“Compound object that defines an entry into

wranIfABsGraConfigTable.”

INDEX { wranIfABsGraConfigIndex }

::= { wranIfABsMultiChnConfigTable 1 }

wranIfABsMultiChnConfigEntry			::= SEQUENCE {

wranIfABsMutliChnConfigIndex			Integer32,

wranIfABsMultiChnMacAddress			MacAddress,

wranIfABsMultiChnMaxAggChns			Integer32,

wranIfABsMutliChnCurNumAggChns		Integer32,

wranIfABsMultiChnAggChnList			OCTET STRING,

wranIfABsMultiChnAggChnListSize		Integer32 }

wranIfABsMutliChnConfigIndex	OBJECT-TYPE

SYNTAX	Integer32 (1..16384)

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

“Index of entry in table.”

::= { wranIfABsMultiChnConfigEntry 1 }

wranIfABsMultiChnMacAddress	OBJECT-TYPE

SYNTAX	MacAddress

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

“MAC address of CPE whose GRA configuration is stored.”

::= { wranIfABsMultiChnConfigEntry 2 }

wranIfABsMultiChnMaxAggChns	OBJECT-TYPE

SYNTAX	Integer32{ 1..64 }

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

“Maximum number of aggregation channels used by CPE.”

::= { wranIfABsMultiChnConfigEntry 3 }

wranIfABsMutliChnCurNumAggChns	OBJECT-TYPE

SYNTAX	Integer32{ 0..64 }

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

“Current number of aggregation channels used by CPE.”

::= { wranIfABsMultiChnConfigEntry 4 }

wranIfABsMultiChnAggChnList	OBJECT-TYPE

SYNTAX	OCTET STRING (SIZE(wranIfABsMutliChnCurNumAggChns))

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

“Octet string containing list of channels assigned to CPE for use

as aggregation channels.”

::= { wranIfABsMultiChnConfigEntry 5 }

wranIfABsMultiChnAggChnListSize	OBJECT-TYPE

SYNTAX	OCTET STRING sizeof(wranIfABsMultiChnAggChnListSize)

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

“Octet string containing list of channels assigned to CPE for use

as aggregation channels.”

::= { wranIfABsMultiChnConfigEntry 6 }

wranIfABsRelayConfigTable		OBJECT-TYPE

SYNTAX	SEQUENCE of wranIfABsRelayConfigEntry

MAX-ACCESS	not-accessible

STATUS	current

DESCRIPTION

“This MIB object defines a table containing DS/US zone

configuration for Repaly CPEs.”

::= { wranIfABsCm 7 }

wranIfABsRelayConfigEntry		OBJECT-TYPE

SYNTAX	wranIfABsRelayConfigEntry

MAX-ACCESS	not-accessible

STATUS	current

DESCRIPTION

“Compound object that defines an entry into

wranIfABsRelayConfigTable.”

INDEX { wranIfABsRelayConfigIndex }

::= { wranIfABsRelayConfigTable 1 }

wranIfABsRelayConfigEntry			::= SEQUENCE {

wranIfABsRelayConfigIndex			Integer32,

wranIfABsRelayConfigMacAddres			MacAddress,

wranIfABsRelayZoneIndex				INTEGER,

wranIfABsRelayZoneMode				INTEGER,

wranIfABsRelayZoneSegStartPos			INTEGER,

wranIfABsRelayZoneSegBandwidth		INTEGER,

wranIfABsRelayZonePhyMode			INTEGER,

wranIfABsRelayZoneOfdmaSymOffset		INTEGER,

wranIfABsRelayCpeSid				Integer32,

wranIfABsRelayZoneDir				INTEGER }

wranIfABsRelayConfigIndex	OBJECT-TYPE

SYNTAX	Integer32 (1..16384)

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

“Index of entry in table.”

::= { wranIfABsRelayConfigEntry 1 }

wranIfABsRelayConfigMacAddress	OBJECT-TYPE

SYNTAX	MacAddress

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

“MAC address of relay CPE.”

::= { wranIfABsRelayConfigEntry 2 }

wranIfABsRelayZoneIndex		OBJECT-TYPE

SYNTAX	INTEGER { 0..255 }

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

“Index of relay zone.”

::= { wranIfABsRelayConfigEntry 3 }

wranIfABsRelayZoneMode		OBJECT-TYPE

SYNTAX	INTEGER { accessZone(0), crz(1), drz(2) }

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

“Type of relay zone: access, centralized relay, distributed

relay.”

::= { wranIfABsRelayConfigEntry 4 }

wranIfABsRelayZoneSegStartPos	OBJECT-TYPE

SYNTAX	INTEGER { segment0(0), segment1(1), segment2(2) }

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

“Start position of segement, if zone mode is distributed relay,

defaults to 0.”

::= { wranIfABsRelayConfigEntry 5 }

wranIfABsRelayZoneSegBandwidth	OBJECT-TYPE

SYNTAX	INTEGER { oneSegmentFromStart(0), twoSegmentsFromStart(1),

threeSegmentsFromStart(2) }

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

“Bandwidth of segment, if zone mode is distributed relay.”

::= { wranIfABsRelayConfigEntry 6 }

wranIfABsRelayZonePhyMode	OBJECT-TYPE

SYNTAX	INTEGER { phyOM1(0), phyOM2(1) }

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

“PHY mode, i.e. OM1 or OM2, of relay zone.”

::= { wranIfABsRelayConfigEntry 7 }

wranIfABsRelayZoneOfdmaSymOffset	OBJECT-TYPE

SYNTAX	INTEGER { 0..127 }

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

“Zone starts at this OFDMA symbol offset, counted after the

preamble of the frame.”

::= { wranIfABsRelayConfigEntry 8 }

wranIfABsRelayCpeSid	OBJECT-TYPE

SYNTAX	Integer32 { 0..511 }

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

“Current Station ID of distributed relay CPE.”

::= { wranIfABsRelayConfigEntry 9 }

wranIfABsRelayZoneDir	OBJECT-TYPE

SYNTAX	INTEGER { downstream(0), upstream(1) }

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

“Indication if this is an US or DS relay zone.”

::= { wranIfABsRelayConfigEntry 10 }

-- wranIfABsPhy: This MIB object contains managed objects related to PHY

-- configuration. All objects described are related to operation of the

-- OFDMA (in PHY-OM1 and PHY-OM2) by the A-BS and A-CPEs.

wranIfABsPhy				OBJECT IDENTIFIER ::= { wranIfABsCm 8 }

wranIfABsOfdmaPhyUsChannelTable	OBJECT IDENTIFIER ::= { wranIfABsPhy 1 }

wranIfABsOfdmaPhyDsChannelTable	OBJECT IDENTIFIER ::= { wranIfABsPhy 2 }

wranIfABsOfdmaPhyUsRegionTable	OBJECT IDENTIFIER ::= { wranIfABsPhy 3 }

wranIfABsOfdmaPhyDsRegionTable	OBJECT IDENTIFIER ::= { wranIfABsPhy 4 }

wranIfABsOfdmaPhyUsChannelTable	OBJECT-TYPE

SYNTAX	SEQUENCE OF wranIfABsOfdmaPhyUsChannelEntry

MAX-ACCESS	not-accessible

STATUS	current

DESCRIPTION

“Table that defines attributes of US channels. Made of multiple

Entries defined by wranIfABsOfdmaPhyUsChannelEntry.”

::= { wranIfABsPhy 1 }

wranIfABsOfdmaPhyUsChannelEntry	OBJECT-TYPE

SYNTAX	wranIfABsOfdmaPhyUsChannelEntry

MAX-ACCESS	not-accesible

STATUS	current

DESCRIPTION

“Compound object that defines entry in

wranIfABsOfdmaPhyUsChannelTable.”

INDEX	{ wranIfABsOfdmaPhyUsChannelIndex }

::= { wranIfABsOfdmaPhyUsChannelTable 1 }

wranIfABsOfdmaPhyUsChannelEntry	::= SEQUENCE {

wranIfABSOfdmaPhyUsChannelIndex		Integer32,

wranIfABsOfdmaPhyUsSid				Integer32,

wranIfABsOfdmaPhyUsMacAddress			MacAddress,

wranIfABsOfdmaPhyUsOM				INTEGER,

wranIfABsOfdmaPhyUsSegmentConfig		INTEGER,

wranIfABsOfdmaPhyUsCenterFrequency		Integer32 }

wranIfABsOfdmaPhyUsChannelIndex	OBJECT-TYPE

SYNTAX	Integer32 (1..131071)

MAX-ACCESS	not-accessible

STATUS	current

DESCRIPTION

“index of entry in this table, defaults to 1.”

::= { wranIfABsOfdmaPhyUsChannelEntry 1 }

wranIfABsOfdmaPhyUsSid			OBJECT-TYPE

SYNTAX	Integer32 (0..511)

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

“A 9-bit SID of CPE whose US is defined in this entry.”

::= { wranIfABsOfdmaPhyUsChannelEntry 2 }

wranIfABsOfdmaUsMacAddress	OBJECT-TYPE

SYNTAX	MacAddress

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

“MAC Address of CPE whose upstream is defined in this table.”

::= { wranIfABsOfdmaPhyUsChannelEntry 3 }

wranIfABsOfdmaPhyUsOM		OBJECT-TYPE

SYNTAX	INTEGER { om1(0), om2(1) }

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

“Phy operational mode of A-CPE US, either OM1 (see Clause 9) or

OM2 (see Clause 10).”

::= { wranIfABsOfdmaPhyUsChannelEntry 4 }

wranIfABsOfdmaPhyUsSegmentConfig	OBJECT-TYPE

SYNTAX	INTEGER { crz_az(0) seg0(1), seg1(2), seg2(3) }

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

“Distributed relay zone US segment that CPE is operating in.

Defaults to ‘0’ indicated CPE operating through CRZ or directly

attached to A-BS.”

::= { wranIfABsOfdmaPhyUsChannelEntry 5 }

wranIfABsOfdmaPhyUsCenterFrequency	OBJECT-TYPE

SYNTAX	Integer32 (0..6000000)

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

“Upstream center frequency in kHz.”

::= { wranIfABsOfdmaPhyUsChannelEntry 6 }

wranIfABsOfdmaPhyDsChannelTable	OBJECT-TYPE

SYNTAX	SEQUENCE OF wranIfABsOfdmaPhyDsChannelEntry

MAX-ACCESS	not-accessible

STATUS	current

DESCRIPTION

“Table that defines attributes of DS channels. Made of multiple

Entries defined by wranIfABsOfdmaPhyDsChannelEntry.”

::= { wranIfABsPhy 2 }

wranIfABsOfdmaPhyDsChannelEntry	OBJECT-TYPE

SYNTAX	wranIfABsOfdmaPhyDsChannelEntry

MAX-ACCESS	not-accesible

STATUS	current

DESCRIPTION

“Compound object that defines entry in

wranIfABsOfdmaPhyDsChannelTable.”

INDEX	{ wranIfABsOfdmaPhyDsChannelIndex }

::= { wranIfABsOfdmaPhyDsChannelTable 1 }

wranIfABsOfdmaPhyDsChannelEntry	::= SEQUENCE {

wranIfABSOfdmaPhyDsChannelIndex		Integer32,

wranIfABsOfdmaPhyDsSid				Integer32,

wranIfABsOfdmaPhyDsMacAddress			MacAddress,

wranIfABsOfdmaPhyDsOM				INTEGER,

wranIfABsOfdmaPhyDsSegmentConfig		INTEGER,

wranIfABsOfdmaPhyDsCenterFrequency		Integer32,

wranIfABsOfdmaPhyDsCellType			INTEGER,

wranIfABsOfdmaPhyDsMacVersion			INTEGER }

wranIfABsOfdmaPhyDsChannelIndex	OBJECT-TYPE

SYNTAX	Integer32 (1..131071)

MAX-ACCESS	not-accessible

STATUS	current

DESCRIPTION

“index of entry in this table, defaults to 1.”

::= { wranIfABsOfdmaPhyDsChannelEntry 1 }

wranIfABsOfdmaPhyDsSid			OBJECT-TYPE

SYNTAX	Integer32 (0..511)

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

“A 9-bit SID of CPE whose US is defined in this entry.”

::= { wranIfABsOfdmaPhyDsChannelEntry 2 }

wranIfABsOfdmaDsMacAddress	OBJECT-TYPE

SYNTAX	MacAddress

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

“MAC Address of CPE whose downtream is defined in this table.”

::= { wranIfABsOfdmaPhyDsChannelEntry 3 }

wranIfABsOfdmaPhyDsOM		OBJECT-TYPE

SYNTAX	INTEGER { om1(0), om2(1) }

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

“Phy operational mode of A-CPE DS, either OM1 (see Clause 9) or

OM2 (see Clause 10).”

::= { wranIfABsOfdmaPhyDsChannelEntry 4 }

wranIfABsOfdmaPhyDsSegmentConfig	OBJECT-TYPE

SYNTAX	INTEGER { crz_az(0) seg0(1), seg1(2), seg2(3) }

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

“Distributed relay zone DS segment that CPE is operating in.

Defaults to ‘0’ indicated CPE operating through CRZ or directly

attached to A-BS.”

::= { wranIfABsOfdmaPhyDsChannelEntry 5 }

wranIfABsOfdmaPhyDsCenterFrequency	OBJECT-TYPE

SYNTAX	Integer32 (0..6000000)

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

“Downstream center frequency in kHz.”

::= { wranIfABsOfdmaPhyDsChannelEntry 6 }

wranIfABsOfdmaPhyDsCellType	OBJECT-TYPE

SYNTAX	INTEGER ( fixed(0), portable(1), crzFixedOnly(1),

drzFixed(2), drzPortable(3),

fixedMultiChannelBulk(4),

fixedMultiChannelTxDiv(5),

portableMultiChannelBulk(6),

portableMultiChannelTxDiv(7) )

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

“Class of BS/cell that CPE is operating on. See ‘CPE Operational

Capability’ IE in 7.9.7.3.4.13.”

::= { wranIfABsOfdmaPhyDsChannelEntry 7 }

wranIfABsOfdmaPhyDsMacVersion		OBJECT-TYPE

SYNTAX	INTEGER	(1..2)

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

“Version of MAC run in the DS/US.”

::= { wranIfABsOfdmaPhyDsChannelEntry 8 }

wranIfABsOfdmaPhyUsRegionTable	OBJECT-TYPE

SYNTAX	SEQUENCE OF wranIfABsOfdmaPhyUsRegionEntry

MAX-ACCESS	not-accessible

STATUS	current

DESCRIPTION

“Table that defines configuration of US subframe. Made of

multiple entries defined by wranIfABsOfdmaPhyUsRegionEntry.”

::= { wranIfABsPhy 3 }

wranIfABsOfdmaPhyUsRegionEntry	OBJECT-TYPE

SYNTAX	wranIfABsOfdmaPhyUsRegionEntry

MAX-ACCESS	not-accesible

STATUS	current

DESCRIPTION

“Compound object that defines entry in

wranIfABsOfdmaPhyUsChannelTable.”

INDEX	{ wranIfABsOfdmaPhyUsRegionIndex }

::= { wranIfABsOfdmaPhyUsRegionTable 1 }

wranIfABsOfdmaPhyUsChannelEntry	::= SEQUENCE {

wranIfABsOfdmaPhyUsRegionIndex		Integer32,

wranIfABsOfdmaphyUsRegionSegmentNum		INTEGER,

wranIfABsOfdmaPhyUsRegionDuration		Integer32 }

wranIfABsOfdmaPhyUsRegionIndex	OBJECT-TYPE

SYNTAX	Integer32 (1..131071)

MAX-ACCESS	not-accessible

STATUS	current

DESCRIPTION

“index of entry in this table, defaults to 1.”

::= { wranIfABsOfdmaPhyUsRegionEntry 1 }

wranIfABsOfdmaphyUsRegionSegmentNum			OBJECT-TYPE

SYNTAX	INTEGER { crz_az(0) seg0(1), seg1(2), seg2(3) }

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

“Segment of US region. Defaults to ‘0’ indicated CPE operating

through CRZ or directly attached to A-BS.”

::= { wranIfABsOfdmaPhyUsRegionEntry 2 }

wranIfABsOfdmaPhyUsRegionDuration	OBJECT-TYPE

SYNTAX	Integer32 (0..65535)

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

“Number of OFDMA slots allocated to region.”

::= { wranIfABsOfdmaPhyUsRegionEntry 3 }

wranIfABsOfdmaPhyDsRegionTable	OBJECT-TYPE

SYNTAX	SEQUENCE OF wranIfABsOfdmaPhyDsRegionEntry

MAX-ACCESS	not-accessible

STATUS	current

DESCRIPTION

“Table that defines configuration of DS subframe. Made of

multiple entries defined by wranIfABsOfdmaPhyDsRegionEntry.”

::= { wranIfABsPhy 4 }

wranIfABsOfdmaPhyDsRegionEntry	OBJECT-TYPE

SYNTAX	wranIfABsOfdmaPhyDsRegionEntry

MAX-ACCESS	not-accesible

STATUS	current

DESCRIPTION

“Compound object that defines entry in

wranIfABsOfdmaPhyDsChannelTable.”

INDEX	{ wranIfABsOfdmaPhyDsRegionIndex }

::= { wranIfABsOfdmaPhyDsRegionTable 1 }

wranIfABsOfdmaPhyDsChannelEntry	::= SEQUENCE {

wranIfABsOfdmaPhyDsRegionIndex		Integer32,

wranIfABsOfdmaphyDsRegionSegmentNum		INTEGER,

wranIfABsOfdmaPhyDsRegionDuration		Integer32 }

wranIfABsOfdmaPhyDsRegionIndex	OBJECT-TYPE

SYNTAX	Integer32 (1..131071)

MAX-ACCESS	not-accessible

STATUS	current

DESCRIPTION

“index of entry in this table, defaults to 1.”

::= { wranIfABsOfdmaPhyDsRegionEntry 1 }

wranIfABsOfdmaphyDsRegionSegmentNum			OBJECT-TYPE

SYNTAX	INTEGER { crz_az(0) seg0(1), seg1(2), seg2(3) }

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

“Segment of DS region. Defaults to ‘0’ indicated CPE operating

through CRZ or directly attached to A-BS.”

::= { wranIfABsOfdmaPhyDsRegionEntry 2 }

wranIfABsOfdmaPhyDsRegionDuration	OBJECT-TYPE

SYNTAX	Integer32 (0..65535)

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

“Number of OFDMA slots allocated to region.”

::= { wranIfABsOfdmaPhyDsRegionEntry 3 }

-- wranIfABsCmMibGroups: This object helps define which MIB groups are available in

-- this module (wranIfABsCm) and which MIG objects are part of each group.

wranIfABsCmMibGroups			OBJECT IDENTIFIER

::= { wranIfABsCm 9 }

wranIfABsCmBasicCapabilityGroup	OBJECT IDENTIFIER

::= { wranIfABsCmMibGroups 1 }

wranIfABsCmGraCapabilityGroup		OBJECT IDENTIFIER

::= { wranIfABsCmMibGroups 2 }

wranIfABsCmMutliChannelGroup		OBJECT IDENTIFIER

::= { wranIfABsCmMibGroups 3 }

wranIfABsCmRelayGroup			OBJECT IDENTIFIER

::= { wranIfABsCmMibGroups 4 }

wranIfABsCmPhyGroup			OBJECT IDENTIFIER

::= { wranIfABsCmMibGroups 5 }

wranIfABsCmBasicCapabilityGroup		OBJECT-GROUP

OBJECTS	{ wranIfABsCpeBasicCapabilityReqIndex,

wranIfABsCpeBasicCapabilityReqCrzModulator,

wranIfABsCpeBasicCapabilityReqCrzDemodulator,

wranIfABsCpeBasicCapabilityReqRelayCpeMode,

wranIfABsCpeBasicCapabilityReqMutlichannelSupport,

wranIfABsCpeBasicCapabilityReqMacAddress,

wranIfABsCpeBasicCapabilityRspIndex,

wranIfABsCpeBasicCapabilityRspCrzModulator,

wranIfABsCpeBasicCapabilityRspCrzDemodulator,

wranIfABsCpeBasicCapabilityRspRelayCpeMode,

wranIfABsCpeBasicCapabilityRspMutlichannelSupport,

wranIfABsCpeBasicCapabilityRspMacAddress,

..wranIfABsBasicCapabilityMacAddress,

wranIfABsCpeBasicCapabilityDefCrzModulator,

wranIfABsCpeBasicCapabilityDefCrzDemodulator,

wranIfABsCpeBasicCapabilityDefRelayCpeMode,

wranIfABsCpeBasicCapabilityDefMutlichannelSupport,

wranIfABsCpeBasicCapabilityDefMacAddress }

STATUS	current

DESCRIPTION

“This group contains objects related to CBC-REQ/RSP process for

A-CPEs.”

::= { wranIfABsCmMibGroups 1 }

wranIfABsCmGraCapabilityGroup		OBJECT-GROUP

OBJECTS	{ wranIfABsGraConfigIndex,

wranIfABsGraCpeMacAddr,

wranIfABsGraGroupSid,

wranIfABsGraGroupId,

wranIfABsGraGroupType,

wranIfABsGraGroupLat,

wranIfABsGraGroupLon,

wranIfABsGraMaxGroupSize }

STATUS	current

DESCRIPTION

“This group contains objects related to group resource allocation

configuration.”

::= { wranIfABsCmMibGroups 2 }

wranIfABsCmMutliChannelGroup		OBJECT-GROUP

OBJECTS	{ wranIfABsMutliChnConfigIndex,

wranIfABsMultiChnMacAddress,

wranIfABsMultiChnMaxAggChns,

wranIfABsMutliChnCurNumAggChns,

wranIfABsMultiChnAggChnList,

wranIfABsMultiChnAggChnListSize }

STATUS	current

DESCRIPTION

“This group contains objects related to multi-channel operation.”

::= { wranIfABsCmMibGroups 3 }

wranIfABsCmRelayGroup		OBJECT-GROUP

OBJECTS	{ wranIfABsRelayConfigIndex,

wranIfABsRelayConfigMacAddres,

wranIfABsRelayZoneIndex,

wranIfABsRelayZoneMode,

wranIfABsRelayZoneSegStartPos,

wranIfABsRelayZoneSegBandwidth,

wranIfABsRelayZonePhyMode,

wranIfABsRelayZoneOfdmaSymOffset,

wranIfABsRelayCpeSid,

wranIfABsRelayZoneDir }

STATUS	current

DESCRIPTION

“This group contains objects related to relay CPE operation.”

::= { wranIfABsCmMibGroups 4 }

wranIfABsCmPhyGroup		OBJECT-GROUP

OBJECTS	{ wranIfABSOfdmaPhyUsChannelIndex,

wranIfABsOfdmaPhyUsSid,

wranIfABsOfdmaPhyUsMacAddress,

wranIfABsOfdmaPhyUsOM,

wranIfABsOfdmaPhyUsSegmentConfig,

wranIfABsOfdmaPhyUsCenterFrequency,

wranIfABSOfdmaPhyDsChannelIndex,

wranIfABsOfdmaPhyDsSid,

wranIfABsOfdmaPhyDsMacAddress,

--				  wranIfABsOfdmaPhyDsOM,

wranIfABsOfdmaPhyDsSegmentConfig,

wranIfABsOfdmaPhyDsCenterFrequency,

wranIfABsOfdmaPhyDsCellType,

wranIfABsOfdmaPhyDsMacVersion,

wranIfABsOfdmaPhyUsRegionIndex,

wranIfABsOfdmaphyUsRegionSegmentNum,

wranIfABsOfdmaPhyUsRegionDuration,

wranIfABsOfdmaPhyDsRegionIndex,

wranIfABsOfdmaphyDsRegionSegmentNum,

wranIfABsOfdmaPhyDsRegionDuration }

STATUS	current

DESCRIPTION

“This group contains objects related to Phy configuration.”

::= { wranIfABsCmMibGroups 5 }

wranIfABsCmMibCompliance	MODULE-COMPLIANCE

STATUS	current

DESCRIPTION

“MIB objects that are optional and mandatory for wranIfABsCm

Compliance.”

MODULE	wranIfABsCm

MANDATORY-GROUPS	{ wranIfABsCmBasicCapabilityGroup,

wranIfABsCmPhyGrouup }

-- OPTIONAL-GROUPS { wranIfABsCmGraCapabilityGroup,

wranIfABsCmMutliChannelGroup,

wranIfABsCmRelayGroup }

::= { wranIfABsCm 10 }

---end of text modification---

14) add text “wranIfABsPm” after “wranIfABsCm”

---start of text modification---

-- wranIfABsPm: This MIB contains various objects related to Performance Management

-- at the A-BS.

wranIfABsPmConfigurationTable		OBJECT IDENTIFIER ::= { wranIfABsPm 1 }

wranIfABsArqMetricsTable		OBJECT IDENTIFIER ::= { wranIfABsPm 2 }

wranIfABsPmMibGroups			OBJECT IDENTIFIER ::= { wranIfABsPm 3 }

wranIfABsPmMibCompliance		OBJECT IDENTIFIER ::= { wranIfABsPm 4 }

wranIfABsPmConfigurationTable		OBJECT-TYPE

SYNTAX	SEQUENCE OF wranIfABsPmConfigurationEntry

MAX-ACCESS	not-accessible

STATUS	current

DESCRIPTION

“The configuration of statistics capture and measure at the A-BS

Configured in this table for each BS sector. The entries are

Defiend by wranIfABsPmConfigurationEntry.”

::= { wranIfABsPm 1 }

wranIfABsPmConfigurationEntry	OBJECT-TYPE

SYNTAX	wranIfABsPmConfigurationEntry

MAX-ACCESS	not-accessible

STATUS	current

DESCRIPTION

“This object defines a compound object that respresents an entry

In wranIfABsPmConfigurationTable.”

INDEX	wranIfABsPmConfigurationIndex

::= { wranIfABsPmConfigurationTable 1 }

wranIfABsPmConfigurationEntry		::= SEQUENCE {

wranIfABsPmConfigurationEntryIndex		INTEGER,

wranIfABsGranularityInterval			INTEGER,

wranIfABsTwoLinkArqConfig			INTEGER }

wranIfABsPmConfigurationEntryIndex		OBJECT-TYPE

SYNTAX	INTEGER (1..1)

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

“Index of entry in this table, defaults to 1.”

::= { wranIfABsPmConfigurationEntry 1 }

wranIfABsGranularityInterval		OBJECT-TYPE

SYNTAX	INTEGER	(1..255)

UNITS		“seconds”

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

“Data rate statistics captured in wranIfABsArqMetricsTable

Are measured over the time interval this object specifics.”

::= { wranIfABsPmConfigurationEntry 2 }

wranIfABsTwoLinkArqConfig	OBJECT-TYPE

SYNTAX	INTEGER { enable(0), disable(1) }

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

“Setting this field enables/disables capturing of Two Link ARQ

metrics.”

::= { wranIfABsPmConfigurationEntry 3 }

wranIfABsArqMetricsTable	OBJECT-TYPE

SYNTAX	SEQUENCE OF wranIfABsArqMetricsEntry

MAX-ACCESS	not-accessible

STATUS	current

DESCRIPTION

“This MIB object provides a table to track performance of Two

Link ARQ being used with S-CPEs attached to A-BS thorugh

distributed scheduling A-CPE. The entries are defined by

wranIfABsMetricsEntry.”

::= { wranIfABsPm 2 }

wranIfABsArqMetricsEntry	OBJECT-TYPE

SYNTAX	wranIfABsArqMetricsEntry

MAX-ACCESS	not-accessible

STATUS	current

DESCRIPTION

“This object defines a compound object that represents an entry

in wranIfABsArqMetricsTable.”

::= { wranIfABsArqMetricsTable 1 }

wranIfABsArqMetricsEntry	::= SEQUENCE {

wranIfABsArqMetricsIndex			INTEGER,

wranIfABsRelayLinkDsNumArqBlocks		Integer32,

wranIfABsAccessLinkDsNumArqBlocks		Integer32,

wranIfABsRelayLinkDsNumArqBlocksDropped	Integer32,

wranIfABsAccessLinkDsNumArqBlocksDropped	Integer32,

wranIfABsRelayLinkDsArqBlockErrorRate	INTEGER,

wranIfABsAccessLinkDsArqBlockErrorRate	INTEGER,

wranIfABsRelayLinkDsArqBlockReTx		Integer32,

wranIfABsAccessLinkDsArqBlockReTx		Integer32,

wranIfABsRelayLinkUsNumArqBlocks		Integer32,

wranIfABsAccessLinkUsNumArqBlocks		Integer32,

wranIfABsRelayLinkUsNumArqBlocksDropped	Integer32,

wranIfABsAccessLinkUsNumArqBlocksDropped	Integer32,

wranIfABsRelayLinkUsArqBlockErrorRate	INTEGER,

wranIfABsAccessLinkUsArqBlockErrorRate	INTEGER,

wranIfABsRelayLinkUsArqBlockReTx		Integer32,

wranIfABsAccessLinkUsArqBlockReTx		Integer32,

wranIfABsRelayLinkDsArqBlockEfficiency	INTEGER,

wranIfABsAccessLinkDsArqBlockEfficiency	INTEGER,

wranIfABsRelayLinkUsArqBlockEfficiency	INTEGER,

wranIfABsAccessLinkUsArqBlockEfficiency	INTEGER }

wranIfABsArqMetricsIndex	OBJECT-TYPE

SYNTAX	INTEGER (1..1)

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

“Index of entry in this table, defaults to 1.”

::= { wranIfABsMetricsEntry 1 }

wranIfABsRelayLinkDsNumArqBlocks	OBJECT-TYPE

SYNTAX	Integer32 (1..4294967295)

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

“The total # of ARQ blocks, including retransmissions, that the

A-BS has sent on the relay link (to distributed scheduling A-

CPE during the reporting period.”

::= { wranIfABsArqMetricsEntry 2 }

wranIfABsAccessLinkDsNumArqBlocks	OBJECT-TYPE

SYNTAX	Integer32 (1..4294967295)

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

“The total # of ARQ blocks, including retransmissions, that the

A-CPE has sent on the access link during the during the

reporting period.”

::= { wranIfABsArqMetricsEntry 3 }

wranIfABsRelayLinkDsNumArqBlocksDropped	OBJECT-TYPE

SYNTAX	Integer32 (1..4294967295)

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

“The total # of ARQ blocks that were dropped, that the A-BS has

sent during the period.”

::= { wranIfABsArqMetricsEntry 4 }

wranIfABsAccessLinkDsNumArqBlocksDropped	OBJECT-TYPE

SYNTAX	Integer32 (1..4294967295)

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

“The total # of ARQ blocks that were dropped, that A-CPEs has

sent on the access link during the during the reporting period.”

::= { wranIfABsArqMetricsEntry 5 }

wranIfABsRelayLinkDsNumArqBlocksErrorRate	OBJECT-TYPE

SYNTAX	INTEGER (0..100)

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

“wranIfABsRelayLinkDsNumArqBlocksErrorRate =

(wranIfABsRelayLinkDsNumArqBlocksDropped /

wranIfABsRelayLinkDsNumArqBlocks) * 10000000, in units of 1e-7.”

::= { wranIfABsArqMetricsEntry 6 }

wranIfABsAccessLinkDsNumArqBlocksErrorRate	OBJECT-TYPE

SYNTAX	INTEGER (0..100)

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

“wranIfABsAccessLinkDsNumArqBlocksErrorRate =

(wranIfABsAccessLinkDsNumArqBlocksDropped /

wranIfABsAccessLinkDsNumArqBlocks) * 10000000,

in units of 1e-7.”

::= { wranIfABsArqMetricsEntry 7 }

wranIfABsRelayLinkDsArqBlockReTx	OBJECT-TYPE

SYNTAX	Integer32 (1..4294967295)

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

“Total # of ARQ blocks that were retransmitted by the A-BS during

the reporting period.”

::= { wranIfABsArqMetricsEntry 8 }

wranIfABsAccessLinkDsArqBlockReTx	OBJECT-TYPE

SYNTAX	Integer32 (1..4294967295)

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

“Total # of ARQ blocks that were retransmitted by the A-CPE

during the reporting period.”

::= { wranIfABsArqMetricsEntry 9 }

wranIfABsRelayLinkUsNumArqBlocks	OBJECT-TYPE

SYNTAX	Integer32 (1..4294967295)

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

“The total # of ARQ blocks, including retransmissions, that the

A-BS has received on the relay link (from distributed scheduling

A-CPE) during the reporting period.”

::= { wranIfABsArqMetricsEntry 10 }

wranIfABsAccessLinkUsNumArqBlocks	OBJECT-TYPE

SYNTAX	Integer32 (1..4294967295)

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

“The total # of ARQ blocks, including retransmissions, that the

A-CPE has received on the access link during the during the

reporting period.”

::= { wranIfABsArqMetricsEntry 11 }

wranIfABsRelayLinkUsNumArqBlocksDropped	OBJECT-TYPE

SYNTAX	Integer32 (1..4294967295)

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

“The total # of ARQ blocks that were dropped, that the A-BS has

received during the period.”

::= { wranIfABsArqMetricsEntry 12 }

wranIfABsAccessLinkUsNumArqBlocksDropped	OBJECT-TYPE

SYNTAX	Integer32 (1..4294967295)

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

“The total # of ARQ blocks that were dropped, that A-CPEs has

received on the access link during the during the reporting

period.”

::= { wranIfABsArqMetricsEntry 13 }

wranIfABsRelayLinkUsNumArqBlocksErrorRate	OBJECT-TYPE

SYNTAX	INTEGER (0..100)

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

“wranIfABsRelayLinkUsNumArqBlocksErrorRate =

(wranIfABsRelayLinkUsNumArqBlocksDropped /

wranIfABsRelayLinkUsNumArqBlocks) * 10000000, in units of 1e-7.”

::= { wranIfABsArqMetricsEntry 14 }

wranIfABsAccessLinkUsNumArqBlocksErrorRate	OBJECT-TYPE

SYNTAX	INTEGER (0..100)

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

“wranIfABsAccessLinkUsNumArqBlocksErrorRate =

(wranIfABsAccessLinkUsNumArqBlocksDropped /

wranIfABsAccessLinkUsNumArqBlocks) * 10000000,

in units of 1e-7.”

::= { wranIfABsArqMetricsEntry 15 }

wranIfABsRelayLinkUsArqBlockReTx	OBJECT-TYPE

SYNTAX	Integer32 (1..4294967295)

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

“Total # of ARQ blocks that were retransmitted to the A-BS during

the reporting period.”

::= { wranIfABsArqMetricsEntry 16 }

wranIfABsAccessLinkUsArqBlockReTx	OBJECT-TYPE

SYNTAX	Integer32 (1..4294967295)

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

“Total # of ARQ blocks that were retransmitted to the A-CPE

during the reporting period.”

::= { wranIfABsArqMetricsEntry 17 }

wranIfABsRelayLinkDsNumArqBlockEfficiency	OBJECT-TYPE

SYNTAX	INTEGER (0..100)

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

“wranIfABsRelayLinkDsNumArqBlockEfficiency =

(wranIfABsRelayLinkDsArqBlockReTx /

wranIfABsRelayLinkDsNumArqBlocks) * 10000000, in units of 1e-7.”

::= { wranIfABsArqMetricsEntry 18 }

wranIfABsAccessLinkDsNumArqBlocksEfficiency	OBJECT-TYPE

SYNTAX	INTEGER (0..100)

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

“wranIfABsAccessLinkDsNumArqBlocksEfficiency =

(wranIfABsAccessLinkDsArqBlockReTx /

wranIfABsAccessLinkDsNumArqBlocks) * 10000000,

in units of 1e-7.”

::= { wranIfABsArqMetricsEntry 19 }

wranIfABsRelayLinkUsNumArqBlockEfficiency	OBJECT-TYPE

SYNTAX	INTEGER (0..100)

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

“wranIfABsRelayLinkUsNumArqBlockEfficiency =

(wranIfABsRelayLinkUsArqBlockReTx /

wranIfABsRelayLinkUsNumArqBlocks) * 10000000, in units of 1e-7.”

::= { wranIfABsArqMetricsEntry 20 }

wranIfABsAccessLinkUsNumArqBlocksEfficiency	OBJECT-TYPE

SYNTAX	INTEGER (0..100)

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

“wranIfABsAccessLinkUsNumArqBlocksEfficiency =

(wranIfABsAccessLinkUsArqBlockReTx /

wranIfABsAccessLinkUsNumArqBlocks) * 10000000,

in units of 1e-7.”

::= { wranIfABsArqMetricsEntry 21 }

-- wranIfABsPmMibGroups: This object helsp define which MIB groups are available in

-- this module (wranIfABsPm) and which MIB objects are part of each group.

wranIfABsPmMibGroups		OBJECT IDENTIFIER

::= { wranIfABsPm 3 }

wranIfABsPmConfigGroup		OBJECT IDENTIFIER

::= { wranIfABsPmMibGroups 1 }

wranIfABsArqGroup			OBJECT IDENTIFIER

::= { wranIfABsPmMibGroups 2 }

wranIfABsPmConfigGroup		OBJECT-GROUP

OBJECTS	{ wranIfABsPmConfigurationEntryIndex,

wranIfABsGranularityInterval,

wranIfABsTwoLinkArqConfig }

STATUS	current

DESCRIPTION

“This group contains objects configuration of measurement records

used in wranIfABsPm.”

::= { wranIfABsPmMibGroups 1 }

wranIfABsArqGroup		OBJECT-GROUP

OBJECTS	{ wranIfABsArqMetricsIndex,

wranIfABsRelayLinkDsNumArqBlocks,

wranIfABsAccessLinkDsNumArqBlocks,

wranIfABsRelayLinkDsNumArqBlocksDropped,

wranIfABsAccessLinkDsNumArqBlocksDropped,

wranIfABsRelayLinkDsArqBlockErrorRate,

wranIfABsAccessLinkDsArqBlockErrorRate,

wranIfABsRelayLinkDsArqBlockReTx,

wranIfABsAccessLinkDsArqBlockReTx,

wranIfABsRelayLinkUsNumArqBlocks,

wranIfABsAccessLinkUsNumArqBlocks,

wranIfABsRelayLinkUsNumArqBlocksDropped,

wranIfABsAccessLinkUsNumArqBlocksDropped,

wranIfABsRelayLinkUsArqBlockErrorRate,

wranIfABsAccessLinkUsArqBlockErrorRate,

wranIfABsRelayLinkUsArqBlockReTx,

wranIfABsAccessLinkUsArqBlockReTx,

wranIfABsRelayLinkDsArqBlockEfficiency,

wranIfABsAccessLinkDsArqBlockEfficiency,

wranIfABsRelayLinkUsArqBlockEfficiency,

wranIfABsAccessLinkUsArqBlockEfficiency }

STATUS	current

DESCRIPTION

“This group contains objects related to tracking Two Link ARQ

performance.”

::= { wranIfABsPmMibGroups 2 }

wranIfABsPmMibCompliance	MODULE-COMPLIANCE

STATUS	current

DESCRIPTION

“Defines which objects are optional and which ones are mandatory

for wranIfABsPm compliance.”

MANDATORY-GROUPS { wranIfABsPmConfigGroup }

-- OPTIONAL-GROUPS { wranIfABsArqGroup }

::= { wranIfABsPm 4 }

END

---end of text modification---

15) adding text for section 14.2.9

---start of text modification---

14.2.9 wranIfACpeMib

IEEE802dot22-WRAN-IF-ACPE-MIB		::=		BEGIN

IMPORTS

MODULE-IDENTITY,

OBJECT-TYPE,

NOTIFICATION-TYPE,

Unsigned32, Integer32

FROM SNMPv2-SMI

SnmpAdminString

FROM SNMP-FRAMEWORK-MIB

TEXTAL-CONVENTION,

RowStatus, TruthValue,

TimeStamp, DateAndTime

FROM SNMPv2-TC

InetAddressType, InetAddress

FROM INET-ADDRESS-MIB

OBJECT-GROUP,

MODULE-COMPLIANCE,

NOTIFICATION-GROUP

FROM SNMPv2-CONF;

wranIfACpeMib	MODULE-IDENTITY

LAST-UPDATED		“201808300000Z” – August 30, 2018

ORGANIZATION		“IEEE 802.22”

CONTACT-INFO

“WG E-mail:	STDS-802-22@LISTSERVE.IEEE.ORG

WG Chair:	Apurva N. Mody

E-mail:	apurva.mody@ieee.org

Maintenance Editor:	Ranga Reddy

E-mail:			ranga.reddy@ieee.org”

DESCRIPTIOIN

“This material was added to support newer functionality

Introduced with the IEEE Std 802.22b-2015 Copyright (c)

2015. This MIB Module defines managed objects for Advanced

Base Station based on IEEE Std 802.22b-2015 and is under

iso(1).std(0).iso8022(8802).wran(22).wranIfACpeMib(9)”

REVISION		“201808300000Z”

DESRIPTION

“The first version of IEEE802dot22-WRAN-IF-ACPE-MIB that

Provides for management of the Advanced CPE.”

::= {iso std(0) iso8802(8802) wran(22) 9}

wranIfACpeConfiguationTable			OBJECT IDENTIFIER

::= { wranIfACpeMib 1 }

wranIfACpeMibGroups				OBJECT IDENTIFIER

::= { wranIfACpeMib 2 }

wranIfACpeMibCompliance				OBJECT IDENTIFIER

::= { wranIfACpeMib 3 }

wranIfACpeSystemParametersTable		OBJECT-TYPE

SYNTAX	SEQUENCE OF wranIfACpeSystemParametersEntry

MAX-ACCESS	not-accessible

STATUS	current

DESCRIPTION

“This table contains objects that define system constants related

to A-BS/A-CPE operation.”

::= { wranIfACpeMib 1 }

wranIfACpeSystemParametersEntry	OBJECT-TYPE

SYNTAX	wranIfACpeSystemParametersEntry

MAX-ACCESS	not-accessible

STATUS	current

DESCRIPTION

“This object defines a compound object that respresents an entry

In wranIfACpeSystemParametersTable.”

INDEX	wranIfACpeSystemParametersIndex

::= { wranIfACpeSystemParametersTable 1 }

wranIfACpeSystemParametersEntry		::= SEQUENCE {

wranIfACpeSystemParametersIndex		INTEGER,

wranIfABsT61					Integer32,

wranIfABsT62					Integer32,

wranIfABsT63					Integer32,

wranIfABsT64					Integer32,

wranIfABsT65					Integer32,

wranIfABsT66					Integer32,

wranIfABsT67					Integer32 }

wranIfACpeSystemParametersIndex		OBJECT-TYPE

SYNTAX	INTEGER (1..1)

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

“Index of entry in this table, defaults to 1.”

::= { wranIfACpeSystemParametersEntry 1 }

wranIfABsT61		OBJECT-TYPE

SYNTAX	Integer32 (10..wranIfBsT9)

UNITS		“milliseconds”

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

“Wait for container ACK timer, T61.”

::= { wranIfACpeSystemParametersEntry 2 }

wranIfABsT62		OBJECT-TYPE

SYNTAX	Integer32 (10..wranIfBsT9)

UNITS		“milliseconds”

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

“Wait for local cell update acknowledgement, T62.”

::= { wranIfACpeSystemParametersEntry 3 }

wranIfABsT63		OBJECT-TYPE

SYNTAX	Integer32 (10..wranIfBsT9)

UNITS		“milliseconds”

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

“Wait for DTT-RSP timeout, T63.”

::= { wranIfACpeSystemParametersEntry 4 }

wranIfABsT64		OBJECT-TYPE

SYNTAX	Integer32 (10..wranIfBsT9)

UNITS		“milliseconds”

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

“Wait for DTT-RPT timeout, T64.”

::= { wranIfACpeSystemParametersEntry 4 }

wranIfABsT65		OBJECT-TYPE

SYNTAX	Integer32 (20..wranIfBsT9)

UNITS		“milliseconds”

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

“Wait for DTT-CFM timeout, T65.”

::= { wranIfACpeSystemParametersEntry 5 }

wranIfABsT66		OBJECT-TYPE

SYNTAX	Integer32 (20..160)

UNITS		“milliseconds”

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

“Wait for DTT-STP timeout, T66.”

::= { wranIfACpeSystemParametersEntry 6 }

wranIfABsT67		OBJECT-TYPE

SYNTAX	Integer32 (10..160)

UNITS		“milliseconds”

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

“Wait for DTT-STP timeout, T66.”

::= { wranIfACpeSystemParametersEntry 7 }

-- wranIfACpeMibGroups: This object helps define which MIB groups are available in

-- this module (wranIfACpeMib) and which MIB objects are part of each group.

wranIfACpeMibGroups		OBJECT IDENTIFIER

::= { wranIfACpeMib 2 }

wranIfACpeConfigGroup		OBJECT IDENTIFIER

::= { wranIfACpeMibGroups 1 }

wranIfACpeConfigGroup		OBJECT-GROUP

OBJECTS	{ wranIfACpeSystemParametersIndex,

wranIfABsT61,

wranIfABsT62,

wranIfABsT63,

wranIfABsT64,

wranIfABsT65,

wranIfABsT66,

wranIfABsT67 }

STATUS	current

DESCRIPTION

“This group contains objects configuration of system timers for

the A-CPE.”

::= { wranIfACpeMibGroups 1 }

wranIfACpeMibCompliance	MODULE-COMPLIANCE

STATUS	current

DESCRIPTION

“Defines which objects are optional and which ones are mandatory

for wranIfACpeMib compliance.”

MANDATORY-GROUPS { wranIfACpeConfigGroup }

::= { wranIfACpeMib 3 }

END

---end of text modification---

16) remove wranIfBsMibGroups and wranIfBsMibCompliance  in 14.2.2 wranIfBsMib

17) update MIB description of wranIfBsCpeSaType in section 14.1.2.5.2.1.4 as follows

---start of text modification---

14.1.2.5.2.1.4  wranIfBsCpeSaType

Type of SA, either Null, Unicast Management, Unicast TransportPrimary, Secondary, or Group.

---end of text modification---

18) update MIB definition of wranIfBsCpeSaType in section 14.2.2 wranIfBsMib

---start of text modification---

wranIfBsCpeSaType	OBJECT-TYPE

SYNTAX	INTEGER { null(0), primary(1),

secondary(2), unicastManagement(1),

unicastTransport(2), group(3) }

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

"Type of SA, either Null, Primary, Secondary, or

Group."

::= { wranIfBsCpeScmSaConfigEntry 4 }

---end of text modification---

19) update MIB description of wranIfBsCpeActiveTekPn in section 14.1.2.5.2.1.9

---start of text modification---

14.1.2.5.2.1.4  wranIfBsCpeActiveTekPn

Current value of the PN counter for the first second (non-active) generation of the two TEKs that are configured for this SA.

---end of text modification---

20) update MIB definition of wranIfBsCpeActiveTekPn in section 14.2.2 wranIfBsMib

---start of text modification---

wranIfBsCpeActiveTekPn	OBJECT-TYPE

SYNTAX	Integer32 (0..167772164294967295)

UNITS		“seconds”

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

"Current value of the PN counter for the current

(active) generation of the two TEKs that are

configured for this SA."

::= { wranIfBsCpeScmSaConfigEntry 9 }

---end of text modification---

21) update MIB definition of wranIfBsCpeNonActiveTekPn in section 14.2.2 wranIfBsMib

---start of text modification---

wranIfBsCpeNonActiveTekPn	OBJECT-TYPE

SYNTAX	Integer32 (0..167772164294967295)

UNITS		“seconds”

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

"Current value of the PN counter for the second

(non-active) generation of the two TEKs that are

configured for this SA."

::= { wranIfBsCpeScmSaConfigEntry 13 }

---end of text modification---

22) update MIB description of wranIfBsCpeScmKeyReqGroupKeyIndicator in section 14.1.2.5.3.1.5

---start of text modification---

14.1.2.5.3.1.5  wranIfBsCpeScmKeyReqGroupKeyIndicator

Indicator of whether Key-Request was for a Unicast Transport or GroupSA.

---end of text modification---

23) update MIB definition of wranIfBsCpeScmKeyReqGroupKeyIndicator in section 14.2.2 wranIfBsMib

---start of text modification---

wranIfBsCpeNonActiveTekPn	OBJECT-TYPE

SYNTAX	INTEGER { unicastTransportSA(0), groupSA(1) }

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

"Type of SA, either Unicast Transport or Group."

::= { wranifBsCpeTekRefreshEntry 5 }

---end of text modification---

24) add a section 14.1.2.2.15.12 “wranIfBasicCapabilityMacAddress” after 14.1.2.2.15.11

---start of text modification---

14.1.2.2.15.12  wranIfBasicCapabilityMacAddress

MAC Address of CPE

---end of text modification---

25) Add entry of wranIfBasicCapabilityMacAddress to beginning MIB definition of wranIfBsCpeBasicCapabilityCmn in 14.2.2

---start of text modification---

wranIfBsCpeBasicCapabilityCmn	OBJECT IDENTIFIER

::= { wranIfBsCm 16 }

wranIfBasicCapabilityIndex  OBJECT IDENTIFIER

::= { wranIfBsCpeBasicCapabilityCmn 1 }

wranIfBasicCapabilityNumAttempts  OBJECT IDENTIFIER

::= { wranIfBsCpeBasicCapabilityCmn 2 }

wranIfBasicCapabilityMacAddress  OBJECT IDENTIFIER

::= { wranIfBsCpeBasicCapabilityCmn 3 }

wranIfBasicCapabilityStationId  OBJECT IDENTIFIER

::= { wranIfBsCpeBasicCapabilityCmn 4 }

wranIfBasicCapabilityMacPduTxandConstruction  OBJECT IDENTIFIER

::= { wranIfBsCpeBasicCapabilityCmn 5 }

wranIfBasicCapabilityMaxCpeTxEirp  OBJECT IDENTIFIER

::= { wranIfBsCpeBasicCapabilityCmn 6 }

wranIfBasicCapabilityCpeDemodulator  OBJECT IDENTIFIER

::= { wranIfBsCpeBasicCapabilityCmn 7 }

wranIfBasicCapabilityCpeModulator OBJECT IDENTIFIER

::= { wranIfBsCpeBasicCapabilityCmn 8 }

wranIfBasicCapabilityCpeScmVersionSupport  OBJECT IDENTIFIER

::= { wranIfBsCpeBasicCapabilityCmn 9 }

wranIfBasicCapabilityCpePnWindowSize  OBJECT IDENTIFIER

::= { wranIfBsCpeBasicCapabilityCmn 10 }

wranIfBasicCapabilityCpeScmFlowControl  OBJECT IDENTIFIER

::= { wranIfBsCpeBasicCapabilityCmn 11 }

wranIfBasicCapabilityMacAddress  OBJECT IDENTIFIER

::= { wranIfBsCpeBasicCapabilityCmn 12 }

---end of text modification---

26) add MIB definition of wranIfBasicCapabilityMacAddress after MIB definition of wranIfBasicCapabilityCpeScmFlowControl

---start of text modification---

wranIfBasicCapabilityMacAddress  	OBJECT-TYPE

SYNTAX	MacAddress

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

"MAC Address of CPE."

---end of text modification---

27) change title of section 14.1.8.1.3.1.9 from wranIfABsTwoLinkArqIndex to wranIfABsArqMetricsIndex

28) delete entry/section 14.1.8.1.3.1.3 wranIfABsNotificationMacAddr, this instance seems to be redundant

29) change correct spelling of “modified” in 14.1.8.1.3.1.4 wranIfABsGroupResourceAllocationConfigType

30) in text for wranIfABsMultiChannelConfigIndex (14.1.8.1.3.1.7), change wranIfABsRelayConfigTable to wranIfABsMultiChnConfigTable

31) in text for wranIfABsRelayCpeConfigIndex (14.1.8.1.3.1.8) change wranIfABsCapabilitiesConfigTable to wranIfABsRelayConfigTable

32) modify text in 14.1.8.2.4.4 wranIfABsBasicCapabilityRelayCpeMode as follows:

---start of text modification---

14.1.8.5.2.4.4  wranIfABsBasicCapabilityRelayCpeMode

Defines whether Relay A-CPE is operating in Centralized or Distributed Scheduling Modedemodulator setting for Centralized Scheduling A-CPE.

---end of text modification---

33) change title of 14.1.8.2.8.1.1.5 to wranIfABsOfdmaPhyUsSegmentConfig, replace all instances of wranIfABsOfdmaPhyUsSegment with wranIfABsOfdmaPhyUsSegmentConfig

34) change title of 14.1.8.2.8.1.1.6 to wranIfABsOfdmaPhyUsCenterFrequency, replace all instances of wranIfABsOfdmaPhyUsChannelNum with wranIfABsOfdmaPhyUsCenterFrequency, modify text of this section as follows:

---start of text modification---

14.1.8.2.8.1.1.6  wranIfABsOfdmaPhyUsChannelNumCenterFrequency

Channel # of USCenter Frequency of US channel in kHz.

---end of text modification---

35) change title of 14.1.8.2.8.2.1.6 to wranIfABsOfdmaPhyDsCenterFrequency, replace all instances of wranIfABsOfdmaPhyDsChannelNum with wranIfABsOfdmaPhyDsCenterFrequency, modify text of this section as follows:

---start of text modification---

14.1.8.2.8.2.1.6  wranIfABsOfdmaPhyDsChannelNumCenterFrequency

Channel # of DSCenter Frequency of DS channel in kHz.

---end of text modification---

36) correct spelling of “Numberf” in 14.1.8.2.8.3.1.3

37) add an index entry 14.1.8.3.1.1.1 wranIfABsPmConfigurationIndex before wranIfABsGranularityInterval, and renumber rest of the objects as parts of wranIfABsPmConfigurationEntry

---start of text modification---

14.1.8.3.1.1.1 wranIfABsPmConfigurationIndex

Index of entry into table, defaults to 1.

---end of text modification---

38) change title of 1st member of wranIfBsPmConfigurationEntry from wranIfBsOtaUsageDataRecordIndex to wranIfBsPmConfigurationEntryIndex

39) change title of 14.1.8.3.2.1.7 to end in “ReTx” instead of “Retransmission”, change MIB definition title for this object accordingly

40) change title of 14.1.8.3.2.1.8 to end in “ReTx” instead of “Retransmission”, change MIB definition title for this object accordingly

41) change title of 14.1.8.3.2.1.15 to end in “ReTx” instead of “Retransmission”, change MIB definition title for this object accordingly

42) change title of 14.1.8.3.2.1.16 to end in “ReTx” instead of “Retransmission”, change MIB definition title for this object accordingly

43) change name of 1st table at start of wranIfACpeMib from wranIfACpeConfigurationTable, wranIfACpeSystemParametersTable

44) change wranIfABsCpeSystemParametersTable to wranIfACpeSystemParametersTable

45) remove reference to wranIfACpeTrapControl, wranIfACpeTrapDefinition, wranIfACpeNotificationObjectsTable