IEEE P802.22
Wireless RANs

| Metadata definition according to SigMF proposal |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2018-10-31 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Gianni Cerro | University of Cassino and Southern Lazio | Via G. Di Biasio 43 – 03043 Cassino, Italy | +3907762993683 | g.cerro@unicas.it |
| Gianfranco Miele | University of Cassino and Southern Lazio | Via G. Di Biasio 43 – 03043 Cassino, Italy | +3907762993686 | g.miele@unicas.it |

Abstract

This document provides metadata definition according to SigMF

Year 2018 – November 13, 2018

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Carl R. Stevenson> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

# Annex B (Normative): Metadata Specification

This appendix presents the structure of metadata used in the information message exchanged between Sensor Manager (SM) and Sensing Device (SD). First, metadata classification is described in B.1. Sub-clause B.2 elaborates on data types and structures required in formally describing metadata. Sub-clause B.3 then formally describes these parameters exchanged based on the data type descriptions given by B.2.

## Metadata categorization

This sub-clause addresses the main categories of metadata included in the messages exchanged between SD and SM. They are categorized into three main classes having different purposes: Class A (System Metadata), Class B (Sensor settings) and Class C (Sensing related metadata).

### System Metadata

Class A (System Metadata) includes all factory information related pieces of data and remain constant for the entire lifespan of the component (SD). Taking into account the metadata reported in table B2, SensorDefinition.Antenna Metadata, SensorDefintion.SignalConditioningUnit Metadata, SensorDefinition.DataExtractionUnit Metadata, Host Metadata are included in this category.

Class A metadata is not subjected to any change since it is offered as a response to a specific query in SD association process.

### Current Status Metadata

Class B (Current Status Metadata) includes data describing the actual configuration of the device, in terms of hardware (positioning, antenna configuration, etc.) and software (frequency settings, sampling rate, sensing algorithm, etc.).

Class B metadata is provided to SM, after a specific user request, and can be subjected to modification and special settings by the Tasking Agent.

### Sensing related Metadata

Class C (Sensing related metadata), specifying parameters strictly related to performed sensing action (scanned time, timestamp, atmosphere conditions, etc.);

Class C metadata is not subjected to any change since it is offered as a response to a specific query in a Sensing request.

## Data Types

This sub-clause defines the primitive data types, simple data types, and derived data types used in the definition of metadata defined in B.3. The physical units used in this definition are based on the International System of Units (SI) and are summarized in Table B1.

Table B1: Units used in the description of Metadata

| Unit | Unit symbol | Value | Note |
| --- | --- | --- | --- |
| second | s |  | SI unit |
| Hertz | Hz | 1 Hz = 1/s | SI derived unit |
| meter | m |  | SI unit |
| Watt | W | kg•m2/s3 | SI derived unit |
| Power ratio | dB | [dB]=10•log10(P1[W]/P2[W]) | dimensionless |
| Power ratio respect to 1 mW | dBm | [dBm]=10•log10(P1[W]/1 mW) | dimensionless |
| Radian | rad | 1 rad = 180/π | SI derived unit, dimensionless |
| Degree of arc | ° | 1° = π/180 rad | dimensionless |
| Power ratio respect to isotropic antenna | dBi | [dBi]=10•log10(P1[W]/Pisotropic[W]) | dimensionless |
| Kelvin | K |  | SI unit |

### Simple data type

In this sub-clause the simple data type considered in this document are summarized.

#### Boolean

A primitive logical data type having one of two values of “true” (1, nonzero) or “false” (0, zero).

#### Integer

A primitive integral data type representing natural numbers and their negatives. Note that common binary representations limit the number range due to machine word length restrictions. In this standard, the integer length is defined as 32-bit value.

#### Unsigned integer

A primitive data type representing non-negative integers.

#### Float

A primitive data type storing real numbers, usually as floating-point numbers. Floating-point number representations as defined in IEEE Std 754TM-2008 can be taken as an example.

#### Array

A simple data type storing a collection of data values of a specified type. The notation array(type) is used to specify the type of array elements.

#### Vector

A simple data type storing a collection of data values of a specified type. The notation vector(type) is used to specify the type of array elements. A vector is a one-dimensional array.

#### String

A simple data type storing a sequence of bytes or characters. A string is a special use of a one-dimensional array.

### Complex data types

This sub-clause summarizes complex data types such as structured types or types that rely on specific interpretation or restriction of the underlying simple type.

#### Enumeration

An enumeration is a listing of elements mapped to an index set consisting of natural numbers. That is, each element of the set is unambiguously represented by an ordinal value.

#### Cluster

A complex data type that aggregates a fixed set of labelled elements, possibly of different simple types, into a single element.

#### Fixed-point

Fixed-point numbers are rational numbers with a fixed length mantissa and a fixed exponent. In contrast to a floating-point representation, using a fixed length but variable exponent, the value range is limited by the mantissa length, but the resolution is constant over the value range. They can be realized by using an integer value in conjunction with an implicit multiplier.

#### Unsigned fixed-point

Unsigned fixed-point numbers are non-negative fixed-point numbers.

## Description of metadata

Metadata descriptions are given in a tabular form throughout B.3.1 through B.3.16. They consist of the metadata name and ID, a short textual description, and a type and size specification, if needed.

⎯ Metadata name and ID

Metadata name provides a unique identification of the parameter in human readable form, whereas the numerical ID is given to unambiguously identify the metadata in the process of information exchange between SCOS entities.

⎯ Metadata type and size

Metadata types are of one of the types defined by B.2. Some parameters may be further restricted in their value range or magnitude. The size field of the parameter description is supplementary information that is either a fixed value, determined by the number of elements contained, or variable if at least one of the elements contained is optional, or is of variable size itself. To avoid implementation dependent specifications for metadata, size is always given in terms of the underlying type.

For metadata based on array types, the size is given as the number of elements stored in the array or as “variable” if the size of the array is unspecified. Note that variable size arrays demand for an implicit array length value in information exchange.

For cluster types, the aggregated size depends on the implementation of the elements enclosed and thus is omitted in the parameter description. The implementation then will decide on the binary representation, encoding, and size in terms of bits or bytes and any tag or length values needed.

Table B2 provides a summary of metadata and categorizes them into each of the three classes introduced in B.1.

Table B2: Summary of metadata

| ID | Metadata name | Sub-clause | Class A | Class B | Class C |
| --- | --- | --- | --- | --- | --- |
| 001 | SensorDefinition.Antenna | B.3.1 | X |  |  |
| 002 | SensorDefinition.SignalConditioningUnit | B.3.2 | X |  |  |
| 003 | SensorDefinition.DataExtractionUnit | B.3.3 | X |  |  |
| 004 | SensorDefinition.Host | B.3.4 | X |  |  |
| 005 | Sensor_ID | B.3.5 | X |  |  |
| 006 | CurrentStatus.Antenna | B.3.6 |  | X |  |
| 007 | CurrentStatus.SignalConditioningUnit | B.3.7 |  | X |  |
| 008 | CurrentStatus.SignalExtractionUnit | B.3.8 |  | X |  |
| 009 | CurrentStatus.EnvironmentalSensors | B.3.9 |  | X |  |
| 010 | CurrentStatus.ReferenceGeolocation | B.3.10 |  | X |  |
| 011 | CurrentStatus.Certificate | B.3.11 |  | X |  |
| 012 | MeasurementTask.Type | B.3.12 |  |  | X |
| 013 | MeasurementTask.TimeDomainDetection | B.3.13 |  |  | X |
| 014 | MeasurementTask.FrequencyDomainDetection | B.3.14 |  |  | X |
| 015 | MeasurementTask.SweptTunedMeasurement | B.3.15 |  |  | X |
| 016 | MeasurementTask.YFactorCalibration | B.3.16 |  |  | X |

### SensorDefinition.Antenna

The Antenna metadata indicates specifications of the antenna as given by the manufacturer. The definition of each parameters and the related unit of measurements are described in table B3.

Table B3: SensorDefinition.Antenna metadata parameters description

| Metadata name: | SensorDefinition.Antenna | Meas. Unit: |  | -------- |  | Data type: | Cluster |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ID: | 001 | Size: |  | Variable |  |  |  |
| Desc.: | List of the SensorDefinition.Antenna specifications according to manufacturer specifications. |  |  |  |  |  |  |
|  | Name | Content |  |  |  | Data type | Meas. Unit |
| .0 | model | It contains a string with the model of the installed antenna |  |  |  | String | ------ |
| .1 | low_frequency | Low frequency of operational range. |  |  |  | Unsigned fixed-point | Hz |
| .2 | high_frequency | High frequency of operational range. |  |  |  | Unsigned fixed-point | Hz |
| .3 | type | Antenna type |  |  |  | String | ------ |
| .4 | gain | Antenna gain |  |  |  | Float | dBi |
| .5 | horizontal_gain_pattern | Antenna gain pattern in horizontal plane from 0° to 359° in 1° steps. |  |  |  | Vector(float) | dBi |
| .6 | vertical_gain_pattern | Antenna gain pattern in vertical plane from -90° to +90° in 1° steps. |  |  |  | Vector(float) | dBi |
| .7 | horizontal_beam_width | Horizontal 3-dB beamwidth |  |  |  | Fixed-point | ° (degree) |
| .8 | vertical_beam_width | Vertical 3-dB beamwidth |  |  |  | Fixed-point | ° (degree) |
| .9 | polarization | Antenna polarization |  |  |  | Enumerator | ------ |
|  |  | “VL” “HL” “LHC” “RHC” “Slant” | Value: Value: Value: Value: Value: |  | 0 1 2 3 4 |  |  |
| .10 | cross_polar_discrimination | Cross-polar discrimination |  |  |  | Float | ------ |
| .11 | voltage_standing_wave_ratio | Voltage wave standing ratio |  |  |  | Unsigned fixed-point | ------ |
| .12 | cable_loss | Attenuation introduced by the cable connecting antenna and the signal conditioning unit. |  |  |  | Float | dB |
| .13 | steerable | Defines if the antenna is steerable or not. |  |  |  | Boolean | ------ |
| .14 | mobile | Defines if the antenna is mobile or not. |  |  |  | Boolean | ------ |
| .15 | negative_horizontal_beam_direction | If the antenna is steerable, it represents the negative selectable deviation of the direction of the main beam in azimuthal respect to 0°. If the antenna is not steerable it is set to 0°. |  |  |  | Fixed-point | ° (degree) |
| .16 | positive_horizontal_beam_direction | If the antenna is steerable, it represents the positive selectable deviation of the direction of the main beam in azimuthal respect to 0°. If the antenna is not steerable it is set to 0°. |  |  |  | Fixed-point | ° (degree) |
| .17 | negative_vertical_beam_direction | If the antenna is steerable, it represents the negative selectable deviation of the direction of the main beam in elevation respect to 0°. If the antenna is not steerable it is set to 0°. |  |  |  | Fixed-point | ° (degree) |
| .18 | positive_vertical_beam_direction | If the antenna is steerable, it represents the positive selectable deviation of the direction of the main beam in elevation respect to 0°. If the antenna is not steerable it is set to 0°. |  |  |  | Fixed-point | ° (degree) |
| .19 | Reserved (for future use) |  |  |  |  |  |  |

### B.3.2. SensorDefinition.SignalConditioningUnit

The Signal conditioning unit metadata indicates specifications of the RF front-end as given by the manufacturer. The definition of each parameters and the related unit of measurements are reported in table B4.

Table B4: SensorDefinition.SignalConditioningUnit metadata parameters description

| Metadata name: | SensorDefinition.SignalConditioningUnit | Meas. Unit: | -------- | Data type: | Cluster |
| --- | --- | --- | --- | --- | --- |
| ID: | 002 | Size: | Variable |  |  |
| Desc.: | List of the SensorDefinition.SignalConditioningUnit specifications according to manufacturer specifications. |  |  |  |  |
|  | Name | Content |  | Data type | Meas. Unit |
| .0 | rf_path_number | Total number of RF paths |  | Unsigned integer | ------ |
| .1 | low_frequency_passband | A vector with a total number of elements equal to rf_path_number. The element i represents low frequency of filter 1-dB passband in the i-th path. |  | Vector(Unsigned fixed-point) | Hz |
| .2 | high_frequency_passband | A vector with a total number of elements equal to rf_path_number. The element i represents high frequency of filter 1-dB passband in the i-th path. |  | Vector(Unsigned fixed-point) | Hz |
| .3 | low_frequency_stopband | A vector with a total number of elements equal to rf_path_number. The element i represents low frequency of filter 60-dB stopband in the i-th path. |  | Vector(Unsigned fixed-point) | Hz |
| .4 | high_frequency_stopband | A vector with a total number of elements equal to rf_path_number. The element i represents high frequency of filter 60-dB stopband in the i-th path. |  | Vector(Unsigned fixed-point) | Hz |
| .5 | lna_gain | A vector with a total number of elements equal to rf_path_number. The element i represents low-noise amplifier gain in the i-th path. |  | Vector(float) | dB |
| .6 | lna_noise_figure | A vector with a total number of elements equal to rf_path_number. The element i represents low-noise amplifier noise figure in the i-th path. |  | Vector(float) | dB |
| .7 | calibration_source | A vector with a total number of elements equal to rf_path_number. The element i is equal to 1 if a calibration source is present in the i-th path. |  | Vector(boolean) | ----- |
| .8 | calibration_source_frequency | A vector with a total number of elements equal to rf_path_number. The element i is equal to the frequency of the calibration source if a calibration source is present in the i-th path otherwise it is set equal to 0. |  | Vector(Unsigned fixed-point) | Hz |
| .9 | calibration_source_amplitude | A vector with a total number of elements equal to rf_path_number. The element i is equal to the magnitude of the calibration source if a calibration source is present in the i-th path otherwise it is sei equal to -1000. |  | Vector(float) | dB |
| .10 | self_calibration | This is set to “1” if the sensor performs a periodical self-calibration procedure. Otherwise it is set to “0” if the self-calibration is performed after a user request. |  | boolean | ----- |
| .11 | Reserved (for future use) |  |  |  |  |

### B.3.3. SensorDefinition.DataExtractionUnit

DataExtractionUnit metadata contains information about computing hardware specifications as given by the manufacturer. The definition of each parameters and the related unit of measurements are reported in table B5.

Table B5: SensorDefinition.DataExtractionUnit metadata parameters description

| Metadata name: | SensorDefinition. DataExtractionUnit | Meas. Unit: | -------- | Data type: | Cluster |
| --- | --- | --- | --- | --- | --- |
| ID: | 003 | Size: | Variable |  |  |
| Desc.: | List of the SensorDefinition.DataExtractionUnit specifications according to manufacturer specifications. |  |  |  |  |
|  | Name | Content |  | Data type | Meas. Unit |
| .0 | model | Manufacturer and model of the receiver. |  | String | ------- |
| .1 | low_frequency | Low frequency of operational range of the receiver. |  | Unsigned fixed-point | Hz |
| .2 | high_frequency | High frequency of operational range of the receiver. |  | Unsigned fixed-point | Hz |
| .3 | noise_figure | Noise figure of the receiver. |  | float | dB |
| .4 | number_of_bits | Number of bits of the ADC. |  | Unsigned integer | ------ |
| .5 | max_power | Maximum input power of the receiver. |  | float | dBm |
| .6 | max_sampling_rate | Maximum sampling rate of the receiver. |  | Unsigned fixed-point | Hz |
| .7 | firmware | Current firmware version |  | String | ------- |
| .8 | Reserved (for future use) |  |  |  |  |

### B.3.4. SensorDefinition.Host

Host metadata contains information about the host controller of the sensor. The host controller can be included in sensor hardware or can be an external device that control and drives the sensor. The definition of each parameters and the related unit of measurements are reported in table B6

Table B6: SensorDefinition.Host metadata parameters description

| Metadata name: | SensorDefinition. SignalConditioningUnit | Meas. Unit: | -------- | Data type: | Cluster |
| --- | --- | --- | --- | --- | --- |
| ID: | 004 | Size: | Variable |  |  |
| Desc.: | List of the SensorDefinition.Host specifications according to manufacturer specifications. |  |  |  |  |
|  | Name | Content |  | Data type | Meas. Unit |
| .0 | Model | Manufacturer and model of the host |  | String | ------ |
| .1 | OS | Operating system installed on the host |  | String | ------ |
| .2 | Reserved (for future use) |  |  |  |  |

### B.3.5. Sensor_ID

Sensor_ID metadata contains information about the Identification Number of the adopted Sensing Device. It can be adopted as a primary key in Sensing Manager Database, and all previous pieces of information can be related to this identifier. The definition of metadata parameter is reported in table B7.

Table B7: Sensor_ID metadata parameters description

| Metadata name: | Sensor_ID | Meas. Unit: | -------- | Data type: | Integer |
| --- | --- | --- | --- | --- | --- |
| ID: | 005 | Size: | 1 |  |  |
| Desc.: | List of the Sensor_ID specifications according to SCOS Manager specifications. |  |  |  |  |
|  | Name | Content |  | Data type | Meas. Unit |
|  | Identifier | A number identifying univoquely the sensor inside the SCOS system |  | Integer | ------ |

### B.3.6 CurrentStatus.Antenna

This is the first section describing metadata reporting current status of the sensing device setttings. CurrentStatus.Antenna metadata describes antenna parameters. that can change dynamically, in their current setting. The definition of each parameters and the related unit of measurements are reported in table B8.

Table B8: CurrentStatus.Antenna metadata parameters description

| Metadata name: | CurrentStatus. Antenna | Meas. Unit: | -------- | Data type: | Cluster |
| --- | --- | --- | --- | --- | --- |
| ID: | 006 | Size: | Variable |  |  |
| Desc.: | List of the SensorDefinition.Host specifications according to manufacturer specifications. |  |  |  |  |
|  | Name | Content |  | Data type | Meas. Unit |
| .0 | horizontal_beam_direction | Current deviation of the direction of the main beam in azimuthal with respect to 0°. |  | Float | ° (degree) |
| .1 | vertical_beam_direction | Current Angle of main beam in elevation plane with respect to 0°. |  | Float | ° (degree) |
| .2 | Reserved (for future use) |  |  |  |  |

### B.3.7 CurrentStatus.SignalConditioningUnit

Signal Conditioning Unit metadata in current status category describes the particular behavior of the receiver front-end at the time instant of the scan request. The definition of each parameters and the related unit of measurements are reported in table B9.

Table B9: CurrentStatus.SignalConditioningUnit metadata parameters description

| Metadata name: | CurrentStatus. SignalConditioningUnit | Meas. Unit: | -------- | Data type: | Cluster |
| --- | --- | --- | --- | --- | --- |
| ID: | 007 | Size: | Variable |  |  |
| Desc.: | List of the SignalConditioningUnit specifications according to SCOS Manager imposed specifications. |  |  |  |  |
|  | Name | Content |  | Data type | Meas. Unit |
| .0 | rf_path_selected_number | Current path number selected for acquisition |  | Unsigned Integer | ------------ |
| .1 | Last_calibration_date | Date of the last certified calibration of the RF front-end of the device. It is denoted as a basic reference time value. Seconds since midnight (UTC) of January 1, 1970 absolute time. |  | Unsigned integer | ------------- |
| .2 | Reserved (for future use) |  |  |  |  |

### B.3.8 CurrentStatus.DataExtractionUnit

Data Extraction Unit metadata in current status category describes the dynamic features of the Analog-to-Digital Converter and Demodulator that are currently selected on the sensing device. The definition of each parameters and the related unit of measurements are reported in table B10.

Table B10: CurrentStatus.DataExtractionUnit metadata parameters description

| Metadata name: | CurrentStatus. DataExtractionUnit | Meas. Unit: | -------- | Data type: | Cluster |
| --- | --- | --- | --- | --- | --- |
| ID: | 008 | Size: | Variable |  |  |
| Desc.: | List of the CurrentStatus.DataExtractionUnit specifications according to SCOS Manager imposed specifications. |  |  |  |  |
|  | Name | Content |  | Data type | Meas. Unit |
| .0 | low_frequency | Current adopted low frequency of the receiver |  | Float | Hz |
| .1 | high_frequency | Current adopted high frequency of the receiver |  | Float | Hz |
| .2 | sampling_rate | Current chosen sampling rate for the A/D conversion |  | Float | Hz |
|  | Reserved (for future use) |  |  |  |  |

### B.3.9 CurrentStatus.EnvironmentalSensors

CurrentStatus.EnvironmentalSensors metadata gives information about the current environmental conditions in the location where the sensing device is placed.

Table B11: CurrentStatus.EnvironmentalSensors metadata parameters description

| Metadata category: | SensorDefinition. EnvironmentalSensors | Meas. Unit: |  | -------- |  | Data type: | Cluster |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ID: | 009 | Size: |  | Variable |  |  |  |
| Desc.: | List of the CurrentStatus.EnvironmentalSensors specifications according to location specifications. |  |  |  |  |  |  |
|  | Name | Content |  |  |  | Data type | Meas. Unit |
| .0 | Environment_type | Environment_type |  |  |  | Enumerator | ------ |
|  |  | “Indoor” “Outdoor” | Value: Value: |  | 0 1 |  |  |
| .2 | Temperature | Current temperature in sensor location. |  |  |  | Float | ° (degree) |
| .3 | Humidity | Value of humidity in sensor location |  |  |  | Float | % |
| .4 | Atmospheric Pressure | Value of atmospheric pressure in sensor location |  |  |  | Float | bar |
| .3 | Reserved (for future use) |  |  |  |  |  |  |

### B.3.10 CurrentStatus.ReferenceGeolocation

CurrentStatus.ReferenceGeolocation metadata gives geographical position information of the sensor and specifications are reported in Table B12.

Table B12: CurrentStatus.EnvironmentalSensors metadata parameters description

| Metadata name: | CurrentStatus. ReferenceGeolocation | Meas. Unit: | -------- | Data type: | Cluster |
| --- | --- | --- | --- | --- | --- |
| ID: | 010 | Size: | Variable |  |  |
| Desc.: | List of the CurrentStatus.ReferenceGeolocation specifications according to sensing device position. |  |  |  |  |
|  | Name | Content |  | Data type | Meas. Unit |
| .0 | Latitude | Current latitude in sensor position expressed with signed float (positive if North, negative if South) |  | Float | ° (degree) |
| .1 | Longitude | Current longitude in sensor position expressed with signed float (positive if east, negative if west) |  | Float | ° (degree) |
| .2 | Altitude | The height of the sensor position above mean sea level |  | Float | m |
| .3 | Reserved (for future use) |  |  |  |  |

### B.3.11 CurrentStatus.Certificate

This metadata gives information about certificate file of the considered sensor. The definition of each parameter and the related unit of measurements are reported in table B13.

Table B13. CurrentStatus.Certificate metadata parameters description

| Metadata name: | SDCert | Meas. Unit: | ----- | Data type: | Cluster |
| --- | --- | --- | --- | --- | --- |
| ID: | 011 | Size: | 3 |  |  |
| Desc.: | It gives information about calibration certificate file of the considered sensor. |  |  |  |  |
|  | Name | Content |  | Data type | Meas. Unit |
| .0 | Path | Calibration certificate file path. |  | String | ------ |
| .1 | Name | Calibration certificate file name. |  | String | ------ |
| .2 | Cert.Auth | Certification authority that have issued the calibration certificate. |  | String | ------ |

### B.3.12 MeasurementTask.Type

Measurement Task describes metadata related to the single measurement task type requested by the SCOS Manager to the sensor.

Table B14. MeasurementTask.Type metadata parameters description

| Metadata name: | MeasurementTask.Type | Meas. Unit: |  | -------- |  | Data type: | Enumerator |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ID: | 012 | Size: |  | 1 |  |  |  |
| Desc.: | List of the MeasurementTask.Type specifications according to SCOS Manager request. |  |  |  |  |  |  |
|  | Name | Content |  |  |  | Data type | Meas. Unit |
|  | Type | Type |  |  |  | Enumerator | ------ |
|  |  | TimeDomainDetection FrequencyDomainDetection SweptTunedMeasurement YFactorCalibration | Value Value Value Value |  | 0 1 2 3 |  |  |

### B.3.13 MeasurementTask.TimeDomainDetection

This metadata is transmitted when the MeasurementTask.Type is equal to 0. It describes settings adopted by the sensor to perform measurement in time domain. The definition of each setting and the related unit of measurements are reported in table B15.

Table B15 MeasurementTask.TimeDomainDetection metadata parameters description

| Metadata name: | MeasurementTask.TimeDomainDetection | Meas. Unit: |  | -------- |  | Data type: | Cluster |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ID: | 012 | Size: |  | Variable |  |  |  |
| Desc.: | List of the MeasurementTask.TimeDomainDetection specifications according to task specifications. |  |  |  |  |  |  |
|  | Name | Content |  |  |  | Data type | Meas. Unit |
| .0 | Detector | Detector |  |  |  | Enumerator | ----------- |
|  |  | Sample_Power Mean_Power Max_power Min_power Median_power M4s_power | Value Value Value Value Value Value |  | 0 1 2 3 4 5 |  |  |
| .1 | Number_of_samples | Samples to be integrated by the detector |  |  |  | Integer | ------------ |
| .2 | Measurement_units | Units |  |  |  | Enumerator |  |
|  |  | dBm volts watts | Value Value Value |  | 0 1 2 |  |  |
| .3 | Reference | Reference Point |  |  |  | Enumerator | ------------ |
|  |  | Receiver_input Antenna_output Out_of_isotropic_antenna | Value Value Value |  | 0 1 2 |  |  |
| .4 | Reserved (for future use) |  |  |  |  |  |  |

### B.3.14 MeasurementTask.FrequencyDomainDetection

This metadata is transmitted when the MeasurementTask.Type is equal to 1. It describes settings adopted by the sensor to perform measurement in frequency domain. The definition of each setting and the related unit of measurements are reported in table B16.

Table B16. MeasurementTask.FrequencyDomainDetection metadata parameters description

| Metadata name: | MeasurementTask.FrequencyDomainDetection | Meas. Unit: |  |  |  | -------- |  |  | Data type: | Cluster |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ID: | 014 | Size: |  |  |  | Variable |  |  |  |  |  |  |
| Desc.: | List of the MeasurementTask.FrequencyDomainDetection specifications according to task specifications. |  |  |  |  |  |  |  |  |  |  |  |
|  | Name | Content |  |  |  |  |  |  | Data type | Meas. Unit |  |  |
| .0 | Detector | Detector |  |  |  |  |  |  | Enumerator | ------------ |  |  |
|  |  | fft_sample_iq fft_sample_power fft_mean_power fft_max_power fft_min_power fft_median_power fft_m4s_power |  |  | Value Value Value Value Value Value Value |  |  | 0 1 2 3 4 5 6 |  |  |  |  |
| .1 | Number_of_ffts | Number of FFTs to be integrated over by detector. |  |  |  |  |  |  | Integer |  | ------------ |  |
| .2 | Measurement Units | Units |  |  |  |  |  |  | Enumerator |  | ------------- |  |
|  |  | dBm volts watts |  | Value Value Value |  |  | 0 1 2 |  |  |  |  |  |
| .3 | Reference | Reference Point |  |  |  |  |  |  | Enumerator |  | ------------- |  |
|  |  | Receiver_input Antenna_output Out_of_isotropic_antenna |  | Value Value Value |  |  | 0 1 2 |  |  |  |  |  |
| .4 | Number_of_samples_in_fft | Number of FFT samples (useful to compute Δf) |  |  |  |  |  |  | Integer |  | ------------- |  |
| .5 | Windowing_function | Window |  |  |  |  |  |  | Enumerator |  | ------------- |  |
|  |  | Blackman_harris Flattop Gaussian_a3.5 Gauss_Top Hamming Hanning Rectangular | Value Value Value Value Value Value Value |  | 0 1 2 3 4 5 6 |  |  |  |  |  |  |  |
| .6 | Equivalent_noise_bandwidth | Bandwidth of brickwall filter that has same integrated noise power as that of the actual filter. |  |  |  |  |  |  | Float |  | ------------- |  |
| .7 | Reserved (for future use) |  |  |  |  |  |  |  |  |  |  |  |

### B.3.15 MeasurementTask.SweptTunedMeasurement

This metadata is transmitted when the MeasurementTask.Type is equal to 2. It describes settings adopted by the sensor to perform swpt tuned measurement. The definition of each setting and the related unit of measurements are reported in table B17.

Table B17. MeasurementTask.SweptTunedMeasurement metadata parameters description

| Metadata name: | MeasurementTask.SweptTunedMeasurement | Meas. Unit: | -------- |  | Data type: | Cluster |
| --- | --- | --- | --- | --- | --- | --- |
| ID: | 015 | Size: | Variable |  |  |  |
| Desc.: | List of the MeasurementTask.SweptTunedMeasurement specifications according to task specifications. |  |  |  |  |  |
|  | Name | Content |  |  | Data type | Meas. Unit |
| .0 | Frequency_start | Frequency value for scan start. |  |  | Float | Hz |
| .1 | Frequency_stop | Frequency value for scan stop. |  |  | Float | Hz |
| .2 | Frequency_step | Frequency value used as scan step. |  |  | Float | Hz |
| .3 | Dwell_time | Integration time of detector at each frequency step |  |  | Float | s |
| .4 | Resolution_bandwidth | Narrowest bandwidth of the IF filters |  |  | Float | Hz |
| .5 | Video_bandwidth | Bandwidth of the video filter |  |  | Float | Hz |
| .6 | Measurement Units | Units |  |  | Enumerator | ------------ |
|  |  | dBm volts watts | Value Value Value | 0 1 2 |  |  |
| .7 | Reference | Reference Point |  |  | Enumerator | ------------- |
|  |  | Receiver_input Antenna_output Out_of_isotropic_antenna | Value Value Value | 0 1 2 |  |  |
| .8 | Reserved for future use |  |  |  |  |  |

### B.3.16 MeasurementTask.YFactorCalibration

This metadata is transmitted when the MeasurementTask.Type is equal to 3. It describes settings adopted by the sensor to perform time-domain y-factor calibrations. The definition of each setting and the related unit of measurements are reported in table B18.

Table B18. MeasurementTask.YFactorCalibration metadata parameters description

| Metadata name: | MeasurementTask.YFactorCalibration | Meas. Unit: |  | -------- |  | Data type: | Cluster |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ID: | 016 | Size: |  | Variable |  |  |  |
| Desc.: | List of the MeasurementTask.YFactorCalibration specifications according to task specifications. |  |  |  |  |  |  |
|  | Name | Content |  |  |  | Data type | Meas. Unit |
| .0 | last_time_performed | Date and time that calibration was performed. |  |  |  | datetime | ISO-8601 compliant |
| .1 | Frequencies | Frequencies that y-factor calibrations are performed. |  |  |  | Array of floats | Hz |
| .2 | excess_noise_ratios | Excess noise ratio of calibrated noise source at frequencies of y-factor calibration. |  |  |  | Array of floats | dB |
| .3 | receiver_setting_name | Receiver Setting Name |  |  |  | Enumerator | ----------- |
|  |  | Attenuation Input_Range | Value Value |  | 0 1 |  |  |
| .4 | receiver_setting_meas_units | Measurement Units |  |  |  | Enumerator | ------------ |
|  |  | dBm dB | Value Value |  | 0 1 |  |  |
| .5 | reference | Reference Point |  |  |  | Enumerator | ------------- |
|  |  | Receiver_input Antenna_output Out_of_isotropic_antenna | Value Value Value |  | 0 1 2 |  |  |
| .6 | calibrations | Receiver settings and corresponding gains and noise_figures arrays |  |  |  | Array of floats | 011.4.4 setting |
| .7 | Reserved (for future use) |  |  |  |  |  |  |