PvarDesignation15™/D5
DrafttxtTrialUsetxtGorRPorSTD for varTitlePAR

Sponsor

varCommittee

of the

IEEE varSociety

Approved varApprovedDate

IEEE-SA Standards Board

Copyright © varCRYear by The Institute of Electrical and Electronics Engineers, Inc.

Three Park Avenue

New York, New York 10016-5997, USA

All rights reserved.

This document is an unapproved draft of a proposed IEEE Standard. As such, this document is subject to change. USE AT YOUR OWN RISK! IEEE copyright statements SHALL NOT BE REMOVED from draft or approved IEEE standards, or modified in any way. Because this is an unapproved draft, this document must not be utilized for any conformance/compliance purposes. Permission is hereby granted for officers from each IEEE Standards Working Group or Committee to reproduce the draft document developed by that Working Group for purposes of international standardization consideration.  IEEE Standards Department must be informed of the submission for consideration prior to any reproduction for international standardization consideration (stds.ipr@ieee.org). Prior to adoption of this document, in whole or in part, by another standards development organization, permission must first be obtained from the IEEE Standards Department (stds.ipr@ieee.org). When requesting permission, IEEE Standards Department will require a copy of the standard development organization's document highlighting the use of IEEE content. Other entities seeking permission to reproduce this document, in whole or in part, must also obtain permission from the IEEE Standards Department.

IEEE Standards Department

445 Hoes Lane

Piscataway, NJ 08854, USA

Abstract: This standard specifies the architecture, abstraction layers, interfaces and metadata requirements for Spectrum Characterization and Occupancy Sensing (SCOS) system, and defines performance parameters, units and measures. This SCOS system comprises one or more semi-autonomous Spectrum Sensing Devices which scan electromagnetic spectrum, digitize it and perform processing, transmitting the resultant data with appropriate metadata to a central storage and processing system, according to rules, policies or instructions imposed on the Spectrum Sensing Devices by a management system.

Keywords: Radio spectrum sensing, spectrum monitoring, signal characterization, cognitive radio, IEEE 802.22.3, WRAN standards

Important Notices and Disclaimers Concerning IEEE Standards Documents

IEEE documents are made available for use subject to important notices and legal disclaimers. These notices and disclaimers, or a reference to this page, appear in all standards and may be found under the heading “Important Notices and Disclaimers Concerning IEEE Standards Documents.” They can also be obtained on request from IEEE or viewed at http://standards.ieee.org/IPR/disclaimers.html.

Notice and Disclaimer of Liability Concerning the Use of IEEE Standards Documents

IEEE Standards documents (standards, recommended practices, and guides), both full-use and trial-use, are developed within IEEE Societies and the Standards Coordinating Committees of the IEEE Standards Association (“IEEE-SA”) Standards Board. IEEE (“the Institute”) develops its standards through a consensus development process, approved by the American National Standards Institute (“ANSI”), which brings together volunteers representing varied viewpoints and interests to achieve the final product. IEEE Standards are documents developed through scientific, academic, and industry-based technical working groups. Volunteers in IEEE working groups are not necessarily members of the Institute and participate without compensation from IEEE. While IEEE administers the process and establishes rules to promote fairness in the consensus development process, IEEE does not independently evaluate, test, or verify the accuracy of any of the information or the soundness of any judgments contained in its standards.

IEEE Standards do not guarantee or ensure safety, security, health, or environmental protection, or ensure against interference with or from other devices or networks. Implementers and users of IEEE Standards documents are responsible for determining and complying with all appropriate safety, security, environmental, health, and interference protection practices and all applicable laws and regulations.

IEEE does not warrant or represent the accuracy or content of the material contained in its standards, and expressly disclaims all warranties (express, implied and statutory) not included in this or any other document relating to the standard, including, but not limited to, the warranties of: merchantability; fitness for a particular purpose; non-infringement; and quality, accuracy, effectiveness, currency, or completeness of material. In addition, IEEE disclaims any and all conditions relating to: results; and workmanlike effort. IEEE standards documents are supplied “AS IS” and “WITH ALL FAULTS.”

Use of an IEEE standard is wholly voluntary. The existence of an IEEE standard does not imply that there are no other ways to produce, test, measure, purchase, market, or provide other goods and services related to the scope of the IEEE standard. Furthermore, the viewpoint expressed at the time a standard is approved and issued is subject to change brought about through developments in the state of the art and comments received from users of the standard.

In publishing and making its standards available, IEEE is not suggesting or rendering professional or other services for, or on behalf of, any person or entity nor is IEEE undertaking to perform any duty owed by any other person or entity to another. Any person utilizing any IEEE Standards document, should rely upon his or her own independent judgment in the exercise of reasonable care in any given circumstances or, as appropriate, seek the advice of a competent professional in determining the appropriateness of a given IEEE standard.

IN NO EVENT SHALL IEEE BE LIABLE FOR ANY DIRECT, INDIRECT, INCIDENTAL, SPECIAL, EXEMPLARY, OR CONSEQUENTIAL DAMAGES (INCLUDING, BUT NOT LIMITED TO: PROCUREMENT OF SUBSTITUTE GOODS OR SERVICES; LOSS OF USE, DATA, OR PROFITS; OR BUSINESS INTERRUPTION) HOWEVER CAUSED AND ON ANY THEORY OF LIABILITY, WHETHER IN CONTRACT, STRICT LIABILITY, OR TORT (INCLUDING NEGLIGENCE OR OTHERWISE) ARISING IN ANY WAY OUT OF THE PUBLICATION, USE OF, OR RELIANCE UPON ANY STANDARD, EVEN IF ADVISED OF THE POSSIBILITY OF SUCH DAMAGE AND REGARDLESS OF WHETHER SUCH DAMAGE WAS FORESEEABLE.

Translations

The IEEE consensus development process involves the review of documents in English only. In the event that an IEEE standard is translated, only the English version published by IEEE should be considered the approved IEEE standard.

Official statements

A statement, written or oral, that is not processed in accordance with the IEEE-SA Standards Board Operations Manual shall not be considered or inferred to be the official position of IEEE or any of its committees and shall not be considered to be, or be relied upon as, a formal position of IEEE. At lectures, symposia, seminars, or educational courses, an individual presenting information on IEEE standards shall make it clear that his or her views should be considered the personal views of that individual rather than the formal position of IEEE.

Comments on standards

Comments for revision of IEEE Standards documents are welcome from any interested party, regardless of membership affiliation with IEEE. However, IEEE does not provide consulting information or advice pertaining to IEEE Standards documents. Suggestions for changes in documents should be in the form of a proposed change of text, together with appropriate supporting comments. Since IEEE standards represent a consensus of concerned interests, it is important that any responses to comments and questions also receive the concurrence of a balance of interests. For this reason, IEEE and the members of its societies and Standards Coordinating Committees are not able to provide an instant response to comments or questions except in those cases where the matter has previously been addressed. For the same reason, IEEE does not respond to interpretation requests. Any person who would like to participate in revisions to an IEEE standard is welcome to join the relevant IEEE working group.

Comments on standards should be submitted to the following address:

Secretary, IEEE-SA Standards Board

445 Hoes Lane

Piscataway, NJ 08854 USA

Laws and regulations

Users of IEEE Standards documents should consult all applicable laws and regulations. Compliance with the provisions of any IEEE Standards document does not imply compliance to any applicable regulatory requirements.  Implementers of the standard are responsible for observing or referring to the applicable regulatory requirements. IEEE does not, by the publication of its standards, intend to urge action that is not in compliance with applicable laws, and these documents may not be construed as doing so.

Copyrights

IEEE draft and approved standards are copyrighted by IEEE under U.S. and international copyright laws. They are made available by IEEE and are adopted for a wide variety of both public and private uses.  These include both use, by reference, in laws and regulations, and use in private self-regulation, standardization, and the promotion of engineering practices and methods. By making these documents available for use and adoption by public authorities and private users, IEEE does not waive any rights in copyright to the documents.

Photocopies

Subject to payment of the appropriate fee, IEEE will grant users a limited, non-exclusive license to photocopy portions of any individual standard for company or organizational internal use or individual, non-commercial use only. To arrange for payment of licensing fees, please contact Copyright Clearance Center, Customer Service, 222 Rosewood Drive, Danvers, MA 01923 USA; +1 978 750 8400. Permission to photocopy portions of any individual standard for educational classroom use can also be obtained through the Copyright Clearance Center.

Updating of IEEE Standards documents

Users of IEEE Standards documents should be aware that these documents may be superseded at any time by the issuance of new editions or may be amended from time to time through the issuance of amendments, corrigenda, or errata. An official IEEE document at any point in time consists of the current edition of the document together with any amendments, corrigenda, or errata then in effect.

Every IEEE standard is subjected to review at least every ten years. When a document is more than ten years old and has not undergone a revision process, it is reasonable to conclude that its contents, although still of some value, do not wholly reflect the present state of the art. Users are cautioned to check to determine that they have the latest edition of any IEEE standard.

In order to determine whether a given document is the current edition and whether it has been amended through the issuance of amendments, corrigenda, or errata, visit IEEE Xplore at http://ieeexplore.ieee.org/ or contact IEEE at the address listed previously. For more information about the IEEE-SA or IEEE’s standards development process, visit the IEEE-SA Website at http://standards.ieee.org.

Errata

Errata, if any, for all IEEE standards can be accessed on the IEEE-SA Website at the following URL: http://standards.ieee.org/findstds/errata/index.html. Users are encouraged to check this URL for errata periodically.

Patents

Attention is called to the possibility that implementation of this standard may require use of subject matter covered by patent rights. By publication of this standard, no position is taken by the IEEE with respect to the existence or validity of any patent rights in connection therewith. If a patent holder or patent applicant has filed a statement of assurance via an Accepted Letter of Assurance, then the statement is listed on the IEEE-SA Website at http://standards.ieee.org/about/sasb/patcom/patents.html. Letters of Assurance may indicate whether the Submitter is willing or unwilling to grant licenses under patent rights without compensation or under reasonable rates, with reasonable terms and conditions that are demonstrably free of any unfair discrimination to applicants desiring to obtain such licenses.

Essential Patent Claims may exist for which a Letter of Assurance has not been received. The IEEE is not responsible for identifying Essential Patent Claims for which a license may be required, for conducting inquiries into the legal validity or scope of Patents Claims, or determining whether any licensing terms or conditions provided in connection with submission of a Letter of Assurance, if any, or in any licensing agreements are reasonable or non-discriminatory. Users of this standard are expressly advised that determination of the validity of any patent rights, and the risk of infringement of such rights, is entirely their own responsibility. Further information may be obtained from the IEEE Standards Association.

Participants

At the time this drafttxtTrialUsetxtGorRPorSTD was completed, the varWorkingGroup Working Group had the following membership:

varWkGrpChair, Chair

varWkGrpViceChair, Vice Chair

Participant1

Participant2

Participant3

Participant4

Participant5

Participant6

Participant7

Participant8

Participant9

The following members of the <individual/entity> balloting committee voted on thistxtTrialUsetxtGorRPorSTD. Balloters may have voted for approval, disapproval, or abstention.

[To be supplied by IEEE]

Balloter1

Balloter2

Balloter3

Balloter4

Balloter5

Balloter6

Balloter7

Balloter8

Balloter9

When the IEEE-SA Standards Board approved thistxtTrialUsetxtGorRPorSTD on varApprovedDate, it had the following membership:

[To be supplied by IEEE]

<Name>, Chair

<Name>, Vice Chair

<Name>, Past Chair

Konstantinos Karachalios, Secretary

SBMember1

SBMember2

SBMember3

SBMember4

SBMember5

SBMember6

SBMember7

SBMember8

SBMember9

*Member Emeritus

Introduction

This introduction is not part of PvarDesignation/DvarDraftNumber, DrafttxtTrialUsetxtGorRPorSTD for varTitlePAR.

<Select this text and type or paste introduction text>

Contents

<After draft body is complete, select this text and click Insert Special->Add (Table of) Contents>

DrafttxtTrialUsetxtGorRPorSTD for varTitlePAR

- Overview

  - Scope

The scope of this document is to define a standard for Spectrum Characterization and Occupancy Sensing (SCOS). It establishes a high-level architecture, defines functional entities and their interfaces, specifies command & control messages and parameters, and defines metadata describing the nature and configuration of the sensing system and the data it gathers. It also describes operating characteristics and behaviors of the components of the SCOS system, with reference implementations for common use cases. Further, it defines measurement parameters in such a way that data gathered by an 802.22.3 compliant system can be consistently and meaningfully interpreted by users.

The focus of this standard is on management and control of a sensor network, the ability for a variety of users to interrogate system capabilities and task scanning functions, the definition of a simple reference implementation for the sensing devices, and for the sensing data to be transmitted to an end-point.

The SCOS architecture is intended to provide incentive for the evolution and implementation of the following three areas of specialization: sensor technology, data acquisition/distribution, and data analysis.

  - Purpose

The purpose of the Spectrum Characterization and Occupancy Sensing (SCOS) system is to characterize and assess the occupancy of spectrum resource towards supporting it’s more efficient and effective use. The intent of the SCOS is to create a high-level architecture to support different spectrum sensing technologies and deployments, to enable specialization and provide incentive, to promote broad adoption of sensing technologies and subsequent economies of scale, and to ultimately achieve broader availability and usage of sensing information from different sources. This will enable clients to acquire and use spectrum sensing information from a multiplicity of predefined independent systems to serve their goals.

Various national regulators and government authorities are developing regulatory and policy frameworks to allow cooperative spectrum sharing approaches in order to optimize spectrum utilization. There is emphasis on greater spectrum efficiencies, spectrum sharing and spectrum utilization, which requires not only database-driven configuration of the radios, but systems that can provide spectrum occupancy at a particular location and at a particular time.

  - Application

Greater efficiencies in spectrum utilization through spectrum sharing and optimized resource allocation, require not only coordinated use by radio systems, but also better information on spectrum occupancy at a particular location and time.

The Spectrum Characterization and Occupancy Sensing (SCOS) System has many applications which include:

(1)	Policy and Planning

(2)	Radio Planning, Management and Engineering

(3)	Regulatory Enforcement where systems can Detect (RF incursion), Locate (source), Classify (by type and severity), Resolve/Remediate

(4)	Research and Technology Development

- Normative references

The following referenced documents are indispensable for the application of this document (i.e., they must be understood and used, so each referenced document is cited in text and its relationship to this document is explained). For dated references, only the edition cited applies. For undated references, the latest edition of the referenced document (including any amendments or corrigenda) applies.

[1] NIST, “Standards for Security Categorization of Federal Information and Information Systems,” FIPS Pub 199, February 2004.

[2] NIST, “Security and Privacy Controls for Information Systems and Organizations,” NIST Special Publication, 800-53, Revision 5, August 2017.

[3] RFC-3339, "Date and Time on the Internet: Timestamps", Klyne, Newman, July 2002.

[4] IETF RFC 2616, “Hypertext Transfer Protocol – HTTP/1.1, June 1999.

[5] IETF RFC 8259, “The JavaScript Object Notation (JSON) Data Interchange Format, December 2017.

- Definitions

For the purposes of this document, the following terms and definitions apply. The IEEE Standards Dictionary Online should be consulted for terms not defined in this clause.

Action: Sensor function that the sensor owner implements and exposes to the API. Actions give the sensor owner control over what the sensor can be tasked to do, e.g,, instrument control, data acquisition, data processing.

Admin: Privileged Sensor/Manager account type with full permissions.

Acquisition: The combination of data and metadata created by an action (though an action does not have to create an acquisition).

Antenna: Sensor hardware component that converts environmental electromagnetic fields into a voltage.

API: Application Programming Interface

Capability: Available actions, installation specifications (e.g., mobile or stationary), and hardware specification (e.g., frequency range of receiver).

Client: Manager account with permissions to query SCOS capabilities, request information on sensed data, and/or request scheduled sensing actions.

GUI: Graphical User Interface

Host controller: Sensor hardware component that provides web services; controls signals/messages to antenna, preselector, and/or receiver; and processes/packages data.

HTTP: Hypertext Transfer Protocol

IP: Internet Protocol

Manager: SCOS entity with hardware resources and software that allows clients to interact with the SCOS platform via web-based GUI and (optional) data service.

Preselector: Sensor hardware component that can provide preselection filtering, improved sensitivity via low noise amplification, calibration signal sources, etc.

Receiver: Sensor hardware component that captures discrete raw data (e.g., baseband representation of the signal) and can apply digital signal processing algorithms to achieve a desired metric.

REST: Representational State Transfer

Schedule: The collection of all schedule entries (active and inactive) on the sensor.

Scheduler: A Sensor process responsible for executing the schedule.

Schedule Entry: Input to the sensor schedule that describes action, start and stop times, interval, and priority.

SCOS: Spectrum Characterization and Occupancy Sensing

SCOS Owner: Manager account type which can approve actions to be performed by the SCOS platform.

Sensor: SCOS entity with hardware resources and software that allow users (e.g., Manager) to interact via web-based API and (optional) data service.

Sensor Operator: Manager account type which can register sensors via authentication key exchange.

SDR: Software Defined Radio

SLA: Service Level Agreement

Task: A representation of an action to be run at a specific time.

Task Result: A record of the outcome of a task.

User: Unprivileged Sensor account type which can create schedule entries and view, modify, and delete things they own, but which cannot modify or delete things they don't own.

- System Concept

Figure 1 illustrates a SCOS context diagram. SCOS is a service provider and Clients are end users. External Clients can discover SCOS capabilities, request data, and request actions to be performed by sensor(s).

Figure 1. SCOS context diagram

  - Entities

The two entities that comprise the SCOS platform are the Manager (server application) and one or more Sensor(s) operating in the field. Together, Manager and Sensor(s) comprise the SCOS Platform.

- Manager: SCOS entity that allows clients (whether individuals or other systems) to interact with the SCOS Platform via web-based Graphical User Interface (GUI) and (optional) data service. It exposes the capabilities of the SCOS platform to users, manages tasks requests by users, and ultimately controls the registered sensors.

- Sensor: SCOS entity that allow users (whether individuals or other applications, e.g., Manager) to interact via web-based API and (optional) data services. The API allows users to discover capabilities, schedule actions, and download data. Sensors package data with required metadata in a standardized data format.

  - Interactions

Figure 2 describes the basic interactions between Client, Manager, and Sensor. The categories of interactions are sensor controller service, capabilities, schedule, acquisition, and data service.

Figure 2. Interactions between Client, Manager, and Sensor

  - General Requirements

The primary goals of the SCOS architecture are to achieve broad sensing and data aggregation. To meet these goals, SCOS developers are required to meet general requirements categorized and described in the following subsections.

    - Connectivity, Control, and Access

The following requirements are related connectivity, sensor control, and data access.

- Connectivity and access: Sensors are to provide easy-to-used control via IP network. Manager is to be accessible to authorized users via the Internet.

- Discoverable capabilities: Entity capabilities and resources are discoverable, so they can be fully utilized by end user applications.

- Data access: SCOS entities are to make data available to authorized users.

- Data standardization: Compliance with a common metadata/data specification is required to allow for collaborative research, large-scale analytics, and sharing of sophisticated tools and methodologies. Data compliance is the subject of Section 8.

    - Security

The following requirements are related to system and information confidentiality, integrity, and availability.

- Security categorization: Information and systems are to be rated at the appropriate level of information security according to range of risk levels [1].

- Service Level Agreement (SLA): A contract between SCOS (service provider) and Client (end user) is required to define the level of service expected from the service provider.

- Security controls: Safeguards or countermeasures are required to avoid, detect, counteract, or minimize security risks to physical property, information, computer systems, or other assets [2]. Examples of relevant security controls include:

  - End user authentication to prevent unauthorized users from accessing internal entity functionality

  - Provisioned and configured Operating Systems (OSs)

  - System administration to manage users and implement data access controls

In general, security requirements are implementation specific.

    - Design Flexibility

The following requirements support the evolution of SCOS technology and broad deployments.

- REST API: Entities are required to expose Representational State Transfer (REST) APIs as an easy-to-use a means for end users to interact with the application over an Internet Protocol (IP) network. REST does not bind the service provider or end user applications to specific implementations.

- Extensible technologies, algorithms, and metrics: SCOS infrastructure enables evolution of centralized and edge processing, sensor technologies, and metrics. SCOS entities and associated interfaces add functionality independent of end user applications.

- Hardware, software, and OS agnostic: Sensor and Manager implementations are not bound to specific hardware, software, or operating system to allow for cost-performance tradeoffs to be considered during design process.

Additional security requirements specific to Sensor and Manager are the subject of Sections Error: Reference source not found and Error: Reference source not found, respectively.

- Sensor

This section provides models and requirements associated with the Sensor hardware and software.

  - Hardware

Figure 3 provides a block diagram of the basic sensor hardware model. Sensors are not required to have each component shown in the block diagram. There can be more sophisticated hardware models, e.g., with multiple antennas and/or multiple receivers connected to a single host controller. Integration can also be achieved, e.g., a mobile phone is an example of all components integrated into a single unit. Sensor definition, measurement type, and algorithm metadata requirements are defined in Section 7.

Figure 3. Sensor hardware model

- Antenna: Optional sensor hardware components that convert environmental electromagnetic fields into a voltage. An RF cable connects the antenna with the next hardware component.

- Preselector: Optional sensor hardware components that can provide preselection filtering, improved sensitivity via low noise amplification, calibration signal sources, etc.

- Receiver: Required sensor hardware components that capture discrete raw data (e.g., baseband representation of the signal) and can apply digital signal processing algorithms to the raw data to achieve a desired metric.

- Host Controller: Optional sensor hardware components that can provide control signals and messages to the preselector and receiver. Host controllers often provide data processing, data packaging, and web-accessible services.

  - Software

Sensor software provides a platform for operating a sensor, such as a software-defined radio (SDR), over a network. The goal is to provide a robust, flexible, and secure starting point for remote spectrum monitoring.

    - Functional Requirements

The following are functional requirements of the Sensor software:

- Provide status: Offer standard HTTP status codes to indicate the success or failure of API calls, storage available, time since last calibration, time since last restart, etc.

- Advertise capabilities: Capabilities are available actions. Actions are a function that the sensor owner implements and exposes to the API. Actions should be used to constrain Sensor tasks to the valid operational ranges of hardware components (e.g., frequency range of antenna, preselector, and receiver)

- Provide onboard scheduler: Task scheduling is to be provided on the sensor. Tasks are to be executed according to start/stop times, interval, and/or priority.

- Responsive schedule entries: Sensors are to be responsive to requested schedule entries by authorized users.

- Provide task results: Task results are to be recorded for each task after the action function returns, and includes metadata such as when the task started, when it finished, its duration, the result (success or failure), and a freeform detail string.

- Categorize security level of acquisitions: Acquisitions are to be tagged with the appropriate security categorization (i.e., low, medium, or high) to enable responsible data management.

- Access acquisitions: Acquisitions and metadata are accessible directly though API, while data is retrievable in an easy-to-use archive format with its associated metadata.

    - Application Programming Interface (API)

The following subsections describe the required Sensor API endpoints to provide the core functionality across four key areas: sensor status, sensor capabilities, sensor scheduling, and obtaining the sensor results.  . Note: {version} in the path indicates the API version. The API uses standard HTTP status codes to indicate the success or failure of the API call. Responses and request bodies use the JSON objects specified below.

      - Status Endpoint

The status interface allows users to request status from the sensor and receive a status object in response.

      - Status Objects

The status request object below is used to perform a status request. Data clients or sensor managers may perform a status request for a sensor.

Table 1: Status Request Object

| Property | R/O/C | Type | Unit | Description | Value |
| --- | --- | --- | --- | --- | --- |
| sm_id | O | String | NA | The unique ID of the sensor manager making the request. |  |
| sd_id | R | String | NA | The unique ID of the sensor. |  |
| client_id | O | String | NA | The unique ID of the data client making the request. |  |
| message_type | R | String | NA | The type of the request (status, capabilities…) | status_request |

The status response object below is returned after a status request to describe the sensor status.

Table 2: Status Response Object

| Property | Required | Type | Unit | Description |  |
| --- | --- | --- | --- | --- | --- |
| sm_id | O | String | NA | The unique ID of the sensor manager making the request. |  |
| sd_id | R | String | NA | The unique ID of the sensor. |  |
| client_id | O | String | NA | The unique ID of the data client making the request. |  |
| scheduler | O | string | N/A | The scheduler status (idle, running, dead) |  |
| location | O | Location | N/A | Sensor location information |  |
| system_time | R | Datetime | ISO-8601 | The current system time on the sensor |  |
| last_calibration_time | O | Datetime | ISO-8601 | The last date/time that the sensor was calibrated. |  |
| message_type | R | String | NA | The message type | status |

Table 3: Location Object

| Property | Required | Type | Unit | Description |
| --- | --- | --- | --- | --- |
| gps | O | boolean | N/A | Indicates whether the location information was pulled from GPS |
| modified | O | datetime | ISO-8601 | Time of the last location update |
| description | O | string | N/A | A textual description of the sensors location |
| latitude | true | double | decimal degrees | The sensor’s latitude |
| longitude | true | double | decimal degrees | The sensor’s longitude |

      - Capabilities Endpoint

The capabilities interface allows entities to request the capabilities of a sensing object.

      - Capabilities Objects

The capabilities request object is used to request the sensing capabilities of a sensor.

Table 4: Capabilities Request Object

| Property | R/O/C | Type | Unit | Description | Value |
| --- | --- | --- | --- | --- | --- |
| sm_id | O | String | NA | The unique ID of the sensor manager making the request. |  |
| sd_id | R | String | NA | The unique ID of the sensor. |  |
| client_id | O | String | NA | The unique ID of the data client making the request. |  |
| message_type | R | String | NA | The message type. | capabilities_request |

The capabilities response object is used to describe the capabilities of a sensor in response to a capabilities request.

Table 5: Capabilities Response Object

| Property | R/O/C | Type | Unit | Description | Value |
| --- | --- | --- | --- | --- | --- |
| sm_id | O | String | NA | The unique ID of the sensor manager making the request. |  |
| sd_id | R | String | NA | The unique ID of the sensor. |  |
| client_id | O | String | NA | The unique ID of the data client making the request. |  |
| sensor | R | Sensor | N/A | A description of the sensor. (Note: this utilizes that same Sensor JSON object described in Error: Reference source not found |  |
| actions | R | Action[] | N/A | An array of Action objects describing each Action the sensor can perform |  |
| message_type | R | String | NA | The message type | capabilities |

The Sensor object is utilized within the capabilities response object.

Table 6:Sensor Object

| Name | Required | Type | Unit | Description |
| --- | --- | --- | --- | --- |
| sensor_spec | R | HardwareSpec | N/A | Metadata describing the sensor hardware |
| antenna | R | Antenna | N/A | Metadata describing the sensor’s antenna |
| preselector | O | Preselector | N/A | Metadata describing the preselector. |
| signal_analyzer | R | SignalAnalyzer | N/A | Metadata describing the signal analyzer. |
| computer_spec | O | string | N/A | Description of host computer. E.g. Make, model, and configuration. |
| mobile | O | boolean | N/A | Indicates if the sensor is mobile |
| latitude | O | double | decimal degrees | The latitude of the sensor. |
| longitude | O | double | decimal degrees | The longitude of the sensor. |
| altitude | O | double | meters | Height above mean sea level. |
| speed | O | double | m/s | Speed of the sensor. |
| bearing | O | double | degrees | Direction (angle relative to true North). |
| gps_nmea | O | string | NMEA | NMEA message from the gps receiver |

Antenna Object

The Antenna object is utilized within the Sensor object and contains the following name/value pairs:

| Name | R/O/C | Type | Unit | Description |
| --- | --- | --- | --- | --- |
| hardware_spec | R | HardwareSpec | N/A | Metadata describing the hardware. |
| type | O | string | N/A | Antenna type. E.g. "dipole", "biconical", "monopole", "conical monopole". |
| low_frequency | O | double | Hz | Low frequency of operational range. |
| high_frequency | O | double | Hz | High frequency of operational range. |
| polarization | O | double | string | Antenna polarization. E.g. "vertical", "horizontal", "slant-45", "left-hand circular", "right-hand circular". |
| cross_polar_discrimination | O | double | N/A | Cross-polar discrimination. |
| gain | O | double | dBi | Antenna gain in direction of maximum radiation or reception. |
| horizontal_gain_pattern | O | double[] | dBi | Antenna gain pattern in horizontal plane from 0 to 359 degrees in 1 degree steps. |
| vertical_gain_pattern | O | double[] | dBi | Antenna gain pattern in vertical plane from -90 to +90 degrees in 1 degree steps. |
| horizontal_beam_width | O | double | degrees | Horizontal 3-dB beamwidth. |
| vertical_beam_width | O | double | degrees | Vertical 3-dB beamwidth. |
| voltage_standing_wave_ratio | O | double | volts | Voltage standing wave ratio. |
| cable_loss | O | double | dB | Cable loss for cable connecting antenna and preselector. |
| steerable | O | boolean | N/A | Defines if the antenna is steerable or not. |
| azimuth_angle | O | double | degrees | Angle of main beam in azimuthal plane from North. |
| elevation_angle | O | double | degrees | Angle of main beam in elevation plane from horizontal. |

    - HardwareSpec Object

The HardwareSpec object has the following properties:

| Name | Required | Type | Unit | Description |
| --- | --- | --- | --- | --- |
| id | R | string | N/A | Unique id of hardware. E.g., serial number. |
| model | O | string | N/A | Hardware make and model. |
| version | O | string | N/A | Hardware version. |
| description | O | string | N/A | Description of the hardware. |
| supplemental_information | O | string | N/A | Information about hardware, e.g., url to on-line data sheets. |

    - Preselector Object

| Name | Required | Type | Unit | Description |
| --- | --- | --- | --- | --- |
| preselector_spec | O | HardwareSpec | N/A | Metadata to describe/specify the preselector. |
| cal_source_spec | O | HardwareSpec | N/A | Metadata to describe/specify the preselector calibration source. |
| lna_spec | O | HardwareSpec | N/A | Metadata to describe/specify the preselector low noise amplifier. |
| filter_spec | O | HardwareSpec[] | N/A | Metadata to describe/specify the preselector RF bandpass filters. |
| rf_paths | O | RFPath[] | N/A | Metadata that describes preselector RF paths. |

    - RFPath Object

The RPatph object may appear in the Preselector object and contains the following properties:

Table 7:RFPath Object

| Name | Required | Type | Unit | Description |
| --- | --- | --- | --- | --- |
| low_frequency_passband_filter | O | double | Hz | Low frequency of filter 1-dB passband. |
| high_frequency_passband_filter | O | double | Hz | High frequency of filter 1-dB passband. |
| low_frequency_stopband_filter | O | double | Hz | Low frequency of filter 60-dB stopband. |
| high_frequency_stopband_filter | O | double | Hz | High frequency of filter 60-dB stopband. |
| gain_lna | O | double | dB | Gain of low noise amplifier. |
| noise_figure_lna | O | double | dB | Noise figure of low noise amplifier. |
| type_cal_source | O | string | N/A | E.g., "calibrated noise source". |

Table 8:SignalAnalyzer Object

| Name | Required |  | Type | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| sigan_spec | O | HardwareSpec |  | N/A | Metadata to describe/specify the signal analyzer. |
| low_frequency | O |  | double | Hz | Low frequency of operational range of the signal analyzer. |
| high_frequency | O |  | double | Hz | High frequency of operational range of the signal analyzer. |
| noise_figure | O |  | double | dB | Noise figure of the signal analyzer. |
| max_power | O |  | double | dBm | Maximum input power of the signal analyzer. |
| a2d_bits | O |  | int | bits | Number of bits in A/D converter. |

Table 9: Action Object

| Property | Required | Type | Unit | Description |
| --- | --- | --- | --- | --- |
| name | R | string | N/A | The name of the action |
| summary | R | string | N/A | A short description of the action |
| description | O | string | N/A | A detailed description of the action |

      - Schedule Interface

The schedule interface allows entities to view, create, and modify sensor schedules. Schedule entry requests are performed with the ScheduleEntryRequest object.

      - Status Objects

Table 10: ScheduleEntryRequest

| Property | Required | Type | Unit | Description |
| --- | --- | --- | --- | --- |
| sm_id | O | String | NA | The unique ID of the sensor manager making the request. |
| sd_ids | R | string[] | NA | The unique ID of the sensor. |
| client_id | O | String | NA | The unique ID of the data client making the request. |
| name | true | string | N/A | The unique name/identifier for the schedule |
| action | true | string | N/A | The name of the action that will be executed |
| start | O | datetime | ISO-8601 | Requested time to schedule the first task. If unspecified, the task will start as soon as possible |
| stop | O | datetime | ISO-8601 | Requested time to end execution of tasks under this schedule. |
| relative_stop | O* | integer | seconds | Seconds after start when the task will end |
| interval | O | integer | seconds | Interval between tasks. If unspecified, the task will run once and then become inactive |
| is_active | true | boolean | N/A | Indicates if tasks will continue to be executed for the schedule |
| is_private | true | Boolean | N/A | True if the entry and data are only visible to admins |
| priority | O | integer | N/A | Priority of the entry. Lower numbers indicate higher priority |
| callback_url | O | string | N/A | The URL to call after the action executes |
| validate_only | O | Boolean | N/A | If true the input will only be validated and no ScheduleEntry will be created |

Table 11: ScheduleEntry Object

| Property | Required | Type | Unit | Description |
| --- | --- | --- | --- | --- |
| sm_id | O | String | NA | The unique ID of the sensor manager making the request. |
| sd_id | R | String | NA | The unique ID of the sensor. |
| client_id | O | String | NA | The unique ID of the data client making the request. |
| id | C | string | N/A | The id for the ScheduleEntry (URL in NTIA implementation) |
| name | R | string | N/A | The unique name for the ScheduleEntry |
| action | R | string | N/A | The name of the action that will be executed |
| start | C | datetime | ISO-8601 | Requested time to schedule the first task. If unspecified, the task will execute as soon as possible |
| stop | C | datetime | ISO-8601 | Requested time stop scheduling tasks |
| interval | O | integer | seconds | Number of seconds between tasks. If unspecified, the task will run exactly once and the schedule will become inactive |
| is_active | C | boolean | N/A | Indicates if tasks will continue to be executed for the schedule |
| validate_only | O |  |  |  |
| is_private | O | boolean | N/A | Indicates whether the entry, and resulting data, are only visible to admins |
| priority | O | integer | N/A | Priority of the entry. Lower numbers indicate higher priority |
| callback_url | O | string | N/A | The URL to call after the action executes |
| next_task_time | O | datetime | ISO-8601 | The time at which the action will execute next under this schedule |
| next_task_id | O | uint | N/A | The id for the next task that will be executed under this schedule |
| created | true | datetime | ISO-8601 | The datetime stamp when the schedule was created |
| modified | true | datetime | ISO-8601 | The datetime stamp when the schedule was last modified |
| owner | O | string | N/A | A URL to the details of the user that created the schedule |
| task_results_id | O | string | N/A | The id to the results from the schedule |

Table 12: ScheduleOverviewRequest Object

| Property | Required | Type | Unit | Description | Value |
| --- | --- | --- | --- | --- | --- |
| sm_id | O | String | NA | The unique ID of the sensor manager making the request. |  |
| sd_id | R | String | NA | The unique ID of the sensor. |  |
| client_id | O | String | NA | The unique ID of the data client making the request. |  |
| offset | O | uint | N/A | The schedule index at which to start the list of results |  |
| limit | O | uint | N/A | The number of schedules |  |
| message_type |  |  |  |  | ScheduleOverviewRequest |

Table 13: ScheduleOverview Object

| Property | Required | Type | Unit | Description |
| --- | --- | --- | --- | --- |
| count | R | uint | N/A | The number of schedules |
| next | R | string | N/A | A URL to the next set of ScheduleEntry (facilitates paging) |
| previous | R | string | N/A | A URL to the previous set of ScheduleEntry objects |
| results | true | ScheduleEntry[] | N/A | The current set of schedules as specified by the limit and offset parameters |

Table 14: ScheduleError Object

| Property | Required | Type | Unit | Description |
| --- | --- | --- | --- | --- |
| sm_id | O | String | NA | The unique ID of the sensor manager making the request. |
| sd_id | R | String | NA | The unique ID of the sensor. |
| client_id | O | String | NA | The unique ID of the data client making the request. |
| detail | R | string | N/A | A message explaining the error. |
| protected_objects | R | string[] | N/A | An array of URLs to the acquisitions associated with the ScheduleEntry |

      - Tasks Interface

The tasks interface allows entities to request information on upcoming and completed tasks.

      - Tasks Objects

Table 15: Completed Tasks Request Object

| Property | Required | Type | Unit | Description | Value |
| --- | --- | --- | --- | --- | --- |
| sm_id | C | String | NA | The unique ID of the sensor manager making the request. |  |
| sd_id | R | String | NA | The unique ID of the sensor. |  |
| client_id | C | String | NA | The unique ID of the data client making the request. |  |
| schedule_name | O | string | N/A | The unique id of the schedule |  |
| task_id | O | uint | N/A | The id of the task in the schedule |  |
| offset | O | uint | N/A | The index at which to start the list of results |  |
| limit | O | uint | N/A | The maximum number or results requested. |  |
| message_type | R | string | N/A | The message type. | delete_result, archive, schedule_results, |

Table 16: TasksCompletedOverview Response Object

| Property | Required | Type | Unit | Description |
| --- | --- | --- | --- | --- |
| sm_id | C | String | NA | The unique ID of the sensor manager making the request. |
| sd_id | R | String | NA | The unique ID of the sensor. |
| client_id | C | String | NA | The unique ID of the data client making the request. |
| count | true | uint | N/A | The number schedule results |
| next | true | string | N/A | The URL of the next set of schedule results |
| previous | true | string | N/A | The URL of the previous set of schedule results |
| results | true | TaskResultOverview[] | N/A | TaskResultOverview for each schedule entry dictated by the limit and offset parameters |

Table 17: TaskResultOverview  Object

| Property | Required | Type | Unit | Description |
| --- | --- | --- | --- | --- |
| task_results_available | true | uint | N/A | The number of results available for a particular schedule |
| schedule_entry | true | string | N/A | The schedule name (NTIA uses URL) |
| archive | R | string | N/A | The URL of the archive |
| task_results | R | String | N/A | The URL to the tasks completed for the schedule entry. |

Table 18: ScheduleTasksCompleted Response Object

| Property | Required | Type | Unit | Description |
| --- | --- | --- | --- | --- |
| sm_id | C | String | NA | The unique ID of the sensor manager making the request. |
| sd_id | R | String | NA | The unique ID of the sensor. |
| client_id | C | String | NA | The unique ID of the data client making the request. |
| count | true | uint | N/A | The number schedule results |
| next | true | string | N/A | The URL of the next set of schedule results |
| previous | true | string | N/A | The URL of the previous set of schedule results |
| results | true | TaskResult[] | N/A | TaskResultOverview for each schedule entry dictated by the limit and offset parameters |

Table 19: TaskResult  Object

| Property | Required | Type | Unit | Description |
| --- | --- | --- | --- | --- |
| self | true | string | N/A | The url for the TaskResult |
| task_id | true | uint | N/A | The task id |
| started | R | datetime | ISO-8601 | The datetime stamp for when the task started |
| finished | C | datetime | ISO-8601 | The datetime stamp for when the task finished |
| duration | C | time | HH:MM:SS.SSSSSS | The duration of the action |
| status | R | string | N/A | success, fail, in-progress |
| data | R | TaskData[] | N/A | The TaskData that results from the action |
| schedule_entry | R | string | N/A | The ScheduleEntry id |
| detail | O | String | N/A | Any details generated by the task execution. |

Table 20: TaskData Object

| Property | Required | Type | Unit | Description |
| --- | --- | --- | --- | --- |
| recording_id | R | uint | N/A | The recording id. |
| metadata | R | SigMF (see section 8 Data Specification) | N/A | The SigMF metadata associated with the action |
| archive | R | string | N/A | The URL to the tar archive for the acquisition |

Table 21: ScheduleTasksUpcoming Response Object

| Property | Required | Type | Unit | Description |
| --- | --- | --- | --- | --- |
| sm_id | C | String | NA | The unique ID of the sensor manager making the request. |
| sd_id | R | String | NA | The unique ID of the sensor. |
| client_id | C | String | NA | The unique ID of the data client making the request. |
| tasks | R | Task[] | N/A | The description of each upcoming Task |

Table 22: Task Object

| Property | Required | Type | Format | Description |
| --- | --- | --- | --- | --- |
| schedule_entry | R | string | URL | The id for the schedule entry. NTIA uses URL. |
| action | R | string | N/A | The name of the action that will be performed |
| priority | R | int | N/A | The priority of the action for the schedule where lower numbers indicate higher priority |
| time | R | datetime | ISO-8601 | The time at which the action will be performed |

  - User Accounts

This section describes sensor authentication, user accounts, and permissions.

Authentication is performed via tokens. Tokens are automatically generated for all users. Non-admin user accounts do not initially have a password and so cannot log in to the browsable API. To set a password for a user (for testing purposes), an admin can do that in the Sensor Configuration Portal, but only the account's token should be stored and used for general purpose API access.

Sensor account types include:

- Admin: Sensor account type with full control over the sensor and can create schedule entries and view, modify, or delete any other user's schedule entries or acquisitions. Admins can create non-privileged user accounts. Admins can mark a schedule entry as private from unprivileged users.

- User (Sensor Operator): Unprivileged sensor account type which can create schedule entries and view, modify, and delete things they own, but which cannot modify or delete things they don't own. Actions marked admin_only are not schedulable, and schedule entries marked private by an admin (along with their results and acquisitions) are not visible to users.

- Manager

This section provides models and requirements associated with the Manager hardware and software.

  - Hardware

Manager is hosted on a server or virtual machine. Resources and configuration can be customized to meet design and SLA requirements.

  - Software

Manager is a web applications that simplifies the management and tasking of a network of sensors.

    - Functional Requirements

The following are software requirements for the Manager software:

- Operations: Sensor locations, status, and other diagnostics are displayed in operations view to allow for early detection of problems.

- Client requests: With appropriate permissions, Clients can download acquisitions already acquired. Based on SCOS platform capabilities, Clients can also request new actions to be scheduled.

- Distributed sensing actions: Schedule actions for groups of sensors

- SCOS Owner control: Clients can request new actions to be scheduled, but actions are not scheduled without SCOS Owner approval. It is the SCOS Owner’s prerogative to optimize data acquisition to maximize profit.

    - Interfaces

The following are required Manager services with associated functions:

- Sensor Controller Service: Allows Sensors to attach to the Manager, be authenticated and made an active node in the SCOS platform. Once a Sensor is authenticated and registered to a Manager, the sensor controller service can discover Sensor capabilities, schedule actions on the appropriate set of Sensors to fulfil the sensing request of the Client(s), and query current tasks and the state of a Sensor.

- Data Service: Receives acquisitions from each associated Sensor, provides acknowledgement of successful data transfer, and distributes data to Clients according to policies established by SCOS Owner.

      - Status Interface

Table 23: Status Request Object

| Property | R/O/C | Type | Unit | Description | Value |
| --- | --- | --- | --- | --- | --- |
| SM_ID | R | String | NA | The unique ID of the sensor manager making the request. |  |
| SDID | C | String | NA | The unique ID of the sensor. |  |
| client_id | R | String | NA | The unique ID of the data client making the request. |  |
| message_type | R | String | NA | The type of the request (status, capabilities…) | sensor_status, sensors_request |

      - Status Objects

Table 24: Sensors Response Object

| Property | R/O/C | Type | Unit | Description | Value |
| --- | --- | --- | --- | --- | --- |
| sm_id | R | String | NA | The unique ID of the sensor manager making the request. |  |
| client_id | R | String | NA | The unique ID of the data client making the request. |  |
| sensors | R | String[] | N/A | An array of sensor IDs. |  |
| url | R | String | N/A | The url of the sensor |  |
| ownerEmail | R | String | N/A | The sensor’s owner’s email |  |
| newAcquisitions | R | uint | N/A | The number of new acquisitions |  |
| message_type | R | String | NA | The type of the request (status, capabilities…) | sesnsors |

The manager also uses the same status response object described in 5.2.2.2.

      - Capabilities Interface

The capabilities interface uses the same object defined in 5.2.2.4

      - Schedule Interface

The schedule interface uses the same object defined in 5.2.5.1

      - Data Interface

The data interface allows for the manager to be notified of data, save data, and allows entities to search and retrieve data.

Table 25: TaskResultNotification Object

| Property | Required | Type | Unit | Description |
| --- | --- | --- | --- | --- |
| SM_ID | R | String | NA | The unique ID of the sensor manager making the request. |
| SDID | C | String | NA | The unique ID of the sensor. |
| client_id | R | String | NA | The unique ID of the data client making the request. |
| self | true | string | N/A | The url for the TaskResult |
| task_id | true | uint | N/A | The task id |
| started | R | datetime | ISO-8601 | The datetime stamp for when the task started |
| finished | C | datetime | ISO-8601 | The datetime stamp for when the task finished |
| duration | C | time | HH:MM:SS.SSSSSS | The duration of the action |
| status | R | string | N/A | success, fail, in-progress |
| data | R | TaskData[] | N/A | The TaskData that results from the action |
| schedule_entry | R | string | N/A | The ScheduleEntry id |
| detail | O | String | N/A | Any details generated by the task execution. |
| message_type | R | String | N/A | TaskResultNotification |

Table 26: TaskData Object

| Property | Required | Type | Unit | Description |
| --- | --- | --- | --- | --- |
| recording_id | R | uint | N/A | The recording id. |
| metadata | C | SigMF (see section 8 Data Specification) | N/A | The SigMF metadata associated with the action |
| archive | C | string | N/A | The URL to the tar archive for the acquisition |
| data | C | SigMF Archive | N/A | The SigMF archive including SigMF metadata and binary sensed data. |

    - User Accounts

This section describes Manager user accounts and associated permissions.

- Admin: Privileged account type with full control to change security settings, install software and hardware, access all files on the computer, and make changes to other user accounts.

- SCOS Owner: Manager account type with permission to approve actions that the SCOS platform will perform.

- Sensor Operator: Manager account type with permission to register sensors via authentication key exchange. With detailed knowledge of the Sensor hardware and software, the Sensor Operator makes required information, e.g, sensor definition and actions, available via Sensor API discovery or manual input.

- Client: Manager account type with permission to query available SCOS capabilities. The Client can request information on sensed data and/or request scheduled sensing actions.

- Procedures

This section describes required steps to achieve SCOS service provider and Client end user functionality.

  - Sensor Owner

  - SCOS Owner

    - Sensor Administrator

  - SCOS Admin

  - Sensor Operator

  - Client

- Data Specification

# NTIA SCOS Implementation

This Annex describes the NTIA/ITS reference implementation of the IEEE 802.22.3 Spectrum Characterization and Occupancy Sensing (SCOS).

## scos-sensor

scos-sensor is NTIA/ITS Spectrum Monitoring group's work-in-progress reference implementation of the IEEE 802.22.3 Spectrum Characterization and Occupancy Sensing (SCOS) sensor. It is a platform for operating a sensor, such as a software-defined radio (SDR), over a network. The goal is to provide a robust, flexible, and secure starting point for remote spectrum monitoring.

### Software

scos-sensor was designed by NTIA/ITS with the following goals in mind:

- Easy-to-use sensor control and data retrieval via IP network

- Low-cost, open-source development resources

- Design flexibility to allow developers to evolve sensor technologies and metrics

- Hardware agnostic

- Discoverable sensor capabilities

- Task scheduling using start/stop times, interval, and/or priority

- Standardized metadata/data format that supports cooperative sensing and open data initiatives

- Security controls that prevent unauthorized users from accessing internal sensor functionality

- Easy-to-deploy with provisioned and configured OS

- Quality assurance of software via automated testing prior to release

Sensor control is accomplished through a RESTful API. The API is designed to be rich enough so that multiple sensors can be automated effectively while being simple enough to still be useful for single-sensor deployments. For example, by advertising capabilites and location, an owner of multiple sensors can easily filter by frequency range, available actions, or geographic location. Yet, since each sensor hosts its own Browsable API, controlling small deployments is as easy as clicking around a website.

When a task acquires data, that data and a significant amount of metadata are stored in a local database. The full metadata can be read directly through the self-hosted website or retrieved in plain text via a single API call. Our metadata and data format is an extension of, and compatible with, the SigMF specification. The SCOS Data Transfer Specification describes the scos namespace.

When deploying equipment remotely, the robustness and security of software becomes a prime concern. scos-sensor sits on top of a popular open-source framework, which provides out-of-the-box protection against cross site scripting (XSS), cross site request forgery (CSRF), SQL injection, and clickjacking attacks, and also enforces SSL/HTTPS (traffic encryption), host header validation, and user session security. In addition to these, we have implemented an unprivileged user type so that the sensor owner can allow access to other users and API consumers while maintaining ultimate control. To minimize the chance of regressions while developing for the sensor, we have written almost 200 unit and integration tests.

We have tried to remove the most common hurdles to remotely deploying a sensor while maintaining flexibility in two key areas. First, the API itself is hardware agnostic, and the implementation assumes different hardware will be used depending on sensing requirements. Second, we introduce the high-level concept of "actions", which gives the sensor owner control over what the sensor can be tasked to do.

### Architecture

Figure 4 illustrates the scos-sensor software architecture. scos-sensor uses an open source software stack that should be comfortable for developers familiar with Python.

- Persistent data is stored on disk in a relational database.

- A scheduler thread running in a Gunicorn worker process periodically reads the schedule from the database and performs the associated actions.

- A website and JSON RESTful API using Django REST framework is served over HTTPS via NGINX, a high-performance web server. These provide easy administration over the sensor.

Figure 4. Sensor Software Architecture

### Quick Start

This section describes how to spin up a production-grade sensor in just a few commands.

We currently support Ettus USRP B2xx software-defined radios out of the box, and any Intel-based host computer should work. ARM-based single-board computers have also been tested, but we do not prepare pre-build Docker containers at this time.

- Install git, Docker, and docker-compose.

- Clone the repository.

$ git clone https://github.com/NTIA/scos-sensor.git

$ cd scos-sensor

- Copy the environment template file and modify the copy if necessary, then source it.

$ cp env.template env

$ source ./env

- Run a Dockerized production-grade stack.

$ docker-compose up –d  # start in background

$ docker-compose exec api /src/manage.py createsuperuser  # create admin user

$ docker-compose logs --follow api  # reattach terminal

### Provisioning and Configuration Management

Large sensor deployments present unique challenges. At NTIA/ITS, we use Foreman and Puppet to handle hardware provisioning and configuration management. While a ground-up introduction to these tools is outside the scope of this repository, The Foreman and Puppet README should be enough to help someone familiar with these tools get up to speed.

### Browsable API

Figure 5 illustrates the scos-sensor API root. Opening the URL to your sensor (localhost if you followed the Quickstart) in a browser, you will see a frontend to the API that allows you to do anything the JSON API allows.

Relationships in the API are represented by URLs which you can click to navigate from endpoint to endpoint. The full API is discoverable simply by following the links.

Scheduling an action is as simple as filling out a short form on /schedule (see Figure 6). Actions that have been scheduled show up in the schedule entry list on /schedule (see Figure 7).

Figure 5. scos-sensor API root

Figure 6. Example Schedule Form

Figure 7. Example schedule entry on schedule endpoint

## scos-manager

scos-manager is NTIA/ITS Spectrum Monitoring group's work-in-progress reference implementation of the IEEE 802.22.3 Spectrum Characterization and Occupancy Sensing (SCOS) Manager.

### Architecture

Figure 8 provides an illustration of the Manager software model. Elements and functions in the Manager software model include:

- Data Database: Stores data collected by the Data Service for a limited period of time, or as long as physical storage limits allow. Transmits data to the appropriate Client(s).

- Data Service: Receives completed scan data (measurement data and metadata) from the Sensors and retransmits to the appropriate Client(s).

- Manager Database: Maintains the information from Sensors obtained during Sensor registration including Sensor capabilities, and information about to-be-scheduled, scheduled, and on-going tasks.

- Sensor Controller: Communicates with the Sensors using the Sensor API, receives the Sensor capabilities and status, and provides a task schedule from the Sensors along with necessary information for sending scan data to the Data Database. Manages and enforces user authentication and authorization, and acquisition policies.

- User Interface: This service provides a graphical interface through which Clients can interact with the Manager and access a set of functions provided by the system.

Figure 8. Manager Software Model

## Data Specification

This SCOS SigMF namespace defines the data format required for the IEEE 802.22.3 draft standard. The goal is to standardize the control method and data transfer format for “time share” access to a networked fleet of sensors. Use cases require a solid solution for basic sensor control and RF data collection with flexibility to incorporate new sensing solutions and metrics in the future.

The SCOS namespace uses and is fully compliant with the SigMF specification and conventions. Building upon the SigMF core namespace, the specification is enhanced through the implementation of a scos namespace, the details of which follow.

We have adopted SigMF’s conventions:

- The key words “MUST”, “MUST NOT”, “REQUIRED”, “SHALL”, “SHALL NOT”, “SHOULD”, “SHOULD NOT”, “RECOMMENDED”, “MAY”, and “OPTIONAL” in this document are to be interpreted as described in RFC 2119.

- JSON keywords are used as defined in ECMA-404. - Augmented Backus-Naur form (ABNF) is used as defined by RFC5234 and updated by RFC7405.

- Fields defined as “human-readable”, a “string”, or simply as “text” shall be treated as plaintext where whitespace is significant, unless otherwise specified.

- Information is given as global, capture, or annotation

The Institute of Electrical and Electronics Engineers, Inc.

3 Park Avenue, New York, NY 10016-5997, USA

Copyright © varCRYear by The Institute of Electrical and Electronics Engineers, Inc.

All rights reserved. Published varPublishedDate. Printed in the United States of America.

IEEE is a registered trademark in the U.S. Patent & Trademark Office, owned by The Institute of Electrical and Electronics 
Engineers, Incorporated.

PDF:	ISBN varISBNpdf	varStdIDpdf

Print:	ISBN varISBNprint	varStdIDprint

IEEE prohibits discrimination, harassment, and bullying.

For more information, visit http://www.ieee.org/web/aboutus/whatis/policies/p9-26.html.

No part of this publication may be reproduced in any form, in an electronic retrieval system or otherwise, without the prior written permission of the publisher.

IEEE Standards Dictionary Online is available at: http://dictionary.ieee.org

.

National Telecommunications and Information Administration (NTIA) regulates government use of radio frequencies in the United States. The Institute for Telecommunication Sciences (ITS) is the NTIA research and development laboratory.