QUESTION ITU-R 236/1

Impact on radiocommunication systems from wireless and wired
data transmission technologies used for the support of
power grid management systems

(2011)

The ITU Radiocommunication Assembly,

considering

a)	that there is increasing demand for and use of power grid and power usage management and sensing for efficiency, reliability and economic purposes;

b)	that data transmission capability is an essential element of power grid management systems;

c)	that the physical design, data rate, bandwidth and frequency requirements for such data transmission capability may vary according to the physical design and operational requirements of the power grid;

d)	that such data transmission capability may be satisfied by telecommunication systems, including Power Line Telecommunication (PLT) systems;

e)	that radiation from such wireless or wired communication systems may cause interference to radiocommunication services;

f)	that power grid management systems may deploy remote sensors on a widespread basis,

decides that the following Questions should be studied

1	What are the technical and operating features and the characteristics of wireless technologies and devices in support of power grid management systems?

Table 1: Technical and operating features of IEEE Std 802.11

| Item | 802.11 | 802.11ah |  | 802.11n | 802.11ac |
| --- | --- | --- | --- | --- | --- |
|  |  | Model 1 | Model 2 |  |  |
| Supported frequency bands (licensed or unlicensed) | 2.4 GHz | 900 MHz | 900 MHz | 2.4 GHz | 5 GHz |
| Nominal operating range | 1.5 km | 2 km | 2 km | 1 km | 1 km |
| Mobility capabilities (nomadic/mobile) | nomadic and mobile | nomadic | nomadic | nomadic and mobile | nomadic and mobile |
| Peak data rate (uplink/downlink if different) | 2 Mb/s | 156 Mb/s | 1.3 Mb/s | 600 Mb/s | 6934 Mb/s |
| Duplex method (FDD, TDD, etc.) | TDD |  |  |  |  |
| Nominal RF bandwidth | 20 MHz | 1, 2, 4, 8, 16 MHz | 2 MHz | 20, 40 MHz | 20, 40, 80, 160 MHz |
| Diversity techniques | Space time |  |  |  |  |
| Support for MIMO (yes/no) | No | Yes | No | Yes | Yes |
| Beam steering/forming | No | Yes | Yes | Yes | Yes |
| Retransmission | ARQ |  |  |  |  |
| Forward error correction | Yes | Convolutional and LDPC | Convolutional and LDPC | Yes | Yes |
| Interference management | Listen before talk | Listen before talk and frequency channel selection | Listen before talk and frequency channel selection | Listen before talk | Listen before talk |
| Power management | Yes |  |  |  |  |
| Connection topology | point-to-point, multi-hop, star |  |  |  |  |
| Medium access methods | CSMA/CA |  |  |  |  |
| Multiple access methods | CSMA | CSMA/TDMA | CSMA/TDMA | CSMA | CSMA |
| Discovery and association method | Passive and active scanning |  |  |  |  |
| QoS methods | Radio queue priority, pass-thru data tagging, and traffic priority |  |  |  |  |
| Location awareness | Yes |  |  |  |  |
| Ranging | Yes |  |  |  |  |
| Encryption | AES-128, AES-256 |  |  |  |  |
| Authentication/replay protection | Yes |  |  |  |  |
| Key exchange | Yes |  |  |  |  |
| Rogue node detection | Yes |  |  |  |  |
| Unique device identification | 48 bit unique identifier |  |  |  |  |

Table 2: Technical and operating features of IEEE Std 802.15.4

| Item |  |
| --- | --- |
| Supported frequency bands, licensed or unlicensed (MHz) | Unlicensed: 169, 450-510, 779-787, 863-870, 902-928, 950-958, 2400-2483.5 Licensed: 220, 400-1000, 1427 |
| Nominal operating range | OFDM – 2 km MR-FSK – 5 km DSSS – 0.1 km |
| Mobility capabilities (nomadic/mobile) | nomadic and mobile |
| Peak data rate (uplink/downlink if different) | OFDM – 860 kb/s MR-FSK – 400 kb/s DSSS – 250 kb/s |
| Duplex method (FDD, TDD, etc.) | TDD |
| Nominal RF bandwidth | OFDM – ranges from 200 kHz to 1.2 MHz MR-FSK – ranges from 12 kHz to 400 kHz DSSS – 5 MHz |
| Diversity techniques | Space and time |
| Support for MIMO (yes/no) | No |
| Beam steering/forming | No |
| Retransmission | ARQ |
| Forward error correction | Convolutional |
| Interference management | Listen before talk, frequency channel selection, frequency hopping spread spectrum, frequency agility. |
| Power management | Yes |
| Connection topology | point-to-point, multi-hop, star |
| Medium access methods | CSMA/CA |
| Multiple access methods | CSMA/TDMA/FDMA (in hopping systems) |
| Discovery and association method | Active and passive scanning |
| QoS methods | Pass-thru data tagging and traffic priority |
| Location awareness | Yes |
| Ranging | Yes |
| Encryption | AES-128 |
| Authentication/replay protection | Yes |
| Key exchange | Yes |
| Rogue node detection | Yes |
| Unique device identification | 64 bit unique identifier |

Table 3: Characteristics of IEEE Std 802.16

| Item | Value |
| --- | --- |
| Supported frequency bands (licensed or unlicensed) | Licensed Frequency bands between 200MHz and 6GHz |
| Nominal operating range | Optimized for range up to 5 km in typical PMP environment, functional up to 100 km |
| Mobility capabilities (nomadic/mobile) | Nomadic and Mobile |
| Peak data rate (uplink/downlink if different) | 802.16-2012: 34.6UL / 60DL Mbps with 1 Tx BS Antenna (10 MHz BW). 69.2 UL / 120DL Mbps with 2 Tx BS Antennas (10 MHz BW) 802.16.1-2012: 66.7UL / 120DL Mbps with 2 Tx BS Antenna (10 MHz BW), 137UL / 240DL Mbps with 4 Tx BS Antennas (10 MHz BW) |
| Duplex method (FDD, TDD, etc.) | Both TDD and FDD defined, TDD most commonly used, Adaptive TDD for asymmetric traffic |
| Nominal RF bandwidth | Selectable: 1.25Mhz to 10Mhz |
| Diversity techniques | Space and Time |
| Support for MIMO (yes/no) | Yes |
| Beam steering/forming | Yes |
| Retransmission | Yes (ARQ and HARQ) |
| Forward error correction | Yes (Convolutional Coding) |
| Interference management | Yes (Fractional Frequency Re-use) |
| Power management | Yes |
| Connection topology | Point to Multipoint, Point to Point, Multihop Relaying |
| Medium access methods | Coordinated contention followed by connection oriented QoS is support through the use of 5 service disciplines |
| Multiple access methods | OFDMA |
| Discovery and association method | Autonomous Discovery, association through CID/SFID |
| QoS methods | QoS differentiation (5 classes supported), and connection oriented QoS support |
| Location awareness | Yes |
| Ranging | Optional |
| Encryption | AES128 - CCM and CTR |
| Authentication/replay protection | Yes |
| Key exchange | PKMv2 ([1], Section 7.2.2) |
| Rogue nodes | Yes, CMAC / HMAC key derivation for integrity protection for control messages. Additionally ICV of AES-CCM for integrity protection of MPDUs. |
| Unique device identification | MAC Address, X.509 certificates, optional SIM Card |

Name of Standard: IEEE802.20 625k-MC Mode

Table 4: Technical and operating features of IEEE Std 802.20 625k-MC mode

| Item | Value |
| --- | --- |
| Supported frequency bands (licensed or unlicensed) | Licensed bands below 3.5 GHz |
| Nominal operating range | 12.7 km (Max) |
| Mobility capabilities (nomadic/mobile) | Mobile |
| Peak data rate (uplink/downlink if different) | The peak downlink user data rates of 1,493 Mbps and peak uplink user data rates of 571 kbps in a carrier bandwidth of 625 kHz. |
| Duplex method (FDD, TDD, etc.) | TDD |
| Nominal RF bandwidth | 2.5 MHz (Accommodates Four 625kHz spaced carriers) |
| 5 MHz (Accommodates Eight 625kHz spaced carriers) | Modulation/coding rate – upstream and downstream |
| Adaptive Modulation and Coding | BPSK, QPSK, 8-PSK,12-PSK,16QAM, 24 QAM, 32QAM and 64 QAM |
| Diversity techniques | Spatial Diversity |
| Support for MIMO (yes/no) | Yes |
| Beam steering/forming | Spatial Channel Selectivity and adaptive antenna array processing. |
| Retransmission | Fast ARQ |
| Forward error correction | YES |
| Block and Convolutional Coding / Viterbi Decoding | Interference management |
| Adaptive Antenna Signal Processing | Power management |
| Yes | Adaptive power control (open as well as closed loop) scheme. The power control will improve network capacity and reduce power consumption on both uplink and downlink. |
| Connection topology | Point to MultiPoint |
| Medium access methods | Random Access, TDMA-TDD |
| Multiple access methods | FDMA-TDMA-SDMA |
| Discovery and association method | By BS-UT Mutual Authentication |
| QoS methods | The 625k-MC mode defines the three QoS classes. that implement IETF’s Diffserv model: Expedited Forwarding (EF), Assured Forwarding (AF) and Best effort (BE) Per Hop Behaviors based on the DiffServ Code Points (DSCP). |
| Location awareness | Yes |
| Ranging | Yes |
| Encryption | Stream Ciphering RC4 and AES |
| Authentication/replay protection | BS authentication and UT authentication based on using digital certificates signed according to the ISO/IEC 9796 standard using the RSA algorithm |
| Key exchange | Elliptic curve cryptography (using curves K-163 and K-233 in FIPS-186-2 standard) |
| Rogue nodes | Protected from Rogue Nodes |
| Unique device identification | Yes |

2	What are the data rates, bandwidths, frequency bands and spectrum requirements needed in support of power grid management systems?

As a standards developing organization that is responsive to the needs of the vendors and users of our technology, we use the information in the SG Network System Requirements Specification, Release Version 5,  to determine the data rate requirements for developing standards to address various problems.

The application requirements of range and data rate dictate that different bandwidths and spectrum are required.  The current frequency bands and bandwidths listed in the response to question 1 provide the minimum requirement for spectrum.  However, the increasing use and importance of wireless Smart Grid applications leads us to recommend that additional licensed-exempt spectrum should be made available to support these networks, both in the sub 1 GHz as well as up to 5 GHz. (Clinton Powell to clean up).

3	What are the interference considerations to radiocommunications associated with the implementation of wireless and wired technologies and devices used in support of power grid management systems?

The IEEE 802 has developed many wireless technologies that have demonstrated interference resilient communications to enable power grid management without interference to others.

- For example, IEEE 802.11 (Wi-Fi™), and IEEE 802.15.1 (Bluetooth™) have demonstrated that they can co-exist while operating in the same band for many years.

- Although thousands of smart grid devices will be deployed, their data rate requirements may be low and it is very likely that all the devices will not be transmitting at the same time. Therefore, they can efficiently share the same spectrum.

- Regulators such as the Federal Communications Commission and UK OfCom have proposed strict emission limits for various bands that strictly need to be adhered to in order to be able to use these bands.

- New cognitive radio sharing technologies developed within the IEEE 802 Standards (e. g. IEEE Std. 802.22-2011™) can make efficient use of spectrum while doing no harm to other primary users operating in these bands or the adjacent bands.

- Features embedded within IEEE 802 standards such as spectrum sensing, spectrum etiquette, channel set management and co-existence will ensure minimal interference to themselves and others.

4	How will spectrum availability be affected by interference associated with widespread deployment of such technologies and devices?

The IEEE 802 believes that the spectrum availability will not be affected by interference associated with wide-spread deployment of such technologies and devices.

- There are currently millions of installed wireless smart grid devices in a variety of countries and regions, e.g., Europe, Australia, North America, that are operating in shared spectrum. These deployments are growing and more are planned in these geographic regions because they have been successful and effective.

- Mobile consumer wireless devices are in wide use globally. Each device may transfer gigabytes of data per month. The data usage of wireless smart grid devices is orders of magnitude smaller. The licensed spectrum, which is managed by wireless carriers, can easily handle the incremental traffic.

- Existing regulations by regulators such as the Federal Communications Commission and UK OfCom have successfully allowed for millions of wireless Smart Grid devices to operate without harm to each other.

- IEEE 802 wireless standards use a variety of technologies, e.g., frequency hopping, mesh routing, fragmentation, coding, and high burst rate, which enable reliable wireless Smart Grid Networks.   In addition, wireless Smart Grid networks are resilient to link breaks and power outages.

- New cognitive radio sharing technologies developed within the IEEE 802 Standards can make efficient use of spectrum while doing no harm to other primary users operating in these bands or the adjacent bands.

- Features embedded within IEEE 802 standards such as spectrum sensing, spectrum etiquette, channel set management and co-existence will ensure minimal interference to themselves and others.

further decides

1	that the results of the above studies should be included in Recommendations(s) and/or Report(s);

2	that the above studies should be completed by 2016.

Category:  S3

This Question should be brought to the attention of ITU-R Study Groups 4, 5, 6 and 7 and ITUT Study Group 15.

The “power grid” in this case is the electricity distribution network that delivers electricity to individual customers in local areas. Power grid management systems are high-capacity, two-way communications networks with embedded sensing that are installed on existing electric distribution networks to transform them into interactive, automated, self-healing smart grids. These grids are managed by monitoring and controlling network elements.

Model 1 is family description + indoor model

Model 2 is specific operating model + outdoor model

http://osgug.ucaiug.org/UtiliComm/Shared%20Documents/Latest_Release_Deliverables/SG%20Network%20SRS%20Version%20V5%20Final.pdf