| Chapter 7.5 and 7.6 amendment for TSN |  |  |  |
| --- | --- | --- | --- |
| Date: 2017-11-30 |  |  |  |
| Authors: |  |  |  |
| Name | Affiliation | Phone | Email |
| Max Riegel | Nokia |  | maximilian.riegel@nokia.com |
| Notice: This document does not represent the agreed view of the OmniRAN TG It represents only the views of the participants listed in the ‘Authors:’ field above. It is offered as a basis for discussion. It is not binding on the contributor, who reserve the right to add, amend or withdraw material contained herein. |  |  |  |
| Copyright policy: The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |  |  |
| Patent policy: The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. |  |  |  |

Abstract

This document proposes amendments to chapter 7.5 and chapter 7.6 to include Time Sensitive Networking in the P802.1CF specification.

The first edition shows the outline of the proposed amendments and some initial content.

The first revision contains further content added after the Nov 2017 plenary meeting.

The second revision was created after the confcall on Nov 28 to better reflect the detailed procedures of service flow management aligned to the TSN configuration models.

The third revision replaces all occurrences of ‘subscriber’.

7	Functional Decomposition and Design	4

7.5	Datapath establishment, relocation, and teardown	4

7.5.1	Introduction	4

7.5.1.1	Link characteristics	4

7.5.1.2	Traffic types	6

7.5.1.3	Time sensitive networking	6

7.5.2	Roles	8

7.5.2.1	Datapath	8

7.5.2.2	Terminal interface (TEI)	8

7.5.2.3	Node of attachment (NA)	9

7.5.2.4	Backhaul (BH)	9

7.5.2.5	Access router interface (ARI)	9

7.5.2.6	Subscription service (SS)	9

7.5.2.7	Access network control (ANC)	9

7.5.2.8	Network management service (NMS)	10

7.5.3	Use cases	10

7.5.3.1	Single, plain access network	10

7.5.3.2	Industrial network for time sensitive applications	10

7.5.3.3	Shared wireless access network	11

7.5.3.4	Multi-operator backhaul infrastructure	12

7.5.4	Functional requirements	13

7.5.5	Datapath-specific attributes	13

7.5.5.1	Node of attachment (NA)	13

7.5.5.2	Backhaul (BH)	13

7.5.5.3	Access router (AR)	13

7.5.5.4	Subscription service (SS)	13

7.5.6	Datapath–specific basic functions	14

7.5.6.1	Retrieval of session-specific datapath configuration values for access network	14

7.5.6.2	Activation of datapath in the NA	14

7.5.6.3	Teardown of datapath in the NA	14

7.5.6.4	Activation of datapath in the BH	14

7.5.6.5	Teardown of datapath in the BH	14

7.5.6.6	AR interface establishment	14

7.5.6.7	AR interface teardown	14

7.5.7	Detailed procedures	14

7.5.7.1	Datapath establishment	15

7.5.7.2	Datapath relocation	16

7.5.7.3	Datapath teardown	17

7.5.8	Mapping to IEEE 802 technologies	17

7.6	Authorization, QoS, and policy control	18

7.6.1	Introduction	18

7.6.1.1	QoS models supported by IEEE 802	18

7.6.1.2	Repository of QoS settings and policy parameters	19

7.6.1.3	QoS policy control architecture and mapping to the NRM	19

7.6.1.4	TSN control architecture and mapping to the NRM	20

7.6.2	Roles	22

7.6.2.1	Service flow	22

7.6.2.2	Subscription	23

7.6.2.3	Subscription service (SS)	23

7.6.2.4	Access network control (ANC)	23

7.6.2.5	Node of attachment and backhaul (NA, BH)	23

7.6.2.6	Terminal control and access router control (TEC, ARC)	23

7.6.3	Use cases	23

7.6.3.1	QoS policy provisioning to AN control	23

7.6.3.2	Default service flow creation, modification, and deletion	23

7.6.3.3	Change of authorization by subscription service	24

7.6.3.4	Access router initiated service flow creation, modification, and deletion	24

7.6.3.5	Terminal initiated service flow creation, modification, and deletion	24

7.6.4	Functional requirements	24

7.6.5	QoS policy-specific attributes	24

7.6.5.1	Service flow parameters	24

7.6.5.2	Policy rules	24

7.6.6	QoS policy control-specific basic functions	25

7.6.6.1	Provisioning of authorization information to policy decision point at session begin	25

7.6.6.2	Change of authorization during session	25

7.6.6.3	Provisioning of QoS parameters to policy enforcement points	25

7.6.6.4	Request of service flow by terminal	25

7.6.6.5	Request of service flow by access router	25

7.6.7	Detailed procedures	25

7.6.7.1	Pre-provisioned service flow establishment	25

7.6.7.2	Service flow initialization by terminal	26

7.6.7.3	Service flow initialization by access router	27

7.6.7.4	Service flow modification by terminal	27

7.6.7.5	Service flow termination by terminal	28

7.6.7.6	Change of authorization by subscription service	28

7.6.8	Mapping to IEEE 802 technologies	29

# Functional Decomposition and Design

## Datapath establishment, relocation, and teardown

### Introduction

The datapath refers to the transport facility for the user payload between the terminal and the access router, or between the terminal and another terminal when direct communication between terminals on the same link is enabled. The datapath is usually preconfigured during access network setup when being shared for multiple terminal sessions, and may be dynamically established just for the lifetime of a terminal session, when having point-to-point characteristic.

Ethernet data frames are carried over the datapath between the data link service access points (DL-SAPs) in the end stations of the communication. Forwarding on the datapath between end stations is performed by IEEE 802.1Q bridging based on the destination MAC address and further information elements in the Ethernet frame, as well as configurations in the network.

In access networks, it is common to denote the forwarding directions either “upstream” or “downstream.” Upstream indicates the direction from TE toward AR; downstream denotes the direction from AR to TE.

- Ethernet datapath

The datapath is either pre-established during access network setup and/or dynamically configured when terminal connects to access network. A terminal resides on one datapath during a terminal session—i.e., is associated with a single datapath at session initiation—but may be assigned to different datapaths for subsequent sessions

#### Link characteristics

Various forwarding behaviors may exist in the NA, depending on the specific IEEE 802 access technology and configuration. Some technologies allow bridging—i.e., forwarding according to destination MAC addresses—to happen directly between TEs associated with the same NA. However, an NA may be configured to enforce that all user data coming from TEs are forwarded over R6 toward the BH. BH may contain functions to enable forwarding between the end stations without passing the data through the access router.

Forwarding in the access network may be restricted to one of the following schemes:

- E-Line (Ethernet-Line) characteristic represents a point-to-point connection carrying Ethernet frames only between the R1 interface of a particular TE and the R3 interface of its AR.

- E-Line characteristic

- Point-to-point connections between TEIs and ARIs require that the AR establish and maintain a dedicated interface for each of the connected TEs. Such configuration is commonly used in mobile networks where the IP connectivity has to be maintained across multiple ANs.

- E-LAN (Ethernet-LAN) characteristic provides multipoint-to-multipoint connectivity for Ethernet frames across a number of interfaces. Any TE connected to an AN with E-LAN characteristic can communicate with any other TE over the same link in that AN. Still, an AN can establish multiple separated links with multipoint-to-multipoint connectivity for groups of TEs by means of VLANs.

- E-LAN service

- E-LAN characteristic is usually deployed when all connected TEs belong to the same security domain and are allowed to communicate directly to each other. A benefit of E-LAN characteristic is that the AR needs only a single interface for a number of TEs and is less loaded, as communication between the connected TEs in an AN does not pass through the AR. Access networks within enterprises or industrial facilities commonly deploy E-LAN characteristic.

- E-Tree (Ethernet-Tree) characteristic distinguishes between leaf interfaces and root interfaces, as depicted in E-Line characteristic. Leaf interfaces are restricted in the exchange of data only with root interface, but never directly with another leaf interface. Root interfaces can exchange data with any leaf interface and with any other root interface.

- E-Tree service

- E-Tree characteristic is usually deployed in ANs that are intended to serve a large number of TEs via a single interface of the AR, as in an E-LAN, but enforce that all user traffic is passing through the AR. E-Tree characteristic is commonly used for efficiently providing public broadband access, for connecting a huge number of small devices to a network such as for IoT, or for delivering multicast services efficiently to multiple interfaces.

Note: E-Tree characteristic is widely deployed in cable networks and DSL networks for aggregating broadband user traffic toward CMTS or BNG.

Note: The distinction of line, LAN, or tree characteristic is also used by the Metro Ethernet Forum (MEF) in its definition of Ethernet services.

#### The datapath is either pre-established during access network setup and/or dynamically configured when terminal connects to access network. A terminal resides on one datapath during a terminal session—i.e., is associated with a single datapath at session initiation—but may be assigned to different datapaths for subsequent sessionsTraffic types

Depending on the forwarding mechanisms used in the bridges various types of traffic can be handled on the data path.

- Prioritized best effort (BE) traffic: Most common is best effort forwarding in a prioritized manner according to the p-bits in the VLAN header. Packet loss may occur when a bridge receives more packets for an output port than the port can handle. In the case of congestion, the bridge forwards frames with the highest priority P-bit setting first, and out of order delivery can happen across the different priorities.

- Rate constraint (RC) traffic: To avoid congestion in the bridges, traffic can be categorized in flows with each flow not exceeding a defined bandwidth through setting the parameters for minimum inter-frame intervals and maximum frame size. Rate constraint traffic can pass bridges without any packet loss.

- Time-trigger (TT) traffic: To avoid congestion, but also to guarantee timed delivery, an accurate time can be set for delivery for frames belonging to traffic flows. Such behavior is usually required for time sensitive networking.

#### Time sensitive networking

Time sensitive networking allows for forwarding of real-time and time-critical traffic. It mandates that data must be delivered within a certain time window, typically not exceeding a specified maximum delay, and that connected devices need to have a common sense of time for synchronization, coordination, phase locking and other processing.

Time sensitive networking results in zero congestion loss. It requires technologies addressing four different aspects of traffic delivery:

- Synchronization: 
It establishes a common clock across the network allowing for timed forwarding and delivery not exceeding an upper bound of end-to-end delay, as well as provides the tool for limiting the frame rates of traffic flows through a precise time at the inbound of the network.

Deployment of IEEE 802.1AS, a layer-2 profile to the IEEE 1588 Precision Time Protocol, is required to synchronize the local clocks located in the network elements to the precise timing of a grandmaster clock. For reliability, the protocol supports selecting a preferred grandmaster clock out of a set of multiple grandmasters, and performs a periodic re-synchronization of the local clock to the grandmaster clock. To equalize for the various transmission delays on the various paths, the protocol measures the forwarding delays through the Bridges and compensates for it.

- Latency: 
Bounded (low) latency ensures timely delivery of frames and avoids congestion in network nodes. Various techniques can be used to schedule and re-order the sequence of frame transmission, and to ensure that the bandwidth used by a particular stream does not exceed specified boundaries. Depending on the requirements, three different kind of mechanisms can be used. Priority scheduling algorithms are well suited to handle best-effort traffic, which is not very sensitive to increased latency due to queuing in buffers. Applying different traffic classes allows for preferred delivery of some packets, but preferred delivery does not guarantee a maximum delay, nor a defined bandwidth. Credit based shapers are well suited for rate-constraint traffic like audio streaming or video streaming. The source encoding of the signals guarantees an upper limit of bandwidth demand, which allows credit based shapers to orderly arrange frames of different sources to become forwarded over a transmission link. Time aware shapers are necessary to control the forwarding process for time-trigger traffic which require guaranteed delivery to its destination within a specified period.

- Reliability:
Time sensitive information often requires additional means to create very high reliability for the transmission path. Path control and per-stream filtering and policing ensures that packet streams stay within defined boundaries regarding bandwidth, inter-frame timing, and packet sizes. For highly increased requirements multiple transmission paths may be used in parallel, with frame replication and elimination feeding the multiple paths and filtering redundant frames at the destination.

- Resource management:
Time sensitive networking requires careful management of the transmission resources to fulfill the various traffic transmission demands. The Stream Reservation Protocol (SRP) registers streams and reserves bandwidth and transmission capabilities along the path from the Talker to the Listener.

Figure 1 - Stream reservation

Streams are reserved by the Talker advertising to the adjacent bridge a stream identified by a StreamID and directed towards a Listener identified through its MAC address and the VLAN-ID. Traffic characteristics is defined by a MaxFrameSize and a maximum number of frames allowed within a given time interval. Further attributes describe priority and ranking among multiple other streams and a maximum accumulated latency. The reservation request is forwarded towards the destination MAC address and each Bridge on the path checks the traffic specification against its available capabilities and resources. When the request can be fulfilled the request is forwarded towards the Listener, otherwise the Bridge responds to the Talker with a message defining the maximum available resources. When the request message successfully receives at the Listener, all the Bridges on the path have sufficient resources for the traffic stream. The resources on the path are allocated when the Listener sends back to the Talker the information that stream transmission is feasible.

### Roles

#### Datapath

The datapath is established from the terminal interface over R1 to the node of attachment, continued over R6 to the backhaul, and carried over R3 to the access router interface.

- NRM, with datapath represented by solid line

The solid line in NRM, with datapath represented by solid line indicates the interfaces in the NRM carrying the datapath. The datapath consists of multiple segments, which usually deploy different IEEE 802 PHY technologies.

The datapath is identified through the

DataPath-ID, which r

epresents one particular datapath instance (link) through the access network. A datapath can be specific to a single user session, or can be shared for multiple user sessions.

The following entities participate in the operation of the datapath.

#### Terminal interface (TEI)

The TEI is the endpoint of the datapath at the terminal. It provides the capability to establish the datapath connection over the R1 interface by negotiation of transmission parameters with the NA according to configuration information provided by the terminal control.

#### Node of attachment (NA)

The NA provides the communication port at the access network to which the terminal connects over R1, and forwards user payload from the terminal toward the backhaul, and vice versa.

The NA has at least two ports, one directing toward the terminal, and another directing toward the backhaul, but may have multiple ports toward terminals, when the NA concurrently connects multiple terminals. Ports toward terminals may be dynamically created and released when terminals establish or release their sessions. For each of the terminal side ports, the NA negotiates with the TEI the configuration parameters of the datapath connection over R1 and forwards the datapath over R6 toward the BH according to configuration information received from the ANC.

Forwarding behavior of the NA may be either point-to-point toward the BH, or it may enable LAN or tree behavior when multiple terminals are connected to the same NA and assigned to the same datapath.

#### Backhaul (BH)

The BH provides the connectivity for the datapath between the NA and the AR. It interfaces to NA over R6 and to the AR over R3. The BH forwards user payload from the NA to the AR, and vice versa. In the case that direct terminal-to-terminal connectivity is enabled on a shared datapath, the BH reflects user payload toward the corresponding terminal, when the terminals are attached to different NAs.

The BH consists of at least two ports—one directing toward the NA and another directing toward the ARI—but may contain many more ports when the AN consists of multiple NAs and eventually has interconnections with multiple ARs. The BH forwards the datapath from the NA over R6 toward the AR over R3 or eventually also to other NAs, when LAN characteristic or Tree characteristic behavior is part of configuration information received from the ANC. Forwarding behavior of the BH may be either point-to-point between a single R6 and a single R3, or it may realize LAN or Tree forwarding characteristics when multiple NAs and/or multiple ARs are connected and assigned to the same datapath.

#### Access router interface (ARI)

The ARI is the endpoint of the datapath at the access router. It terminates the datapath toward the router, which forwards user payload to communication peers not residing on the same datapath based on IP addresses.

An access router may terminate multiple datapaths either over a single access router interface serving multiple datapaths or over multiple access router interfaces attached to an AR instance.

#### Subscription service (SS)

The subscription service provides configuration information for the datapath of a successfully authenticated user as part of the authorization to the access network control, and forwards related configuration information for that user to the access router.

#### Access network control (ANC)

The access network control generates and distributes configuration information for the datapaths of users based on the authorization received from the subscription service, eventually taking into account system configuration directives received from the coordination and information service.

#### Network management service (NMS)

The network management service provides the initial configuration of the network entities involved in the establishement of the datapath and well as the configuration information for pre-established datapathes which exist for longer periods carrying the traffic of multiple user sessions, and allowing for direct terminal-to-terminal connectivity. The network management service also performs monitoring and accounting of the network elements, and guides fault discovery and maintenance procedures when something is going wrong in the access network.

### Use cases

#### Single, plain access network

A single, plain wireless access network consists of several nodes of attachments connected to a bridge, with connection to a single access router and a single subscription server.

- Simple, plain access network

LAN characteristics are provided by the single Ethernet bridge, which connects all the nodes of attachment with the access router. All terminals are assigned to the common datapath, which allows for direct terminal-to-terminal communication and inherently provides mobility support when the terminals move from node of attachment to node of attachment, as the terminals stay on the same access router interface, ensuring that IP addressing is maintained during movements.

#### Industrial network for time sensitive applications

#### Industrial networks are used to connect IoT devices, control processes of machineries and robots, surveillance equipment and building control gear to their respective central control entities, but also to allow for peer-to-peer communication among themselves. Often industrial networks are connected as well to local datacenters and to the cloud to enable communication of industrial devices with services residing in a local or remote cloud environment. The local area part of industrial networks are meanwhile built through Ethernet networks, which are connected through routers to local data centers and the cloud. Ethernet technology allowed for combining multiple different services and multiple independent networks into a common infrastructure separated through VLAN technology. Such local area network can be further enhanced to provide time sensitive networking between the devices and the access gateway (router) towards external services.

#### Figure 40+: Industrial (access) network

#### The figure above shows a schematic network of a factory floor with four robots at an assembly line, each supervised through a surveillance camera forwarding live video to a central control room not depicted here. A Wi-Fi network throughout the factory floor allows the operators to remotely access the control processes of the robots and live video streams and other information while walking along the assembly line.

#### Control processes of robots require highest reliability and immediate response to their messaging. Video streaming is not as critical as robot control, but sufficient bandwidth has to be reserved on the network to ensure that the control center has latest insights into the factory hall. The applications running on smartphones, tablets and notebook computers allowing operators to access control information while walking around in the factory hall may be able to deal with packet losses and higher delays as usual when accessing services over the Internet. However, the network has to ensure through scheduling and prioritization that bulk data transferred to wireless devices does not impact the transmission of the low-delay real-time messaging and streaming data.

#### Shared wireless access network

A shared wireless access network leverages the same access network infrastructure for multiple separate services, each with its own access router and its own subscription service. It isolates each of the connected terminals to block direct terminal-to-terminal communication and enforces that all communication passes through the access router, providing enhanced capabilities for user-specific services and centralized policy enforcement and accounting.

Still, mobility support by the bridged infrastructure has to be supported, to enable user to move around in the wireless coverage area without losing IP connectivity or application session states.

- Shared wireless access network

Bridges and nodes of attachment provide two independent access networks in a virtualized manner realizing the same coverage area for each of the service providers operating its own access router and subscription service. Forwarding behavior in the nodes of attachment and in the bridges is restricted to point-to-point behavior, enforcing that all user payload packets pass through the access router.

#### Multi-operator backhaul infrastructure

The third use case shows the scenario when a backhaul infrastructure composed of networks provided by multiple Ethernet operators is used by multiple service providers, each with its own access points and its own subscription service and access router.

- Multi-operator backhaul infrastructure

The backhaul operators provide dedicated Ethernet connectivity to each of the wireless service providers for aggregation and interconnection of their nodes of attachment to their access routers, respectively.

The backhaul connectivity of the Ethernet operators is statically configured according to the requirements of the individual service providers. Nevertheless, access network operators may provide different services to their users, with the need to provide point-to-point service as well as tree service and LAN service forwarding behavior through the same infrastructure.

### Functional requirements

- Datapath should be configurable as either point-to-point or multipoint-to-multipoint or rooted multipoint (point-to-multipoint) behavior

- Datapath should allow for differentiated services based on C-VIDs and priority bits.

- Datapath should be able to support time sensitive networking in addition to best effort traffic.

- Successful completion of datapath establishment should be indicated.

- Relocation of datapath within the access network should be supported.

- Datapath should be configurable to support the transport of C-VIDs (Customer WLAN Identifiers) between terminal and access router

- Datapath should protect integrity of user payload.

- Datapath should support encrypted transport of user payload

- Datapath should allow for differentiated services based on C-VIDs and priority bits.

- Datapath should support wired and wireless links in the access and backhaul.

### Datapath-specific attributes

#### Node of attachment (NA)

- R1 MAC and PHY configuration parameters

- R1 performance and QoS parameters

- E.g. supported service classes (Throughput up/down, delay, jitter)

- R6 configuration parameters

- VLAN configuration and mapping

- TSN configurations

#### Backhaul (BH)

- R6 configuration parameters

- R3 configuration parameters

- Service specification

- Service mapping table

- TSN configurations

#### Access router (AR)

- R3 configuration parameters

- Network interface performance

- E.g. supported service classes (throughput up/down, delay, jitter)

#### Subscription service (SS)

- User-specific service specification

### Datapath–specific basic functions

#### Retrieval of session-specific datapath configuration values for access network

Session-specific datapath configuration values are usually provided by the SS as part of the authorization information, which is forwarded from the SS to the AN as part of the final message of a successful authentication procedure.

#### Activation of datapath in the NA

The ANC generates the session-specific configuration values for the datapath establishment in the NA, and forwards the values into the NA.

#### Teardown of datapath in the NA

The ANC sends a command to NA to tear down a particular datapath and to release the used resources. It may happen either due to the termination of a session or due to movement of the TE into the coverage area of another NA and a relocation of the connectivity to the other NA.

#### Activation of datapath in the BH

The ANC generates the session-specific configuration values for the establishment of the datapath in the BH. Depending on deployment scenario and concurrent usage of the AN, a new session may not require the activation of a datapath in the BH, but may leverage an existing one.

#### Teardown of datapath in the BH

The ANC sends a command to BH to tear down a particular datapath and to release the used resources. It may only happen after the teardown of the last related datapath in any of the connected NAs, i.e., only when all sessions making use of the particular datapath have been terminated.

#### AR interface establishment

When datapaths are dynamically established for each of the sessions, the ANC informs the AR about the demand for a new interface and provides its R3-related configuration parameters. The AR acknowledges the proposed parameters or provides other values when the proposed values are not feasible. ANC and AR agree on configuration parameters after one or more roundtrips, or terminate the session establishment with a failure notice.

AR may request, for a particular session, further configuration values from the SS.

#### AR interface teardown

When session-specific datapaths are deployed, the ANC MAY inform the AR about the termination of a datapath when the last session making use of that datapath has been released.

### Detailed procedures

Datapath procedures take place as part of the session establishment, as part of the session termination, or during a session when the TE changes its NA due to a handover process.

The following figure shows the occurrence of datapath procedures during the life cycle of a terminal session.

- Datapath actions

#### Datapath establishment

Datapath establishment occurs after the association of the terminal to the access network and after successful completion of the authentication procedure.

- Datapath establishment

- The service-specific configuration details of the datapath are provided by the subscription service through the authorization following the successful authentication.

- The SS informs the AR about the network attachment of a new user and the related service parameter.

- The ANC creates the user-specific datapath configuration information for the BH and delivers such configuration to the BH.

- The ANC creates the user-specific datapath configuration information for the NA and forwards the information to the NA.

- The NA performs the configuration of the data connectivity for the new terminal.

- When the NA is ready for accepting user data, the NA sends an Authentication Success message to the TE to inform about the possibility to transfer data.

- Once the TE has enabled the TEI, the datapath is established and allows for forwarding user data between the terminal and the access router.

#### Datapath relocation

Various versions exist for the relocation of the datapath due to a handover from one NA to another NA.

In the case of break-before-make, the TE terminates the connectivity to its current NA, searches through Network Discovery and Selection for a more appropriate NA, and establishes the datapath across the new NA. In this case, the datapath relocation is essentially a datapath teardown followed by a datapath establishment.

Make-before-break describes a procedure that allows the TE to leverage the current datapath connectivity to prepare for the preparation of handover and to immediately use the prepared datapath when the association changes to the new NA. The following figure shows the make-before-break procedure.

- Datapath relocation

- In the case of make-before-break, the TE uses the established datapath to communicate with the new NA. The TE sends the indication that it would like to handover to the new NA by an HO request message over the established datapath to the new NA.

- The new NA informs the ANC about the request of a TE to relocate its attachment to the new NA.

- The ANC arranges for BH connectivity for such TE to the new NA.

- The ANC sends the configuration information for the TE to the new NA. With this information, the NA can pre-install and pre-activate the datapath for the TE

- Once the preparation is completed, the new NA informs the TE about the possibility to handover immediately.

- The TE informs its current NA about the end of its attachment.

- The TE informs the new NA that the attachment moved to the new NA and the datapath is reestablished over the new NA.

#### Datapath teardown

When a TE has terminated its connectivity to an access network, the access network tears down the datapath and releases the allocated resources.

- Datapath teardown

- When a TE intends to terminate its service, it sends a detach message to the NA, to inform the NA about the end of the connectivity.

- The NA informs the ANC about the end of the connectivity of the TE.

- The ANC instructs the NA to remove the connectivity for the TE and to release the allocated resources.

- Once the NA has terminated the connectivity and the remaining data frames have been delivered, the ANC informs the BH about the termination of the service for the TE and requests the removal of the configuration and the release of the allocated resources.

- Thereafter the ANC indicates to the SS that the service for the TE has been terminated.

- The SS informs the AR to remove the configuration for the TE and to release the allocated resources.

### Mapping to IEEE 802 technologies

The following table provides IEEE 802 technology-specific attributes for the datapath configuration.

- IEEE 802 technology-specific attributes for datapath configuration

## Authorization, QoS, and policy control

### Introduction

The functional description of authorization, QoS, and policy control has several aspects to address:

- QoS models supported by IEEE 802

- Repository of QoS and policy parameters

- QoS policy control architecture

The following sections describe each of the aspects mentioned above.

#### QoS models supported by IEEE 802

IEEE 802 supports both prioritization of packets and resource reservation on communication links. These approaches are denoted in the IP domain as diffserv and intserv, respectively. As QoS mainly depends on packet forwarding behavior across networks, the IEEE 802.1Q specification on bridging provides the foundations for QoS and policy control in IEEE 802 technologies. Part of the IEEE 802 specification are the priority marking of packets, which was initially introduced by IEEE 802.1p, as well as the resource reservation for particular service streams, which is supported by the Resource Reservation Protocol and was introduced into IEEE 802.1Q by IEEE 802.1Qat. Both methods are widely supported in IEEE 802 technologies. An even stricter and more powerful method was recently introduced by IEEE 802.3br, which provides means for even tighter control of jitter and transmission delays for real time data.

Highest level of control of traffic passing through IEEE 802 networks is provided by the means of time sensitive networking, comprising tight time synchronization among the end stations and bridges, traffic shaping at the ingress port of the forwarding infrastructure, dedicated path control and other means to increase reliability, as well as protocols for strict reservations of forwarding capabilities at bridges.

Also, the IEEE 802 radio technologies support both the diffserv and the intserv QoS model. The technologies operating in unlicensed spectrum mainly deploy the diffserv model, as it is not feasible to guarantee transmission of data over spectrum that is not under full control and can be blocked by some other radio device at any time.

IEEE 802.11 was amended by comprehensive diffserv-based QoS support through IEEE 802.11e. As resource reservation is difficult in unlicensed spectrum, only the diffserv based EDCF is nowadays widely used in equipment.

IEEE 802.16, which is designed for operation in licensed spectrum, supports 5 different data delivery service, not only following the diffserv model, but also providing the capabilities to reserve fixed bandwidth for particular service flows. As IEEE 802.22 inherits most of the MAC from IEEE 802.16, it also provides the QoS capabilities of IEEE 802.16.

#### Repository of QoS settings and policy parameters

Another aspect for consideration is the repository of the QoS settings and policy parameters. As provisioning of QoS is tightly coupled with the subscription, the obvious location for the repository is the subscription service. Providing users individually assigned QoS profiles is the native purpose of the authorization signaling during session establishment. However, the subscription service provides authorization not only at the beginning of a session, it has the possibility to change authorization parameters also during a session by invoking a change of authorization.

Aside of provisioning of QoS configurations out of the authorization information delivered by the SS at the successful end of the authentication procedure, QoS settings can also be requested through R8 from the TE, and R9 from the AR. QoS requests from the endstations are subject of QoS policy control according to the rules delivered by SS to guarantee that configured QoS behavior is within the capabilities and limitations specified in the subscription.

Configuration of TSN streams and forwarding as defined within IEEE 802.1Qcc can be performed either from the end-stations or centrally through a database. Both models are supported by this specification

#### QoS policy control architecture and mapping to the NRM

The architecture of QoS policy control in this specification for IEEE 802 access network follows the common approach of distributing the storage of the policies in a dedicated policy repository, processing the resource demands against the policy rules in a policy decision point, and enforcing the QoS settings at the various network elements through policy enforcement points. The following Generic policy control architecture shows the generic policy control architecture applied in this specification.

- Generic policy control architecture

The generic architecture can be easily mapped to the NRM, as the entities of the policy control architecture have clear relations to entities of the Network Reference Model.

Figure 2- Mapping of policy control architecture to NRM

The following mappings can be applied:

- The policy data base resides in the subscription service.

- The policy decision point is part of the access network control.

- The policy enforcement points are the node of attachment as well as the backhaul.

- Resource requests are forwarded either from the terminal control over R8 or the access router control over R9 to the policy decision in the ANC.

#### TSN control architecture and mapping to the NRM

The IEEE 802.1Qcc introduced three different management models for configuration of TSN streams. The models differ in the way, how user and network configuration information is stored and forwarded between the network elements and end-stations. Either centralized or distributed models are foreseen. A User/Network Interface (UNI) is defined for allowing the user side, which is either a talker or a listener, to exchange TSN stream configuration values with the adjacent Bridge in the network. The defined models are:

- Distributed TSN configuration model
End stations (i.e. Talkers and Listeners) communicate the user requirements directly over the TSN user/network protocol. The network is configured in a fully distributed manner without a centralized network configuration entity. TSN user/network configuration information is propagated along the active topology for the Stream (i.e. Bridges in the tree from Talker to Listeners). Bridge’s resources are effectively managed locally without knowledge of the entire network.
This model fundamentally differs from the network management model used in this specification, that assumes that a central control of resources in the Bridges of the network exists.

- Centralized network / distributed user model
More complex TSN use cases benefit from a complete knowledge of all Streams in the network, which is stored and processed through a centralized entity. It gathers information for the entire network in order to find the best configuration. Similar to the fully distributed model the end stations communicate their Talker/Listener requirements directly over the TSN UNI. The CNC has a complete view of the physical topology of the network, as well as the capabilities of each Bridge. This enables the CNC to centralize complex computations.

Figure 3 - Centralized network /distributed user

The edge Bridges are configured as proxy, transferring Talker/Listener information directly between the edge Bridge and the CNC, rather than propagate the information to the interior of the network. However, the configuration information is directed to/from a Centralized Network Configuration (CNC) entity. All configuration of Bridges for TSN Streams is performed by this CNC using a network management protocol

- Fully centralized TSN configuration model
Many TSN use cases require significant user configuration in the end stations that act as Talkers and Listeners. The fully centralized model enables a Centralized User Configuration (CUC) entity to discover end stations, retrieve end station capabilities and user requirements, and configure TSN features in end stations.

Figure 4 - Fully centralized TSN configuration

All user requirements are exchanged between the CNC and CUC, locating the TSN UNI between the CNC and CUC.

Out of the three configuration models introduced through IEEE 802.1Qcc, the fully centralized TSN configuration model, as well as the centralized network / distributed user configuration model can be well mapped to the NRM. In the NRM, the Listener / Talker reside in the TE or AR, respectively, and according to the direction of the stream. Both, the NA as well as the BH can be TSN enabled through implementation of TSN forwarding functions in the Bridges residing in the data path. The UNI for exchange of stream configuration information is defined through R8 towards the Listener/Talker in the TE, and through R9 towards the Listener / Talker in the AR. The centralized network configuration resides mainly in the ANC with default configurations delivered through the NMS during network initialization. For the case of centralized user configuration, the CUC is located in the Subscribtion Service.

Figure 5 - Mapping of TSN configuration to NRM

For TSN capabilities User/Network Configuration parameters are embedded in the messages and are propagated as part of the QoS control messaging across the reference points.

### Roles

#### Service flow

The service flow is the basic means of QoS policy control. It defines a traffic forwarding class with defined QoS parameters, which is uniquely identified by an ServiceFlow-ID.

For time sensitive networking, service flows are denoted as ‘Streams’ starting at a ‘Talker’ and ending at a ‘Listener’. Talker and listener are end stations in the notation of IEEE 802 networking and are represented by Terminal and Access Router in this specification.

User datagrams can be assigned to Service Flows by filtering rules, which allows network elements to preferably process some user datagrams against some others.

Service Flows have operational states:

- Provisioned: Service flow parameters are provided by subscription service

- Permitted: Requested service flow parameters fit to allowed range defined by authorization

- Active: service flow is established in network elements

Service Flows can be either static over a session or dynamic

- Static flows keep same behavior throughout a session

- Dynamic flows change behavior during a session

#### Subscription

The subscription contains user-specific QoS policy information, which consists of the permitted QoS parameters and some other information about the applicability of the policies.

#### Subscription service (SS)

The subscription service acts as repository of QoS policies. It provides the QoS policies to the access network at the beginning of sessions and initiates policy changes during sessions if there is need for it, e.g., by expiring service credits.

The subscription service also provides the QoS parameters of the default service flow, which is established at connection establishment.

#### Access network control (ANC)

The access network control contains the policy decision point, which decides about assignment of QoS settings to service flows based on available resources, the QoS policies provided by the subscription service, and the demands for additional service flows or changes to service flow parameters coming from either the terminal control or the access router control.

#### Node of attachment and backhaul (NA, BH)

Node of attachment and backhaul are the network elements enforcing the QoS parameters to the user datagrams traversing the datapath. They both act as policy enforcement points.

#### Terminal control and access router control (TEC, ARC)

Terminal control and access router control are the entities requesting the establishment of new service flows during ongoing sessions, or for the change of QoS parameters of active service flows. They negotiate with the ANC whether the resource requests are permitted.

### Use cases

#### QoS policy provisioning to AN control

The initial use case is the provisioning of the subscription-specific QoS policy to the policy decision point at session begin, which is denoted as authorization in the AAA procedures.

#### Default service flow creation, modification, and deletion

The default service flow is the initial service flow established by the access network to initiate communication between terminal and access router. This default service flow has to be established at session begin, and has to be teared down as last service flow at the session end. It is possible, but not very common, to change the QoS parameters of the default service flow during an ongoing session.

#### Change of authorization by subscription service

The subscription service has the possibility to change the authorization for a particular subscription and terminal at any time during a session. Such change may be required, when service periods or service credits for a particular subscription expire.

#### Access router initiated service flow creation, modification, and deletion

Access router control can request the establishment of new service flows, the change of the QoS parameters of existing service flows, or the termination of service flows, when the service flow is not required anymore. The ARC forwards the request for creation, modification, or deletion to the policy decision point in the ANC.

#### Terminal initiated service flow creation, modification, and deletion

Terminal control can request as well the establishment of new service flows, the change of the QoS parameters of existing service flows, or the termination of service flows, when the service flow is not required anymore. The TEC forwards the request for creation, modification, or deletion to the policy decision point in the ANC.

### Functional requirements

The following requirements apply to the QoS policy control:

- QoS policy control should support user-specific QoS parameters.

- QoS policy control should allow for intserv, diffserv as well as TSN QoS models.

- QoS policy control should support static and dynamic service flows.

- QoS policy control should allow subscription service to change active service flows.

- QoS policy control should allow for dynamic service flow creation on request from terminal or access router.

- QoS policy control should create static service flows based on the authorization information delivered in the admission accept message.

- QoS policy control should be able to handle any number of terminals, any number of access routers, and any number of subscription services.

- QoS policy control should support the resource reservation protocols specified by IEEE 802.

### QoS policy-specific attributes

#### Service flow parameters

- Datagram filter

- Priority

- Bandwidth

- Delay

- Jitter

#### Policy rules

- Traffic specification

- Priority

- Usage limits (time, volume, delay)

### QoS policy control-specific basic functions

#### Provisioning of authorization information to policy decision point at session begin

The subscription service provides at session begin the authorization information through the AAA protocol to the ANC, which contains the policy decision point.

#### Change of authorization during session

During an ongoing session, the subscription service can update the authorization by issuing a change of authorization, which is a function of the AAA protocol. The new authorization will be immediately applied by the policy decision point, and when the new authorization does not permit for ongoing service flows, such service flows are either terminated or modified to the new authorization.

#### Provisioning of QoS parameters to policy enforcement points

The policy decision point in the ANC derives from authorization and resource requests the QoS-related configuration parameters for both the NA and BH, and forwards the parameters over R5 and R7, respectively.

#### Request of service flow by terminal

When required, the terminal control can request the allocation of new service flows, modification of QoS parameters for active service flows, or termination of service flows not needed anymore by issuing a Resource Request to the policy decision point in the ANC.

#### Request of service flow by access router

When required, the access router control can request the allocation of new service flows, modification of QoS parameters for active service flows, or termination of service flows not needed anymore by issuing a resource request to the policy decision point in the ANC.

### Detailed procedures

#### Pre-provisioned service flow establishment

The user-specific QoS policy is passed to ANC from SS during the authorization process, together with the information about pre-provisioned service flows (SF), such as the number of pre-provisioned service flows and the relative QoS parameters. After that, ANC assigns SFID to each pre-provisioned service flow, and initiates the service flow establishment through sending SF registration requests to TE, NA, BH, and AR, as shown in pre-provisioned service flow establishment procedure.

- Upon receiving the authorization of the pre-provisioned service flow, ANC may reject or permit the establishment based on the admission control policy toward the pre-provisioned service flows and its available resources.

- If permitted, the service flow will be attached to a datapath (DP), and ANC will send SF registration requests to TE, NA, BH, and AR including the permitted QoS parameters. If these parameters are accepted by the entities to their QoS capabilities, the pre-provisioned service flow is permitted. All entities then reserves the resource and notifies ANC with SF registration response about the service flow establishment. The pre-provisioned service flow is finally activated by ANC through SF registration ack to TE, NA, BH, and AR.

- If rejected, the entity not being able to accept the request responses to ANC with the preferred QoS parameters for negotiation. In this case, ANC may adjust the QoS parameters for the pre-provisioned service flow and restart the whole procedure. If it is rejected for certain times, ANC will declare failure of the pre-provisioned service flow establishment to SS.

- Pre-provisioned service flow establishment procedure

#### Service flow initialization by terminal

Terminal may initiate service flows with the preferred QoS parameters after authorization during the user session by sending service flow addition request to ANC. Upon receiving the request, ANC should reply with a service flow addition received message to TE as an immediate acknowledgment with the integrity verification, then decide to admit or reject the request based on the admission control policy.

If the request is accepted by ANC applying the user-specific policy, it will assign SFID for the service flow and notify NA, BH, and AR with the SF registration including the QoS parameters of the service flow and datapath ID. NA, BH, and AR send SF registration response that sufficient resources are available and will be reserved for the admitted service flow. ANC sends service flow addition response to TE to notify the service flow establishment.

TE replies with service flow addition ack, and ANC  sends SF registration ack to confirm with NA, BH and AR, the activation of the new service flow.

- Service flow initialization by terminal

#### Service flow initialization by access router

Access router may initiate service flows with the preferred QoS parameters after authorization by sending service flow addition request to ANC. Upon receiving the request, ANC should reply with a service flow addition received message to AR as an immediate acknowledgment with the integrity verification, then decide to admit or reject the request based on the admission control policy.

If the request is accepted by ANC applying the user-specific QoS policy, it attaches the permitted service flow to a datapath, assigns a SFID for the service flow, and sends SF registration request to NA, BH, and AR, including the permitted QoS parameters of the service flow and datapath ID, so that NA, BH, and AR will reserve resources for the accepted service flow. NA, BH, and AR notify ANC with SF registration response, that the service flow can be established.  ANC will send service flow addition response to AR to notify the service flow establishment.

AR should reply service flow addition ack, and ANC will send SF registration ack to TE, NA and BH to confirm addition of the service flow.

- Fig. 34 Service flow initialization by access router

#### Service flow modification by terminal

Terminal can modify the ongoing service flows by sending service flow change request to ANC, including the modified QoS parameter and the specified service flow. Upon receiving the request, ANC should reply with a service flow change received message to TE as an immediate acknowledgment with the integrity verification, then decide to admit or reject the request based on the admission control policy.

If the request is accepted by ANC applying the user-specific QoS policy, ANC sends SF modification request to NA, BH, and AR, including the permitted modification on the QoS parameter. NA, BH and AR will verify that the modification can be accomplished and change the reserved resources for the service flow. NA, BH, and AR will send SF modification response to ANC as a confirmation. When ANC received confirmation from all involved entities, it sends SF change response to TE to notify the modification.

TE should reply SF change ack, and ANC will send SF modification ack to NA, BH, and AR to confirm the activation of the new parameters.

- Service flow modification by terminal

#### Service flow termination by terminal

Terminal can terminate the ongoing service flows by sending service flow delete request to ANC, including information of the service flows to be terminated. Upon receiving the request, ANC should reply with a service flow delete response to TE indicating that the resource reserved for the specified service flow has been found and the owner has been verified, and send SF de-registration request to NA, BH, and AR including the related information about service flow and datapath. The entities then deactivate the specified service flow, terminate the accounting, and reply with a datapath de-registration response to ANC indicating that the service flow has been set to inactive.

ANC will then finally release the reserved resources and force the deletion of the service flow through sending SF de-registration ack to NA, BH, and AR.

- Service flow termination by terminal

#### Change of authorization by subscription service

SS can change the QoS policies of a specific user by sending the authorization change request to ANC. Upon receiving the request, ANC should update the QoS policies specified to the users, and initiate deletion and modification on the related service flows which may conflict with the new QoS policies.

- Change of authorization by subscription service

While the generic QoS policy control architecture is applicable to all IEEE 802 access technologies, differences exist in the capabilities of the access technologies in supporting various QoS models and parameters. In the introduction to this chapter, main differences between IEEE 802 technologies were already mentioned. The following section provides more details about the capabilities of the IEEE 802 technologies to support the concepts presented above.

### Mapping to IEEE 802 technologies

While the generic QoS policy control architecture is applicable to all the IEEE 802 access technologies, differences exist in the capabilities of the access technologies in supporting various QoS models and parameters. In the introduction to this chapter, main differences between IEEE 802 technologies were already mentioned. This section provides more details about the capabilities of the IEEE 802 technologies to support the concepts presented above.

All IEEE 802 technologies support the divserv QoS model; some of the technologies also support an intserv QoS model with strict reservation of resources for particular service streams.

The following table provides references to the QoS-related sections of IEEE 802 technologies.

However, there is one deficiency of the E-Tree characteristic for establishing public access networks: IPv6 operation requires direct host-to-host connectivity for neighbor discovery messages to allow secure neighbor discovery.