PvarDesignation™/D01
Draft [Report] for [Wired/Wireless Use Cases and Communication Requirements for Flexible Factories IoT Bridged Network]

Sponsor

[ICCOM]

of the

IEEE [Computer Society/Standards Development Board (COM/SDB)]

Approved varApprovedDate

IEEE-SA Standards Board

Copyright © varCRYear by The Institute of Electrical and Electronics Engineers, Inc.

Three Park Avenue

New York, New York 10016-5997, USA

All rights reserved.

This document is an unapproved draft of a proposed IEEE Standard. As such, this document is subject to change. USE AT YOUR OWN RISK! IEEE copyright statements SHALL NOT BE REMOVED from draft or approved IEEE standards, or modified in any way. Because this is an unapproved draft, this document must not be utilized for any conformance/compliance purposes. Permission is hereby granted for officers from each IEEE Standards Working Group or Committee to reproduce the draft document developed by that Working Group for purposes of international standardization consideration.  IEEE Standards Department must be informed of the submission for consideration prior to any reproduction for international standardization consideration (stds.ipr@ieee.org). Prior to adoption of this document, in whole or in part, by another standards development organization, permission must first be obtained from the IEEE Standards Department (stds.ipr@ieee.org). When requesting permission, IEEE Standards Department will require a copy of the standard development organization's document highlighting the use of IEEE content. Other entities seeking permission to reproduce this document, in whole or in part, must also obtain permission from the IEEE Standards Department.

IEEE Standards Department

445 Hoes Lane

Piscataway, NJ 08854, USA

Abstract: <Select this text and type or paste Abstract—contents of the Scope may be used>

Keywords: <Select this text and type or paste keywords>

Important Notices and Disclaimers Concerning IEEE Standards Documents

IEEE documents are made available for use subject to important notices and legal disclaimers. These notices and disclaimers, or a reference to this page, appear in all standards and may be found under the heading “Important Notices and Disclaimers Concerning IEEE Standards Documents.” They can also be obtained on request from IEEE or viewed at http://standards.ieee.org/IPR/disclaimers.html.

Notice and Disclaimer of Liability Concerning the Use of IEEE Standards Documents

IEEE Standards documents (standards, recommended practices, and guides), both full-use and trial-use, are developed within IEEE Societies and the Standards Coordinating Committees of the IEEE Standards Association (“IEEE-SA”) Standards Board. IEEE (“the Institute”) develops its standards through a consensus development process, approved by the American National Standards Institute (“ANSI”), which brings together volunteers representing varied viewpoints and interests to achieve the final product. IEEE Standards are documents developed through scientific, academic, and industry-based technical working groups. Volunteers in IEEE working groups are not necessarily members of the Institute and participate without compensation from IEEE. While IEEE administers the process and establishes rules to promote fairness in the consensus development process, IEEE does not independently evaluate, test, or verify the accuracy of any of the information or the soundness of any judgments contained in its standards.

IEEE Standards do not guarantee or ensure safety, security, health, or environmental protection, or ensure against interference with or from other devices or networks. Implementers and users of IEEE Standards documents are responsible for determining and complying with all appropriate safety, security, environmental, health, and interference protection practices and all applicable laws and regulations.

IEEE does not warrant or represent the accuracy or content of the material contained in its standards, and expressly disclaims all warranties (express, implied and statutory) not included in this or any other document relating to the standard, including, but not limited to, the warranties of: merchantability; fitness for a particular purpose; non-infringement; and quality, accuracy, effectiveness, currency, or completeness of material. In addition, IEEE disclaims any and all conditions relating to: results; and workmanlike effort. IEEE standards documents are supplied “AS IS” and “WITH ALL FAULTS.”

Use of an IEEE standard is wholly voluntary. The existence of an IEEE standard does not imply that there are no other ways to produce, test, measure, purchase, market, or provide other goods and services related to the scope of the IEEE standard. Furthermore, the viewpoint expressed at the time a standard is approved and issued is subject to change brought about through developments in the state of the art and comments received from users of the standard.

In publishing and making its standards available, IEEE is not suggesting or rendering professional or other services for, or on behalf of, any person or entity nor is IEEE undertaking to perform any duty owed by any other person or entity to another. Any person utilizing any IEEE Standards document, should rely upon his or her own independent judgment in the exercise of reasonable care in any given circumstances or, as appropriate, seek the advice of a competent professional in determining the appropriateness of a given IEEE standard.

IN NO EVENT SHALL IEEE BE LIABLE FOR ANY DIRECT, INDIRECT, INCIDENTAL, SPECIAL, EXEMPLARY, OR CONSEQUENTIAL DAMAGES (INCLUDING, BUT NOT LIMITED TO: PROCUREMENT OF SUBSTITUTE GOODS OR SERVICES; LOSS OF USE, DATA, OR PROFITS; OR BUSINESS INTERRUPTION) HOWEVER CAUSED AND ON ANY THEORY OF LIABILITY, WHETHER IN CONTRACT, STRICT LIABILITY, OR TORT (INCLUDING NEGLIGENCE OR OTHERWISE) ARISING IN ANY WAY OUT OF THE PUBLICATION, USE OF, OR RELIANCE UPON ANY STANDARD, EVEN IF ADVISED OF THE POSSIBILITY OF SUCH DAMAGE AND REGARDLESS OF WHETHER SUCH DAMAGE WAS FORESEEABLE.

Translations

The IEEE consensus development process involves the review of documents in English only. In the event that an IEEE standard is translated, only the English version published by IEEE should be considered the approved IEEE standard.

Official statements

A statement, written or oral, that is not processed in accordance with the IEEE-SA Standards Board Operations Manual shall not be considered or inferred to be the official position of IEEE or any of its committees and shall not be considered to be, or be relied upon as, a formal position of IEEE. At lectures, symposia, seminars, or educational courses, an individual presenting information on IEEE standards shall make it clear that his or her views should be considered the personal views of that individual rather than the formal position of IEEE.

Comments on standards

Comments for revision of IEEE Standards documents are welcome from any interested party, regardless of membership affiliation with IEEE. However, IEEE does not provide consulting information or advice pertaining to IEEE Standards documents. Suggestions for changes in documents should be in the form of a proposed change of text, together with appropriate supporting comments. Since IEEE standards represent a consensus of concerned interests, it is important that any responses to comments and questions also receive the concurrence of a balance of interests. For this reason, IEEE and the members of its societies and Standards Coordinating Committees are not able to provide an instant response to comments or questions except in those cases where the matter has previously been addressed. For the same reason, IEEE does not respond to interpretation requests. Any person who would like to participate in revisions to an IEEE standard is welcome to join the relevant IEEE working group.

Comments on standards should be submitted to the following address:

Secretary, IEEE-SA Standards Board

445 Hoes Lane

Piscataway, NJ 08854 USA

Laws and regulations

Users of IEEE Standards documents should consult all applicable laws and regulations. Compliance with the provisions of any IEEE Standards document does not imply compliance to any applicable regulatory requirements.  Implementers of the standard are responsible for observing or referring to the applicable regulatory requirements. IEEE does not, by the publication of its standards, intend to urge action that is not in compliance with applicable laws, and these documents may not be construed as doing so.

Copyrights

IEEE draft and approved standards are copyrighted by IEEE under U.S. and international copyright laws. They are made available by IEEE and are adopted for a wide variety of both public and private uses.  These include both use, by reference, in laws and regulations, and use in private self-regulation, standardization, and the promotion of engineering practices and methods. By making these documents available for use and adoption by public authorities and private users, IEEE does not waive any rights in copyright to the documents.

Photocopies

Subject to payment of the appropriate fee, IEEE will grant users a limited, non-exclusive license to photocopy portions of any individual standard for company or organizational internal use or individual, non-commercial use only. To arrange for payment of licensing fees, please contact Copyright Clearance Center, Customer Service, 222 Rosewood Drive, Danvers, MA 01923 USA; +1 978 750 8400. Permission to photocopy portions of any individual standard for educational classroom use can also be obtained through the Copyright Clearance Center.

Updating of IEEE Standards documents

Users of IEEE Standards documents should be aware that these documents may be superseded at any time by the issuance of new editions or may be amended from time to time through the issuance of amendments, corrigenda, or errata. An official IEEE document at any point in time consists of the current edition of the document together with any amendments, corrigenda, or errata then in effect.

Every IEEE standard is subjected to review at least every ten years. When a document is more than ten years old and has not undergone a revision process, it is reasonable to conclude that its contents, although still of some value, do not wholly reflect the present state of the art. Users are cautioned to check to determine that they have the latest edition of any IEEE standard.

In order to determine whether a given document is the current edition and whether it has been amended through the issuance of amendments, corrigenda, or errata, visit the IEEE Xplore at http://ieeexplore.ieee.org/ or contact IEEE at the address listed previously. For more information about the IEEE-SA or IEEE’s standards development process, visit the IEEE-SA Website at http://standards.ieee.org.

Errata

Errata, if any, for all IEEE standards can be accessed on the IEEE-SA Website at the following URL: http://standards.ieee.org/findstds/errata/index.html. Users are encouraged to check this URL for errata periodically.

Patents

Attention is called to the possibility that implementation of this standard may require use of subject matter covered by patent rights. By publication of this standard, no position is taken by the IEEE with respect to the existence or validity of any patent rights in connection therewith. If a patent holder or patent applicant has filed a statement of assurance via an Accepted Letter of Assurance, then the statement is listed on the IEEE-SA Website at http://standards.ieee.org/about/sasb/patcom/patents.html. Letters of Assurance may indicate whether the Submitter is willing or unwilling to grant licenses under patent rights without compensation or under reasonable rates, with reasonable terms and conditions that are demonstrably free of any unfair discrimination to applicants desiring to obtain such licenses.

Essential Patent Claims may exist for which a Letter of Assurance has not been received. The IEEE is not responsible for identifying Essential Patent Claims for which a license may be required, for conducting inquiries into the legal validity or scope of Patents Claims, or determining whether any licensing terms or conditions provided in connection with submission of a Letter of Assurance, if any, or in any licensing agreements are reasonable or non-discriminatory. Users of this standard are expressly advised that determination of the validity of any patent rights, and the risk of infringement of such rights, is entirely their own responsibility. Further information may be obtained from the IEEE Standards Association.

Participants

At the time this drafttxtTrialUsetxtGorRPorSTD [report] was completed, the varWorkingGroup IEEE 802 Network Enhancement for the Next Decade Industry Connections Activity Working Group had the following membership:

Glenn Parsons, Chair

varWkGrpViceChair, Vice Chair

Participant1

Participant2

Participant3

Participant4

Participant5

Participant6

Participant7

Participant8

Participant9

The following members of the <individual/entity> balloting committee voted on thistxtTrialUsetxtGorRPorSTD. Balloters may have voted for approval, disapproval, or abstention.

[To be supplied by IEEE]

Balloter1

Balloter2

Balloter3

Balloter4

Balloter5

Balloter6

Balloter7

Balloter8

Balloter9

When the IEEE-SA Standards Board approved thistxtTrialUsetxtGorRPorSTD on varApprovedDate, it had the following membership:

[To be supplied by IEEE]

<Name>, Chair

<Name>, Vice Chair

<Name>, Past Chair

Konstantinos Karachalios, Secretary

SBMember1

SBMember2

SBMember3

SBMember4

SBMember5

SBMember6

SBMember7

SBMember8

SBMember9

*Member Emeritus

Introduction

This introduction is not part of PvarDesignation/DvarDraftNumber, DrafttxtTrialUsetxtGorRPorSTD for varTitlePAR.

Communication used in factories has until now been mainly wired communication, which has been preferred for its reliability. However, in recent years the shorter times of product development cycles demands greater flexibility in the layout of machines and pipeline construction, and there are increasing expectations for the use of radio links amongst of the sensors and machines used in the manufacturing and factory processes.

This report is developed under the IEEE 802 Network Enhancements for Next Decade Industry Connections Activity (NEND-ICA). It addresses the converged and bridged wired and wireless IoT communications in the factory environment. The report includes use cases and requirements within the factory wireless environment. It presents problems and challenges observed within the factory and reports on feasibility for overcoming these issues leading towards a proposal to initiate new standards development makes the case for a new project to be conducted within the IEEE 802.1 WG. Material supporting the development of a new PAR is included in this report.

The report analyses the factories environments considering its foreseen evolution that includes dense radio devices utilization. It summarizes wireless use cases and their communication requirements which are obtained based on information gathered from literature and factories survey.

The report then presents an underlying End to End network architecture which encompasses the operation and control of the various services in the factory network according to their dynamic QoS requirements. It analyses the applicable standards and features in wired and wireless IEEE 802 technologies for managing requirements in End-to-End (E2E) network connectivity and TSN profiling for converged wired and wireless connectivity in factory environment.

Contents

1. Overview	9

1.1 Scope	9

1.2 Purpose	9

2. Definitions	10

3. Factory Overview and Operation environment	10

4. less use cases and communication requirements	14

4.1 Introduction	14

4.2 Wireless use cases and communication requirements	14

4.3 Categories of wireless use cases and communication requirements	15

4.4 Details of wireless use cases and communication requirements	17

5. Factory Usage scenarios	25

5.1 Usage scenarios example: Metal processing site	25

5.2 Usage scenarios example: Mechanical assembly site	26

5.3 Usage scenarios example: Elevated and high temperature work site	27

5.4 Usage scenarios example: Logistics warehouse site	28

6. Factory End to End Network architecture	30

6.1 Concept of architecture	30

6.2 Expectation for Enhancement of existing  IEEE 802 functions	30

7. New Project description and proposal	30

Annex A (informative) Bibliography	31

Draft Report for [Wired/Wireless use cases and communication requirements for Flexible Factories IoT Bridged Network]

- Overview

Communication used in factories has until now been mainly wired communication, which has been preferred for its reliability. However, in recent years the shorter times of product development cycles demands greater flexibility in the layout of machines and line construction, and there are increasing expectations for the use of wireless communications.

The report addresses the converged and bridged wired and wireless Internet of Things (IoT) communications in the factory environment. It analyses factory environment considering its foreseen evolution that includes dense radio devices utilization. It gives summary of use cases and requirements within the factory wireless environment which are obtained based on information gathered from literature and factories survey. It presents problems and challenges observed within the factory and reports on feasibility for overcoming these issues leading towards a proposal to initiate new standards development. The new project is targeted to be conducted within the IEEE 802.1 WG based on material in this report.

  - Scope

The scope of this report is to capture current and future network requirements taking into consideration dense use of radio devices and its operation in factory environment. The report presents analysis of issues and challenges identified in maintaining reliable and time sensitive/constraint deliverable of control messages and data traffic across wired and wireless bridged network within the identified factory environment. Also to present analysis of applicable standards and features in wired and wireless IEEE802 technologies for managing requirements in E2E network connectivity.

  - Purpose

The purpose of this report is to present an overview of issues and challenges in managing a reliable and time sensitive connectivity in E2E wired and wireless network characterized by dense radio devices installation and noisy factory environment. The report will also present technical analyses of the applicable features and functions in wired and wireless IEEE802 technologies for managing requirements in E2E network connectivity which can be used in an IEEE 802 standard solution based on TSN profiling for converged wired and wireless connectivity within the factory environment.

- Definitions

For the purposes of this document, the following terms and definitions apply. The IEEE Standards Dictionary Online should be consulted for terms not defined in this clause.

- Factory Overview and Operation environment

Recent trends to introduce IoT devices, such as sensors and cameras in the factories are accelerated by strong demand for improving productivity under the constraints of decreasing works in aging population society and pressure for cost reduction. Digitalization of the factories as well as connection of information on production process and supply chain management within a factory and across factories becomes important. It is no doubt that commutation networks will be changing in factories for the next decade.

There are several system applications, e.g., preventive maintenance, management of materials and products, monitoring of movements and machine monitors which are integrated in the network. Future industrial network for a factory may consist of wired and wireless bridges for such systems above. The successful convergence of wired and wireless systems is indispensable and more efforts will be required for wireless communication because of its narrow bandwidth and sensitive nature of environment.

Traditionally, wireless communications have not been so popular in the manufacturing field. There are still many stand-alone machines managed manually by skilled workers. Advanced factories, on the other hand, have been using communication networks called fieldbus which is a wireline network. One of reasons not to use wireless commutations is that they have been believed to operate possibly instable in some cases. Technology developments as well as standardization are keys to success for wireless utilization. Once success, it facilities more use of IoT devices resulting in productivity improvement in the factories.

It is true that wireless commutations are not always difficult everywhere in factories.  However, we have to consider that some applications require high-reliable, low-latency and low-jitter data transmission compared with other application in other places like offices and homes in general. Furthermore, the measurement results have revealed that some factories are facing difficulties coming from (a) severe environment for wireless communications, and/or (b) existence of uncoordinated and independent systems in the same space.

(a) The Severe Environment for Wireless Communications

There are two source of impairment to radio signal within the factory environment that cause unpredictable variations to channel capacity, namely:

- Fluctuation of signal strength

- Noises at 1.9GHz and 2.4GHz.

As follows are examples of such impairments observed within the factory environment.

Example of Fluctuation of signal Strength:

Typical layout of the environment for which measurements are made is shown in the figure below

Figure 3-1 Typical Layout in factory for which measurement of RSSI is recorded.

The observed RSSI measurement in LOS condition is shown in figure 3-2 below.

Figure 3-2 RSSI Fluctuation in Factory (LOS)

This fluctuation in RSSI is due to motions of materials, parts, products and carriers in closed space.

Example of Noises at 1.9GHz and 2.4GHz:

While carrying radio measurement within the factory environment strong noise signals are observed within the 1.9 GHz band and the 2.4 GHz band. These are shown in Figure 3-3.

- 1.9 GHz band		(b) 2.4GHz band

Figure 3-3 Measured noise spectral density within (a) 1.9 GHz band and (b) 2.4 GHz band

In the 1.9 GHz band, the noise appears to cause problems for the communication with particular machines as well as problem for using the 1.9GHz band for internal telephone system.

The source of these noises is attributed to some kinds of manufacturing machines that are causing interference for wireless communications.

(b) Uncoordinated and Independent Systems

This issue within the factory environment is attributed to the progressive nature which leads to stepped approach of addition and installation of machines and equipment in the factory and due to coexistence of heterogeneous and legacy devices/systems used within the factory.

An example of using wireless technology in the factory is shown in Figure 3-4.

Figure 3-4 video monitor application as an example of using wireless technology

In this example it illustrates an application in which the data flow across the wired network and bridged across to the wireless domain. In this application there are QoS requirements and latency constraints for both the video signal and the control signal. Potential problem is a bottleneck for which delay or uncoordinated signal flow may occur due to disturbance and/or degradation in the radio signal.

When considering the coexistence of uncoordinated wireless systems, we observe the problem of interference between the legacy wireless communications used by some machinery in the factory with the new systems using Wi-Fi. The overlapping of signal causing potential interference is illustrated in Figure 3-5.

Figure 3-5 Coexistence of different wireless technologies

Some of the problems observe relates to the packet delivery delay. In Figure 3-6 shows packet latency increases from 8ms in case of no interference to around 2 second in the presence interference due to lack of coordination amongst the used wireless systems used in the factory.

Figure 3-6 Impact of interference on latency in uncoordinated wireless systems

In this document, wireless use cases and communication requirements are described to understand what shall be improved and enhanced for successful convergence of wired and wireless systems.

- less use cases and communication requirements

  - Introduction

The wireless use cases considered in this clause illustrate the use of wireless systems that are currently used –or will be used in the near future - in factories and in factory related facilities, such as factory warehouses. The use cases correspond to wireless systems that are installed for specific purposes.

For example, in the case of wireless systems inside a factory as shown in Fig. 4-1, there are individual systems (within the dotted lines) introduced for specific purposes such as “Collection of Management Information”, and a wireless network consisting of a collection of multiple such wireless systems and transmitting information aggregated by them. In this case, each individual system corresponds to a wireless use case as described in this following sub sections, but not the whole wireless network. That is, each wireless segment is considered as a separate use case.

Section 5 considers actual factory sites with large needs for wireless communication and describes usage scenes where multiple wireless use cases coexist.

Figure 4-1 Scope of wireless use cases

  - Wireless use cases and communication requirements

In our survey of uses of wireless communication in factories, we collected various use cases, classified them according to their purpose, and organized their communication requirements. The classification categories are shown in Table 1. Use cases were classified into six categories, consisting of five specific categories concerning the quality of manufactured products, managing the factory, and so forth, and one category for all other cases.

Table 4-1: Categories of wireless use cases

| Category | Description of category |
| --- | --- |
| Quality | Providing means for collecting information related to products and states of machines during production |
| Management | Providing means for collecting information about whether production is proceeding under proper environmental conditions, and whether personnel and things contributing to productivity enhancement are being managed appropriately |
| Display | Providing means for workers to receive necessary support information, and for managers to monitor the production process and production status |
| Control | Providing means for sending commands to mobile vehicles and production equipment |
| Safety | Providing means for collecting information about dangers to workers |
| Other | Communication infrastructure with non-specific purposes, and others |

The communication requirements of each wireless use case are shown as the following parameters - mobility, number of terminals in the factory, density of terminals, average number of terminals influencing at a location, transmission data size, regularity of communications, communication frequency, delivery time tolerance, tolerance jitter, communication range, wireless terminal (master station) height, wireless terminal (sub-station) height, communication standard, frequency band and network topology.

Footnote #2: Physical objects such as materials and equipment related to production are called “things”

  - Categories of wireless use cases and communication requirements

As the result of organizing the wireless use cases and communication requirements, it can be seen that there is a variety of data sizes, data generation frequency, number of wireless terminals and so forth, and in each case, there is an optimal wireless communication according to the features of the communication environment and wireless use case. Figure 4-2 shows representative wireless use cases and their features of wireless communication. Values of data size, data generation frequency, number of wireless terminals, and so forth are different for different systems in factories, and according to the required functions of the systems, they use different wireless frequency bands and wireless standards. High frequency bands such as the 60 GHz band are expected to be effective for systems with relatively large data volume requirements (image inspection equipment, etc.), 5 GHz band and 2.4 GHz band are being used for systems with medium requirements of data sizes and data generation frequency, such as distributing control programs and control of mobile equipment, and relatively low wireless frequency bands such as the 920 MHz band are being used for applications with low power requirements (such as environmental sensing).

Figure 4-2 Representative wireless use cases and their features of wireless communication

Figure 4-3 shows the permissible delay for representative wireless use cases in each category. There are wireless use cases, such as robot control and urgent announcements, for which the urgency and accuracy of timing of arrival of information requires latency of less than one millisecond. On the other hand, particularly in the categories of quality (inline inspection, etc.) and management (preventive maintenance, etc.), there are many wireless use cases that tolerate latencies larger than ten milliseconds.

Figure 4-3 Permissible latencies of representative wireless use cases

  - Details of wireless use cases and communication requirements

Within each category, classification is made according to purpose, as shown in Table 4-2.

Table 4-2 Purposes within each category

| Category | Classification of purpose within category |
| --- | --- |
| Quality | Checking that products are being produced with correct precision Checking that production is proceeding with correct procedure and status |
| Management | Checking that the production environment is being appropriately managed Monitoring movement of people and things Checking the management status of equipment and materials (stock) Checking that the production equipment is being maintained Appropriate recording of work and production status |
| Display | Providing appropriate work support Visually display whether the process is proceeding without congestion or delay Visually display the production status |
| Control | Control, operation and command of production equipment and auxiliary equipment Ensuring the safety of workers |
| Other | Cases other than the above |

Each table shows the purpose of the wireless use case, the corresponding information, and the communication requirements of transmitted data size, communication rate, and delivery time tolerance.

An example of “List of wireless use case and communication requirements” and will be added.

| No. | Category | Use Case |  | Data Size (bytes) | Communication Rate | Delivery Time Tolerance |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Purpose | Target Information | Data Size (ave.) |  |  |
| 1 | Quality | size inspection by line camera (line sensor) | size measurements | 30K | once per sec. | 5 sec. |
| 2 | Quality | detect defect state | defect information (video) | 500 | one per 100 millisec. | 500 millisec. |
| 3 | Quality | detect incorrect operation | anomalous behavior due to adding impurities | 1M | once per sec. | 10 sec. |

- Factory Usage scenarios

  - Usage scenarios example: Metal processing site

  - Usage scenarios example: Mechanical assembly site

  - Usage scenarios example: Elevated and high temperature work site

  - Usage scenarios example: Logistics warehouse site

- Factory End to End Network architecture

  - Concept of architecture

  - Expectation for Enhancement of existing  IEEE 802 functions

- New Project description and proposal

# (informative)
Bibliography

Bibliographical references are resources that provide additional or helpful material but do not need to be understood or used to implement this standard. Reference to these resources is made for informational use only.

The Institute of Electrical and Electronics Engineers, Inc.

3 Park Avenue, New York, NY 10016-5997, USA

Copyright © varCRYear by The Institute of Electrical and Electronics Engineers, Inc.

All rights reserved. Published varPublishedDate. Printed in the United States of America.

IEEE is a registered trademark in the U.S. Patent & Trademark Office, owned by The Institute of Electrical and Electronics 
Engineers, Incorporated.

PDF:	ISBN varISBNpdf	varStdIDpdf

Print:	ISBN varISBNprint	varStdIDprint

IEEE prohibits discrimination, harassment, and bullying.

For more information, visit http://www.ieee.org/web/aboutus/whatis/policies/p9-26.html.

No part of this publication may be reproduced in any form, in an electronic retrieval system or otherwise, without the prior written permission of the publisher.

IEEE Standards Dictionary Online is available at: http://dictionary.ieee.org

.